3GPP TSG-RAN WG4 Meeting #106		R4-2303238

Athens, Greece, February 27 – March 3, 2023

Agenda item:			9.25.6

Source:	Qualcomm

Title:	WF on NR NTN enhancement RRM requirements

Document for:	Approval

# Introduction

The WF covers the contributions submitted under the following AIs

- 9.25.5	RRM core requirements	[NR_NTN_enh-Core]

* R1-2213001 Reply LS on RACH-less handover in NTN

# Topic #1: Response to incoming LS on RACH-less handover in NTN

### Issue 1-1: UL synchronization requirement specified in Table 7.1C.2-1 of TS38.133 applies to the first UL transmission in the target cell?

Agreement:

  - RAN4 to send a reply-LS to RAN1 with the following contents:

    - For NTN-NTN FR1-FR1 handover, RAN4 to confirm that Note 1 in the LS (R1-2213001) is correct, i.e. the timing requirement specified in Table 7.1C.2-1 of TS38.133 applies to the first UL transmission, including PUCCH, PUSCH, SRS, PRACH, and msgA, in the target cell, provided that

      - at least one SSB is available at the UE during the last 160 ms.

      - UE is provided with information such that the UE has valid NTA,adjcommon and NTA,adjUE upon handover execution.

      - the network and UE have a common understanding of NTA component upon HO execution.

    - RAN4 assumes the determination of the value for NTA for the different scenarios is up to RAN1

    - The timing requirement specified in Table 7.1C.2-1 of TS38.133 are applicable to FR1 only and there are no requirements for other frequency ranges

    - There are no existing RAN4 NR RRM requirements for RACH-less HO and additional RAN4 work will be required to introduce requirements and identify side conditions

# Topic #2: Coverage enhancement

- No agreement

# Topic #3: NR-NTN deployment in above 10 GHz bands

### Issue 3-1: RRM impacts due to FDD-based NTN support in above 10 GHz

- No agreement

# Topic #4: Network verified UE location

- No agreement

# Topic #5: NTN-TN and NTN-NTN mobility and service continuity enhancements

### Issue 5-1: NTN-NTN Cell reselection enhancements for earth moving cell

- No agreement

### Issue 5-2: NTN-TN Cell reselection enhancements (to reduce UE power consumption)

- No agreement

### Issue 5-3: NTN-NTN handover (to reduce the signalling overhead)

- No agreement

### Issue 5-4: NTN-TN Handover

Conclusion:

- It is a common understanding of the group that NTN-TN handover is not in the immediate scope of Rel-18 NR NTN enhancement.

Agreement:

- RAN4 to not define NTN-TN handover, i.e. mobility requirements between NTN and TN are applicable only when UE is in RRC Idle/Inactive mode.

# Topic #6: Applicable scope of this WI

### Issue 6-1: Applicable scope of this WI

Agreement:

- Unless any compelling reasons are identified and agreed, RRM requirements will be defined for non-CA.

- The following aspects will be discussed under relevant items:

  - Whether/How to define RRM requirements for mobility between NTN and FR1/FR2 TN will be discussed under sub-AI of “NTN-TN and NTN-NTN mobility and service continuity enhancements” if needed. The discussion also includes whether TN is limited to NR or other RATs.

  - Whether/How to define RRM requirements for mobility within NTN-Ka bands and between FR1/FR2 and NTN-Ka bands will be discussed under sub-AI of “NR-NTN deployment in above 10 GHz bands” if needed.

# References

[1] R4-2302783, “Topic summary for [106][225] NR_NTN_enh,” 3GPP TSG-RAN WG4 Meeting #106