3GPP TSG-RAN WG4 Meeting #106	R4-2303275

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2844 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on clarification for UE acquiring system information during handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The corrections for NTN HO and CHO requirements are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for NTN SAN handover and conditional handover are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C, 7.1C.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2300598. |  |  |  |  |  |  |  |  |

--------------------------------------------------------First change request------------------------------------------------------------

6.1C	Handover for SAN

6.1C.1	NR SAN Handover

6.1C.1.1	Introduction

The purpose of NR SAN handover is to change the NR SAN PCell to another NR SAN cell. The requirements in this clause are applicable to SA NR SAN.

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR SAN FR1 cell to NR SAN FR1 cell.

6.1C.1.2.1	Handover delay

When the UE receives a RRC message implying handover to NR SAN cell, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command.

Where:

-	Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1C.1.2.2.

6.1C.1.2.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover to NR SAN cell is commanded,

If the valid parameters of ephemeris information, epoch time of the ephemeris, common TA, validity timer information, DL and UL Polarization information, Koffset, and Kmac for target NR SAN cell are send to UE,

the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Otherwise, no interruption time requirement is applied.

Where:

-	Tsearch is the time required to search the target NR SAN cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

-	Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

-	Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

-	TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and [10] ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover.

6.1C.2	NR SAN Conditional Handover

6.1C.2.1	Introduction

The requirements in this clause are applicable to conditional handover to change the NR SAN PCell to another NR SAN cell.

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR SAN FR1 cell to NR SAN FR1 cell.

6.1C.2.2.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. UE should start RRM measurement before the time or distance condition is met, the time/distance condition is defined in clause 5.5.4 in TS 38.331[2]

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

-	TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

-	TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover

-	Tmeasure is the measurements time stated in clause 6.1C.4.2.2.

-	TCHO_execution is the UE conditional execution preparation time for conditional handover in clause 6.C1.2.2.3.

-	Tinterrupt is the interruption time stated in clause 6.1C.2.2.4.

The conditional handover delay requirements are applied if condition T1-2 is later than the end of Tmeasure for time based CHO, or both condition D1-1 and condition D1-2 are fulfilled before the end of Tmeasure for location-based CHO, otherwise no CHO requirement is applied.

6.1C.2.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For intra-frequency handover, the requirements for identifying a new detectable intra frequency cell measured without Time To Trigger (TTT) and L3 filtering, Tidentify_intra_with_index or Tidentify_intra_without_index, defined in clause 9.2C.5.1 and clause 9.2C.6.2 are used.

For time-based conditional intra-frequency handover:

-	If condition T1-1 occurs earlier than TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index assuming UE only performs the measurements within SMTC window of the target cell.

-	If condition T1-1 occurs later than TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For location-based conditional intra-frequency handover:

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index assuming UE only performs measurements within SMTC window of the target cell.

-	If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU plus Tidentify_intra_with_index or Tidentify_intra_without_index for intra-frequency handover, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

For inter-frequency handover, the requirements for identifying a new detectable inter frequency cell measured without Time To Trigger (TTT) and L3 filtering, Tidentify_inter_with_index or Tidentify_inter_without_index, defined in clause 9.3C.7.1 are used.

For time-based conditional inter-frequency handover:

-	If condition T1-1 occurs earlier than TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index assuming that the UE uses only the SMTC window of the target inter-frequency carrier for performing the measurements. In this case Ksatellite=1, CSSFinter=1.

-	If condition T1-1 occurs later than TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until condition T1-1.

For location-based conditional inter-frequency handover,

-	If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index, assuming that the UE uses only the SMTC window of the target inter-frequency carrier for performing the measurements. In this case Ksatellite=1, CSSFinter=1.

-	If both condition D1-1 and condition D1-2 are fulfilled later than TEvent_DU plus Tidentify_inter_with_index or Tidentify_inter_without_index, then the measurement time delay equal to the time from the end of Tevent_DU until time of both condition D1-1 and condition D1-2 are fulfilled.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period [Tidentify_intra_without_index] or [Tidentify_intra_with_index] for intra-frequency handover or [Tidentify_inter_without_index] for inter-frequency handover.

6.1C.2.2.3	Preparation time

TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up to 10ms.

6.1C.2.2.4	Interruption time

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

-	Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

-	TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and [10] ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

-	Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

--------------------------------------------------------Second change request------------------------------------------------------------

7.1C.2.1	Gradual timing adjustment

When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE is required to adjust its timing to within Te_NTN. The reference timing shall be NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN.

2)	The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last one second, shall be Tp_NTN per second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200ms, shall be Tq_NTN per 200 ms.

Where, the maximum autonomous time adjustment step Tq_NTN and the aggregate adjustment rate Tp_NTN are specified in Table 7.1C.2.1-1.

Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq_NTN | Tp_NTN |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | N/A | N/A |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |