3GPP TSG-RAN WG4 Meeting #106	R4-2303489

Athens, GR, February 27th ‒ March 3rd, 2023

Title: 	WF on RF requirements for 5G Broadcast

Agenda Item:	10.3.5

Source: 	Qualcomm Incorporated

Document for:	Approval

# Band plan

From [1] two band plan options not mutually exclusive were identified

- Define a single band covering the entire UHF frequency range 470 – [698/702] MHz

- The requirements are relaxed for this band (TBD which requirements and how much) and/or

- This band is assumed to be implemented with more than one filter in the UE (TBD how many filters and the characteristics of those filters)

- Define smaller bands

- At least one of the bands overlaps with Band n105 DL to reuse n105 Rx filter

- Specify the following bands 470-542 MHz, 540-606 MHz and 602-702 MHz.

- Other

Agreement from online session:

- Define the band plan based on available data for BS and UE.

  - Full UHF band, i.e., 470 – 698MHz,

    - with reduced UE blocking

  - Narrow band, i.e., 612 – 652 MHz, which is the same as the downlink operating band of n105.

  - If data for other frequency ranges becomes available before the conclusion of the work item, it can be considered either as a separate band or as a modification to initially defined bands.

Way Forward

- Further clarification and study is needed to elaborate on and quantify requirements with respect to reduced UE blocking for the full UHF band.

- All technical requirements for these bands are still to be defined.

# Reference sensitivity

For discussion

- The values of -99.2, -98.5, and -97.9 dBm are agreed for 6, 7, and 8 MHz PMCH channel assuming 10 MHz UE receive channel filter.

- FFS on whether all bandwidths are needed to be tested.

Way forward: At least the following points are to be considered in deriving the reference sensitivity

- Which bandwidth shall be used for noise integration?  Possibilities include 6, 7, 8, and 10 MHz.

- What signal shall be defined for refsens?  Possibilities include a newly defined 6, 7, and 8 MHz PMCH or an existing 10 MHz PDSCH.

# ACS

Way Forward:

- Blocker placement for ACS is 6, 7, or 8 MHz

- ACS is studied as [-16 to -33] dB for 6 MHz channel with the assumption of 10 MHz UE channel filter and coordinated network deployment.

- Also to be studied is 8 MHz channel with the assumption of 10 MHz UE channel filter, coordinated network deployment, and 1.25 kHz SCS.

# Reference

- R4-2220501, “WF on UE RF for 5G Broadcast,” Qualcomm Incorporated