3GPP TSG-RAN WG4 Meeting #106 										R4-2303693

Athens, GR, 27 Feb - 03 Mar, 2023

Source:	China Telecom

Title:	WF on Multi-carrier enhancements for NR

Agenda Item:	9.22.4

Document for:	Approval

- Topic 1: Tx switching across 3/4 bands with single TAG

- Issue 1-1-3: Fallback of Rel-18 Tx switching to Rel-16/17 Tx switching

Candidate options:

- Option 1:

UE will report the 3/4 band combination with Tx switching capability. It is expected that this gives the network sufficient information on UE capability for Tx switching across all fallback combinations.

- Option 2:

For a band pair supported Rel-18 1T-2T switching, Rel-16 1T-2T switching is supported as well.

For a band pair supported Rel-18 2T-2T switching, Rel-17 2T-2T switching is supported as well.

- Other options are not precluded

- Sub-topic 1-3: Applicability of DL interruption

- Issue 1-3-1: Issues of different band pairs having different capabilities of DL interruption

Way forward:

- If it is possible for two different band pairs conducting Tx switching across the same time and with overlapping switching period, and if the UE capability of DL interruption on any of the band(s) is different, the worst case for each band should be considered, i.e., DL interruption on one band is allowed if it is allowed when Tx switching is conducted on one of the two band pairs.

- Issue 1-3-2: On previous agreements for combinations of SUL+TDD and TDD+TDD CA band combinations

- Way forward for “synchronized CA between SUL configuration (with a SUL band and a TDD band) and TDD band(s) with the same UL-DL pattern across all TDD bands”

  - Whether to mandate no DL interruption can be discussed in a case by case (i.e., per band combination basis) manner.

- Way forward for “synchronized CA among 3 or 4 TDD band(s) [‘with the same UL-DL pattern’ or ‘without mandatory simultaneous Rx/Tx requirement’] across all TDD bands, DL interruption is not required”

  - No consensus on whether to use ‘with the same UL-DL pattern’ or ‘without mandatory simultaneous Rx/Tx requirement’, and it is recommended to discuss whether to mandate no DL interruption in a case by case (i.e., per band combination basis) manner.

- Sub-topic 1-4: CR related issues

- Issue 1-4-1: 2-layer UL-MIMO support for carrier(s) capable of 2Tx

Way forward:

- Further discuss on whether or not to mandate 2-layer UL-MIMO support for carrier(s) capable of 2Tx.

- Issue 1-4-2: Location of switching period in relation to time T0

Background:

RAN1 agreement for Rel-16 maintenance on Wednesday of this week (the green highlighted part has been agreed by RAN1)

Way forward:

- Further discuss on whether or not to define the time relationship between the location of the switching period and T_0 in RAN4 specification, taking into account the RAN1 agreement.

  - Note: Similar change to Rel-16/17/18 if any change is needed.

- Issue 1-4-3: Time mask for dualUL related switching scenarios

Background:

RAN4 #104-e-bis:

- For the location of switching period, in RAN4 #104-e-bis, RAN4 agreed to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1.

RAN1 #112 (Athens meeting):

The following Updated Proposal 3-2 from RAN1 FL is being discussed in RAN1:

Way forward:

- Further discuss whether and how to cover the following scenario based on the RAN1/2 further agreement.

- Further discuss whether and how to cover the following scenario based on the RAN1/2 further agreement.

Time mask for one transmitter switching between band X and band Z, and between band Y and band Z, where UE is capable of uplink transmission on band Y during time period of {switching period #1 - switching period #2}, i.e., the UE indicates [TBD-1] in the capability [TBD tx-on-non-affected-band]

Time mask for one transmitter switching between band X and band Z, and between band Y and band Z, where UE is not capable of uplink transmission on band Y during time period of {switching period #1 - switching period #2}, i.e., UE does not indicate [TBD-1] in the capability [TBD tx-on-non-affected-band]

- Topic 2: Tx switching with dual TAGs

- Sub-topic 2-1: RF specification impact

Way forward:

- RAN4 can further discuss the following two options in RAN4 #106e-bis meeting:

  - Option 1: Introduce new time mask requirements

    - Option 1a: To ensure that UE functionality can be properly tested, modify the existing time masks such that Tx switching with uplink slot timing difference between carriers is verified when symbols containing the switching period preceding the time T0 are not scheduled by the test system on either carrier to avoid UE malfunction in the field.

  - Option 2: Do not modify the time mask for Tx switching for multiple TAGs. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction.

- Sub-topic 2-2: Clarificaiton on UE behaviour in 2-TAG case

Way forward:

- For UEs declaring the Tx switching support with 2-TAG, the baseline and advanced UE behaviors applies to both 1-TAG and 2-TAG cases.