3GPP TSG-RAN WG4 Meeting # 106												R4-2303746

Athens, Greece, 27 February –03 March, 2023

Agenda item:			5.4

Source:	Moderator (Ericsson)

Title:	Topic summary for [106][301] BSRF_maintenance

Document for:	Information

# Introduction

The thread covers BS RF maintenance agenda items. Topics are divided according to the agenda:

- BS RF requirements and conformance test for LTE and NR (up to Rel-16) 	(4.2)

- BS RF requirements and conformance test for LTE and NR (Rel-17)	(5.2.8.1)

# Topic #1: BS RF requirements and conformance test for LTE and NR (up to Rel-16)

## Companies’ contributions summary

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2300576 | CATT | CR for TS 38.141-2, Correction on transmitter spatial emissions declaration in Annex G Summary of change: 1) Change Dxx.1 Dxx.2 Dxx.3 and Dxx.4 in Table G.2-1 to DE.1 DE.2 DE.3 and DE.4 respectively. 2) Change D9.x in Table G.2-1 to D.8, D.10. |
| R4-2300672 | Nokia, Nokia Shanghai Bell | CR to TS 37.145-1 with modulation quality and TPDR tests for NR updates Summary of change: Removal of details for NR in procedure for modulation quality and for TPDR, and instead included direct reference to NR conformance test specification TS 38.141-1. |
| R4-2300675 | Nokia, Nokia Shanghai Bell | CR to TS 38.176-2 with updates to OTA modulation quality Summary of change: Addition to procedure for IAB-MT type 2-O step with single PRB allocation i.e. IAB-MT-FR2-TM2 and IAB-MT-FR2-TM2a. |
| R4-2300677 | Nokia, Nokia Shanghai Bell | CR to TS 38.176-1 with corrections to test models acronyms Summary of change: Test model acronyms updated in clauses 6.5.3.4.3, 6.5.4.4.2, 6.7.4.2 |
| R4-2300742 | Ericsson, Verizon, China Telecom | Clarification of carrier resource grid mapping Summary of change: It is clarified that the carrier resource grid with size as indicated by carrierBandwidth in SIB1 shall be mapped to the channel raster for at least one numerology: the channel raster for the “RF channel” maps to a carrier advertised in SIB1 and in signaling of cell-specific parameters. The CA channel spacing with UE-specific channel bandwidths is specified by reference to the UE specifications. Clause 5.4.1: the CA channel spacing for CA configurations with UE-specific channel bandwidths (configured by the BS) is specified by reference to the corresponding clauses of 38.101-1 and 38.101-2. Clause 5.4.2.1: clarification of the RF channel position and relation to the carrier resource grid. Clause 5.4.2.2: clarification that the carrier resource grid with size as indicated by carrierBandwidth in signaling of common parameters (including SIB1) shall be mapped to the channel raster for at least one numerology. The defintions of nPRB and k are corrected. |
| R4-2301434 | Huawei, HiSilicon | CR to TS 37.105: The aplicability of additional BC3 requirements Summary of change: Add a sentence on the applicability for BC3 transmitter intermodulation and blocking requirements |
| R4-2301437 | Huawei, HiSilicon | CR to TS 37.145-1: The aplicability of additional BC3 requirements Summary of change: Add a sentence on the applicability for BC3 transmitter intermodulation and blocking requirements |
| R4-2301440 | Huawei, HiSilicon | CR to TS 37.145-2: The aplicability of additional BC3 requirements Summary of change: Add a sentence on the applicability for BC3 transmitter intermodulation and blocking requirements |
| R4-2301443 | Huawei, HiSilicon | CR to 37.105: Operating band unwanted emission requirements Summary of change: Add the OBUE requirement for NR operation with UTRA to align with TS 37.104. |
| R4-2301446 | Huawei, HiSilicon | CR to 37.145-1: operating band unwanted emission requirements Summary of change: 1. Add the OBUE requirement for NR operation with UTRA to align with TS 37.104. 2. Add the Option 1 WA BS OBUE requirements in BC1 and BC3 bands > 3GHz |
| R4-2302254 | Huawei, HiSilicon | CR to 37.145-2: Operating band unwanted emission requirements Summary of change: Add the OBUE requirement for NR operation with UTRA to align with TS 37.104. |
| R4-2301452 | Huawei, HiSilicon | CR to 38.141-2: NOTE for n104 in BS co-existence table Summary of change: Add the missing NOTE for n104 in BS co-existence table |
| R4-2301490 | Ericsson | CR to TS 37.105: Wrong reference to NR additional spurious requirement Summary of change: Align the wording used for UTRA and E-UTRA by removing the confusing reference to co-location. |
| R4-2301748 | ZTE Corporation | CR to TS 38.176-1: Maintenance for IAB-MT performance requirement R16 Summary of change: Correct in the corresponding places where the requirements are for IAB-MT, not UE. |
| R4-2301750 | ZTE Corporation | CR to TS36.141[R15] E-UTRA slot TTI Test Model 2-1 Summary of change: Test model label “sE-TM2-1” should be modified as “sE-TM2-2” |
| R4-2302224 | Ericsson | CR to TS 38.141-2: Addition of missing note for FR2 RX spurious emissions in subclause 7.7.5 Summary of change: 1. A note is added in Table 7.7.5.2-1 for FR2 |

## Open issues summary

-

# Topic #2: BS RF requirements and conformance test for LTE and NR (Rel-17)

## Companies’ contributions summary

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2300679 | Nokia, Nokia Shanghai Bell | CR to TS 38.176-1 with bracket removal for measurement uncertainties for timing error between IAB-DU and IAB-MT Summary of change: Brackets removal in clause 4.1.2.2 for 6.6.5 Timing error between IAB-DU and IAB-MT. |
| R4-2300680 | Nokia, Nokia Shanghai Bell | CR to TS 38.176-2 with bracket removal for measurement uncertainties for OTA timing error between IAB-DU and IAB-MT Summary of change: Brackets removal in clause 4.1.2.2 for 6.6.5 Timing error between IAB-DU and IAB-MT. |
| R4-2301494 | Ericsson, Nokia, Nokia Shanghai Bell | CR to TS 37.104: Additional requirements for BC3 Summary of change: Clarify and align the transient period requirements and add the missing requirement for NR+UTRA Clarify the additional IM and blocking BC3 requirements are only applicable if the BS is intended to be deployed in the same geographical area where an UTRA TDD network is operating. |
| R4-2301496 | Ericsson, Nokia, Nokia Shanghai Bell | CR to TS 37.141: Additional requirements for BC3 BSs Summary of change: Clarify the IM and blocking BC3 requirements are only applicable if the BS is intended to be deployed in the same geographical area where an UTRA TDD network is operating. |

## Open issues summary

-