3GPP TSG-RAN WG4 Meeting #106	R4-2302126

Athens, Greece, Feb. 27 – Mar. 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7193 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NR RedCap Idle mode (TS36.133) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-3-3 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Table 4.2.2.5.8-2 is for FR1 other than 1 Rx and Table 4.2.2.5.8-3 is for FR2 other than 2 Rx. - LTE eDRX configuration froms 5.12s and PTW configuration from 10.24s. - Table 4.2.2.5.8-1 is the same as Table 4.2.2.5.6-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change 1Rx and 2RX with FR1 and FR2. - Update the eDRX table to align with LTE eDRX configuration. - Delete the redundant reference Table 4.2.2.5.6-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The related spec. is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------NEXT CHANGE----------------------------

##### 4.2.2.5.8	Measurements of NR cells for RedCap

This clause contains requirements on the NR capable E-UTRAN RedCap UE regarding measurement in RRC_IDLE state. The requirements are specified for inter-RAT NR frequency measurements for serving E-UTRAN cell.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT NR layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT NR layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT NR layers shall be the same as that defined below for lower priority RATs.

The requirements in this section apply for inter-RAT NR measurements. When the measurement rules indicate that inter-RAT NR cells are to be measured, the UE shall measure SS-RSRP and SS-RSRQ of detected NR cells in the neighbour frequency list at the minimum measurement rate specified in this section. The parameter NNR_carrier_RedCap is the total number of configured NR carriers in the neighbour frequency list. The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured NR cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

The UE shall be able to evaluate whether a newly detectable inter-RAT NR cell meets the reselection criteria defined in TS 36.304 [1] within (NNR_carrier_RedCap) * TdetectNR_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or 6 dB in FR1 or 7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or 4 dB in FR1 and 4 dB in FR2 for SS-RSRQ reselections based on absolute priorities.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_RedCap. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT NR cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

Cells which have been detected shall be measured at least every (NNR_carrier_RedCap) * TmeasureNR_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT NR cell has met reselection criterion defined in TS 36.304 [1] within (NNR_carrier_RedCap) * TevaluateNR_RedCap when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or 6 dB in FR1 or 7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or 4 dB in FR1 and 4 dB in FR2 for SS-RSRQ reselections based on absolute priorities.

For UE not configured with eDRX_IDLE cycle, Tdetect,NR_RedCap, Tmeasure,NR_RedCap and Tevaluate,NR_RedCap are specified in Table 4.2.2.5.8-1 .

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_RedCap, Tmeasure,NR_RedCap and Tevaluate,NR_RedCap are specified in Table 4.2.2.5.8-2 and Table 4.2.2.5.8-3 for FR1 and FR2 respectively.

If Treselection timer has a non zero value and the inter-RAT NR cell is satisfied with the reselection criteria which are defined in TS 36.304 [1], the UE shall evaluate this NR cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2.2.5.8-1: Tdetect,NR_RedCap, Tmeasure,NR_RedCap and Tevaluate,NR_RedCap

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_RedCap [s] (number of DRX cycles) | Tmeasure,NR_RedCap [s] (number of DRX cycles) | Tevaluate,NR_RedCap [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE 1: Applies for RedCap UE of all power classes. |  |  |  |  |  |

Table 4.2.2.5.8-2: Tdetect,NR_RedCap, Tmeasure,NR_RedCap and Tevaluate,NR_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] |  | DRX cycle length [s] |  | PTW length [s] (number of 1.28s periods) | Tdetect,NR_RedCap [s] (number of DRX cycles) |  | Tmeasure,NR_RedCap [s] (number of DRX cycles) |  | Tevaluate,NR_RedCap [s] (number of DRX cycles) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.12 |  | - |  | - | 117.76 (23) |  | 5.12 (1) |  | 10.24 (2) |  |
| 10.24 ≤ eDRX_IDLE cycle length ≤2621.44 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x M2 (1 x M2) | 0.64 x M2 (2 x M2) |
|  |  | 0.64 |  | ≥1.28 (1) |  |  | 0.64 (1) |  | 1.28 (2) |  |
|  |  | 1.28 |  | ≥2.56 (2) |  |  | 1.28 (1) |  | 2.56 (2) |  |
|  |  | 2.56 |  | ≥5.12 (4) |  |  | 2.56 (1) |  | 5.12 (2) |  |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals, 5.12s. Otherwise, number of DRX cycles. |  |  |  |  |  |

Table 4.2.2.5.8-3: Tdetect,NR_RedCap, Tmeasure,NR_RedCap and Tevaluate,NR_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_RedCap [s] (number of DRX cycles) | Tmeasure,NR_RedCap [s] (number of DRX cycles) | Tevaluate,NR_RedCap [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 ≤ eDRX_IDLE cycle length ≤2621.444 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| NOTE 1: Applies for RedCap UE of all power classes. NOTE 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 4: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s. Otherwise, number of DRX cycles. NOTE 5: The lower bound of PTW length is derived based on Tevaluate,NR_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |  |

----------------------END OF CHANGES----------------------------