3GPP TSG-RAN WG4 Meeting #106	R4-2302390

# Athens, Greece, February 27 - March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7199 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections in TS 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Errors in current specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Current specification contains editorial erors which are corrected in this CR. This CR is based on agreed R4-2008657. Removed remaining [ ] and fixed the missing numbers which were mistakenly deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.6, 9.1.20.2, 9.1.21.11, 9.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

#### 4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage

The UE shall be able to identify new inter-frequency cells and perform NRSRP measurements of identified inter-frequency cells if carrier frequency information is provided by the serving NB-IoT cell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Pcarrier * Tdetect,NB_Inter_EC. An inter-frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.5 for a corresponding Band.

The UE shall not cause any interruption to the paging reception and acquisition of SI while performing measurement on serving or any neighbor cells.

The UE shall filter NRSRP measurements of each measured inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NB_Inter _EC/2.

If an inter-frequency cell has been already detected but that has not been reselected to the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Pcarrier * Tevaluate,NB_Inter_EC. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and inter-frequency NB-IoT cells.

If Treselection timer has a non zero value and the inter-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For a UE not configured with eDRX_IDLE cycle, Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate, NB_inter_EC are specified in Table 4.6.2.6-1. For UE configured with eDRX_IDLE cycle, Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate, NB_inter_EC are specified in Table 4.6.2.6-2 for the UE in enhanced coverage, where the requirements apply provided that the serving NB-IoT cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate, NB_inter_EC when multiple PTWs are used.

Table 4.6.2.6-1 : Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate,NB_Inter_EC

| SCH Ês/Iot of neighboring cell: Q2 | DRX cycle length [s] | Tdetect,NB_Inter_ EC [s] (number of DRX cycles) | Tmeasure,NB_Inter_ EC [s] (number of DRX cycles) | Tevaluate,NB_Inter_ EC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0.32 | 256 (800) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 266 (415) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 532 (415) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 532 (208) | 2.56 (1) | 15.36 (6) |
|  | 5.12 | 1063 (208) | 5.12 (1) | 20.48 (4) |
|  | 10.24 | 1063 (104) | 10.24 (1) | 30.72 (3) |
| Q2-6 | 0.32 | 26 (80) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 29 (45) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 58 (45) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 59 (23) | 2.56 (1) | 15.36 (6) |
|  | 5.12 | 113 (22) | 5.12 (1) | 20.48 (4) |
|  | 10.24 | 113 (11) | 10.24 (1) | 30.72 (3) |

Table 4.6.2.6-2: Tdetect,NB_Inter_EC, Tmeasure,NB_Inter_EC and Tevaluate, NB_inter_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] |  | PTW length [s] (number of 2.56s periods) | Tdetect,NB_Inter_EC [s] (number of DRX cycles) | Tmeasure,NB_Inter_EC [s] (number of DRX cycles) | Tevaluate,NB_interEC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥12.8 (5) |  | (406) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | ≥12.8 (5) |  |  | 1.28 (2) | 10.24 (16) |
|  | 1.28 | ≥15.36 (6) |  |  | 1.28 (1) | 12.8 (10) |
|  | 2.56 | ≥17.92 (7) |  |  | 2.56 (1) | 15.36 (6) |
|  | 5.12 | ≥23.04 (9) |  |  | 5.12 (1) | 20.48 (4) |
|  | 10.24 | ≥33.28 (13) |  |  | 10.24 (1) | 30.72 (3) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |  |  |  |

Table 4.6.2.6-3: Conditions on NSCH Ês/Iot of identified and of the neighbour cell

| NSCH Ês/Iot of already identified cell including serving cell: Q1 | Neighbouring cell NSCH Ês/Iot: Q2 | Cell Reselection Margin ‘Y’ |
| --- | --- | --- |
| -15≤Q1<-6 | -15≤ Q2 < -6 | 9.3 |
| -15≤Q1<-6 | Q2-6 | 9.3 |
| Q1 -6 | Q2-6 | 5 |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s inter-frequency measurement is not required to meet Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate,NB_intra_EC as defined in Table 4.6.2.6-1 and Table 4.6.2.6-2.

------------------------------------------------------------- End of change 1 ------------------------------------------------------------

------------------------------------------------------------- Start of change 2 ------------------------------------------------------------

#### 9.1.20.2	SSTD Measurement Report Mapping

SFN and subframe timing difference (SSTD) measurement report comprises 3 elements:

SFN offset between MeNB and SeNB (X)

Reporting range of X is between frame number # 0 to frame number # 1023 as defined in TS 36.331 [2].

Frame boundary offset between MeNB and SeNB (Y)

Reporting range of Y is between subfrane number #-5 and subframe number# 4 as defined in TS 36.331 [2].

Subframe boundary offset between MeNB and SeNB (Z)

The reporting range of value of Z is between -1320Ts and +1320Ts with the reporting granularity of 10Ts in the intervals [-1320Ts, -700Ts] and [700Ts, 1320Ts].

The mapping of measured Subframe boundary offset (Z) is defined in Table 9.1.20.2-1.

Table 9.1.20.2-1: SSTD report mapping

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| SUBFRAME_BOUNDARY_OFFSET_00 | Z -1320 | Ts |
| SUBFRAME_BOUNDARY_OFFSET_01 | -1320 < Z -1310 | Ts |
| SUBFRAME_BOUNDARY_OFFSET_02 | -1310 < Z -1300 | Ts |
|  |  | … |
| SUBFRAME_BOUNDARY_OFFSET_62 | -710 < Z -700 | Ts |
| SUBFRAME_BOUNDARY_OFFSET_63 | -700 < Z 0 | Ts |
| SUBFRAME_BOUNDARY_OFFSET_64 | 0 < Z 700 | Ts |
| SUBFRAME_BOUNDARY_OFFSET_65 | 700 < Z 710 | Ts |
| … | … | … |
| SUBFRAME_BOUNDARY_OFFSET_125 | 1300 < Z 1310 | Ts |
| SUBFRAME_BOUNDARY_OFFSET_126 | 1310 < Z 1320 | Ts |
| SUBFRAME_BOUNDARY_OFFSET_127 | 1320 < Z | Ts |

------------------------------------------------------------- End of change 2 ------------------------------------------------------------

------------------------------------------------------------- Start of change 3 ------------------------------------------------------------

#### 9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B

The requirements for absolute accuracy of RSRP in this clause apply to a cell on another frequency than that of the serving cell for UE category M1.

The accuracy requirements in Table 9.1.21.11-1 and Table 9.1.21.11-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.11-1: RSRP Inter frequency absolute accuracy for UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 8 | 11 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
| 7 | 10 | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 10 | 13 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| 9 | 12 | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.11-2: RSRP Inter frequency absolute accuracy for UE category M1 with CE mode B for HD-FDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 8 | 11 | -15≤Ês/Iot≤-12 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
| 7 | 10 | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 10 | 13 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, FDD-M1_D, FDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| 9 | 12 | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

------------------------------------------------------------- End of change 3 ------------------------------------------------------------

------------------------------------------------------------- Start of change 4 ------------------------------------------------------------

### 9.1.23	Power Headroom for UE Category NB1

The requirements in this clause shall apply for power headroom for UE Category NB1 as defined in [31].

The power headroom provides the serving eNB with information about the differences between the UE configured maximum output power (PCMAX,) defined in TS 36.101 [5] and the estimated power for UL-NSCH transmission of the serving cell [3].

Table 9.1.23 -1: The applicability of power headroom report mapping requirements for different power class UE

| Power class | Power headroom report mapping |
| --- | --- |
| PC3 and PC5 | As defined in section 9.1.23.3 |
| PC6 | As defined in section 9.1.23.4 |

#### 9.1.23.1	Period

The reported power headroom shall be estimated over 1 slot of NPUSCH transmissions.

#### 9.1.23.2	Reporting Delay

The power headroom reporting delay is defined as the time between the beginning of the power headroom reference period and the time when the UE starts transmitting the power headroom over the radio interface. The reporting delay of the power headroom shall be 0 ms, which is applicable for all configured triggering mechanisms for power headroom reporting.

#### 9.1.23.3	Report Mapping for UE Category NB1

The power headroom reporting range is from -54 dB ...+11 dB for UE category NB1 when the enhanced coverage level 0 is selected during the random access procedure [17]. The report mapping is defined in Table 9.1.23.3-1 for UEs not supporting enhanced PHR, and in Table 9.1.23.3-1A for UEs supporting enhanced PHR [31].

Table 9.1.23.3-1: Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR [31] when the enhanced coverage level 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH 5 |
| POWER_HEADROOM_1 | 5 PH 8 |
| POWER_HEADROOM_2 | 8 PH 11 |
| POWER_HEADROOM_3 | PH ≥ 11 |

Table 9.1.23.3-1A: Power headroom report mapping for UE category NB1 UEs supporting enhanced PHR [31] when the enhanced coverage level 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -5 PH -37 |
| POWER_HEADROOM_1 | -37 PH -33 |
| POWER_HEADROOM_2 | -33 PH -29 |
| POWER_HEADROOM_3 | -29 PH -25 |
| POWER_HEADROOM_4 | -25 PH -21 |
| POWER_HEADROOM_5 | -21 PH -17 |
| POWER_HEADROOM_6 | -17 PH -13 |
| POWER_HEADROOM_7 | -13 PH -9 |
| POWER_HEADROOM_8 | -9 PH -5 |
| POWER_HEADROOM_9 | -5 PH -1 |
| POWER_HEADROOM_10 | -1 PH 3 |
| POWER_HEADROOM_11 | 3 PH 7 |
| POWER_HEADROOM_12 | 7 PH 11 |
| POWER_HEADROOM_13 | 11 PH 15 |
| POWER_HEADROOM_14 | 15 PH 19 |
| POWER_HEADROOM_15 | PH ≥ 19 |

The power headroom reporting range is from -54 dB ...+6 or 11 dB for UE category NB1 when enhanced coverage level other than 0 is selected during the random access procedure [17]. The report mapping is defined in Table 9.1.23.3-2 for the UEs not supporing enhanced PHR, and in Table 9.1.23.3-2A for UEs supporting enhanced PHR [31].

Table 9.1.23.3-2: Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR [31] when the enhanced coverage level other than 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

Table 9.1.23.3-2A: Power headroom report mapping for UE category NB1 supporting enhanced PHR [31] when the enhanced coverage level other than 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -45 |
| POWER_HEADROOM_1 | -45 PH -41 |
| POWER_HEADROOM_2 | -41 PH -37 |
| POWER_HEADROOM_3 | -37 PH -33 |
| POWER_HEADROOM_4 | -33 PH -29 |
| POWER_HEADROOM_5 | -29 PH -25 |
| POWER_HEADROOM_6 | -25 PH -21 |
| POWER_HEADROOM_7 | -21 PH -17 |
| POWER_HEADROOM_8 | -17 PH -13 |
| POWER_HEADROOM_9 | -13 PH -9 |
| POWER_HEADROOM_10 | -9 PH -5 |
| POWER_HEADROOM_11 | -5 PH -1 |
| POWER_HEADROOM_12 | -1 PH 3 |
| POWER_HEADROOM_13 | 3 PH 7 |
| POWER_HEADROOM_14 | 7 PH 1 |
| POWER_HEADROOM_15 | PH ≥ 11 |

##### 9.1.23.3.1	Void

##### 9.1.23.3.2	Void

#### 9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6

The power headroom reporting range is -54 dB … +11 dB for UE category NB1 when the enhanced coverage level 0 is selected during the random access procedure [17] for UE power class 6 [5]. The report mapping is defined in Table 9.1.23.4-1 for the UEs not supporting enhanced PHR, and in Table 9.1.23.4-1A for UEs supporting enhanced PHR [31].

Table 9.1.23.4-1: Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH 5 |
| POWER_HEADROOM_1 | 5 PH 8 |
| POWER_HEADROOM_2 | 8 PH 11 |
| POWER_HEADROOM_3 | PH ≥ 11 |

Table 9.1.23.4-1A: Power headroom report mapping for UE category NB1 for UE PC6 and supporting enhanced PHR [31] when the enhanced coverage level 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -45 |
| POWER_HEADROOM_1 | -45 PH -41 |
| POWER_HEADROOM_2 | -41 PH -37 |
| POWER_HEADROOM_3 | -37 PH -33 |
| POWER_HEADROOM_4 | -33 PH -29 |
| POWER_HEADROOM_5 | -29 PH -25 |
| POWER_HEADROOM_6 | -25 PH -21 |
| POWER_HEADROOM_7 | -21 PH -17 |
| POWER_HEADROOM_8 | -17 PH -13 |
| POWER_HEADROOM_9 | -13 PH -9 |
| POWER_HEADROOM_10 | -9 PH -5 |
| POWER_HEADROOM_11 | -5 PH -1 |
| POWER_HEADROOM_12 | -1 PH 3 |
| POWER_HEADROOM_13 | 3 PH 7 |
| POWER_HEADROOM_14 | 7 PH 11 |
| POWER_HEADROOM_15 | PH ≥ 11 |

The power headroom reporting range is from -54 dB ...0 dB for UE category NB1 when the enhanced coverage level other than 0 is selected during the random access procedure [17] for UE power class of 6 [5]. The report mapping is defined in Table 9.1.23.4-2.

Table 9.1.23.4-2: Power headroom report mapping for UE category NB1 when the enhanced coverage level other than 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -20 |
| POWER_HEADROOM_1 | -20 PH -10 |
| POWER_HEADROOM_2 | -10 PH 0 |
| POWER_HEADROOM_3 | PH ≥ 0 |

------------------------------------------------------------- End of change 4 ------------------------------------------------------------