3GPP TSG-RAN WG4 Meeting # 106												R4-2302777

Athens, Greece, 27 February –03 March, 2023

Agenda item:			9.14.6

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Topic summary for [106][219] NR_FR1_lessthan_5MHz_BW

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

# Topic #1: RRM Core Requirements impact analysis

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-23xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2300284 | Apple | In this contribution, we discuss the RRM impacts for the spectrum less than 5MHz. Proposal 1: RAN4 to discuss this WI(spectrum less than 5MHz) based on R15 framework as starting point; and R16/R17/R18 features with less than 5MHz can be discussed case by case if operators have such deployments. Proposal 2: RAN4 to discuss the RRM impacts like in table 1. |
| R4-2300481 | Intel | Observation 1: Cell search performance in terms of search time and detection probability may be impacted from RAN1 design decision. Observation 2: Degradation of time-domain resolution from limited bandwidth is inevitable and it needs to be evaluated based on RAN1 design decision especially for UE transmit timing requirements. Other signalling characteristic requirements also may depend on the number of available DMRS in the new PBCH. Observation 3: RAN4 can decide whether relax the accuracy or extend time / samples after evaluating the RAN1 design details on PBCH / CORESET0. Proposal 1: RAN4 to firstly identify the potentially impacted RRM requirements. After RAN1 design details are fixed, RAN4 to evaluate performance degradation and finally move forward to decide the specification changes. |
| R4-2301984 | Huawei, HiSilicon | Proposal 1: Existing cell detection and measurement requirements can be used as a starting point for less than 5MHz BW. Proposal 2: RAN4 to study whether existing SBI reading requirements can be re-used for less than 5MHz BW after RAN1 concludes on PBCH design. Proposal 3: RAN4 to study the PBCH decoding performance for SFTD and CGI reading requirements after RAN1 concludes on PBCH design. Proposal 4: RAN4 to study whether existing timing requirements can be re-used for 3MHz BW. |
| R4-2302241 | Nokia, Nokia Shanghai Bell | Observation 1: For FRMCS gradual migration, 3 MHz and 5 MHz channel bandwidths are sufficient from RF viewpoint, while it is important to support a larger set of L1 transmission bandwidths to facilitate co-existence with up to 14 GSM-R carriers on the n100 band. Proposal 1: RAN4 can start the RRM impact analysis with 15 RBs and clarify the exact number (e.g., 16 or 15RBs) and other possible numbers when RAN1 agrees on them. Observation 2: PBCH occupies bandwidth of 3.6 MHz, or 20 PRBs, and PBCH needs to be narrowed down to the desired transmission bandwidth. Observation 3: no impact on PSS / SSS is expected. Therefore PSS / SSS can be used without adaptation mechanism (e.g., puncturing) Observation 4: DMRS detection performance may degrade because of PBCH puncturing Observation 5: PBCH decoding performance may degrade because of PBCH puncturing Observation 6: RAN4 PBCH puncturing impact will depend on RAN1 design. Proposal 2: RAN4 to evaluate the d degradation Observation 7: There is negative performance impact of using less PRBs on PBCH DMRS detection. 2.7dB loss seen with 12PRB BW and 1.4dB loss with 15PRB BW. Proposal 3: RAN4 to evaluate the impact on RRM requirements due to the PBCH-DMRS detection performance degradation. Observation 8: CORESETs can be flexibly configured to the UE after the initial access but there are limited configuration options for the CORESET#0 that is used for the PDCCH scheduling the transmission of SIB1. Observation 9: During system information acquisition, UE monitors PDCCH on resources spanning at least 4.32 MHz (i.e., 24 PRBs). To support scenarios with narrower Tx BW, changes are needed. Proposal 4: PDCCH changes are necessary to support NR in narrow spectrum allocations. RAN4 shall evaluate PDCCH impact of reduced bandwidth to RRM requirements. Proposal 5: 15 RBs is sufficient for SSB and CORESET#0 for the 3MHz – 4Mhz (15 - 19 RBs) case. Proposal 6: RAN4 to consider the RRM impact of PDCCH detection performance of CORESET#0 Observation 11: CSI-RS for mobility is having configurable bandwidth with minimum bandwidth being 24 PRBs and there is no relation to the BWP size. Proposal 7: RAN4 to wait for RAN1 before proceeding on CSI-RS for mobility Proposal 8: RAN4 to study with simulations the performance impact of the reduced bandwidth |
| R4-2302318 | MediaTek inc | Proposal 1: No impact of less than 5 MHz on PSS/SSS detection. Also, no simulation is needed. Proposal 2: RAN4 shall wait for RAN1 to complete their study on PDCCH and PBCH detection. Proposal 3: RAN4 should leverage the link-level simulation assumptions and parameters from RAN1 to speed up the progress and discussion in RAN4. |
| R4-2302658 | Ericsson | Mobility: Proposal 1: Cell search requirements and HO requirements shall be further investigated for potential impact. Proposal 2: RAN4 to agree that RACH requirements do not need to be enhanced. Timing: Proposal 3: Reuse the LTE requirement for UE initial transmission timing error, Te = 12 Ts for a channel bandwidth of 3 MHz, regardless of we use 15 or 16 PRB. Proposal 4: MRTD and MTTD requirements of legacy NR to be reused for NR less than 5MHz Proposal 5: The timing related requirements Timer accuracy requirements, Timing advance requirements and Cell phase synchronisation accuracy can reuse the requirements of legacy NR. RLM: Proposal 6: For RLM OOS and IS for CBW less than 5MHz, thresholds Qout and Qin should be derived with the assumption the PDCCH is transmitted with the same parameters as TS38.133 8.1.2.1 and 8.1.3.1 even if the PDCCH transmission bandwidth exceeds the transmission channel bandwidth. Proposal 7: RAN4 can keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB and TEvaluate_in_SSB for CBW less then 5MHz. Proposal 8: RAN4 should extend the CSI-RS based RLM evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for CBW less then 5MHz, where Mout = [30] (=1.5*20) for OOS and Mout = [15] (=1.5*10) for IS for CSI-RS within the channel bandwidth below 24RB. Interruption, CA and DC: Proposal 9: RAN4 to agree that interruption requirements on other active serving cells need not be studied and specified Proposal 10: RAN4 to agree that following requirements are not impacted due to introduction of NR in less than 5 MHz bandwidth a. SCell activation and deactivation requirements b. PSCell addition/change c. SCG activation/deactivation d. Conditional PSCell change requirements Proposal 11: RAN4 to agree that UL carrier reconfiguration requirements can be reused for NR less than 5 MHz. Link recovery: Proposal 12: For BFD for CBW less than 5MHz, threshold Qout_LR, should be derived with the assumption the PDCCH is transmitted with the same parameters as TS38.133 Table 8.5.2.1-1 even if the PDCCH transmission bandwidth exceeds the transmission channel bandwidth. Proposal 13: RAN4 apply the existing SSB based BFD evaluation period TEvaluate_BFD_SSB for CBW less then 5MHz. Proposal 14: RAN4 should extend the CSI-RS based BFD evaluation period TEvaluate_BFD_CSI-RS for CBW less then 5MHz, where MBFD = [15] (=1.5*10) for CSI-RS within the channel bandwidth below 24RB. Proposal 15: RAN4 apply the existing SSB based CBD evaluation period TEvaluate_CBD_SSB for CBW less then 5MHz. Proposal 16: RAN4 should extend the CSI-RS based CBD evaluation period TEvaluate_CBD_CSI-RS for CBW less then 5MHz, where MBFD = [5] (= 1.5*3) for CSI-RS within the channel bandwidth below 24RB. BWP and TCI state switching: Proposal 17: RAN4 to agree that BWP switching and CBW requirements are not impacted with introduction of NR less than 5 MHz. Proposal 18: RAN4 to request RAN1 about the TCI state switching framework to be used for NR less than 5MHz. RAN4 to send LS to RAN1 to confirm the same. Proposal 19: TCI state switching requirements impact can be further studied after the framework is clear. Measurements: Proposal 20: RAN4 apply the exiting L3 measurement requirements for CBW less than 5MHz. Proposal 21: RAN4 apply the existing SSB based L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB for CBW less then 5MHz. Proposal 22: RAN4 apply the existing CSI-RS based L1-RSRP measurement period TL1-RSRP_Measurement_Period_CSI-RS for CBW less then 5MHz. Proposal 23: In the performance part, RAN4 should discuss whether to define CSI-RS based L1-RSRP measurement accuracy requirements with CSI-RS is less than 24RB. Proposal 24: RAN4 should discuss whether L1-SINR measurement requirements are applicable for CBW less than 5MHz. Proposal 25: If RAN4 agree that L1-SINR measurement requirements are applicable for CBW less than 5MHz, apply the same conclusion as L1-RSRP measurement requirements for CBW less than 5MHz. Proposal 26: RAN4 to agree that defining positioning requirements are out of scope of the WID Proposal 27: RAN4 to agree that measurement performance requirements are studied during the performance stage. Proposal 28: RAN4 to discuss whether TMIB/TSI is relaxed due to the PBCH puncturing during the performance stage. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

6 companies have submitted discussion papers for this topic for RAN4#106 meeting.

All companies address the possible scope and impacted RRM requirements from introducing UE operations in spectrum allocations from approximately 3 MHz up to below 5 MHz.

From the WID as input to facilitate the discussion:

  - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix.

  - For SSB:

    - Reuse PSS/SSS specification without puncturing.

    - PBCH based on current design

  - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization.

RAN1 has made some agreements which should be accounted in the discussions.

Based on the input from companies it is proposed to look at the RRM requirements impact identified by the different companies. Each company view has been captured under each issue (issue 1-1 – issue 1-26) and summary is in below table.

Based on the collected views from the contributions the impact on the RRM requirements has been collected and concluded in the table such that if a company listed possible impact or more study needed, it is listed as ‘FFS’ in the table below. If no company had identified any impact, it is concluded as ‘no impact’.

Hence, the following table collect the current status based on contributions before the meeting.

| RRM Requirement | approximately 3 MHz | Below 5 MHz | Issue |
| --- | --- | --- | --- |
| Idle mode Requirements | FFS | FFS | 1-1 |
| Inactive Mode Requirements | FFS | FFS | 1-2 |
| Handover | FFS | FFS | 1-3 |
| RRC Re-establishment | FFS | FFS | 1-4 |
| RRC Connection Release with Redirection | FFS | FFS | 1-5 |
| Random Access | no impact | no impact | 1-6 |
| UE Transmit timing | FFS | FFS | 1-7 |
| MRTD/MTTD | no impact | no impact | 1-8 |
| RLM | FFS | FFS | 1-9 |
| Interruptions | no impact | no impact | 1-10 |
| SCell Activation delay | FFS | FFS | 1-11 |
| UE UL carrier RRC reconfiguration delay | no impact | no impact | 1-12 |
| Link Recovery Procedures (SSB based BFD, CSI-RS based BFD, SSB based CBD, CSI-RS based CBD | FFS | FFS | 1-13 |
| Active BWP switch delay | no impact | no impact | 1-14 |
| PSCell Addition | FFS | no impact | 1-15 |
| TCI state switching | FFS | FFS | 1-16 |
| PSCell change | no impact | no impact | 1-17 |
| SCG Activation | FFS | no impact | 1-18 |
| General measurement requirements (e.g. measurement gaps, measurement capability etc) | no impact | no impact | 1-19 |
| Intra-frequency measurements (SSB, CSI-RS) | FFS | FFS | 1-20 |
| Inter-frequency measurements (SSB, CSI-RS) | FFS | FFS | 1-21 |
| Inter-RAT (LTE) | no impact | no impact | 1-22 |
| L1-RSRP | FFS | FFS | 1-23 |
| L1-SINR | FFS | FFS | 1-24 |
| NR positioning | not in scope of WI | not in scope of WI | 1-25 |
| Measurement performance requirements | FFS | FFS | 1-26 |

Each of the following Issues will address each RRM Requirement in the table. Conclusion will be captured in the table (possibly in a WF).

### Sub-topic 1-1

Sub-topic description:

RRM requirements impact analysis from introducing UE operations in spectrum allocations from approximately 3 MHz up to below 5 MHz.

Issue 1-0: Initial RRM impact analysis assumption

- Proposal: RAN4 can start the RRM impact analysis with 15 RBs and clarify the exact number (e.g., 16 or 15RBs) and other possible numbers when RAN1 agrees on them?

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - To progress the work where possible it is proposed to agree that RAN4 can start the RRM impact analysis with 15 RBs and clarify the exact number (e.g., 16 or 15RBs) and other possible numbers when RAN1 agrees on them.

Issue 1-1: Impact on Idle mode Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Idle mode Requirements | Apple: FFS: EMR measurement with SSN index reading (section: 4.4.2.2 Measurements of inter-frequency CA/DC candidate cells) Intel: FFS Huawei: FFS MediaTek: FFS Ericsson: FFS Nokia: FFS | Apple: No impact Huawei: No impact MediaTek: no impact Ericsson: FFS Nokia: FFS |

- Recommended WF

  - FFS

Issue 1-2: Impact on Inactive mode Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Inactive mode Requirements | Apple: FFS: EMR measurement with SSN index reading (section: 4.4.2.2 Measurements of inter-frequency CA/DC candidate cells) Intel: FFS Huawei: FFS MediaTek: FFS Ericsson: FFS Nokia: FFS | Apple: No impact Huawei: No impact MediaTek: no impact Ericsson: FFS Nokia: FFS |

- Recommended WF

  - FFS

Issue 1-3: Impact on Handover Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Handover Requirements | Apple: HO requirement: FFS if PBCH BW reduction will impact SSB index reading during Tsearch Intel: FFS Huawei: FFS MediaTek: FFS Ericsson: FFS Nokia: FFS | Apple: No impact Ericsson: FFS Nokia: FFS |

- Recommended WF

  - FFS

Issue 1-4: Impact on RRC Re-establishment Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| RRC Re-establishment | Apple: Reestablishment delay requirement: FFS if PBCH BW reduction will impact SSB index reading during Reestablishment Huawei: FFS MediaTek: FFS Nokia: FFS | Apple: No impact Nokia: FFS |

- Recommended WF

  - FFS

Issue 1-5: Impact on RRC Connection Release with Redirection Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| RRC Connection Release with Redirection | Apple: Redirection delay requirement: FFS if PBCH BW reduction will impact SSB index reading during Redirection Huawei: FFS MediaTek: FFS Nokia: FFS | Apple: No impact Nokia: FFS |

- Recommended WF

  - FFS

Issue 1-6: Impact on Random Access Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Random Access | Apple: No impact Ericsson: no impact | Apple: No impact Ericsson: no impact |

- Recommended WF

  - no impact

Issue 1-7: Impact on UE Transmit timing Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| UE Transmit timing | Apple: No impact | Apple: No impact |
|  | Intel: FFS Huawei: FFS Ericsson: no impact | Huawei: FFS Ericsson: no impact |

- Recommended WF

  - FFS

Issue 1-8: Impact on UE MRTD/MTTD Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| MRTD/MTTD | Apple: No impact Ericsson: no impact | Apple: No impact Ericsson: no impact |

- Recommended WF

  - no impact

Issue 1-9: Impact on UE RLM Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| RLM | Apple: (1)Hypothetical PDCCH transmission parameter: FFS due to reduced BW (Current hypothetical BW for SSB is 24PRBs (4.32MHz) and for CSI-RS is 48PRBs(8.64MHz)). (2)CSI-RS based OOS/IS evaluation: FFS due to reduced BW (Current hypothetical BW for CSI-RS based evaluation is 24PRBs (4.32MHz)). Intel: FFS Ericsson: FFS Nokia: FFS | Apple: Hypothetical PDCCH transmission parameter: FFS due to reduced BW (Current hypothetical BW for SSB is 24PRBs (4.32MHz) and for CSI-RS is 48PRBs(8.64MHz)). Ericsson: no impact Nokia: FFS |

- Recommended WF

  - FFS

Issue 1-10: Impact on UE Interruption Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Interruptions | Apple: No impact Ericsson: no impact | Apple: No impact Ericsson: no impact |

- Recommended WF

  - no impact

Issue 1-11: Impact on UE SCell Activation delay Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| SCell Activation delay | Apple: Activation delay requirement: (1)FFS if PBCH BW reduction will impact SSB index reading during SCell activation (2)FFS if CSI-RS BW reduction will impact L1-RSRP measurement delay during SCell activation (Current CSI-RS L1-RSRP is based on 48PRBs, L1-RSRP related side conditions given in clauses 10.1.19.2) Ericsson: no impact Nokia: FFS | Apple: Activation delay requirement: FFS if CSI-RS BW reduction will impact L1-RSRP measurement delay during SCell activation (Current CSI-RS L1-RSRP is based on 48PRBs, L1-RSRP related side conditions given in clauses 10.1.19.2) Ericsson: no impact |

- Recommended WF

  - FFS

Issue 1-12: Impact on UE UL carrier RRC reconfiguration delay Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| UE UL carrier RRC reconfiguration delay | Apple: No impact Ericsson: no impact | Apple: No impact Ericsson: no impact |

- Recommended WF

  - no impact

Issue 1-13: Impact on UE Link Recovery Procedure Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Link Recovery Procedures (SSB based BFD, CSI-RS based BFD, SSB based CBD, CSI-RS based CBD | Apple: BFD: (1)Hypothetical PDCCH transmission parameter: FFS due to reduced BW (Current hypothetical BW for SSB is 24PRBs (4.32MHz) and for CSI-RS is 48PRBs(8.64MHz)). (2)CSI-RS based evaluation: FFS due to reduced BW (Current hypothetical BW for CSI-RS based evaluation is 24PRBs (4.32MHz)). Apple: CBD: CSI-RS based evaluation: FFS due to reduced BW (Current hypothetical BW for CSI-RS based evaluation is 24PRBs (4.32MHz)). Intel: FFS Ericsson: FFS Nokia: FFS | Apple: BFD: No impact Apple: CBD: No impact Ericsson: FFS Nokia: FFS |

- Recommended WF

  - FFS

Issue 1-14: Impact on UE Active BWP switch delay Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Active BWP switch delay | Apple: No impact Ericsson: no impact | Apple: No impact Ericsson: no impact |

- Recommended WF

  - no impact

Issue 1-15: Impact on UE PSCell Addition Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| PSCell Addition | Apple: PSCell addition: FFS if PBCH BW reduction will impact SSB index reading during Tsearch Huawei: FFS MediaTek: FFS Nokia: FFS | Apple: No impact |

- Recommended WF

  - FFS

Issue 1-16: Impact on UE TCI state switching Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| TCI state switching | Ericsson: FFS | Ericsson: FFS |

- Recommended WF

  - FFS

Issue 1-17: Impact on UE PSCell change Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| PSCell change | Ericsson: no impact | Ericsson: no impact |

- Recommended WF

  - no impact

Issue 1-18: Impact on UE SCG Activation Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| SCG Activation | Apple: SCG activation: FFS if PBCH BW reduction will impact SSB index reading during Tsearch Huawei: FFS MediaTek: FFS Ericsson: no impact Nokia: FFS | Apple: No impact Ericsson: no impact |

- Recommended WF

  - FFS

Issue 1-19: Impact on UE General measurement Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| General measurement requirements | Apple: Measurement capability: No impact | Apple: Measurement capability: No impact |

- Recommended WF

  - no impact

Issue 1-20: Impact on UE Intra-frequency measurements Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Intra-frequency measurements | Apple: SSB based Cell identification/measurement delay: FFS SSB index reading delay due to reduced BW of PBCH DMRS CSI-RS based L3 measurement delay: FFS CSI-RS based L3 measurement delay due to reduced CSI-RS BW (Current CSI-RS L3 measurement requirement is based on 48PRBs) CGI reading with autonomous gaps: FFS on the reduced BW for MIB and SIB1 Huawei: FFS MediaTek: FFS Nokia: FFS Other measurement requirement: No impact Intel: FFS Ericsson: FFS | Apple: SSB based Cell identification/measurement delay: No impact CSI-RS based L3 measurement delay: FFS CSI-RS based L3 measurement delay due to reduced CSI-RS BW (Current CSI-RS L3 measurement requirement is based on 48PRBs) CGI reading with autonomous gaps: No impact (current band n100, n8, n26 and n28 has min BW=5MHz) Huawei: FFS Other measurement requirement: No impact Huawei: No impact MediaTek: no impact Ericsson: FFS |

- Recommended WF

  - FFS

Issue 1-21: Impact on UE Inter-frequency measurements Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Inter-frequency measurements | Apple: SSB based Cell identification/measurement delay: FFS SSB index reading delay due to reduced BW of PBCH DMRS CSI-RS based L3 measurement delay: FFS CSI-RS based L3 measurement delay due to reduced CSI-RS BW (Current CSI-RS L3 measurement requirement is based on 48PRBs) CGI reading with autonomous gaps: FFS on the reduced BW for MIB and SIB1 Huawei: FFS MediaTek: FFS Nokia: FFS Other measurement requirement: No impact Intel: FFS Ericsson: FFS | Apple: SSB based Cell identification/measurement delay: No impact CSI-RS based L3 measurement delay: FFS CSI-RS based L3 measurement delay due to reduced CSI-RS BW (Current CSI-RS L3 measurement requirement is based on 48PRBs) CGI reading with autonomous gaps: No impact (current band n100, n8, n26 and n28 has min BW=5MHz) Huawei: FFS Other measurement requirement: No impact Huawei: No impact MediaTek: no impact Ericsson: FFS |

- Recommended WF

  - FFS

Issue 1-22: Impact on UE Inter-RAT Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Inter-RAT (LTE) | Apple: No impact | Apple: No impact |

- Recommended WF

  - no impact

Issue 1-23: Impact on UE L1-RSRP Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| L1-RSRP | Apple: FFS if CSI-RS BW reduction will impact L1-RSRP measurement delay (Current CSI-RS L1-RSRP is based on 48PRBs, L1-RSRP related side conditions given in clauses 10.1.19.2) | Apple: FFS if CSI-RS BW reduction will impact L1-RSRP measurement delay (Current CSI-RS L1-RSRP is based on 48PRBs, L1-RSRP related side conditions given in clauses 10.1.19.2) |

- Recommended WF

  - FFS

Issue 1-24: Impact on UE L1-SINR Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| L1-SINR | Ericsson: FFS | Ericsson: FFS |

- Recommended WF

  - FFS

Issue 1-25: Impact on UE NR positioning Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| NR positioning | Ericsson: not in scope | Ericsson: not in scope |

- Recommended WF

  - not in scope

Issue 1-26: Impact on UE Measurement performance Requirements

- Proposals

| RRM Requirement | approximately 3 MHz | Below 5 MHz |
| --- | --- | --- |
| Measurement performance requirements | Apple: CSI-RS based measurement accuracy: FFS CSI-RS based L3 measurement accuracy due to reduced CSI-RS BW (Current CSI-RS L3 measurement accuracy requirement is based on 48PRBs) Intel: FFS Ericsson: FFS | Apple: CSI-RS based measurement accuracy: FFS CSI-RS based L3 measurement accuracy due to reduced CSI-RS BW (Current CSI-RS L3 measurement accuracy requirement is based on 48PRBs) Ericsson: FFS |

- Recommended WF

  - FFS

Issue 1-27: Features to consider in this WI

- Proposals

  - RAN4 to discuss this WI(spectrum less than 5MHz) based on R15 framework as starting point; and R16/R17/R18 features with less than 5MHz can be discussed case by case if operators have such deployments.

- Recommended WF

  - FFS

# Topic #2: Simulation Assumptions

Following preliminary simulation assumptions for DMRS detection, DMRS demodulation, and PDCCH detection loss due to puncturing, have been proposed as baseline for upcoming simulations.

Companies are encouraged to comment on the listed simulation assumptions.

Please add any comments directly in the table with Company name.

Table 1 Preliminary simulation assumptions for DMRS detection, DMRS demodulation, and PDCCH detection loss due to puncturing

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 900MHz |
| Subcarrier spacing | kHz | 15 kHz; |
| Number of Tx antennas | - | 1 |
| Number of Rx antennas | - | 2 |
| DMRS | - | 3GPP NR PBCH DMRS |
| Other assumptions |  | Tx BW known at Rx side Rx BW known at TX side |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | Baseline 0.9 |
| PBCH symbols within the SS block |  | PSS-PBCH-SSS-PBCH |
| Data and Control Power offset with respect to PSS and SSS | dB | Baseline 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | No changes expected |
| PBCH-DMRS sequences | - | No changes expected |
| PBCH-DMRS RE positions within the PBCH resource | - | No changes expected except for puncturing impact |
| PBCH Channel coding |  | No changes expected to actual Channel coding (Polar code with 512 length and 24bit CRC) |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) ¦ FFS |
| PBCH SNR | dB | -10: 0 dB, with 1 dB spacing |
| Propagation Condition / Channel models | - | FR1: AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns |
| UE speed |  | FR1: 3 km/h, Higher speeds FFS |
| Detection Method |  | One shot detection (No combination for different PBCHs) |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |