3GPP TSG-RAN WG4 Meeting # 106												R4-2302813

Athens, Greece, 27 February –03 March, 2023

Agenda item:			8.33.5

Source:	Moderator (Ligado Networks)

Title:	Topic summary for [106][120] R18_NR_TDD_n54

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Background:

In RAN#98e, CRs related to Introduction of E-UTRA Band in 1670 – 1675 MHz (Band 54) was completed [1,2]. A new work item (WI) to introduce a corresponding NR band n54 was approved [3].

[1] RP-223315, RAN4 CRs to Introduction of LTE TDD band in 1670 – 1675 MHz, RAN#98-e

[2] RP-222830, Status Report for WI: Introduction of LTE TDD band in 1670 – 1675 MHz, RAN#98-e

[3] RP-223514, New WID: Introduction of NR band in 1670 – 1675 MHz, RAN#98-e

Given the discussions that have occurred related Tx and Rx requirements for E-UTRA Band 54 in previous meetings, the agreements and WFs [4, 5, 6, 7, 8] related to B54 Tx/Rx requirements and specifications are likely applicable and can be leveraged for specifying the re-farmed NR band n54.

[4] R4-2212090, Proposed key changes to TS 36.101 to support introduction of new LTE TDD Band in 1670 – 1675 MHz, RAN4#104-e

[5] R4-2214238, Email discussion summary for [104-e][127] R18_LTE_TDD_1.6GHz, RAN4#104-e

[6] R4-2214440, WF on Introduction of LTE TDD band in 1670-1675 MHz, RAN4#104-e

[7] R4-2207966, Emissions Requirements for LTE TDD band in 1670 – 1675 MHz, RAN4#103-e

[8] R4-2210572, WF on Introduction of LTE TDD band in 1670-1675 MHz, RAN4#103-e

Scope:

This thread is to summarize the proposals and CRs submitted for the introduction of re-farmed NR band n54, and the issues resulting from the submissions.

Discussion structure:

- Topic 1: Identify and summarize issues related to system parameters and UE requirements and CRs

- Topic 2: Identify and summarize issues related to BS/IAB/Repeater (core and conformance testing related) requirements and CRs

- Topic 3: Identify and summarize issues related to RRM requirements (band grouping) and CR

# Topic #1: System Parameters and UE RF

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300070 | Ligado Networks | Proposal 1: It is proposed that CRs for TS 38.101-1 and TS 38.104 with above system parameters for n54 be agreed. |
| R4-2301614 | MediaTek, Inc. | Proposes system parameters for n54 |
| R4-2302350 | Mediatek India Technology Pvt. | Observation 1: Regarding NR band n54 UE RF requirements for TS 38.101-1, to leverage band 54 most RF requirements from TS 36.101 are feasible. Proposal 1: To agree the proposal in the Table 2.1-1, or at least use them as the starting points for further discussions on NR band n54 UE TX/RX requirements for TS 38.101-1. |
| R4-2300071 | Ligado Networks | Proposal 1: It is proposed that the CR with above changes to TS 38.101-1 for the Introduction of n54 in R4-2300072 be agreed. |
| R4-2300072 | Ligado Networks | CR to TS 38.101-1 based on assessment performed in R4-2300071 and system parameters proposed in R4-2300070 |
| R4-2300073 | Ligado Networks | CR to TS 38.101-5 to protect n54 (Spurious Emissions UE Co-existence) |
| R4-2301211 | ZTE Corporation | Draft CR to TS 38.101-1 based on assessment performed in R4-2301219 |
| R4-2301219 | ZTE Corporation | Proposal 1: For the new NR TDD band, to define NR-ARFCN as Table 2.1-1. Proposal 2: For the new NR TDD band, to define sync raster as Table 2.1-2. Proposal 3: For the new NR TDD band, to define UE power class as Table 2.2-1. Proposal 4: For band n54, to follow the reference sensitivity requirement for B54 defined in 36.101. |
| R4-2302028 | Nokia, Nokia Shanghai Bell | CR to TS 38.104 (also proposes system parameters for n54 for inclusion in 38.104) |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Table 1 below compares the proposals for system parameters and identifies any potential issues for discussion during the meeting:

Table 1: Comparing proposals for system parameter

| System Parameters (Clause 5) | Ligado (From discussion, paper, CR) | Media Tek (From discussion paper) | Nokia (From BS CR) | ZTE (From UE and BS Draft CRs) |
| --- | --- | --- | --- | --- |
| Operating bands | No issues identified: all contributors have proposed same frequency range for the TDD band n54: 1670 – 1675 MHz |  |  |  |
| UE channel bandwidth | No issues identified: all contributors have proposed same values of 5 MHz with 15 kHz SCS |  |  |  |
| Channel raster | No issues identified: all contributors have proposed the same range: 334000 - <20> - 335000 for UL and DL with ΔFRaster = 100 kHz |  |  |  |
| Sync raster |  |  |  |  |
| SS block SCS | No issues identified: all contributors have proposed 15 kHz |  |  |  |
| SS block pattern | No issues identified: all contributors have proposed Case A |  |  |  |
| GSCN range | 4181 - <1> - 4182 | 4179 - <1> - 41874184* | 4181 - <1> - 4182 | 4181 - <1> - 4182 |

*Moderator Note: based on offline comment received from Mediatek, the larger GSCN range was proposed to future proof it for eventual 3 MHz channel support being contemplated as part of WI NR_FR1_lessthan_5MHz_BW. Also, the upper value of the range was calculated incorrectly and a new value of 4184 was provided via email (Mediatek is willing to submit a revised tdoc if consideration of CBW of 3 MHz is accepted).

Table 2 below compares the proposals for Tx/Rx requirements and identifies any potential issues for discussion during the meeting:

Table 2: Comparing proposals for changes to Tx/Rx clauses

| TS 38.101-1 Clauses with band specific requirements | Band-specific? | Reuse 36.101 requirement? | Ligado | Media Tek | ZTE |
| --- | --- | --- | --- | --- | --- |
| Clause 6: Transmitter Characteristics |  |  |  |  |  |
| UE maximum output power | Yes | Yes | No issues identified: all contributors have proposed reuse values from 36.101 for Band 54: 23 +/- 2 dBm |  |  |
| UE additional maximum output power reduction | Yes | A-MPR: Yes CBW: Update NS value: Yes | No issues identified: all contributors have proposed that like Band 54, no A-MPR is required to meet additional spurious emissions requirements; however, CBW needs to be only 5 MHz for NR. Also, like B54, NS_62 is proposed for signalling additional spurious emissions requirements if it is not used for another NR band (which is currently the case) |  |  |
| Spurious emissions for UE co-existence | Yes | Yes | No issues identified: all contributors have proposed reusing requirements from 36.101 as done for majority of NR bands |  |  |
| Additional spurious emissions | Yes | Yes | No issues identified: all contributors have proposed adding additional spurious emissions requirements from 36.101 |  |  |
| Clause 7: Receiver characteristics |  |  |  |  |  |
| Minimum requirements (QPSK) | Yes | Yes | No issues identified: all contributors have proposed to reuse -100 dBm for the 5 MHz CBW from TS 36.101 |  |  |
| In-band blocking | Yes | Yes | No issues identified: all contributors have proposed the same requirement as majority of the NR bands in the same way as for B54 |  |  |
| Out-of-band blocking | Yes | Yes | No issues identified: all contributors have proposed the same requirement as majority of the NR bands in the same way as for B54 |  |  |
| Narrow band blocking | Yes | N/A | No issues identified: all contributors have proposed the same requirement as majority of the NR bands |  |  |

### Sub-topic 1-1: System Parameters

Sub-topic description:

Review of proposals and change requests for system parameters for n54.

Open issues and candidate options before meeting:

Several companies have submitted proposals for the system parameters for n54. Table 1 above summarized the proposals. Except for GSCN range, all companies have the same proposal for other system parameters.

Issue 1-1: GSCN Range

- Proposals

  - Option 1: Based on support for CBW of 5 MHz

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n54 | 15 kHz | Case A | 4181 – <1> – 4182 |

  - Option 2: Based on future support for CBW of 3 MHz

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n54 | 15 kHz | Case A | 4179 – <1> – 41874184 |

- Recommended WF

  - Discuss and if necessary, revise the values in the formal CRs for 38.101-1 and 38.104 in R4-2300070 and R4-2302028.

  - Moderator comment: It might be best not to prejudice work related to NR_FR1_lessthan_5MHz_BW WI by assuming size of PBCH to be 12 PRBs (as done by Mediatek); GSCN range based on standard PBCH size and 5 MHz channel bandwidth is preferable.

### Sub-topic 1-2: UE Tx Requirements

Sub-topic description :

Review of proposals and change requests for the UE Tx Requirements for n54 (Clause 6 of TS 38.101-1)

Open issues and candidate options before meeting:

There are no open issues

Recommended WF

- Review and if acceptable, merge (add ZTE corp, Mediatek to list of companies to the coversheet) R4-2301211 into R4-2300070 since it is a formal CR and if necessary, update the GSCN range based on agreement reached for sub-topic 1-1

### Sub-topic 1-3: UE Rx Requirements

Sub-topic description

Review of proposals and change requests for the UE Rx Requirements for n54 (Clause 7 of TS 38.101-1)

Open issues and candidate options before meeting:

There are no open issues

Recommended WF

- Review and if acceptable, merge (add ZTE corp, Mediatek to list of companies to the coversheet) R4-2301211 into R4-2300070 since it is a formal CR. R4-2300070 also includes the additional spurious emissions table and the configuration for reference sensitivity that seem to be missing in R4-2301211.

# Topic #2: BS RF

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2301220 | ZTE Corporation | Proposal 1: For the new NR TDD band, to define NR-ARFCN as Table 2.1-1. Proposal 2: For the new NR TDD band, to define sync raster as Table 2.1-2. Proposal 3: To use the proposals in table for band n54 BS RF requirements. |
| R4-2301218 | ZTE Corporation | Draft CR for TS 38.104 based on assessment performed in R4-2301220 |
| R4-2301190 | Nokia, Nokia Shanghai Bell | CR to TS 36.104 |
| R4-2301191 | Nokia, Nokia Shanghai Bell | CR to TS 36.141 |
| R4-2301192 | Nokia, Nokia Shanghai bell | CR to TS 37.104 |
| R4-2301193 | Nokia, Nokia Shanghai bell | CR to TS 37.141 |
| R4-2301195 | Nokia, Nokia Shanghai bell | Discusses and proposes OBUE, spurious emissions coexistence and BS co-location requirements for n54 |
| R4-2300681 | Nokia, Nokia Shanghai bell | CR to TS 38.115-1 |
| R4-2300682 | Nokia, Nokia Shanghai bell | CR to TS 38.106 |
| R4-2300683 | Nokia, Nokia Shanghai bell | CR to TS 38.176-1 |
| R4-2300684 | Nokia, Nokia Shanghai bell | CR to TS 38.176-2 |
| R4-2300685 | Nokia, Nokia Shanghai bell | CR to TS 38.174 |
| R4-2301474 | Ericsson, Ligado Networks | CR to 37.105 |
| R4-2301475 | Ericsson, Ligado Networks | CR to 37.145-1 |
| R4-2301476 | Ericsson, Ligado Networks | CR to 37.145-2 |
| R4-2302030 (Rev of R4-2301188) | Nokia, Nokia Shanghai bell | CR to 38.141-1 |
| R4-2302029 (Rev of R4-2301189) | Nokia, Nokia Shanghai bell | CR to 38.141-2 |
| R4-2302028 (Rev of R4-2301194) | Nokia, Nokia Shanghai bell | CR to 38.104 based on assessment performed in R4-2301195 |

Moderator comments:

R4-2300682: WI code on the coversheet needs to be NR_TDD_n54-Core (instead of NR_TDD_n54-Perf)

R4-2300685: WI code on the coversheet needs to be NR_TDD_n54-Core (instead of NR_TDD_n54-Perf)

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Table 3 below compares the proposals for Tx/Rx clauses and identifies any potential issues for discussion during the meeting. The system parameters comparison was performed in sub-topic 1-1.

Table 3: Comparing proposals for changes to BS system parameters and Tx/Rx clauses

| TS 38.104 Clauses with band specific requirements | Band-specific? | Reuse 36.104 requirement? | Nokia (From CR) | ZTE (From discussion paper and CR) |
| --- | --- | --- | --- | --- |
| Tx Requirements |  |  |  |  |
| OBUE | Yes | Yes | No issues identified: both have proposed Category A BS OBUE |  |
| Additional spurious emissions | Yes | Yes | No issues identified: both have proposed -52 dBm/1 MHz value (Note; Nokia captures the additional spurious emissions requirements for n54 in Table 6.6.5.2.3-14 as well) Note: Ericsson/Ligado are using the same value in their CRs for 37.105 as well |  |
| Co-location requirements | Yes | Yes | No issues identified: both have proposed -96, -91 and -88 dBm values for WA, MR and LA BS with MBW of 100 kHz Note: Ericsson/Ligado are using the same value in their CRs for 37.105 as well |  |
| Rx Requirements |  |  |  |  |
| Dynamice range, ACS, inband blocking, out of band blocking narrowband blocking, Rx IMD, Rx spurious emissions | No | N/A | No issues identified: Band agnostic general requirements are applicable for n54 |  |

There are no open issues other than the GSCN range issue 1-1 of sub-topic 1-1

### Sub-topic 2-1: BS Tx Requirements (Core/Performance)

Sub-topic description

Review of proposals and change request for BS requirements (OBUE, spurious emissions for coexistence and BS co-location requirements) to core and conformance testing specifications for n54

Open issues and candidate options before meeting:

No open issues

Recommended WF

- Review and if acceptable, merge (add ZTE corp to list of companies to the coversheet) changes from R4-2301211 into R4-2300070 since it is a formal CR.

- Review and if necessary, revise n54 CRs for other BS core and conformance testing specifications related to OBUE, coexistence and co-location (TS 36.104, 37.104, 37.105, 36.141, 37.141, 37.145-1, 37.145-2, 38.141-1, 38.141-2)

### Sub-topic 2-2: Repeater Tx Requirements (Core/Performance)

Sub-topic description

Review of proposals and change requests for repeater requirements (OBUE, spurious emissions for coexistence and BS co-location requirements) to core and conformance testing specifications for n54

Open issues and candidate options before meeting:

No open issues

Recommended WF

- Review and if necessary, revise n54 CRs for repeater core and conformance testing specifications (TS 38.106, 38.115-1)

### Sub-topic 2-3: IAB Tx Requirements (Core/Performance)

Sub-topic description

Review of proposals and change requests for IAB requirements (spurious emissions for coexistence and BS co-location requirements) to core and conformance testing specifications for n54

Open issues and candidate options before meeting:

No open issues

Recommended WF

- Review and if necessary, revise n54 CRs for IAB core and conformance testing specifications (TS 38.174, 38.176-1, 38.176-2)

# Topic #3: RRM Requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2301477 | Ericsson, Ligado Networks | Proposes band grouping for n54 |
| R4-2301207 | ZTE | Proposes band grouping for n54 |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Band grouping

Sub-topic description:

Band grouping for n54 in the RRM specification is proposed. Both are proposing Group A for n54, and no issues have been identified.

Open issues and candidate options before meeting:

There are no open issues

Recommended WF

- Review and if acceptable, revise R4-2301477 to include ZTE as co-signing company.