3GPP TSG-RAN WG4 Meeting # 106												R4-2302825

Athens, Greece, 27 February –03 March, 2023

Agenda item:			9.8.5

Source:	Moderator (Apple)

Title:	Topic summary for [106][132] FR2_multiRx_UERF_part2

Document for:	Information

# Introduction

This email thread is focused on AI 9.8.2.1 System parameter assumption and UE architecture.

# Topic #1: System parameter assumption and UE architecture

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300195 | Nokia, Nokia Shanghai Bell | Observation 1: Single fixed AoA offset will not cover all the relevant practical use cases. Observation 2: In single-DCI scheme, the performance of a given pair of AoAs for a given UE orientation would be dominated by the worst link and may be inferior to that of single direction reception. Observation 3: The performance of UE will be the best when the orthogonality of the 2 beams is achieved. Otherwise, the interference of the 2 beams will be introduced into the system. Based on the statistics of the tests for the specific UE (see Figure 3), the interference happens more often when the AoA of 2 beams become smaller. Observation 4: In real deployment scenarios, small AoA offset is possible. Proposal 1: For fixed relative AoA separation for UE RF requirement test set up, single fixed AoA offset should not be considered. Proposal 2: With fixed test-probe for AoA1, the offset angles for defining the AoA2 should be selected from the candidates 30∘, 60∘, 90∘, 120∘ and 150∘. Proposal 3: With small AoA separation and single-DCI scheme, the UE RF requirements may need to be relaxed as compared to the current requirements for single direction reception. Proposal 4: For the multi-DCI scheme, we suggest keeping the legacy spherical coverage requirement in the standard, but we can also consider reasonable relaxation depending on AoA separation due to the inter-beam interference between the 2 beams. Proposal 5: RAN4 to define a different set of UE RF requirements for single-DCI and multi-DCI schemes. Proposal 6: For deriving UE RF requirements, small AoA offset must not be ignored. |
| R4-2300267 | Apple | Proposal 1: To have the unified requirement concept for UEs supporting multi-DCIs as for UEs supporting single DCI, RAN4 can focus on fully overlapping in time and in frequency, supported by UE capabilities “multiDCI-MultiTRP-r16” and “overlapPDSCHsFullyFreqTime-r16.” Proposal 2: Use MMSE-IRC as receiver assumptions for defining RF requirement for both sDCI and mDCI. Besides the two UE processing assumptions, no further consideration of other “joint detect/decode” aspect in UE BB processing for mDCI is assumed. Proposal 3: If there is some performance difference between sDCI and mDCI, such difference can be reflected in the requirement. Proposal 4: It remains to be seen if it is necessary to specify minimum network benefit for enhanced UE, given RAN4 is going to specify some spherical coverage requirement. |
| R4-2301233 | ZTE | Observation 1: In order to match both inter-cell and intra-cell TRP, different fixed AoA offset values are needed. Observation 2: In order to match the variable locations of UE, different fixed AoA offset values are needed. Proposal 1: Multiple fixed AoA offset values should be introduced, the number of fixed AoA offset values are FFS. Proposal 2: To define single set of requirements for both sDCI and mDCI, fully overlapping in time and non-overlapping/fully overlapping in frequency should be taken as the starting point for mDCI. |
| R4-2301461 | OPPO | Proposal 1: The range AoA angular separation is proposed as 30 degree to 150 degree. Proposal 2: It is proposed that UE can satisfy the RF requirement with UE’s declaration on the preferred fixed AoA separation between 30 degree to 150 degree. |
| R4-2301573 | vivo | Observation 1: For indoor and Dense Urban scenario, the AoA separation is distributed between 0° and 180° and it is hard to further narrow down the AoA separation range. Observation 2: The impact of AoA separation will be mitigated when separation becomes larger but the impact is also related to the dedicated UE implementation. Proposal 1: UE declares 2 AoA separation values from 2 AoA separation pools for verification - one is a range for small AoA separation [30, 90], and another is a range for larger separation (90,180]. - Each pool has only a limited number of separation values, the specific value is FFS Proposal 2: The AoA separation value should be an integer multiple of the test grid. |
| R4-2302522 | Keysight | Observation 1: For single DCI schemes, only the total throughput (TP) can be measured, i.e., the TP and thus EIS metric cannot be determined per AoA. Observation 2: From a TE vendor perspective, the Joint 2 AoA Sensitivity approach 1 (single-DCI scheme) is preferred ease of implementation, reduced test time, and lack of ambiguity Observation 3: The number of polarization combinations and the joint 2 AoA sensitivity approaches (single-DCI schemes) have a large effect on test time while the number of AoA2 probes does not affect test time significantly. Observation 4: For multi-DCI schemes, TP and EIS/sensitivity can be determined per AoA. Proposal 1: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, apply a full rotation in  and a half rotation in . Proposal 2: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, utilize constant-step size grids only. Proposal 3: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, place the AoA2 probes in the xz plane. Proposal 4: OEMs and chipset vendors to comment on the directionality of the AoA1-AoA2 DL orientation vectors, e.g., whether reciprocity of the TRPs will tolerate the observed directionality. Proposal 5: Limit the polarization combinations for the 2-DL spherical coverage test case pending feedback from OEMs and chipset vendors. Proposal 6: OEMs and chipset vendors to provide feedback on the 2 presented Joint 2 AoA Sensitivity approaches in Table 4 for single-DCI schemes and whether two different AoA1 and AoA2 DL levels can yield the same target total TP. Proposal 7: OEMs and chipset vendors to provide feedback on how the Total Joint 2 AoA Sensitivity, TJ2AS, is determined from the Joint 2 AoA Sensitivities for each polarization combination, i.e., . TJ2AS = f(J2ASAoA1q, AoA2q, J2ASAoA1q, AoA2f, J2ASAoA1f, AoA2q, J2ASAoA1f, AoA2f) Proposal 8: For single-DCI scheme with parametric test approach, select the 2 AoA spherical coverage test procedure outlined in Figure 4. Proposal 9: In order to keep multi-AoA DL spherical coverage test times manageable for single-DCI scheme with parametric test approach, it is proposed to limit the number of polarization combinations to 2 and to select the 2 AoA sensitivity approach #1. Proposal 10: For multi-DCI scheme with parametric test approach, select the 2 AoA spherical coverage test procedure outlined in Figure 6. Proposal 11: For the non-parametric test approach, select the multi-AoA spherical coverage test procedure outlined in Figure 8. Proposal 12: Feedback from industry is requested whether the test approach for multi-AoA spherical coverage should be based on a parametric or a non-parametric test |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: fixed AoA offset

Sub-topic description:

Open issues and candidate options before f2f meeting:

Issue 1-1-1: number of fixed AoA offsets needed for defining the requirement

- Proposals

  - Option 1: Multiple fixed AoA offset values should be introduced. (Nokia, ZTE, LGE)

  - Option 2: Small AoA offset must not be ignored. (Nokia)

  - Option 3: Single UE declared offset. (OPPO)

  - Option 4: UE declares 2 AoA separation values from 2 AoA separation pools for verification (vivo)

    - one is a range for small AoA separation [30, 90], and another is a range for larger separation (90,180].

    - Each pool has only a limited number of separation values, the specific value is FFS

- Recommended WF

  - TBA

Issue 1-1-2: Range of AoA offset values

- Proposals

  - Option 1: 30∘, 60∘, 90∘, 120∘ and 150∘ (Nokia, OPPO).

  - Option 2: 30∘, 60∘, 90∘, 120∘, 150∘ and 180∘  (vivo)

- Recommended WF

  - TBA

Issue 1-1-3: AoA offset value should be an integer multiple of the test grid. (vivo)

- Proposals

- Recommended WF

  - TBA

### Sub-topic 1-2: sDCI and mDCI

Sub-topic description

Open issues and candidate options before f2f meeting:

Issue 1-2-1: requirements for sDCI and mDCI

- Proposals

  - Option 1: RAN4 to define a different set of UE RF requirements for single-DCI and multi-DCI schemes. (Nokia)

  - Option 2: If there is some performance difference between sDCI and mDCI, such difference can be reflected in the requirement. (Apple)

- Recommended WF

  - TBA

Issue 1-2-2: mDCI scenario

- Proposals

  - Option 1: To have the unified requirement concept for UEs supporting multi-DCIs as for UEs supporting single DCI, RAN4 can focus on fully overlapping in time and in frequency, supported by UE capabilities “multiDCI-MultiTRP-r16” and “overlapPDSCHsFullyFreqTime-r16.” (Apple)

  - Option 2: To define single set of requirements for both sDCI and mDCI, fully overlapping in time and non-overlapping/fully overlapping in frequency should be taken as the starting point for mDCI. (ZTE)

- Recommended WF

  - TBA

Issue 1-2-3: receiver assumption

- Proposals

  - Option 1: Use MMSE-IRC as receiver assumptions for defining RF requirement for both sDCI and mDCI. Besides the two UE processing assumptions, no further consideration of other “joint detect/decode” aspect in UE BB processing for mDCI is assumed. (Apple)

    - Process only 2 TRP-RX pairs– In this case we only consider two TRP to RX pairs - TRP1-RX1 and TRP2-RX2. The signal from TRP2 to RX1 and TRP1 to RX2 is treated as interference.

    - Jointly process all TRP-RX combinations – In this case we consider all combinations of TRP to RX pairs – TRP1-RX1, TRP1-RX2, TRP2-RX1, TRP2-RX2, like 4x4 MIMO.

- Recommended WF

  - TBA

### Sub-topic 1-3: minimum network benefit for enhanced UE

Sub-topic description

Open issues and candidate options before f2f meeting:

Issue 1-3-1:  minimum network benefit for enhanced UE

- Proposals

  - Option 1: It remains to be seen if it is necessary to specify minimum network benefit for enhanced UE, given RAN4 is going to specify some spherical coverage requirement. (Apple)

- Recommended WF

  - TBA

### Sub-topic 1-4: Testing

Sub-topic description

Open issues and candidate options before f2f meeting:

Issue 1-4-1:  Testing aspects

- Proposals

  - From Keysight:

  - Proposal 1: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, apply a full rotation in  and a half rotation in .

  - Proposal 2: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, utilize constant-step size grids only.

  - Proposal 3: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, place the AoA2 probes in the xz plane.

  - Proposal 4: OEMs and chipset vendors to comment on the directionality of the AoA1-AoA2 DL orientation vectors, e.g., whether reciprocity of the TRPs will tolerate the observed directionality.

- Recommended WF

  - TBA