3GPP TSG-RAN WG4 Meeting # 106												R4-2303753

Athens, Greece, 27 February –03 March, 2023

Agenda item:			9.14.6

Source:	Man Hung Ng (Nokia)

Title:	Topic summary for [106][308] NR_FR1_lessthan_5MHz_BW_BSRF

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Email discussion for contributions submitted under agenda items 9.14.4 for NR support for dedicated spectrum less than 5MHz for FR1.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Discussion and agreement on open issues listed below.

- 2nd round: Continue discussion and agreement on open issues listed below.

# Topic #1: BS RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300198 | Nokia, Nokia Shanghai Bell | Proposal 1: It is proposed to approve the proposals for NR 3 MHz in Table 1 to guide the specification changes. NR BS RF Tx/Rx requirement; Current LTE specification for 3 MHz; Proposal for NR 3 MHz. 6.2 Base station output power; Minimum requirements are channel bandwidth agnostic, no additional regional requirement for 3 MHz.; No specification impact. 6.3.2 RE power control dynamic range; Channel bandwidth agnostic; No specification impact. 6.3.3 Total power dynamic range; Minimum requirement calculated according to transmission bandwidth configuration.; Minimum requirement calculated according to transmission bandwidth configuration in Table 6.3.3.2-1.. 6.3.4 NB-IoT RB power dynamic range for NB-IoT operation in NR in-band; No minimum requirement for 3 MHz.; No specification impact. 6.4 Transmit ON/OFF power; Channel bandwidth agnostic; No specification impact. 6.5 Transmitted signal quality; Channel bandwidth agnostic; No specification impact. 6.6.2 Occupied bandwidth; Channel bandwidth agnostic; No specification impact. 6.6.3 Adjacent Channel Leakage Power Ratio; Requirement limits are channel bandwidth agnostic, but assumed adjacent channel carriers are different for 3 MHz in unpaired spectrum.; Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.2-1 and Table 6.6.3.2-2a.. 6.6.4 Operating band unwanted emissions; Specific operating band unwanted emission limits for 3 MHz.; Specific operating band unwanted emission limits for 3 MHz.. 6.6.4 Transmitter spurious emissions; Channel bandwidth agnostic; No specification impact. 6.7 Transmitter intermodulation; Channel bandwidth agnostic; No specification impact. 7.2 Reference sensitivity level; Channel bandwidth dependent; Add minimum requirements for 3 MHz in Table 7.2.2-1, Table 7.2.2-2 and Table 7.2.2-3. Scaling from 5 MHz can be used if the same SU as LTE 3 MHz is adopted.. 7.3 Dynamic range; Channel bandwidth dependent; Add minimum requirements for 3 MHz in Table 7.3.2-1, Table 7.3.2-2 and Table 7.3.2-3. Scaling from 5 MHz can be used if the same SU as LTE 3 MHz is adopted.. 7.4.1 Adjacent Channel Selectivity (ACS); Requirement limits are channel bandwidth agnostic, but assumed adjacent channel carriers are different for 3 MHz.; Same requirement limits as other channel bandwidth in Table 7.4.1.2-1, but assume different adjacent channel carriers in Table 7.4.1.2-2.. 7.4.2 In-band blocking; Requirement limits are channel bandwidth agnostic, but assumed adjacent channel carriers are different for 3 MHz general blocking requirement.; Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 7.4.2.2-1, Table 7.4.2.2-2 and Table 7.4.2.2-3.. 7.5 Out-of-band blocking; Channel bandwidth agnostic; No specification impact. 7.6 Receiver spurious emissions; Channel bandwidth agnostic; No specification impact. 7.7 Receiver intermodulation; Requirement limits are channel bandwidth agnostic, but assumed adjacent channel carriers are different for 3 MHz general blocking requirement.; Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 7.7.2-2 and Table 7.7.2-4.. 7.8 In-channel selectivity; Channel bandwidth dependent; Add minimum requirements for 3 MHz in Table 7.8.2-1, Table 7.8.2-2 and Table 7.8.2-3. Requirement limits depend on ICS FRC to be defined for 3MHz.. 9.2 Radiated transmit power; Channel bandwidth agnostic; No specification impact. 9.3 OTA base station output power; Channel bandwidth agnostic; No specification impact. 9.4.2 OTA RE power control dynamic range; Channel bandwidth agnostic; No specification impact. 9.4.3 OTA total power dynamic range; Minimum requirement calculated according to transmission bandwidth configuration.; No specification impact, referring to requirements in Table 6.3.3.2-1.. 9.5 OTA transmit ON/OFF power; Channel bandwidth agnostic; No specification impact. 9.6 OTA transmitted signal quality; Channel bandwidth agnostic; No specification impact. 9.7.2 OTA occupied bandwidth; Channel bandwidth agnostic; No specification impact. 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR); Requirement limits are channel bandwidth agnostic, but assumed adjacent channel carriers are different for 3 MHz in unpaired spectrum.; No specification impact, referring to requirements in Clause 6.6.3.2.. 9.7.4 OTA operating band unwanted emissions; Specific operating band unwanted emission limits for 3 MHz.; No specification impact, referring to requirements in Clause 6.6.4.1.. 9.7.5 OTA transmitter spurious emissions; Channel bandwidth agnostic; No specification impact, referring to requirements in Clause 6.6.5.2.1.. 9.8 OTA transmitter intermodulation; Channel bandwidth agnostic; No specification impact. 10.2 OTA sensitivity; Channel bandwidth dependent; No specification impact, referring to requirements in Clause 7.2.. 10.3 OTA reference sensitivity level; Channel bandwidth dependent; Add minimum requirements for 3 MHz in Table 10.3.2-1, Table 10.3.2-2 and Table 10.3.2-3. Scaling from 5 MHz can be used if the same SU as LTE 3 MHz is adopted.. 10.4 OTA dynamic range; Channel bandwidth dependent; Add minimum requirements for 3 MHz in Table 10.4.2-1, Table 10.4.2-2 and Table 10.4.2-3. Scaling from 5 MHz can be used if the same SU as LTE 3 MHz is adopted.. 10.5.1 OTA adjacent channel selectivity; Requirement limits are channel bandwidth agnostic, but assumed adjacent channel carriers are different for 3 MHz.; Same requirement limits as other channel bandwidth carriers in Table 10.5.1.2-1 but assume different adjacent channel carriers in Table 10.5.1.2-2.. 10.5.2 OTA in-band blocking; Requirement limits are channel bandwidth agnostic, but assumed adjacent channel carriers are different for 3 MHz general blocking requirement.; Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 10.5.2.2-1, Table 10.5.2.2-2 and Table 10.5.2.2-3.. 10.6 OTA out-of-band blocking; Channel bandwidth agnostic; No specification impact. 10.7 OTA receiver spurious emissions; Channel bandwidth agnostic; No specification impact. 10.8 OTA receiver intermodulation; Requirement limits are channel bandwidth agnostic, but assumed adjacent channel carriers are different for 3 MHz general blocking requirement; Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 10.8.2-2 and Table 10.8.2-4.. 10.9 OTA in-channel selectivity; Channel bandwidth dependent; Add minimum requirements for 3 MHz in Table 10.9.2-1, Table 10.9.2-2 and Table 10.9.2-3. Requirement limits depend on ICS FRC to be defined for 3MHz.. Observation 1: |
| R4-2301224 | ZTE Corporation | Proposal 1: To use the proposals in Table 1 for BS RF requirements for NR support for dedicated spectrum less than 5MHz for FR1. Tx RF requirements part. Base station output power; All of these requirements are defined as band or channel bandwidth agnostic except for output power dynamics, therefore it’s not expected to have any impacts with the introduction of 3 MHz channel bandwidth.. Output power dynamics. Transmit ON/OFF power. Transmitted signal quality. OBW. Tx intermodulation. ACLR; To add ACLR requirement for 3MHz CBW.. Rx RF requirements part. REFSENS; To add REFSENS requirement for 3MHz CBW.. Dynamic range; To add dynamic range requirement for 3MHz CBW and define additional FRC.. ACS; To add ACS requirement for 3MHz CBW.. OOBB; To add OOBB requirement for 3MHz CBW.. RX IMD; To add RX IMD requirement for 3MHz CBW.. Rx spurious emission; No requirement impacts.. ICS; To add ICS requirement and define additional FRC for 3MHz CBW.. Observation 1: |
| R4-2301485 | Ericsson | Proposal 1: As starting point, align NR 3 MHz channel BW BS RF requirements with the corresponding LTE 3MHz channel BW requirements. Requirements; Impact. BS output power; For band n100, the additional requirement shall be scaled for 3 MHz to keep PSD constant.. Output power dynamics. RE power control dynamic range; No impact expected.. Total power dynamic range; Requirement for 3 MHz channel BW shall be added .. NB-IoT RB power dynamic range for NB-IoT operation in NR in-band; Not applicable, NB-IoT in-band support is not mentioned in the objectives of this WI.. Transmit ON/OFF power; No impact expected.. Transmitted signal quality. Frequency error; No impact expected.. Modulation quality; No impact expected.. Time alignment error; No impact expected.. Unwanted emissions. Occupied bandwidth; No impact expected.. Adjacent Channel Leakage Power Ratio; Requirement for 3 MHz channel BW shall be added.. Operating band unwanted emissions; A new OBUE mask shall be specified, based on LTE 3 MHz one. For band n100, an additional requirement for 3 MHz channel BW should also be added.. Transmitter spurious emissions; No impact expected.. Transmitter intermodulation; No impact expected.. Requirements; Impact. Reference sensitivity level; Requirement for 3 MHz channel BW shall be added.. Dynamic range; Requirement for 3 MHz channel BW shall be added.. ACS; Requirement for 3 MHz channel BW shall be added.. In-band blocking; Requirement for 3 MHz channel BW shall be added For band n100, an additional narrow band blocking requirement for 3 MHz channel BW shall also be added.. Out of band blocking; No impact expected.. Receiver spurious emission; No impact expected.. Receiver intermodulation; Requirement for 3 MHz channel BW (interferer) shall be added. For band n100, an additional narrowband intermodulation requirement for 3 MHz channel BW shall also be added.. In-channel selectivity; Requirement for 3 MHz channel BW (interferer) shall be added.. Observation 1: |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description: BS RF requirements

Open issues and candidate options before meeting:

Issue 1-1: Conducted requirements

- Proposals

  - See contributions summary in 1.1

- Recommended WF

  - Combine all proposals into one table

| NR BS RF Tx/Rx requirement | Proposal for NR 3 MHz |
| --- | --- |
| 6.2 Base station output power | Option 1: No specification impact (Nokia, ZTE) Option 2: For band n100, the additional requirement shall be scaled for 3 MHz to keep PSD constant. (Ericsson) |
| 6.3.2 RE power control dynamic range | No specification impact |
| 6.3.3 Total power dynamic range | Option 1: Minimum requirement calculated according to transmission bandwidth configuration in Table 6.3.3.2-1. (Nokia, Ericsson) Option 2: No specification impact (ZTE) |
| 6.3.4 NB-IoT RB power dynamic range for NB-IoT operation in NR in-band | No specification impact |
| 6.4 Transmit ON/OFF power | No specification impact |
| 6.5 Transmitted signal quality | No specification impact |
| 6.6.2 Occupied bandwidth | No specification impact |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.2-1 and Table 6.6.3.2-2a. |
| 6.6.4 Operating band unwanted emissions | Option 1: A new OBUE mask shall be specified, based on LTE 3 MHz one. (Nokia, Ericsson) Option 2: For band n100, an additional requirement for 3 MHz channel BW should also be added. (Ericsson) |
| 6.6.4 Transmitter spurious emissions | No specification impact |
| 6.7 Transmitter intermodulation | No specification impact |
| 7.2 Reference sensitivity level | Add minimum requirements for 3 MHz in Table 7.2.2-1, Table 7.2.2-2 and Table 7.2.2-3. Scaling from 5 MHz can be used if the same SU as LTE 3 MHz is adopted. |
| 7.3 Dynamic range | Add minimum requirements for 3 MHz in Table 7.3.2-1, Table 7.3.2-2 and Table 7.3.2-3. Scaling from 5 MHz can be used if the same SU as LTE 3 MHz is adopted. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Same requirement limits as other channel bandwidth in Table 7.4.1.2-1, but assume different adjacent channel carriers in Table 7.4.1.2-2. |
| 7.4.2 In-band blocking | Option 1: Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 7.4.2.2-1, Table 7.4.2.2-2 and Table 7.4.2.2-3. (Nokia, Ericsson) Option 2: For band n100, an additional narrow band blocking requirement for 3 MHz channel BW shall also be added. (Ericsson) |
| 7.5 Out-of-band blocking | Option 1: No specification impact (Nokia, Ericsson) Option 2: To add OOBB requirement for 3MHz CBW. (ZTE) |
| 7.6 Receiver spurious emissions | No specification impact |
| 7.7 Receiver intermodulation | Option 1: Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 7.7.2-2 and Table 7.7.2-4. (Nokia, Ericsson) Option 2: For band n100, an additional narrowband intermodulation requirement for 3 MHz channel BW shall also be added. (Ericsson) |
| 7.8 In-channel selectivity | Add minimum requirements for 3 MHz in Table 7.8.2-1, Table 7.8.2-2 and Table 7.8.2-3. Requirement limits depend on ICS FRC to be defined for 3MHz. |

Issue 1-2: Radiated requirements

- Proposals

  - Option 1: Use table below (Nokia)

| NR BS RF Tx/Rx requirement | Proposal for NR 3 MHz |
| --- | --- |
| 9.2 Radiated transmit power | No specification impact |
| 9.3 OTA base station output power | No specification impact |
| 9.4.2 OTA RE power control dynamic range | No specification impact |
| 9.4.3 OTA total power dynamic range | No specification impact, referring to requirements in Table 6.3.3.2-1. |
| 9.5 OTA transmit ON/OFF power | No specification impact |
| 9.6 OTA transmitted signal quality | No specification impact |
| 9.7.2 OTA occupied bandwidth | No specification impact |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | No specification impact, referring to requirements in Clause 6.6.3.2. |
| 9.7.4 OTA operating band unwanted emissions | No specification impact, referring to requirements in Clause 6.6.4.1. |
| 9.7.5 OTA transmitter spurious emissions | No specification impact, referring to requirements in Clause 6.6.5.2.1. |
| 9.8 OTA transmitter intermodulation | No specification impact |
| 10.2 OTA sensitivity | No specification impact, referring to requirements in Clause 7.2. |
| 10.3 OTA reference sensitivity level | Add minimum requirements for 3 MHz in Table 10.3.2-1, Table 10.3.2-2 and Table 10.3.2-3. Scaling from 5 MHz can be used if the same SU as LTE 3 MHz is adopted. |
| 10.4 OTA dynamic range | Add minimum requirements for 3 MHz in Table 10.4.2-1, Table 10.4.2-2 and Table 10.4.2-3. Scaling from 5 MHz can be used if the same SU as LTE 3 MHz is adopted. |
| 10.5.1 OTA adjacent channel selectivity | Same requirement limits as other channel bandwidth carriers in Table 10.5.1.2-1 but assume different adjacent channel carriers in Table 10.5.1.2-2. |
| 10.5.2 OTA in-band blocking | Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 10.5.2.2-1, Table 10.5.2.2-2 and Table 10.5.2.2-3. |
| 10.6 OTA out-of-band blocking | No specification impact |
| 10.7 OTA receiver spurious emissions | No specification impact |
| 10.8 OTA receiver intermodulation | Same requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 10.8.2-2 and Table 10.8.2-4. |
| 10.9 OTA in-channel selectivity | Add minimum requirements for 3 MHz in Table 10.9.2-1, Table 10.9.2-2 and Table 10.9.2-3. Requirement limits depend on ICS FRC to be defined for 3MHz. |

  - Option 2: FFS if radiated requirements are applicable

- Recommended WF

  - TBA