3GPP TSG-RAN WG4 Meeting #106										    			       R4-2303755

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:			9.19.4

Source:	Moderator (Samsung)

Title:	Topic summary for [106][310] FS_NR_duplex_evo_Part1

Document for:	Information

# Introduction

This thread is on Rel-18 SI for Study on evolution of NR duplex operation, in which the following highlighted agenda items are supposed to be covered:

- Study on evolution of NR duplex operation [FS_NR_duplex_evo] - General and work plan [FS_NR_duplex_evo] - Study the feasibility of and impact on RF requirements [FS_NR_duplex_evo] - Adjacent channel co-existence evaluation [FS_NR_duplex_evo] - Study the feasibility of and impact on RF requirements [FS_NR_duplex_evo] - BS aspect [FS_NR_duplex_evo] - UE aspect [FS_NR_duplex_evo] - Summary of regulatory aspects [FS_NR_duplex_evo] - Moderator summary and conclusions [FS_NR_duplex_evo]

# Topic #1: Feasibility study and RF impact from BS aspects

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300144 (under AI 9.19) | Charter Communications, Inc | Proposal 1: RAN4 should decide on the acceptable range for RSI, to guarantee the operation of SBFD without self-interference. |
| R4-2300061 | Kumu Networks | Observation 1: Using 3GPP_38.901_Uma_LOS channel, we show that having RF cancellation before the receiver LNA have the benefits of achieving the desired self-interference cancellation residue floor as well as preventing saturation of the Rx LNAs. When considering the viability of SBFD, RF cancellation plays a critical part and should be considered in the evaluation of overall RSIC capability. Observation 2: Using 3GPP_38.901_Uma_LOS channel, we show that the worst case rx sensitivity per Rx link is 0.9 dB and the mean case rx sensitivity per Rx link is 0.6 dB. Observation 3: A small (~2 dB) degradation of beam-forming gain is observed with RF cancellation and beam-nulling working jointly to reduce self-interference residue when operating in 3GPP_38.901_Uma_LOS channel Observation 4: While the worst case RSI value are useful to provide insight to how each receiver link performance is impacted by self-interference residue, overall mean value is also useful to provide more typical performance expectation. In our simulation, the worst case RSIC capability is 148 dB and the mean case RSIC capability is 153 dB. Proposal 1: RF cancellation should be used in SBFD to mitigate self-interference pre Rx LNA in terms of minimizing non-linearity effects and overall self-interference residue. Proposal 2: Overall RSIC reporting should include pre-LNA RF mitigation as a separate capability. For example, total RSIC can be given by: - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-RFIC)+dB(αSI-digtial)+… - αSI-spatial denotes the spatial isolation. - αSI-frequency (n,m) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n. - αSI-beam denotes the beam-nulling or beam isolation. - αSI-RFIC denotes the pre-LNA RF interference cancellation capability. - αSI-digtial denotes the digital cancellation capability. Proposal 3: We support using overall Rx sensitivity degradation of 1 dB for SBFD system simulation and evaluation. Proposal 4: When evaluating self-interference cancellation methodology, it is useful to also report the achievable beam-forming gain. Proposal 5: When reporting simulation results, it would be useful to clarify if it is the worst case or typical case being reported. |
| R4-2300445 | Samsung | RSIC Analysis Framework: Observation 1: Before digital IC, the residual self-interference comes from two sources: - PSI_leakage, Interference leakage in gNB RX subband due to non-ideal TX, which can be derived as dBPSI_leakage=dBPTX-dBαSI-frequency, TX-dBαSI-spatial-dBαSI-beam, TX-dBαSI-RFIC,TX = ①-②-③-④-⑤ dBm , with the reference point being set at RX antenna. - PSI_Selectivity, Interference in gNB RX subband caused by non-ideal RX selectivity, which can be derived as dBPSI_Selectivity=dBPTX-dBαSI-spatial-dBαSI-beam,RX-dBαSI-RFIC,RX-dBαSI-frequency, RX=①-③-⑨-⑧-⑥ dBm, which is the gain-normalized value with reference point being set at RX antenna. Observation 2: Before digital IC, the residual self-interference in RX subband is PSI=PSI_leakage+PSI_Selectivity. Observation 3: After digital IC, the residual self-interference in the RX subband can be derived as dBPSIleakage+PSISelectivity-dBαSI-digtial=①-③-dB11linear②×linear④xlinear⑤+1linear⑨×linear⑧×linear⑥-⑦ dBm. Proposal 1: Based on component capabilities in the analysis framework, the overall RSIC capability can be derived as dBRSIC=dBαSI-spatial+dB11linear②×linear④xlinear⑤+1linear⑨×linear⑧×linear⑥+dBαSI-digtial =③+dB11linear②×linear④xlinear⑤+1linear⑨×linear⑧×linear⑥+⑦ dBc. RSIC Capability for FR1 and FR2-1 BS: Observation 4: Samsung’s input for RSIC budget calculation for FR1 BS is provided in Table-1. Observation 5: According to SIC budget calculation in Table-1, it’s feasible to ensure 1dB de-sensitivity based on achievable spatial isolation, frequency isolation, RF IC and digital IC applied, for FR1 BS. Observation 6: Samsung’s input for RSIC budget calculation for FR2-1 BS is provided in Table-1. Observation 7: According to SIC budget calculation in Table-2, it’s feasible to ensure 1dB de-sensitivity based on achievable spatial isolation, frequency isolation and digital IC applied, for FR2-1 BS. Proposal 2: RSIC analysis framework shall be adopted for SBFD BS RF feasibility study to be captured in TR38.858, and subsection for different component capabilities can be reserved to encourage companies’ inputs. Observation 8: Simply following existing RAN4 requirement to derive the required LNA IIP3 metrics and accordingly the feasibility conclusion of SBFD is very pessimistic assumption. Observation 9: According to calculation presented in Table 5, the requested ADC dynamic range is still within the range of commercialized available component. Co-channel Inter-Subband gNB-gNB CLI Modeling: Proposal 3: Based on the analysis framework for self-interference, the below table is proposed for analysis for co-channel co-site inter-sector gNB-gNB CLI modeling, with different fields highlighted as below. Observation 10: The RF simulation has shown that numerical analysis on the antennal isolation for co-channel co-site inter-sector gNB-gNB CLI: - In the range of 62-93dB, depending on different antenna pair and co/cross-polarization - Note: the results are obtained for the 3-sector scenario at 3.5GHz with detailed parameters provided. Observation 11: Antennal isolation for co-channel co-site inter-sector gNB-gNB CLI can be further improved to the range of [90-100]dB, by having the methods e.g., installing EM conjugated structure between sectors, larger horizontal distance, or vertical antenna arrangement, or different boresight angle directions, or different electrical tilts or combination thereof. BS RF Requirement Impact for SBFD capable gNB: Observation 12: It is difficult for RAN4 to agree on a single RF architecture to derive the potential new requirements for (1) in-channel adjacent subblock leakage ratio (new), (2) in-channel adjacent subblock blocking (new) and (3) in-channel adjacent subband selectivity. Proposal 4: RAN4 consider the SBFD performance requirement for receiver sensitivity with the simultaneous TX in the SBFD time slot, in which the in-channel adjacent subblock leakage ratio, and in-channel adjacent subblock blocking and in-channel adjacent subband selectivity requirements can be guaranteed implicitly while no explicit requirement needed. |
| R4-2300545 | CATT | • gNB Self-interference Observation 1: The Tx leakage and Rx contribution requirements to the SI can be divided to -105 dBm each if each of them contributes to half of noise. Observation 2: The overall RSIC requirement for Tx leakage should be at least 148 dB for WA SBFD BS. Observation 3: The RSIC requirement for Rx blocking issue should be 89 dB for WA SBFD BS. Observation 4: If IM3 contribution is -108 dBm for WA BS SBFD Rx noise, IIP3 should be at least-10.5 dBm in whole operation temperature range, which is very challenging. Observation 5: 65 dB ACS capability is needed for WA BS. It is chanllenging if SU is the same with current specification. Observation 6: If Rx blocking level is assumed to be the current WA BS requirement, ADC dynamic range can cover the whole signal level including wanted signal and interference signals. Observation 7: AGC is not need to be analysed for BS Rx path because ADC dynamic is not a problem. • Co-channel inter-cell co-site inter-sector gNB-to-gNB inter-subband CLI Observation 8: The Tx leakage and Rx contribution requirements to the CLI can be divided to -108 dBm each if each of them contributes to half of noise. Observation 9: The RSIC requirement for Tx leakage issue should be 151 dB for co-channel co-site inter-sector CLI, which is very challenging. Observation 10: The RSIC requirement for Rx blocking issue should be 92 dB for co-channel co-site inter-sector CLI. Observation 11: If IM3 contribution is -108 dBm for co-channel co-site inter-sector CLI Rx noise, IIP3 should be at least-9 dBm in whole operation temperature range, which is very challenging. Observation 12: 68 dB ACS capability is needed for co-channel co-site inter-sector CLI. It is chanllenging if SU is the same with current specification. • OBUE performance impact Proposal 1: OBUE performance impact should be discussed and decided for Tx leakage analysis. • RF Requirement Impact o New RF requirements for SBFD Proposal 2: New RF requirements for SBFD listed in Table 7 should be defined. o Existing requirement with different value Proposal 3: Transmit ON/OFF power requirement is not applicable for SBFD BS. Observation 13: Coupling loss for co-location minimum requirements needs to be changed. REFSENS impact can be discussed later when SI, CLI analysis is clear and decided. Proposal 4: OTA receiver spurious emissions can’t be defined for SBFD BS. Observation 14: Co-location assumption for transmitter intermodulation and OTA transmitter intermodulation need to be further studied for SBFD BS. o Below requirements would be remained unchanged with respect to SBFD operation. Proposal 5: Existing requirements listed in Table 8 are unchanged for SBFD BS. |
| R4-2300690 | Nokia, Nokia Shanghai Bell | Proposal 1: Set the upper limit for the DL EIRP impact due to TX beam nulling at 1 dB. Proposal 2: Use RMS average power for the saturation, non-linearity, and AGC models in the feasibility study and system level simulations. In aspects requiring more precise understanding of the signal peaks, a reasonable RX PAPR (at least 10 dB) should be added to the RMS power. Proposal 3: The AGC model captures impacts of receiver non-linearity, gain control, and ADC dynamic range, and the following parameters are used for wide area base stations: Proposal 4: The RSIC framework can be used in co-channel inter-sector case. Proposal 5: The spatial isolation for co-channel inter-sector case is 60 dB for FR1 wide area base stations. Proposal 6: Further study is needed to conclude that no new requirements are needed for In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity. Proposal 7: gNB OTA sensitivity shall be relaxed for SBFD gNB. The DL signal shall be active in the test. Proposal 8: gNB OTA adjacent channel selectivity, in-band blocking, and receiver intermodulation tests shall have the DL signal active. For out-of-band blocking, it is FFS whether to activate the DL signal. |
| R4-2300793 | CMCC | Proposal 1: for inter-sector interference feasibility criteria, it’s suggested to use SLS to analysis minimum inter-sector interference cancellation ratio to avoid blocking. Observation 1: commercial gNB could at least achieve 50dB Rx selectivity. Proposal 2: it’s suggested to at least use 50dB for inter-site Rx selectivity. |
| R4-2301382 | China Telecom | We provide values based on the following RSIC analysis framework table and the information on how the intermediate results are derived. |
| R4-2301426 | Huawei, HiSilicon | In this contribution, we provide some consideration on RSIC analysis. Our analysis and evaluation is sumarized in Table 2.1-1 for FR1 and Table 2.1-2 for FR2. |
| R4-2301734 | ZTE Corporation | Proposal 1: to consider the clutter impact on digital IC in RSIC performance if needed. Proposal 2: to consider in-band blocking level as baseline for gNB receiver saturation, non-linearity and higher capability should depend on vendors’ declaration. |
| R4-2301885 | Ericsson | Observation 1 The RSIC provides a snapshot breakdown of the self-interference for a certain gNB output power and does not generalize to the gNB operating at other power levels. Observation 2 The components of the RSIC are not independent of one another; changing some assumptions for one component can change other components Observation 3 The RSIC breakdown is a snapshot of the gNB at full power with a certain set of assumptions. Observation 4 Antenna isolation varies with beam direction (and varies between different RX sub-arrays). The RSIC values in this section are a snapshot with average spatial isolation per RX sub-array. With some directions the suppression may be better, for other directions worse (for WA and MR). Observation 5 For FR1 WA BS, the receiver is saturated and transmitter leakage is large. It is not feasible to give an RSIC. Observation 6 SBFD for MR BS may achieve self-interference suppression, although a more complex receiver may be needed or the gNB TX power may need to be lower than the 3GPP maximum to achieve 1dB sensitivity degradation. Observation 7 SBFD with 1dB sensitivity degradation can likely be achieved for FR1 LA BS with digital IC. Observation 8 For a WA BS with 53dBm TRP, the receiver is likely to be saturated. Observation 9 For FR2 SBFD, in general receiver linearity is not a major issue for output power below around 35dBm. Observation 10 An RF filtering solution prior to the LNA for a single carrier, custom designed BS for a specific carrier with little DUD configuration flexibility would incur an NF increase of up to 6dB (with questionable feasibility due to filter size) Observation 11 A reasonable RF filtering solution prior to the LNA is not feasible. Observation 12 RX beam nulling is part of the receiver SINR optimization in the receive baseband. Observation 13 For good SINR optimization, the self-interference must be sufficiently supressed that channel estimations are possible at each receiver and that there is no distortion form the receiver analogue front end. Observation 14 The receiver combining performance should not be assessed considering only self-interference. A reference scenario(s) is needed considering all sources of inter-sector, inter-gNB and other cell UE interferences. Observation 15 A reference baseline scenario is needed considering other UE interferences at the BS in order to assess desensitization caused by SBFD when considering RX combining algorithms. Observation 16 For FR1 WA BS (without considering radome), for panel to sub-array isolation better than 70 dB over sufficiently large bandwidth is achievable as long as the beam is steered in boresight. When the beam is steered elsewhere, the isolation may reduce by ~up to 15-20dB. Observation 17 For FR2 WA (without considering radome), using a structure with RF chokes, ~80dB of isolation is achievable over a reasonable bandwidth. Unlike FR1, the isolation does not seem to vary significantly with beam steering. Observation 18 The IC gain depends on the interference structure Observation 19 The interference suppression depends on the interference power (INR) Observation 20 The interference suppression has dependencies on the TX power, antenna isolation and frequency isolation. Observation 21 A number of implementation issues can impact the level of interference suppression, such as modelling of receiver behavior, non-linearities and filtering and TX-RX timing alignment. Observation 22 The interference suppression for reflections depends on the number of taps provided, the coherence time and channel estimation. Observation 23 The gain from beam nulling increases when the TX beam is steered and the antenna isolation decreases. Thus, beam nulling can to some extent reduce the variation of the overall spatial isolation due to beam steering. It may also reduce the frequency variation. However, with increasing steering, the cost in DL of beam nulling increases. Observation 24 The cost of beam nulling in downlink can be substantial; we have observed up to 5dB DL power loss. There may be further DL losses due to lower degrees of freedom for MIMO operation. Observation 25 When deciding beam nulling gains, downlink impacts should be taken into account. Observation 26 The BS is usually a multi-carrier node by default and multi-carrier aspects affect many related feasibility aspects such as improved linearization, CFR, filtering, PIM, beam nulling and digital interference cancellation. Thus, feasibility study assuming single carrier operation for BS is not sufficient. |
| R4-2302433 | Qualcomm Incorporated | Observation self-interference framework for FR1: Table-1 shows that it is feasible to meet the 1 dB sensitivity degradation for FR1 SBFD-capable BS considering the different self-interference mitigation stages (i.e., spatial isolation, frequency isolation, beam nulling, and digital cancellation). Observation self-interference framework for FR2: Table-2 shows that it is feasible to meet the 1 dB sensitivity degradation for FR1 SBFD-capable BS considering the different self-interference mitigation stages (i.e., spatial isolation, frequency isolation, beam nulling, and digital cancellation). Observation FR1 BS NF: For FR1 BS NF, beyond the total input power value of -52 dBm, the AGC impacts are observed and lead to increase in the BS noise figure. Proposal FR1 BS IIP3 model: For FR1 BS IM3 model, RAN4 to adopt a Pin-dependent (average total input power) piecewise linear model as shown in the Figure above to characterizes IIP3. The proposed model captures IM3 contributions and AGC impact on IIP3 which can be utilized by RAN4 to progress the SBFD feasibility work. Observation FR1 BS IM3: The IM3 contributions are not significant when the total input power signal + jammer is lower than -52 dBm. Observation: For FR1 BS, other distortions such as ADC quantization noise and distortions were considered in our simulation and measurements, and it was observed that ADC performance is not limiting. Similarly, phase noise and residual sideband are not significant contributors Observation FR1 BS RX subband filtering: Enhanced Rx selectivity can be realized in a SBFD-capable BS visa subband analog filtering. Proposal FR1 BS maximum blocking level: We are OK with -43 dBm Proposal FR2 BS NF: For FR2-1 BS NF = 5 dB up to -52 dBm Pin, and then a sloped section rising 1 dB/dB with Pin > -52 dBm. Proposal FR2 BS interference model with co-channel jammer: FR2 BS interference can be modelled as a fixed level of interference 34 dB below the total input power. Proposal: For co-channel co-site inter-sector inter-gNB CLI, RAN4 to reuse the self-interference analysis framework with revisited mitigation capabilities if found necessary. Proposal: For co-channel co-site inter-sector inter-gNB CLI, at least similar or improved spatial isolation compared to the self-interference framework (e.g., by means of additional electromagnetic absorbers between the different sectors or radiation mask) should be considered to provide sufficient inter-gNB CLI mitigation. Proposal: For co-channel inter-site inter-gNB CLI, RAN4 to reuse existing BS ACLR and ACS to model inter-gNB CLI in the feasibility study as well as the adjacent channel coexistence study. Proposal: RAN4 to adopt baseline ACS requirements given in TS 38.104, which are 46 dB and 24 dB for FR1 and FR2, respectively for all base station classes (i.e., wide area, medium range, and local area gNBs). Proposal: RAN4 to investigate the applicability of RAN4 baseline ACLR and ACS for the inter-subband leakage and inter-subband selectivity depending on the findings in the adjacent channel coexistence work. |
| R4-2302728 | Intel Corporation | Observation 1: Beam nulling can be of use to intermittently reduce CLI from other base-stations, but it is not reasonable to tie up MIMO layers and beam steering capabilities full-time to reduce always on self-interference. Observation 2: While academic literature shows Rx digital self-interference cancellation values exceeding 30dB, these are primarily the results of single Tx to single Rx scenarios. For large MIMO arrays we see the achievable Rx digital self-interference cancellation as somewhat lower. Observation 3: The value of Rx IIP3 seen at the LNA input comes from the numerous trade-offs between gain, noise and IIP3 for all the Rx components that make up the Rx chain. In order to achieve a feasible Rx IIP3, some re-design of the Rx chain to enable higher IIP3 may be required. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: BS Aspects: Self-interference model

#### Issue 1-1-1: Assumption on site deployment aspects

- Proposals:

  - Proposal 1 (ZTE): to consider the clutter impact on digital IC in RSIC performance if needed.

  - Observation 1 (Nokia): To alleviate the rooftop deployment clutter, the different gNB sectors may need to be installed on separate poles at the opposite corners:

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-1-2: Impact of multi-carrier support at BS

- Proposals from Nokia:

  - Observation 1 (Nokia): Such an implementation (sub-band filtering after downconversion) would not be compatible with multi-carrier support, as the receiver would only be able to receive the UL sub-band during the SBFD slots.

  - Observation 2 (Ericsson): The BS is usually a multi-carrier node by default and multi-carrier aspects affect many related feasibility aspects such as improved linearization, CFR, filtering, PIM, beam nulling and digital interference cancellation. Thus, feasibility study assuming single carrier operation for BS is not sufficient.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 1-2: RSIC capability summary and overall feasibility analysis

#### Issue 1-2-1: Residual Self-Interference Cancellation (RSIC) Analysis Framework

- Proposals on how to interpret RSIC analysis framework from Samsung:

  - Observation 1 (Samsung): Before digital IC, the residual self-interference comes from two sources:

  - PSI_leakage, Interference leakage in gNB RX subband due to non-ideal TX, which can be derived as dBPSI_leakage=dBPTX-dBαSI-frequency, TX-dBαSI-spatial-dBαSI-beam, TX-dBαSI-RFIC,TX = ①-②-③-④-⑤ dBm , with the reference point being set at RX antenna.

  - PSI_Selectivity, Interference in gNB RX subband caused by non-ideal RX selectivity, which can be derived as dBPSI_Selectivity=dBPTX-dBαSI-spatial-dBαSI-beam,RX-dBαSI-RFIC,RX-dBαSI-frequency, RX=①-③-⑨-⑧-⑥ dBm, which is the gain-normalized value with reference point being set at RX antenna.

  - Observation 2 (Samsung): Before digital IC, the residual self-interference in RX subband is PSI=PSI_leakage+PSI_Selectivity.

  - Observation 3 (Samsung): After digital IC, the residual self-interference in the RX subband can be derived as

dBPSIleakage+PSISelectivity-dBαSI-digtial=①-③-dB11linear②×linear④xlinear⑤+1linear⑨×linear⑧×linear⑥-⑦  dBm.

  - Proposal 1 (Samsung): Based on component capabilities in the analysis framework, the overall RSIC capability can be derived as

dBRSIC=dBαSI-spatial+dB11linear②×linear④xlinear⑤+1linear⑨×linear⑧×linear⑥+dBαSI-digtial

=③+dB11linear②×linear④xlinear⑤+1linear⑨×linear⑧×linear⑥+⑦ dBc.

- Proposal on analysis framework refinement from Kumu:

  - Proposal 2 (Kumu): Overall RSIC reporting should include pre-LNA RF mitigation as a separate capability. For example, total RSIC can be given by:

    - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-RFIC)+dB(αSI-digtial)+…

      - αSI-spatial denotes the spatial isolation.

      - αSI-frequency (n,m) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n.

      - αSI-beam denotes the beam-nulling or beam isolation.

      - αSI-RFIC denotes the pre-LNA RF interference cancellation capability.

      - αSI-digtial denotes the digital cancellation capability.

  - Proposal 3 (Kumu): When evaluating self-interference cancellation methodology, it is useful to also report the achievable beam-forming gain.

  - Proposal 4 (Kumu): When reporting simulation results, it would be useful to clarify if it is the worst case or typical case being reported.

- Observations on analysis framework from Ericsson:

  - The RSIC provides a snapshot breakdown of the self-interference for a certain gNB output power and does not generalize to the gNB operating at other power levels.

  - The components of the RSIC are not independent of one another; changing some assumptions for one component can change other components

  - Antenna isolation varies with beam direction (and varies between different RX sub-arrays). The RSIC values in this section are a snapshot with average spatial isolation per RX sub-array. With some directions the suppression may be better, for other directions worse (for WA and MR).

- Proposals on related TR drafting:

  - Proposal 5 (Samsung): RSIC analysis framework shall be adopted for SBFD BS RF feasibility study to be captured in TR38.858, and subsection for different component capabilities can be reserved to encourage companies’ inputs.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-2-2: RSIC capability for FR1 WA-BS

- Proposals/Observations:

Table FR1 RSIC budget calculation Summary for WA-BS

| FR1 |  |  |  | Qualcomm | Intel | Samsung | Nokia | Ericsson | Huawei |  | Kumu |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Wide Area BS | Wide Area BS | Wide Area BS | Wide Area BS | Wide Area BS example 1 | Wide Area BS example 2 | Wide Area BS (High) | Wide Area BS (Mean) |
| BS TX Power dBPTX = ① dBm |  |  |  | [49] dBm | 49 dBm | 49 dBm | 54 dBm | 53 dBm | 47 | 53 | 53 dBm | 53 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 | 45 | 45 dB | 45 |
|  |  | Frequency isolation techniques used |  | DPD or sub-band filtering |  | DPD utilized | Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion | Digital filtering, CFR, DPD | DPD | DPD |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 70 dBc | 80 dBc | 65 dBc | 70 dBc | 80 | 80 | 65 dB | 65 |
|  |  | Spatial isolation techniques used |  | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation |  | TX/RX panel separation and RF barrier structure | Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels | A combination of spatial isolation, chokes, absorption, mushroom EBG. Note that 65dB is an “average” and the exact value depends on TX and RX beam steering direcitons, varying between 55dB and 80dB. | spatial separation between TX/RX panel | spatial separation between TX panel to single RX |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 15dBc | 0 dBc | 5 dBc | 5-10 dBc | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. | 10 （Beam nulling /isolation in TX sub-band） | 10 （Beam nulling /isolation in TX sub-band） | 17.87 dB | 21.3 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  | Limited, ~0dB | 1 dB maximum | 3-5dB, depending on TX beam direction | Less than 0.5 dB loss | Less than 0.5 dB loss |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -91 dBm (=①-②-③-④) | = 49 – 45 -70 = -66dBm | -81 dBm Note: provided by ①-②-③-④ dBm | -62 dBm/20 MHz | -72 dBm | =-94 | -88 | -57 dBm ①-②-③ Tx beam nulling is not helping in Rx subband because of random phases in non-linearirites from DPD | -57 dBm ①-②-③ Tx beam nulling is not helping in Rx subband because of random phases in non-linearirites from DPD |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  |  | 0 dB | 25 dBc | 0 dBc | 0 dBc | N/A | N/A | 15.39 dB | 16.6 |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  |  | 11 dB | 0dBc | 0 dBc | 0 dBc | N/A | 10 | 26.03 dB | 26.2 |
|  |  | RF IC techniques and other tech. (before LNA) |  |  |  | subband filtering | None apply due to feasibility concerns | Analogue interference cancellation incurs RX sensitivity loss due to insertion and also severe limitations on sub-band pre-coding and multi-carrier. Also, high routing complexity with large number of TX and RX. Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Insertion loss would degrade sensitivity. | N/A | Analog filter is put after LNA |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | - | 1dB | TBA | N/A dBc | >=5dBc if e.g. filtering or analogue IC would be applied. | N/A |  | 0.2 dB (assuming 15 dB Rx coupler) | 0.2 |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | x dBc (①-③-④-⑤) | = 49-70-11 = -32dBm | -61 dBm Note: provided by ①-③-⑨-⑤dBm | -21 dBm to -16 dBm depending on TX beam | -27 dBm | -43 | -37 (Equivalent to -47 when 10 dB filtering is counted.) | <-45.26 dBm | -49.9 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 15dBc | 45 dBc | 40 dBc | 0 dBc | xxx dBc | 80 | digital filtering | 0 dB | 0 |
|  |  | Frequency isolation techniques |  | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). |  | Filtering | None apply due to feasibility concerns | The RX input level is -27 dBm, and hence the receiver is blocked; no possibility for interference mitigation as part of the digital receive combining algorithms. Filtering after the LNA reduces insertion loss but does not suppress the RX level sufficiently prior to the filter. | digital filtering | digital filtering |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | Not a significant contributor on the gNb Rx capability. When the total input power (Pin) (signal + jammer) is lower than -52 dBm, IM3 contribution is not significant (see Section 3.12 for more details). | -15 dBm | -20dBm | -10 dBm at maximum sensitivity; +10 dBm at maximum linearity (at NF penalty) | -32dBm (Minimum for RAN4 requirement) -22dBm (Realistic for AAS) -10dBm (optimistic for AAS) | -10 | -10 | -20 | -20 |
|  |  |  | Rx IM3 contribution (dBm) |  | -66 dBm | -143dBm | -87 dBm/20 MHz to -74 dBm/20 MHz depending on the TX beam Assuming sufficient isolation at a given TX power, IM3 contribution should be minor in the input range of interest, if using front-end design with suitable gain control elements close to the antenna (NF penalty) | Note: ADC will be overloaded unless AGC is used (which would significantly increase noise figure) or sufficient filtering prior to ADC. Even without ADC overload: -17 dBm (RAN4 minimum receiver) -37 dBm (Realistic) -61 dBm (Optimistic) | -109 | -121 | -95.78 | -109.7 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Increase in Noise figure when total input power (Pin) exceeds -52 dBm. Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model as show Section 3.1.1. |  | N/A | At an RX input signal level of -21 dBm to -16 dBm, the NF would increase to 22 dB to 27 dB. NF penalty is due to gain control at input levels above -45 dBm RMS; phase noise is minor at 3.5 GHz | ADC will be overloaded unless AGC used (which would significantly increase noise figure) or sufficient filtering prior to ADC. | ADC noise: -109 reciprocal phase noise mixing:-112 | ADC noise: -113 reciprocal phase noise mixing:-116 | 5 dB (noise figure) | 5 |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | - (①-③-④-⑤-⑥) | =-66dBm + (-32-45) = -65.6 dBm | -101 dBm Note: provided by ①-③-⑨-⑤-⑥dBm | TBD dBm due to spillover in FFT, if lacking sufficient digital filtering (the self-interference signal in the example is -21 dBm to -16 dBm) | Receiver saturated | =-105 | -111 | -95.78 dBm | -109.7 |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | - | 0 dBc | 5 dBc | 0 dBc | RX processing does not mitigate saturation | 10 | 10 | 12.28 dB | 17.5 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | - | 0 dBc | Limited, ~0dB | 0 dBc; should not assume further UL beamforming loss to maintain any UL gains | Receiver saturated | Less than 0.5 dB loss | Less than 0.5 dB loss | TBD (?) | TBD |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc | 25 dBc | 20 dBc | 0 dBc | Digital IC not possible due to receiver saturation and would anyhow be highly complex due to large number of TX/RX for wide area. | 15 | 15 | 0 dB | 0 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 155 dBc (②+③+④+⑦) | =-65.6dBm – 25dB = -90.6 dBm, (49- -90.6dBm) = 139.6 dB | 150.0 dBc | 115 dBc to 120 dBc for TX sub-band 110 dBc for RX sub-band | Transmitter: 125 dB Receiver: N/A due to receiver saturation | 150 | 155 | 148.31 dB | 153.7 |
| Noise floor ⑩dBm |  |  |  | -96 dBm/20 MHz @ 5dB noise figure | xxx dBm/CBW | -95dBm/20MHz | -96 dBm/CBW (20 MHz) | -96 dBm/CBW | -96 dBm/20 MHz | -96 dBm/20 MHz | -89 dBm/100MHz | -89 dBm/100MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -102 dBm | xxx dBm | -101 dBm | -102 dBm | -102 dBm | -102 dBm | -102 dBm | -95 dBm | -95 |
| Required RSIC budget (①-⑪dBc) |  |  |  | 151 dBc | xxx dBc | 150 dBc | 156 dBc | 155 dBc | 149 | 155 | 148 dB | 148 |
| SBFD configuration |  |  |  | DUD |  | DUD(40-20-40MHz) | DUD (40/20/40 MHz) | See Annex. 40-20-40 MHz | [40, 20, 40] | [40, 20, 40] |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRBs |  | 5 PRB | 5 RB (1.8 MHz) | See Annex. 5 PRB. | Existing SU | Existing SU+ additionally few RBs |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |  | 20MHz |  | >300 MHz |  |  |  |  |
| Others |  |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-2-3: RSIC capability for FR1 MR-BS

- Proposals/Observations:

Table FR1 RSIC budget calculation Summary for MR-BS

| FR1 |  |  |  | Samsung | ZTE | Qualcomm | Ericsson |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Medium Range BS | Medium Range BS | Medium Range BS | Medium Range (3GPP minimum requirements) | Medium range (Realistic) | Medium Range (Optimistic RX) | Medium Range (Realistic, lower power) |
| BS TX Power dBPTX = ① dBm |  |  |  | 38 dBm | 31dBm for 100MHz DL and 30dBm for 80MHz DL | 38 dBm | 38 dBm | 38 dBm | 38 dBm | 35 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45dBc+10*log10(80/20)=51dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD | DPD or sub-band filtering | Digital filtering, CFR, DPD |  |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 50-60 | 80 dBc | 65-70 dBc | 65-70 dBc | 65-70 dBc | 65-70 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | Spatial separation between TX/RX | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation | A combination of spatial isolation, chokes, absorption, mushroom EBG. Note that 65dB is an “average” and the exact value depends on TX and RX beam steering directions, varying between 55dB and 80dB. |  |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 dBc | N/A | 10 dBc | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. |  |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB | N/A | - | 3-5dB, depending on TX beam direction |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -92 dBm Note: provided by ①-②-③-④ dBm | 30dBm-51(ACLR)-50(antenna isolation)-30(digital cancellation/sub-band filtering)-10*log10(20*10) =-124.01dBm/100kHz 30dBm-51(ACLR)-60(antenna isolation)-20(digital cancellation/sub-band filtering)-10*log10(20*10) =-124.01dBm/100kHz Note 1: the digital IC and sub-band filter in the transmitter could achieve the same purpose for suppress the transmitter leakage into its receiver Note 2: with increasing antenna isolation, then sub-band filter attenuation or digital IC could be reduced as well. | -97 dBm (=①-②-③-④) | -87 dBm | -87 dBm | - 87 dBm | -92 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 25 dBc | N/A |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | 30dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | subband filtering | analog filter for Rx sub-band |  | Analogue interference cancellation incurs RX sensitivity loss due to insertion and also severe limitations on sub-band pre-coding and multi-carrier. Also, high routing complexity with large number of TX and RX. Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Insertion loss would degrade sensitivity. |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | TBA |  |  | >=5dBc if e.g. filtering or analogue IC would be applied. |  |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -72 dBm Note: provided by ①-③-⑨-⑤dBm | 30dBm-50(antenna isolation)-30 (sub-band filter) = -50dBm equal to 50dBm for ACS requirement and -38dBm for IBB requirements of 20MHz reception |  | -42 dBm | -42 dBm | -42 dBm | -47 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 25 dBc | 46dBc for ACS requirement | 15dBc | xxx dBc |  |  |  |
|  |  | Frequency isolation techniques |  | Filtering | 30dBc for analog filter of Rx receiver | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. For RX, the 3rd column represents improved receiver linearity in the analogue domain. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. This assumption should be reviewed by RAN1. |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm | TBD | Not a significant contributor on the gNb Rx capability. When the total input power (Pin) (signal + jammer) is lower than -52 dBm, IM3 contribution is not significant (see Section 3.12 for more details). | -27.6 dBm | -17.6 dBm | -13 dBm | -17.6 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -176dBm | =3*Pinf-2*IIP3 |  | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |  | Increase in Noise figure when total input power (Pin) exceeds -52 dBm. Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model as show Section 3.1.1. | No significant issues for medium range BS power level other than mentioned above. Phase noise reciprocal mixing is not significant for this frequency range and power levels. |  |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -97 dBm Note: provided by ①-③-⑨-⑤-⑥dBm | 30dBm-50(antenna isolation)-30(sub-band filter)-46dB (ACS)-10*log10(20*10) = -118.01dBm/100kHz 31dBm-60(antenna isolation)-30(sub-band filter)-46dB (ACS)-10*log10(20*10) = -128.01dBm/100kHz | - (①-③-④-⑤-⑥) | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc | N/A | - | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | N/A | - | TBC dBc |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 20 dBc | 20 dBc | 10 dBc | 10-15 dBc (Transmitter) |  |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 148.8 dBc | [120.9dBss] | 145 dBc (②+③+④+⑦) | 109 dBc | 128 dBc | 135 dBc | 135 dBc |
| Noise floor ⑩dBm |  |  |  | -90dBm/20MHz |  | -91 dBm/20 MHz @ 10 dB NF | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -96 dBm | -174dBm/Hz+10*log10(100*10^3)+10dB-6dB= -120dBm/100kHz | -97 dBm | -96 dBm | -96 dBm | -96 dBm | -96 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 134 dBc | 30dBm-10*log10(80*10)-(-120dBm/100kHz) = 120.9dB for single antenna 36dBm-10*log10(80*10)-(-120dBm/100kHz) = 126.9dB for four antenna | 135 dBc | 134 dBc | 134 dBc | 134 dBc | 131 dBc |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | 80MHz DL, 20MHz | DUD | 40-20-40, see annex |  |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB | Left up to the implementation. This highly depend on how to achieve the ACS performance and Q factor of analog filter if needed | 5 PRBs | 5 PRB, see annex |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | 20MHz | 80MHz DL and 20MHz UL | 100MHz | <300MHz |  |  |  |
| Others |  |  |  |  |  |  | Preliminary estimates due to lack of final conclusion on RX ACS and scattering effects. |  |  |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-2-4: RSIC capability for FR1 LA-BS

- Proposals/Observations:

Table FR1 RSIC budget calculation Summary for LA-BS

| FR1 |  |  |  | Samsung | ZTE | Ericsson (preliminary) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Local Area BS | Local Area BS | Local Area BS (3GPP minimum) | Local Area BS (Realistic RX) |
| BS TX Power dBPTX = ① dBm |  |  |  | 24 dBm | 24 for 100MHz DL and 23.0dBm for SBFD DL | 24 dBm | 24 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45dBc+10*log10(80/20)=51dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD | Digital filtering, CFR, DPD | Digital filtering, CFR, DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 50 | 70 dBc | 70 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | Spatial separation between TX/RX | Physical distance, isolation structures | Physical distance, isolation structures |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 dBc | N/A | 0 dBc | 0 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB |  | TX beam nulling not assumed due to array size | TX beam nulling not assumed due to array size |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -106 dBm Note: provided by ①-②-③-④ dBm | 23dBm-51(ACLR)-50(antenna isolation)-20(digital cancellation)-10*log10(20*10) =-121.01/100kHz | -91 dBm | -91 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 25 dBc | N/A | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | N/A | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | subband filtering | N/A | Analogue IC could be considered for this case, but is restrictive on pre-coding and multi-carrier. Digital IC has instead been assumed. |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | TBA |  | 0 dBc | 0 dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -86 dBm Note: provided by ①-③-⑨-⑤dBm | 23dBm-50(antenna isolation)=-27dBm Which is a bit higher than in-band blocking requirement of LA -35dBm | -46 dBm | -46 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 20 dBc | 46dBc | xxx dBc | xxx dBc |
|  |  | Frequency isolation techniques |  | N/A | N/A | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. This assumption should be reviewed by RAN1. |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm | -2dBm | -24.6 dBm | -14 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -218dBm | 3*(-35)-2(-2)-10*log10(20*10)=-124.01dBm/100kHz | -88.8 dBm | -110 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |  | No other significant impacts other than those mentioned above |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -106 dBm Note: provided by ①-③-⑨-⑤-⑥dBm | 23dBm-50(antenna isolation)-46dB (ACS)-10*log10(20*10)-20= -116.01dBm/100kHz | -88.8 dBm | -110 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc | N/A | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | N/A | TBC dBc |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 20 dBc | 20dBc | 15 dBc on transmitter | 15 dBc on transmitter |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 147.0 dBc | 110.9dB | 112 dBc | 124 dBc |
| Noise floor ⑩dBm |  |  |  | -87dBm/20MHz |  | -87 dBm/CBW | -87 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93dBm | -174dBm/Hz+10*log10(100*10^3)+13dB-6dB= -117dBm/100kHz | -93 dBm | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 117 dBc | 23dBm-10*log10(80*10)-(-117dBm/100kHz) = 110.9dB for single antenna 29dBm-10*log10(80*10)-(-117dBm/100kHz) = 116.9dB for four antenna | 117 dBc | 117 dBc |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | 80MHz DL, 20MHz | 40-20-40, see annex |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB | Left up to the implementation. This highly depend on how to achieve the ACS performance and Q factor of analog filter if needed | 5 PRB, see annex |  |
| bandwidth over which suppression is achieved |  |  |  | 20MHz | 80MHz DL and 20MHz UL | <300MHz |  |
| Others |  |  |  |  |  | Preliminary estimates due to lack of final conclusion on RX ACS and scattering effects. |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-2-5: RSIC capability for FR2 BS

- Proposals/Observations:

Table: FR2-1 RSIC budget calculation Summary

| FR2-1 |  |  |  | Qualcomm | Samsung | China Telecom |  | Ericsson |  |  | Huawei |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | FR2-1 BS | Case I | Case II | 40 dBm TRP | 30 dBm TRP | 24 dBm TRP | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm | 30 dBm | 30 dBm@FR2 | 30 dBm@FR2 | 40 dBm | 30 dBm | 24 dBm | 35 dBm/200MHz |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc |
|  |  | Frequency isolation techniques used |  | DPD or sub-band filtering | Without DPD | Filtering, CFR | N\A | Filtering, CFR |  |  | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 85-95 dBc | 87 dBc | 80 dBc | 80 dBc | 80 dBc | 80 dBc | 80 dBc | 85~95 dBc |
|  |  | Spatial isolation techniques used |  | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation | TX/RX panel separation and RF barrier structure | A combination of spatial isolation, chokes, absorption, mushroom EBG. | spatial separation between TX/RX panel; | A combination of spatial isolation, chokes, absorption, mushroom EBG. |  |  | spatial separation between TX/RX panel |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5-10 dBc | 10 dBc | 0dBc | 5 dBc | 5 dBc | 5 dBc | 5 dBc | 10 dBc （Beam nulling /isolation in TX sub-band） |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | - | Limited, ~0dB | <1.5dB | TBC | TBC | TBC | TBC | Less than 0.5 dB loss |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -88 dBm (=①-②-③-④) | -95 dBm Note: provided by ①-②-③-④ dBm | −100dBm | -73 dBm | -73 dBm | -83 dBm | -89 dBm | -94~-104 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | - | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | - | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | - | Not applicable |  |  |  |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | - | No impact | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | - (①-③-④-⑤) | -67 dBm Note: provided by ①-③-⑨-⑤dBm | −70dBm | -45 dBm | -45 dBm | -55 dBm | -61 dBc | -60 ~ -70 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 15 dBc | 24 dBc |  |  |  |  |  |  |
|  |  | Frequency isolation techniques |  | Filtering | Filtering |  |  |  |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | Similar conclusion as FR1 (i.e., IIP3 and IM3 are not dominant) following the same analysis that was conducted for FR1 (Section 3.1.2). | -35 dBm | -35 dBm | -35 dBm | -35 dBm | -35 dBm | -35 dBm |  |
|  |  |  | Rx IM3 contribution (dBm) |  | -65 dBm | - 95 dBm | -113 dBm | -65 dBm | - 95 dBm | -113 dBm | negligible |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Noise figure degradation for FR2 is given in Section 3.2. | Reciprocal phase noise mixing will add noise at around -95dBm. | Reciprocal phase noise mixing will add noise at around -105dBm | Reciprocal phase noise mixing will add noise at around -110dBm | Reciprocal phase noise mixing will add noise at around -95dBm. | Reciprocal phase noise mixing will add noise at around -105dBm | Reciprocal phase noise mixing will add noise at around -110dBm | negligible |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | - (①-③-④-⑤-⑥) | -91 dBm Note: provided by ①-③-⑨-⑤-⑥dBm | −90dBm | -65 dBm | -65 dBm | -94.6 dBm | -108 dBm | negligible |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | - | 10 dBc | 0dBc | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  | 10 dB （Beam nulling /isolation in RX sub-band） |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | - | Limited, ~0dB | <1dB | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  | Less than 0.5 dB loss |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 dBc | 15 dBc | 0 dBc | 10 dBc | 10 dBc | 10 dBc | 10 dBc |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 128 dBc (②+③+④+⑦) | 134.5 dBc | 120 dBc | 130 dBc | 105 dBc | 120,7dBc | 122.5 dBc | 129~139 dBc |
| Noise floor ⑩dBm |  |  |  | -88 dBm/40 MHz @ 10dB noise figure | -83 dBm/100MHz | −83dBm/100MHz | −83dBm/100MHz | -87 dBm/CBW | -87 dBm/CBW | -87 dBm/CBW | -88 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -94 dBm | -89 dBm | −89 dBm | −89 dBm | -93 dBm | -93 dBm | -93 dBm | -94 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 124 dBc | 119 dBc | 119 dBc | 119 dBc | 133 dBc | 123 dBc | 117 dBc | 129 dBc |
| SBFD configuration |  |  |  | DUD | DU (100MHz-100MHz) |  |  | 75-50-75 (See Annex) |  |  | 75-50-75 (See Annex) |
| Guardband assumption (if exist) |  |  |  | 5 PRBs | 5PRB |  |  | 3 RB (See Annex) |  |  | 3 RB (See Annex) |
| bandwidth over which suppression is achieved |  |  |  | 200MHz | 100MHz | 100MHz | 100MHz | Several GHz |  |  | 200MHz |
| Others |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-2-6: Other inputs

- Proposals/Observations from Charter Communications/Cable Labs:

- Note: it is expected the proponents of above proposal can be incorporated into the analysis framework agreed in the last meeting.

### Sub-topic 1-3: RSIC Component Capability Analysis

#### Issue 1-3-1: TX/RX Beam nulling

- Observation from Intel:

  - Observation 1 (Intel): Beam nulling can be of use to intermittently reduce CLI from other base-stations, but it is not reasonable to tie up MIMO layers and beam steering capabilities full-time to reduce always on self-interference.

- Observation from Ericsson:

  - Observation 1 (Ericsson): The gain from beam nulling increases when the TX beam is steered and the antenna isolation decreases. Thus, beam nulling can to some extent reduce the variation of the overall spatial isolation due to beam steering. It may also reduce the frequency variation. However, with increasing steering, the cost in DL of beam nulling increases.

  - Observation 2 (Ericsson): The cost of beam nulling in downlink can be substantial; we have observed up to 5dB DL power loss. There may be further DL losses due to lower degrees of freedom for MIMO operation.

  - Observation 3 (Ericsson): When deciding beam nulling gains, downlink impacts should be taken into account.

  - Observation 4 (Ericsson): RX beam nulling is part of the receiver SINR optimization in the receive baseband.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-3-2: RX combining

- Proposal from Ericsson:

  - Proposal 1 (Ericsson): RAN4 should discuss and identify appropriate scenarios for assessing RX combining algorithm performance, including all sources of interference.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-3-3: RX sub-band selectivity

- Observation from Qualcomm:

  - Observation 1 (Qualcomm): FR1 BS RX subband filtering: Enhanced Rx selectivity can be realized in a SBFD-capable BS visa subband analog filtering.

- Observation from Ericsson:

  - Observation 2 (Ericsson): An RF filtering solution prior to the LNA for a single carrier, custom designed BS for a specific carrier with little DUD configuration flexibility would incur an NF increase of up to 6dB (with questionable feasibility due to filter size)

  - Observation 3 (Ericsson): A reasonable RF filtering solution prior to the LNA is not feasible.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-3-4: RF SIC

- Proposals/Observations from Kumu on RF SIC:

  - Observation 1 (Kumu): Using 3GPP_38.901_Uma_LOS channel, we show that having RF cancellation before the receiver LNA have the benefits of achieving the desired self-interference cancellation residue floor as well as preventing saturation of the Rx LNAs. When considering the viability of SBFD, RF cancellation plays a critical part and should be considered in the evaluation of overall RSIC capability.

  - Proposal 1 (Kumu): RF cancellation should be used in SBFD to mitigate self-interference pre Rx LNA in terms of minimizing non-linearity effects and overall self-interference residue.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-3-5: Digital IC

- Observations from Intel:

  - Observation 1 (Intel): While academic literature shows Rx digital self-interference cancellation values exceeding 30dB, these are primarily the results of single Tx to single Rx scenarios.  For large MIMO arrays we see the achievable Rx digital self-interference cancellation as somewhat lower.

- Observations from Ericsson

  - The IC gain depends on the interference structure

  - The interference suppression depends on the interference power (INR)

  - The interference suppression has dependencies on the TX power, antenna isolation and frequency isolation.

  - A number of implementation issues can impact the level of interference suppression, such as modelling of receiver behavior, non-linearities and filtering and TX-RX timing alignment.

  - The interference suppression for reflections depends on the number of taps provided, the coherence time and channel estimation.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-3-6: Overall Desense

- Observations/Proposals from Kumu:

  - Observation 1 (Kumu): Using 3GPP_38.901_Uma_LOS channel, we show that the worst case rx sensitivity per Rx link is 0.9 dB and the mean case rx sensitivity per Rx link is 0.6 dB.

  - Proposal 1 (Kumu): We support using overall Rx sensitivity degradation of 1 dB for SBFD system simulation and evaluation.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-3-7: Desense allocation between TX and RX (based on 1dB desense)

- Proposals from CATT:

  - Observation 1: The Tx leakage and Rx contribution requirements to the SI can be divided to -105dBm each if each of them contributes to half of noise.

  - Observation 2: The overall RSIC requirement for Tx leakage should be at least 148 dB for WA SBFD BS

  - Observation 3: The RSIC requirement for Rx blocking issue should be 89 dB for WA SBFD BS.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 1-4: RSI dependency on blocking, AGC and ADC

#### Issue 1-4-1: Assumption for input power metric to LNA

- Proposals:

  - Proposal 1 (Nokia): Use RMS average power for the saturation, non-linearity, and AGC models in the feasibility study and system level simulations. In aspects requiring more precise understanding of the signal peaks, a reasonable RX PAPR (at least 10 dB) should be added to the RMS power.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-4-2: Analysis on LNA non-linearity and blocking level

- Observation and proposals on LNA non-linearity:

  - Observation 1 (Samsung): Simply following existing RAN4 requirement to derive the required LNA IIP3 metrics and accordingly the feasibility conclusion of SBFD is very pessimistic assumption.

  - Observation 2 (Intel): The value of Rx IIP3 seen at the LNA input comes from the numerous trade-offs between gain, noise and IIP3 for all the Rx components that make up the Rx chain.  In order to achieve a feasible Rx IIP3, some re-design of the Rx chain to enable higher IIP3 may be required.

  - Proposal 1 (Qualcomm): FR1 BS IIP3 model: For FR1 BS IM3 model, RAN4 to adopt a Pin-dependent (average total input power) piecewise linear model as shown in the Figure above to characterizes IIP3. The proposed model captures IM3 contributions and AGC impact on IIP3 which can be utilized by RAN4 to progress the SBFD feasibility work.

  - Observation 3 (Qualcomm): FR1 BS IM3: The IM3 contributions are not significant when the total input power signal + jammer is lower than -52 dBm.

  - Observation 4 (CATT): If IM3 contribution is -108dBm for WA BS SBFD Rx noise, IIP3 should be at least-10.5 dBm in whole operation temperature range, which is very challenging.

  - Observation 5 (Ericsson) A reasonable RF filtering solution prior to the LNA is not feasible

- Observation and proposals on blocking level:

  - Proposal 1 (ZTE): to consider in-band blocking level as baseline for gNB receiver saturation, non-linearity and higher capability should depend on vendors’ declaration.

  - Proposal 2 (Qualcomm): FR1 BS maximum blocking level: We are OK with -43 dBm+

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-4-3: Analysis on AGC and NF model

- Proposals from Qualcomm:

  - Proposal 1 (Qualcomm): FR1 BS NF model: For FR1 BS NF, RAN4 to adopt a noise figure model (shown below) that is dependent on the total average (not peak as in [1]) input power (signal + jammer) with a piecewise linear model. The NF is 5 dB below -52 dBm, and 25.2 dB above -21 dBm. In between there is a linear slope.

  - Observation 1 (Qualcomm): FR1 BS NF: For FR1 BS NF, beyond the total input power value of -52 dBm, the AGC impacts are observed and lead to increase in the BS noise figure.

  - Proposal 2 (Qualcomm): FR2 BS NF:  For FR2-1 BS NF = 5 dB up to -52 dBm Pin, and then a sloped section rising 1 dB/dB with Pin > -52 dBm.

  - Observation 2 (CATT): AGC is not need to be analysed for BS Rx path because ADC dynamic is not a problem.

  - Proposal 3 (Nokia): The AGC model captures impacts of receiver non-linearity, gain control, and ADC dynamic range, and the following parameters are used for wide area base stations:

| Snf | Small signal noise figure | 5 | dB |
| --- | --- | --- | --- |
| a | Input power threshold 1 (RMS) | -44 | dBm |
| b | Input power threshold 2 (RMS) | -38.5 | dBm |
| SL1 | Noise figure slope 1 | 0.364 |  |
| SL2 | Noise figure slope 2 | 0.778 |  |

  - Ericsson: Adopt the piecewise linear model of figure/table 1 for FR1.

| Parameter | Value |
| --- | --- |
| Noise figure (F) | 5 dB |
| Total input average power threshold 1 (P1) | -57.9 dBm |
| Total input average power threshold 2 (P2) | -41.7 dBm |
| Sensitivity degradation slope 1 (k1) | 0.4 dB/dBm |
| Sensitivity degradation slope 2 (k2) | 3.0 dB/dBm |
| Total input average power blocking limit (P3) | -32.5 dBm |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-4-4: Analysis on other distortions (Phase noise, ADC quantization noise, Residual sideband, ADC distortions)

- Observations:

  - Observation 1 (Qualcomm): For FR1 BS, other distortions such as ADC quantization noise and distortions were considered in our simulation and measurements, and it was observed that ADC performance is not limiting. Similarly, phase noise and residual sideband are not significant contributors

  - Observation 2 (Samsung): According to calculation presented in Table 5, the requested ADC dynamic range is still within the range of commercialized available component.

  - Observation 3 (CATT): If Rx blocking level is assumed to be the current WA BS requirement, ADC dynamic range can cover the whole signal level including wanted signal and interference signals.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-4-5: Interference from co-channel jammer for FR2

- Observation from Qualcomm:

  - Proposal FR2 BS interference model with co-channel jammer: FR2 BS interference can be modelled as a fixed level of interference 34 dB below the total input power.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 1-5: BS Aspects: Co-channel co-site gNB-gNB CLI model

#### Issue 1-5-1: Sensitivity degradation for feasibility criteria

- Proposals/Observations:

  - Proposal 1 (CMCC): for inter-sector interference feasibility criteria, it’s suggested to use SLS to analysis minimum inter-sector interference cancellation ratio to avoid blocking.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-5-2: Antenna Isolation

- Proposals/Observations from Samsung:

  - Observation 1 (Samsung): The RF simulation has shown that numerical analysis on the antennal isolation for co-channel co-site inter-sector gNB-gNB CLI:

    - In the range of 62-93dB, depending on different antenna pair and co/cross-polarization

    - Note: the results are obtained for the 3-sector scenario at 3.5GHz with detailed parameters provided.

  - Observation 2 (Samsung): Antennal isolation for co-channel co-site inter-sector gNB-gNB CLI can be further improved to the range of [90-100]dB, by having the methods e.g., installing EM conjugated structure between sectors, larger horizontal distance, or vertical antenna arrangement, or different boresight angle directions, or different electrical tilts or combination thereof.

- Proposals/Observations from Qualcomm:

  - Proposal 1 (QC): For co-channel co-site inter-sector inter-gNB CLI, at least similar or improved spatial isolation compared to the self-interference framework (e.g., by means of additional electromagnetic absorbers between the different sectors or radiation mask) should be considered to provide sufficient inter-gNB CLI mitigation.

- Proposal from Ericsson:

  - Observation 3 (Ericsson): The simulation results in figure 2.4.2-2 indicate that the panel-panel isolation for the 3-GHz Wide Area system in varies from 55 – 70dB (as discussed in [2]).

  - Observation 4 (Ericsson): The simulation results for the 30-GHz systems are reported in Figures 2.4.2-5 and 2.4.2-7. For 100 mm edge-to-edge distance in azimuth direction, the best achievable panel-panel isolation is 75 dB while the worst is 60 dB.

- Proposal from Nokia:

  - Proposal 2 (Nokia): The spatial isolation for co-channel inter-sector case is 60 dB for FR1 wide area base stations.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-5-3: Digital IC

- Proposals/Observations:

  - Option 1: Digital IC can be applied for co-site inter-sector case.

  - Option 2: No digital IC can be applied for co-site inter-sector case.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-5-4: Analysis Framework

- Proposals/Observations:

  - Proposal 1 (Qualcomm, Nokia, Samsung): For co-channel co-site inter-sector inter-gNB CLI, RAN4 to reuse the self-interference analysis framework with revisited mitigation capabilities if found necessary.

  - Proposal 1a (Samsung): Based on the analysis framework for self-interference, the below table is proposed for analysis for co-channel co-site inter-sector gNB-gNB CLI modeling, with different fields highlighted as below:

- Table 4: Analysis framework for  co-channel co-site inter-sector gNB-gNB CLI

| FR1 (or FR2-1) |  |  |  | Company-A |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | xxx dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | xxx dBc |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | xxx dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | xxx dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  |
|  |  |  | Rx IM3 contribution (dBm) |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | xxx dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | xxx dBc |
| Noise floor ⑩dBm |  |  |  | xxx dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | xxx dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | xxx dBc |
| SBFD configuration |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 1-6: BS Aspects: Co-channel inter-site gNB-gNB CLI model

#### Issue 1-6-1: RX selectivity level

- Proposals/Observations for improved receiver impairment performance compared to gNB ACS:

  - Observation 1 (CMCC): commercial gNB could at least achieve 50dB Rx selectivity.

  - Proposal 1 (CMCC): it’s suggested to at least use 50dB for inter-site Rx selectivity.

  - Proposal 2 (QC): RAN4 to adopt baseline ACS requirements given in TS 38.104, which are 46 dB and 24 dB for FR1 and FR2, respectively for all base station classes (i.e., wide area, medium range, and local area gNBs).

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-6-2: How to determine the enhancement over baseline (ACLR and ACS)

- Proposals/Observations:

  - Proposal 1 (QC): For co-channel inter-site inter-gNB CLI, RAN4 to reuse existing BS ACLR and ACS to model inter-gNB CLI in the feasibility study as well as the adjacent channel coexistence study.

  - Proposal 2 (QC): RAN4 to investigate the applicability of RAN4 baseline ACLR and ACS for the inter-subband leakage and inter-subband selectivity depending on the findings in the adjacent channel coexistence work.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 1-7: BS Aspects: RF requirement impact

Sub-topic description:

[Moderator] In last meeting, BS RF requirement impact has been initially discussed, with the following agreements in WF [R4-2220244]:

Agreement: - In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity: - Option 1: No such requirement needed. RAN4 consider the SBFD performance requirement for receiver sensitivity with the simultaneous TX in the SBFD time slot, in which the in-channel adjacent subblock leakage ratio and in-channel adjacent subblock blocking requirements can be guaranteed implicitly. - Option 2: New requirements are needed for In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking + Adjacent subband selectivity. - OTA sensitivity: - New requirements are needed - ACLR, ACS, in-band blocking, intermodulation: - FFS. - Other requirements not precluded

#### Issue 1-7-1: Potentially impacted RF requirement for SBFD capable gNB

- Proposals/Observations:

  - Observation 1 (Samsung): It is difficult for RAN4 to agree on a single RF architecture to derive the potential new requirements for (1) in-channel adjacent subblock leakage ratio (new), (2) in-channel adjacent subblock blocking (new) and (3) in-channel adjacent subband selectivity.

  - Proposal 1 (Samsung): RAN4 consider the SBFD performance requirement for receiver sensitivity with the simultaneous TX in the SBFD time slot, in which the in-channel adjacent subblock leakage ratio, and in-channel adjacent subblock blocking and in-channel adjacent subband selectivity requirements can be guaranteed implicitly while no explicit requirement needed.

  - Proposal 2 (CATT): New RF requirements for SBFD listed in Table 7 should be defined.

Table 7: New RF requirements analysis results

In-channel adjacent subband leakage ratio

In-channel adjacent subband Blocking + Adjacent subband selectivity

In-channel Receiver intermodulation

In-channel operating subband unwanted emission

  - Proposal 3 (Nokia): Further study is needed to conclude that no new requirements are needed for In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity.

  - Proposal 4 (Nokia): gNB OTA sensitivity shall be relaxed for SBFD gNB. The DL signal shall be active in the test.

  - Proposal 5 (Nokia): gNB OTA adjacent channel selectivity, in-band blocking, and receiver intermodulation tests shall have the DL signal active. For out-of-band blocking, it is FFS whether to activate the DL signal.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 1-7-2: Existing requirement without impact for SBFD capable gNB

- Proposals/Observations from CATT:

  - Proposal 1: Transmit ON/OFF power requirement is not applicable for SBFD BS.

  - Observation 2: Coupling loss for co-location minimum requirements needs to be changed. REFSENS impact can be discussed later when SI, CLI analysis is clear and decided.

  - Proposal 2: OTA receiver spurious emissions can’t be defined for SBFD BS.

  - Observation 3: Co-location assumption for transmitter intermodulation and OTA transmitter intermodulation need to be further studied for SBFD BS.

  - Proposal 3: Existing requirements listed in Table 8 are unchanged for SBFD BS.

Table 8: Existing requirements would be remained unchanged

|  | Requirements |
| --- | --- |
| Conducted RF requirement | Adjacent Channel Leakage power Ratio (ACLR) |
|  | Operating band unwanted emissions |
|  | Transmitter spurious emissions |
|  | Adjacent channel selectivity (ACS) |
|  | In-channel selectivity |
|  | Receiver intermodulation |
| Radiated RF requirement Radiated RF requirement | OTA Adjacent Channel Leakage Power Ratio (ACLR) |
|  | OTA operating band unwanted emissions |
|  | OTA transmitter spurious emission |
|  | OTA Adjacent channel selectivity (ACS) |
|  | OTA in-channel selectivity |
|  | OTA receiver intermodulation |

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 1-8: BS Aspects: OBUE performance impact

#### Issue 1-8-1: OBUE performance impact

- Proposals/Observations from CATT:

  - OBUE performance impact should be discussed and decided for Tx leakage analysis.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

# Topic #2: Feasibility study and RF impact from UE aspects

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300266 | Apple | Proposal 1: It is proposed to use a simple model to combine the above two aspects and make some common assumptions for further evaluation. Proposal 2: Any proposal to improve UE RF capabilities should be postponed to R19 or future releases. Proposal 3: If RAN4 continues to pursue this AGC model, further discussion is needed to revise it. |
| R4-2300446 | Samsung | Observation 1: The definition of sub-band selectivity needs to be clarified firstly in RAN4. Proposal 1: Sub-band selectivity is the ratio of the receive filter attenuation on the assigned sub-band to the receive filter attenuation on the adjacent sub-band. Proposal 2: For the verification/evaluation of X dBc sub-band selectivity, test parameters are provided as below table (comparable to the test parameters for ACS). Table 2. Test parameter example for the verification/evaluation of X dBc sub-band selectivity RX parameter; Units; Sub-band Channel bandwidth (MHz). BWSB_Channel = 10, 20, 40, 60, 80 ... (candidate DL subband BW smaller than channel BW). Power in transmission bandwidth configuration; dBm; REFSENS + 14 dB. Pinterferer; dBm; REFSENS + 14 dB + X dB - 1.5 dB. BWinterferer; MHz; BWinterferer = 20MHz, 40MHz ... (candidate UL subband BW smaller than channel BW). Finterferer (offset); MHz; (BWSB_Channel + BWinterferer)/2 + Guardband / - (BWSB_Channel + BWinterferer)/2 - Guardband. timing or/and frequency offset; TBD e.g., yy us timing advance for interference signal ahead of DL subband signal. Observation 2: There are several factors which could impact UE receiver sub-band selectivity performance: (1) non-linearity in RF components; (2) ADC saturation, and (3) spectral leakage by FFT block. Observation 3: From theoretical analysis, for legacy UE with normal FFT operation, at least 30dB subband selectivity can be achieved at the outermost DL subcarrier, if 12 subcarrier guardband is assumed. Observation 4: UE receiver sub-band selectivity can be further improved with the FFT operating on the DL subband. Proposal 3: RAN4 shall not introduce new RAN4 requirement for sub-band selectivity for legacy UE, which is not capable of SBFD operation. Proposal 4: For SBFD-capable UE, whether or not new sub-band selectivity requirement shall be introduced can be further discussed in Rel-18 duplex evolution study item, and will be decided in the normative work item phase if any. |
| R4-2300544 | Ericsson | Observation 1: Frequency and time synchronization errors are not the primary factors influencing UE-UE interference for FR1. Observation 2: Frequency and time synchronization errors are not the primary factors influencing UE-UE interference for FR2. Proposal 1: Propose to use 33 dB as the in-band selectivity in FR1 Proposal 2: Propose to use 23 dB as the in-band selectivity in FR2 |
| R4-2300557 | CATT | For UE adjacent channel AGC and NF, Observation 1: UE ACS performance already considers the NF and filter contribution. Proposal 1: No need to model AGC and NF for UE Rx adjacent channel model when ACS performance is assumed. For UE co-channel Rx model, Observation 2: No Rx subband filter being assumed for UE is more reasonable for SBFD analysis. Observation 3: 6 dB power imbalance for 64 QAM wanted signal is defined in DEMOD requirement, which leads to 25 dBc ICS performance for UE. Proposal 2: -25 dBc ICS performance can be assumed for UE co-channel Rx model. Proposal 3: If -25 dBc ICS performance is agreed as the co-channel UE-UE CLI Rx model, UE NF change related to AGC is not necessary to be modelled. |
| R4-2300692 | Nokia, Nokia Shanghai Bell | Proposal 1: Typical performance for at least the cases listed above need to be understood and known in order to predict UE and system performance related to SBFD. Proposal 2: Requirements for “In-channel selectivity” for UE’s should be part of RAN4 discussions related to SBFD capable UE’s. Observation 1: In channel selectivity is specified in 38.104, [3] chapter 7.8; however, such test case is not mentioned in [2], (UE RF spec.). The meaning of “in channel” is adjacent RB’s not overlapping as might be implied by “Co-channel”. |
| R4-2300794 | CMCC | Observation 1: from RAN4 simulation perspective, we don’t need to model the case that allocation less than full UL sub-band. Observation 2: the probability of larger than -25dBm UE-UE inter-sub band interference is very near to zero. It could be assumed that legacy ADC dynamic range is enough to maintain constant NF for UE-UE inter-sub band interference case. Proposal 1: 9dB and 10dB constant NF assumption are still suggested for FR1 and FR2-1 SBFD inter-sub band UE-UE interference analysis. Observation 3: if there is no ICS requirement, it’s impossible to configure aggressor UE and victim UE in the same cell if victim UE only allow 1dB REFSENSE degradation. |
| R4-2301427 | Huawei, HiSilicon | Observation 1: at least for DUD configuration, the IQ image also can be ignored. Observation 2: the receiver’s effective noise figure as a function of the input power can be modelled for co-channel sub-band receiver. Meanwhile it is open to discuss a simplified model. Observation 3: At current stage, it is not necessary to model allocations that are less than fully allocated uplink sub-bands. We can revisit the discussion on 2 steps model if UE-UE CLI becomes significant. |
| R4-2301534 | vivo | Proposal 1: Apply existing UR RF requirements for both legacy and SBFD-aware UEs. Observation 1: The value 25dB co-channel selectivity is even higher than ACS for FR2. Proposal 2: For receiver sub-band selectivity, consider co-channel selectivity for FR1 and FR2 separately, or a lower level, such as 20 dB for both FR1 and FR2. Proposal 3: No need to apply the selectivity and performance of the FFT for adjacent channel case. |
| R4-2301610 | MediaTek Inc. | Observation 1: Realistic frequency and timing offsets need to be considered when analyzing the ability of FFT to perform rejection of interference. Observation 2: The maximum frequency offset could be 1.2kHz for FR1 and 10.52kHz for FR2-1, plus any doppler due to mobility. Observation 3: Timing mismatch will depend on the fixed timing offset between UL and DL (as defined in TS 38.133) and also the timing advance applied by the transmitting UE. Observation 4: the STO has largest impact compared to the “relevant” CFO levels. Observation 5: IBE should largely dominate over FFT leakage, with some guardband required, 1 RB if assuming 3GPP IBE QPSK requirements and a larger guardband for other modulation schemes. A real UE may or may not have improved IBE relative to the 3GPP specification. Proposal 1: Inform RAN1 that the agreed IBE model actually dominates with respect to “selectivity due to FFT rejection”, assuming some guardband depending on the modulation scheme and possibly UE implementation. Proposal 2: Use the simplified AGC model with a fixed noise figure only for the purpose of system level simulation for SBFD. For any other consideration, a more representative AGC modelling corresponding to real implementation would need to be considered. Proposal 3: Do not consider improving selectivity requirements for Rx sub-bands. |
| R4-2301735 | ZTE Corporation | Proposal 1: for EVM requirement in the IBE mode, propose to consider it based on the following approach: the received SINR->CQI-> MCS->Modulation order; Proposal 2: from UE receiver perspective, it’s proposed to only consider CLI of co-channel inter sub-band with its arrival timing beyond the CP of wanted signal of victim UE or when victim UE’s received signal including both co-channel inter sub-band signals and wanted signals beyond its maximum input power. Proposal 3: from UE receiver perspective, to set one check point for its received signal including both co-channel inter sub-band signals and wanted signals beyond its maximum input power, if it’s above the maximum input power, then throughput of UE could be set as 0 with some performance degradation. Proposal 4: X and Y should be equal to the FFT selectivity. |
| R4-2302434 | Qualcomm Incorporated | Proposal channel configuration: RAN4 to focus it’s effort on legacy UE, i.e. a UE that does not configure BW to the sub-band BW. Proposal FR1 UE noise figure value: Use 7 dB typical NF for FR1 Proposal FR2-1 UE noise figure value: Use 7.5 dB typical NF for FR2-1 Proposal FR1 interference from co-channel jammer (typical UE): Model as Pin - 33 dB Proposal FR2-1 interference from co-channel jammer (typical UE) : Pinterference_co-channel_FR2-1 = Pin – (34 dB + 10*log10(max(1,BWinterference /BWvictim_subband))) Proposal FR2-1 interference from adjacent channel jammer(typical UE) : Pinterference_adjacent_channel_FR2-1 = Pin – (34 dB + 10*log10(max(1,BWinterference /BWvictim_subband))) Proposal FFT leakage: We recommend ignoring the FFT leakage interference since IBE-based interference is stronger and dominates. Proposal typical FR1 ACLR mode for SBFD sims: UE ACLR is modeled as 30 dB at max power, and improves 1dB/dB with backoff up to a maximum 10 dB of improvement. So this means at 10 dB backoff the ACLR is 40 dB. Proposal typical FR2-1 ACLR mode for SBFD sims: 24 dB based value improved 1 dB/dB for up to 10 dB, similar approach as FR1. Proposal confirmation for use of FR2-1 IBE model: IBE model to be used for FR2-1 UE. Proposal FR1 IBE interference : For DUD configuration use -29 dB as the interference in each downlink sub-band. Proposal FR2-1 IBE interference : For DUD configuration use -25 dB as the interference in each downlink sub-band. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: Sub-band selectivity modeling for RX Modeling

#### Issue 2-1-1: General aspects for sub-band selectivity

- Proposals on general aspects:

  - Proposal 1 (Apple): It is proposed to use a simple model to combine the above two aspects (subband selectivity due to FFT operation at the RX, and RX modelling) and make some common assumptions for further evaluation.

- Proposals on definition of subband selectivity:

  - Proposal 2 (Samsung): Sub-band selectivity is the ratio of the receive filter attenuation on the assigned sub-band to the receive filter attenuation on the adjacent sub-band.

  - Proposal 3 (Samsung): For the verification/evaluation of X dBc sub-band selectivity, test parameters are provided as below table (comparable to the test parameters for ACS).

Table. Test parameter example for the verification/evaluation of X dBc sub-band selectivity

| RX parameter | Units | Sub-band Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | BWSB_Channel = 10, 20, 40, 60, 80 ... (candidate DL subband BW smaller than channel BW) |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 14 dB + X dB - 1.5 dB |
| BWinterferer | MHz | BWinterferer = 20MHz, 40MHz ... (candidate UL subband BW smaller than channel BW) |
| Finterferer (offset) | MHz | (BWSB_Channel + BWinterferer)/2 + Guardband / - (BWSB_Channel + BWinterferer)/2 - Guardband |
| timing or/and frequency offset |  | TBD e.g., yy us timing advance for interference signal ahead of DL subband signal |

  - Observation 1 (CMCC): if there is no ICS requirement, it’s impossible to configure aggressor UE and victim UE in the same cell if victim UE only allow 1dB REFSENSE degradation.

- Proposals on how to proceed the discussion on sub-band selectivity

  - Observation 2 (MediaTek): IBE should largely dominate over FFT leakage, with some guardband required, 1 RB if assuming 3GPP IBE QPSK requirements and a larger guardband for other modulation schemes. A real UE may or may not have improved IBE relative to the 3GPP specification.

  - Proposal 4 (MediaTek, Qualcomm): Inform RAN1 that the agreed IBE model actually dominates with respect to “selectivity due to FFT rejection”, assuming some guardband depending on the modulation scheme and possibly UE implementation.

  - Proposal 5 (ZTE): to further evaluate the FFT selectivity based on gap between FFT PSD and its practical PSD of wanted signal.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 2-1-2: Sub-band selectivity performance level for legacy UE

- [Moderator] Pls. note this issue is about the achievable receiver sub-band selectivity performance for legacy UE, i.e., based on current UE implementation without additional optimization.

- Proposals/observations on assumptions for sub-band filtering:

  - Observation 1 (CATT): No Rx subband filter being assumed for UE is more reasonable for SBFD analysis.

- Proposals/observations on frequency/time offset:

  - Observation 2 (MediaTek): The maximum frequency offset could be 1.2kHz for FR1 and 10.52kHz for FR2-1, plus any doppler due to mobility.

  - Observation 3 (MediaTek): Timing mismatch will depend on the fixed timing offset between UL and DL (as defined in TS 38.133) and also the timing advance applied by the transmitting UE.

  - Observation 4 (MediaTek): the STO has largest impact compared to the “relevant” CFO levels.

  - Proposal 1 (ZTE): Regarding timing/freq offset for adjacent channel scenario, not consider it and follow the legacy approach.

  - Observation 5 (Ericsson): Frequency and time synchronization errors are not the primary factors influencing UE-UE interference for FR1 and FR2.

- Proposals on factors impacting sub-band selectivity:

  - Observation 6 (Samsung): There are several factors which could impact UE receiver sub-band selectivity performance: (1) non-linearity in RF components; (2) ADC saturation, and (3) spectral leakage by FFT block.

  - Proposal 2 (ZTE): X and Y should be equal to the FFT selectivity. (Note: X and Y values for the agreed clarification on co-channel RX model).

- Proposals/observations on sub-band selectivity performance:

  - Proposal 3 (Samsung): From theoretical analysis, for legacy UE with normal FFT operation, at least 30dB subband selectivity can be achieved at the outermost DL subcarrier, if 12 subcarrier guardband is assumed.

  - Proposal 4 (CATT): -25 dBc ICS performance can be assumed for UE co-channel Rx model.

  - Proposal 5 (Nokia): Requirements for “In-channel selectivity” for UE’s should be part of RAN4 discussions related to SBFD capable UE’s.

  - Proposal 6 (vivo): For receiver sub-band selectivity, consider co-channel selectivity for FR1 and FR2 separately, or a lower level, such as 20 dB for both FR1 and FR2.

  - Proposal 7 (Ericsson): 33 dB as the in-band selectivity in FR1; 23 dB as the in-band selectivity in FR2.

  - Proposal 8 (Qualcomm):

    - FR1 interference from co-channel jammer (typical UE): Model as Pin - 33 dB

    - FR2-1 interference from co-channel jammer (typical UE):

      - Pinterference_co-channel_FR2-1 = Pin – (34 dB + 10*log10(max(1,BWinterference /BWvictim_subband)))

    - FR2-1 interference from adjacent channel jammer(typical UE):

      - Pinterference_adjacent_channel_FR2-1 = Pin – (34 dB + 10*log10(max(1,BWinterference /BWvictim_subband)))

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 2-1-3: Sub-band selectivity requirement for legacy UE

- Proposals/observations:

  - Proposal 1 (Samsung, vivo): RAN4 shall not introduce new RAN4 requirement for sub-band selectivity for legacy UE, which is not capable of SBFD operation.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 2-1-4: Sub-band selectivity performance level for SBFD-capable UE

- Proposals/Observations:

  - Proposal 1 (Apple): Any proposal to improve UE RF capabilities should be postponed to R19 or future releases.

  - Observation 1 (Samsung): UE receiver sub-band selectivity can be further improved with the FFT operating on the DL subband.

  - Proposal 2 (MediaTek): Do not consider improving selectivity requirements for Rx sub-bands.

  - Proposal 3 (Qualcomm): RAN4 to focus it’s effort on legacy UE, i.e. a UE that does not configure BW to the sub-band BW.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 2-1-5: Sub-band selectivity requirement for SBFD-capable UE

- Proposals/Observations:

  - Proposal 1 (Samsung): For SBFD-capable UE, whether or not new sub-band selectivity requirement shall be introduced can be further discussed in Rel-18 duplex evolution study item, and will be decided in the normative work item phase if any.

  - Proposal 2 (Nokia): Requirements for “In-channel selectivity” for UE’s should be part of RAN4 discussions related to SBFD capable UE’s.

  - Proposal 3 (vivo): Apply existing UR RF requirements for SBFD-aware UEs.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 2-2: AGC and NF for RX Modelling

#### Issue 2-2-1: AGC and NF modelling for co-channel CLI

- General proposal:

  - Proposal 1 (Apple): If RAN4 continues to pursue this AGC model, further discussion is needed to revise it.

- Proposals to agree on a constant NF model:

  - Proposal 2 (CATT): No need to model AGC and NF for UE Rx adjacent channel model when ACS performance is assumed.

  - Proposal 3 (CATT): If -25 dBc ICS performance is agreed as the co-channel UE-UE CLI Rx model, UE NF change related to AGC is not necessary to be modelled.

  - Proposal 4 (CMCC): 9dB and 10dB constant NF assumption are still suggested for FR1 and FR2-1 SBFD inter-sub band UE-UE interference analysis.

  - Proposal 5 (MediaTek): Use the simplified AGC model with a fixed noise figure only for the purpose of system level simulation for SBFD. For any other consideration, a more representative AGC modelling corresponding to real implementation would need to be considered.

  - Proposal 6 (Qualcomm): Use 7 dB typical NF for FR1; Use 7.5 dB typical NF for FR2-1.

- Proposals for NF modelling containing AGC:

  - Observation 1 (Huawei): the receiver’s effective noise figure as a function of the input power can be modelled for co-channel sub-band receiver. Meanwhile it is open to discuss a simplified model.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 2-2-2: AGC and NF modelling for adjacent-channel CLI

- Proposals to agree on a constant NF model:

  - Proposal 1 (CATT): No need to model AGC and NF for UE Rx adjacent channel model when ACS performance is assumed.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 2-3: Remaining issues on TX modeling:

#### Issue 2-3-1: IQ image contribution for IBE model (co-channel)

- [Moderator] Pls. note similar discussion is given in Sub-topic 3-3 for the response to Agreement-3 in the RAN1 LS.

- Proposals/Observations:

  - Observation 1 (Huawei): at least for DUD configuration, the IQ image also can be ignored.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 2-3-2: Non-fully allocated UL subband for TX modelling (co-channel)

- Proposal:

  - Observation 1 (Huawei): At current stage, it is not necessary to model allocations that are less than fully allocated uplink sub-bands. We can revisit the discussion on 2 steps model if UE-UE CLI becomes significant.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 2-3-3: Improved TX modeling (co-channel)

- Proposal:

  - Proposal 1 (Qualcomm): FR1 IBE interference: For DUD configuration use -29 dB as the interference in each downlink sub-band.

  - Proposal 2 (Qualcomm): Proposal FR2-1 IBE interference : For DUD configuration use -25 dB as the interference in each downlink sub-band.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 2-3-4: Improved TX modeling (adjacent channel)

- Proposal:

  - Proposal 1 (Qualcomm): typical FR1 ACLR mode for SBFD sims:  UE ACLR is modeled as 30 dB at max power, and improves 1dB/dB with backoff up to a maximum 10 dB of improvement. So this means at 10 dB backoff the ACLR is 40 dB.

  - Proposal 2 (Qualcomm): typical FR2-1 ACLR mode for SBFD sims: 24 dB based value improved 1 dB/dB for up to 10 dB, similar approach as FR1.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 2-4: Other aspects

#### Issue 2-4-1: Applying selectivity and performance of FFT for adjacent channel case

- Proposal:

  - Proposal 1 (vivo, ZTE): No need to apply the selectivity and performance of the FFT for adjacent channel case.

- Recommended WF

  - Seems the understanding of applying ACS for adjacent channel case already means P1 is taken into account. With the conclusion for adjacent channel case, seems no need to discuss this issue.

# Topic #3: Reply LS to RAN1

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300558 | CATT | Answer to the question 1: There’s no blocking issue for inter-site SBFD gNB. RAN1 ICS understanding is not correct. RAN4 assumes ACLR performance for Tx sub-band signal and ACS performance for Rx sub-band that sub-band filters are assumed for Tx/Rx, which is different with ICS performance where there’s no in channel filter. So the interference falling into the victim gNB should be calculated according to ACIR value. Answer to the question 2: RAN1 consideration for the co-site inter-sector co-channel inter-subband CLI modelling is not complete. The blocking issue should be considered and the contribution to interference power is not complete either. RAN1 can use the following assumption which is used in RAN4 co-existence simulation calibration. The assumption can be updated when RAN4 updates the evaluation results. SBFD Co-site co-channel interference consideration Using following assumption for calibration purpose: - For co-site self-interference scenario, it is assumed the interference level from gNB self-interference is: Noise floor – 6dB. - For co-site inter-sector scenario, it is assumed the interference level from co-site inter-sector gNBs is: Noise floor + X dB  For medium and local BS: X = -6 dB;  For wide-area BS: X = -6 dB.  Note 1: this is the sum of all inter-sector gNB-gNB CLI per site.  Note 2: Final co-existence study simulation set-up need to be aligned with the conclusion on co-site inter-sector interference modelling and isolation.  Note 3: for FR1 wide-area, this means the inter-sector isolation should be not less than [144dB] Answer to the question 3: RAN4 has agreed that IBE-based model as mentioned in LS R4-2220243, please RAN1 use that model in the simulation. |
| R4-2300689 | Nokia, Nokia Shanghai Bell | Observation 1: With respect to the selectivity modelling in Agreement-1 of the RAN1 LS R1-2212963, the interference is calculated per UL RB and it depends solely on the average power spectral density in DL RBs, while the total power of the (aggressor) DL signal, the configuration (DL signal only on one or on both sides) and the difference between the DL and UL subband size do not play a role in the model. This assumption does not seem correct as the aggressor bandwidth, its total power and the arrangement (DU or DUD) have a strong impact on the receiver performance. Proposal 1: Regarding the question to RAN4 in Agreement-1 of the RAN1 LS R1-2212963 on the modelling of in-channel selectivity (ICS): - RAN4 to communicate to RAN1 that UL reception impairment due to receiver selectivity consists of a combination of linear and non linear effects: - Linear effect is primarily due to crosstalk effect between subcarriers which is dependent on the filtering and on how much OFDM symbol orthogonality there is between the aggressor/interfering and the wanted signal. Note that this interference effect is dependent on the frequency separation between aggressor and victim subcarriers. - Non-linear effect is due to receiver densensitization caused by blocking in the receiver. - For the linear component, the RAN1 model in agreement-1 above could be applied as a simplification in the absence of a frequency-dependent model. - RAN4 to inform RAN1 that the impairment due to selectivity is dependent on the total power and the bandwidth occupied by the aggressor signal, as well as the position of the DL and UL subbands (e.g. DUD vs DU). - As a starting point, for the non-linear component, RAN4 to communicate to RAN1 that the gNB UL reception impairments due to receiver selectivity and receiver blocking are not dependent on the total power received from one specific aggressor gNB (referred to as Pblocker in the RAN1 agreement), but on the total power passing the front-end analogue filter of the gNB receiver, including gNB-gNB co-channel and adjacent channel interference, self-interference, legacy co-channel and adjacent-channel UL interference, UL desired signal transmissions, as well as the signals from other networks in the same operating band. - RAN4 to discuss and agree on the modelling of receiver blocking effects which can then be forwarded to RAN1. - As one modelling approach, it can be assumed that the NF increases as a function of total received RMS input power in the gNB receiver Pblocker(j), e.g. using a piece-wise linear approximation with the parameters (a, b, SL1, SL2), where the first and the second input threshold are a and b, with the slopes SL1 and SL2, respectively. Example parameters of the model (a, b, SL1, SL2) for wide-area base stations are derived in [3] and summarized in Table 1 and can be further discussed by RAN4. Proposal 2: With regards to the RAN1 question on inter-sector co-site isolation: - RAN4 to discuss possible values for co-site inter-sector isolation. For FR1 sectorized deployments, a range between 55-70 dB can be assumed as the staring point. - For ACLRBS and ACSBS, the ACLR and ACS requirements defined by RAN4 can be used as starting point, e.g. 45 dB and 46 dB respectively for FR1. - On the definition of αco-site, RAN4 to inform RAN1 that the impairment due to selectivity is dependent on the total power as well as the size and the arrangement of the DL and UL subbands (e.g. DUD vs DU), while current agreement from RAN1 seems to assume that both DL and UL subband are of equal size. |
| R4-2301532 | vivo | For now, RAN4 did not consider ICS model as the BS co-channel Rx model. It was agreed to use BS ACS for co-channel Rx model. BS ACS can be considered as frequency flat model, 46 dBc for FR1 BS and 24 dBc for FR2 BS. This model is not dependent on Pblocker. A reply LS [2] was sent to RAN1 for interference modelling and RAN4 recommended: - Iselectivity can be obtained based on the RX power and the ACS. - RAN4 has not yet preclude further study on the possibility of improved receiver impairment performance compared to gNB ACS. RAN4 would like RAN1 to consider gNB ACS for co-channel Rx model. For now, spatial isolation is not concluded and it will be discussed in the RSIC analysis framework [3]. Once it is settled, RAN4 will send the value ranges for spatial isolation. For BS ACLR/ACS, existing BS RF requirements can be referenced: - FR1: BS ACLR (45 dB) BS ACS (46 dB) - FR2: BS ACLR (28 dB) BS ACS (23 dB) For now, RAN4 does not consider to simplify the UE IBE model as an equivalent frequency flat model. In the Reply LS [2], RAN4 informed RAN1 that: - The IBE-based model shall be used for TX modelling for UE-UE CLI for the co-channel case in RAN1 system-level simulation: - IBE models provided in clause 6.4.2.3 in TS38.101-1 and clause 6.4.2.3.4 in TS38.101-2 shall be followed. - The general and IQ Image part of in-band emission model shall be considered, while the carrier leakage part can be ignored. |
| R4-2301884 | Ericsson | Proposal 1 Calculate receiver power based on the total power due to self-interference, inter-site and inter-cell interference Proposal 2 Adopt the piecewise linear model of figure/table 1 for FR1 and table 2 for FR2. Proposal 3 In addition to the receiver blocking model, 10*log10ACLRBS and 10*log10ACSBS should be added for the inter-site and inter-sector interference (together with assumed desense due to the self-interference) Proposal 4 When replying on the value range for spatial isolation, capture also the assumptions on inter-sector distance and antenna structure for each art of the range. Proposal 5 FR1 range of inter-sector TX panel – RX sub-array isolation is 75-90dB, with 75dB being typical. Proposal 6 FR2 range of inter-sector TX panel – RX sub-array isolation is 75-98dB, with 88dB being typical. |
| R4-2301428 | Huawei, HiSilicon | Observation 1: For FR1 WA, ICS_BS can be modelled as below with ICS_1 and P_1/P_2 reported by companies ICSBS= ICS1, Pblocker≤P1 ICS1-2Pblocker-P1, P1< Pblcoker≤P2Saturated, Pblocker>P2 Observation 2: For FR1 WA, The achievable equivalent selectivity is much higher than 45 dB. Observation 3: For co-site inter-sector case better spatial isolation than RSI case is achievable. Observation 4: The values of ACLR_BS and ACS_BS for co-channel inter-sub-band should be the same for all the cases, e.g. co-site inter-sector and inter site gNB-gNB. Observation 5: the IQ image falls into the UL sub-band for DUD configuration hence IQ image also can be ignored. |
| R4-2300445 | Samsung | Response to RAN1 LS (R1-2212963): Proposal 5: RAN4 reply the following information related to Agreement-1: - RAN4 confirm RAN1’s understanding on ICS performance, and the following agreement has been agreed: Agreement: - For co-channel inter-site gNB-gNB CLI modeling, gNB ACS shall be used as baseline for system level simulation and feasibility study. - Further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.. Proposal 6: RAN4 reply the following information related to Agreement-2: - RAN4 confirm RAN1’s understanding on this model; - For co-site inter-sector co-channel inter-subband CLI modelling, the value range of spatial isolation in the range can be in the range of [90,100]dB, by having the methods e.g., installing EM conjugated structure between sectors, larger horizontal distance, or vertical antenna arrangement, or different boresight angle directions, or different electrical tilts or combination thereof. - For the values of ACLR and ACS for candidate for TX leakage and RX impairment respectively, the following agreement has been agreed in RAN4: Agreement on feasibility and how to model co-site inter-sector gNB-gNB CLI modelling: similar modelling as for self-interference (RSI) can be applied but may with different parameters especially on antenna isolation - FFS on possibility to apply digital IC for this case. Proposal 7: RAN4 reply the following information related to Agreement-3: - RAN4 confirm Option 1 is aligned with RAN4 agreement, and as confirmed in previous reply LS (R4-2220243), the IBE-based model shall be used for TX modelling for UE-UE CLI for the co-channel case in RAN1 system-level simulation: - IBE models provided in clause 6.4.2.3 in TS38.101-1 and clause 6.4.2.3.4 in TS38.101-2 shall be followed. - The general and IQ Image part of in-band emission model shall be considered, while the carrier leakage part can be ignored. |
| R4-2300266 | Apple | Proposal 4: It is preferred to follow the IBE requirement as defined in TS38.101-1 and TS38.101-2 if the effort to derive an equivalent frequency flat model is time-consuming. |
| R4-2302433 | Qualcomm Incorporated | Proposal reply LS: RAN4 to consider the draft LS in this paper. - Regarding RAN1 Agreement-1 in R1-2212963: - RAN4 confirms RAN1 modeling of the inter-site inter-gNB co-channel CLI considering the large scale and small-scale fading components. - RAN4 recommends that ICSBS to be modelled by a single value which equals the gNB ACS for the different gNB classes (i.e., 46 dB and 24 dB for FR1 and FR2, respectively). - RAN4 would like to point out that inter-subband selectivity and its enhancements should be considered for only SBFD-capable gNBs while legacy gNBs should not be impacted. - Regarding RAN1 Agreement-2 in R1-2212963: - RAN4 confirms RAN1 the co-site inter-sector co-channel inter-subband CLI modelling carried out in RAN1. - RAN4 has considered so far baseline values for ACLR and ACS (based on TS 38.104) to be reused for inter-subband co-channel CLI modeling. Additionally, RAN4 has not yet precluded possible improvements on receiver performance compared to baseline gNB ACS. The ACLR/ACS values for FR1 and FR2 are shown in the table below. Range; ACLR [dB]; ACS [dB]. FR-1; 45; 46. FR-2; 28; 24. - For spatial isolation values, RAN4 is still discussing whether similar spatial isolation capabilities as the self-interference framework should be considered for the co-site inter-sector or not. - Regarding RAN1 Agreement-3 in R1-2212963: - RAN4 recommends option 1. RAN4 has not yet converged on average IBEs for FR1 and FR2 value that can be assumed. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

[Moderator] In Nov. RAN1 meeting, LS R1-2212963 has been approved which contains four RAN1 agreement, and accordingly confirmation and reply from RAN4 are expected.

Open issues and candidate options before e-meeting:

### Sub-topic 3-1: Response to Agreement-1 in R1-2212963

[Moderator] The following Agreement-1 is contained in RAN1 LS.

Agreement-1 Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise - σz,n2=PblockerICSBS×NDLRB - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2 - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm, - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1, - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB. - NDLRB is the total number of DL RBs in the DL subbands, - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received. - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g., ICSBS= ICS1 Pblocker<P1 ICS2, P1< Pblcoker<P2ICS3 Pblocker>P2 - Note: Pblocker can be reported by companies

#### Issue 3-1-1: General modeling:

- Proposals on UL reception impairment due to receiver selectivity consists of a combination of linear and non linear effects:

  - Proposal 1 (Nokia):

    - RAN4 to communicate to RAN1 that UL reception impairment due to receiver selectivity consists of a combination of linear and non linear effects:

      - Linear effect is primarily due to crosstalk effect between subcarriers which is dependent on the filtering and on how much OFDM symbol orthogonality there is between the aggressor/interfering and the wanted signal. Note that this interference effect is dependent on the frequency separation between aggressor and victim subcarriers.

      - Non-linear effect is due to receiver densensitization caused by blocking in the receiver.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 3-1-2: Confirmation on gNB ACS value used for ICS

- Proposals on the confirmation of RAN1 understanding for the modelling of inter-site gNB-gNB co-channel inter-subband CLI

  - Option 1 (CATT, vivo): RAN1 ICS understanding is not correct.

  - Option 1a (CATT): Further clarification: RAN4 assumes ACLR performance for Tx sub-band signal and ACS performance for Rx sub-band that sub-band filters are assumed for Tx/Rx, which is different with ICS performance where there’s no in channel filter. So the interference falling into the victim gNB should be calculated according to ACIR value.

  - Option 2 (Samsung, Qualcomm): Confirm gNB ACS as co-channel RX modeling, based on the following agreement:

Agreement: - For co-channel inter-site gNB-gNB CLI modeling, gNB ACS shall be used as baseline for system level simulation and feasibility study. - Further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.

  - Option 3 (Nokia): RAN1 model in agreement-1 above could be applied as a simplification in the absence of a frequency-dependent model.

    - RAN4 to inform RAN1 that the impairment due to selectivity is dependent on the total power and the bandwidth occupied by the aggressor signal, as well as the position of the DL and UL subbands (e.g. DUD vs DU).

  - Option 4 (Huawei): For FR1 WA, the achievable equivalent selectivity is much higher than 45 dB.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 3-1-3: In-channel selectivity and noise modeling depending on blocker interference

- Proposals on whether ICSBS can be modelled depending on the value of the blocker interference:

  - Option 1 (CATT): No blocking issue for inter-site SBFD gNBs.

  - Option 2 (vivo, Samsung): The model is not dependent on P_blocker.

  - Option 3 (Huawei): For FR1 WA, ICS_BS can be modelled as below with ICS_1  and P_1/P_2 reported by companies

ICSBS= ICS1,            Pblocker≤P1 ICS1-2Pblocker-P1,   P1< Pblcoker≤P2Saturated,             Pblocker>P2

  - Option 4 (Nokia): RAN4 to communicate to RAN1 that the gNB UL reception impairments due to receiver selectivity and receiver blocking are not dependent on the total power received from one specific aggressor gNB (referred to as P_blocker in the RAN1 agreement), but on the total power passing the front-end analogue filter of the gNB receiver, including gNB-gNB co-channel and adjacent channel interference, self-interference, legacy co-channel and adjacent-channel UL interference, UL desired signal transmissions, as well as the signals from other networks in the same operating band.

- Proposals on the level of blocker(s) interference to be considered:

  - Option 1 (Nokia, Ericsson): Non-linear part depends on total power due to self-interference, inter-site and inter-cell interference

- Proposals/observation noise modeling based on input total power:

  - Proposal 1 (Ericsson): Adopt the piecewise linear model of figure/table 1 for FR1.

| Parameter | Value |
| --- | --- |
| Noise figure (F) | 5 dB |
| Total input average power threshold 1 (P1) | -57.9 dBm |
| Total input average power threshold 2 (P2) | -41.7 dBm |
| Sensitivity degradation slope 1 (k1) | 0.4 dB/dBm |
| Sensitivity degradation slope 2 (k2) | 3.0 dB/dBm |
| Total input average power blocking limit (P3) | -32.5 dBm |

  - Proposal 2 (Nokia): the NF increases as a function of total received RMS input power in the gNB receiver P_blocker^((j)), e.g. using a piece-wise linear approximation with the parameters (a, b, SL1, SL2):

| Snf | Small signal noise figure | 5 | dB |
| --- | --- | --- | --- |
| a | Input power threshold 1 (RMS) | -44 | dBm |
| b | Input power threshold 2 (RMS) | -38.5 | dBm |
| SL1 | Noise figure slope 1 | 0.364 |  |
| SL2 | Noise figure slope 2 | 0.778 |  |

  - Observation 1 (Ericsson): If 1dB desense is assumed to model self-interference, then the self-interference power input to the model should be the value assumed to get 1dB desense.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 3-2: Response to Agreement-2 in R1-2212963

[Moderator] The following Agreement-2 is contained in RAN1 LS.

Agreement-2 For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows. Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB - Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB. - Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x. - αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI. - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment. - Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived. - Send LS to RAN4 confirming the model and asking the value ranges for spatial isolation, and values of 10*log10ACLRBS and 10*log10ACSBS.

#### Issue 3-2-1: Range of spatial isolation for co-site inter-sector CLI modelling

- General proposal:

  - Proposal 1 (Ericsson): When replying on the value range for spatial isolation, capture also the assumptions on inter-sector distance and antenna structure for each art of the range.

  - Proposal 2 (Huawei): The values of ACLR_BS and ACS_BS for co-channel inter-sub-band should be the same for all the cases, e.g. co-site inter-sector and inter site gNB-gNB.

- Proposals for FR1:

  - Option 1 (Samsung): the value range of spatial isolation in the range can be in the range of [90,100] dB for FR1, by having the methods e.g., installing EM conjugated structure between sectors, larger horizontal distance, or vertical antenna arrangement, or different boresight angle directions, or different electrical tilts or combination thereof.

  - Option 2 (Ericsson): FR1 range of inter-sector TX panel – RX sub-array isolation is 75-90dB, with 75dB being typical, based on a separation of sectors of 400mm horizontal and 300mm vertical.

  - Option 3 (Nokia): For FR1 sectorized deployments, a range between 55-70 dB can be assumed as the starting point.

  - Option 4 (Huawei): For co-site inter-sector case better spatial isolation than RSI case is achievable.

- Proposals for FR2:

  - Option 1 (Ericsson): FR2 range of inter-sector TX panel – RX sub-array isolation is 75-98dB, with 88dB being typical based on an edge to edge separation of 400mm (horizontal and vertical).

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

#### Issue 3-2-2: TX leakage and RX impairment modeling for co-site inter-sector CLI

- Proposals on gNB ACLR and gNB ACS for TX leakage and RX impairment in the CLI modelling:

  - Option 1 (CATT): RAN1 consideration for the co-site inter-sector co-channel inter-subband CLI modelling is not complete. The blocking issue should be considered and the contribution to interference power is not complete either. RAN1 can use the following assumption and update when RAN4 updates the evaluation results.

    - SBFD co-site co-channel interference consideration

      - Using following assumption for calibration purpose:

      - For co-site self-interference scenario, it is assumed the interference level from gNB self-interference is: Noise floor – 6dB.

      - For co-site inter-sector scenario, it is assumed the interference level from co-site inter-sector gNBs is: Noise floor + X dB

        - For medium and local BS: X = -6 dB;

        - For wide-area BS: X = -6 dB.

        - Note 1: this is the sum of all inter-sector gNB-gNB CLI per site.

        - Note 2: Final co-existence study simulation set-up need to be aligned with the conclusion on co-site inter-sector interference modelling and isolation.

        - Note 3: for FR1 wide-area, this means the inter-sector isolation should be not less than [144dB]

  - Option 2 (vivo, Samsung, Qualcomm): Confirm the reuse of gNB ACLR/ACS.

  - Option 2a (Nokia): Confirm the reuse of gNB ACLR/ACS as starting point, e.g. 45 dB and 46 dB respectively for FR1.

    - On the definition of α_(co-site) , RAN4 to inform RAN1 that the impairment due to selectivity is dependent on the total power as well as the size and the arrangement of the DL and UL subbands (e.g. DUD vs DU), while current agreement from RAN1 seems to assume that both DL and UL subband are of equal size.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

### Sub-topic 3-3: Response to Agreement-3 in R1-2212963

[Moderator] The following Agreement-3 is contained in RAN1 LS.

Agreement-3 For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point. - Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point. - Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

#### Issue 3-3-1: Tx leakage model of UE-UE co-channel inter-subband CLI modelling

- Proposals

  - Option 1 (CATT, Samsung, vivo, Apple, Qualcomm, Huawei): RAN4 confirm RAN1’s Option 1 (IBE model) aligned with RAN4 agreement, already contained in RAN4 LS R4-2220243.

  - Option 1a (Huawei): In addition to Option 1, below information can be further replied with:

    - the IQ image falls into the UL sub-band for DUD configuration hence IQ image also can be ignored.

- Recommended WF

  - Further discussion on proposals/observations in 1st round.

# Topic #4: Regulatory survey

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300447 | Samsung | Proposal 1: it is proposed to approve the above-mentioned TP towards Chapter 11 of TR 38.585 |
| R4-2300693 | Nokia, Nokia Shanghai Bell | Proposal 1: The study on the regulatory aspects for deploying the identified duplex enhancements should at least include the TDD unpaired spectrum in the 3400-3800 MHz frequency range. Observation 1: In many CEPT countries, the same frame format is effectively mandated both indoor and outdoor in the 3400-3800 MHz frequency band. Proposal 2: The conclusions on the regulatory aspects of dynamic TDD are also applicable to SBFD. Proposal 3: It is proposed to approve TP to the TR 38.585 for clause 11. |
| R4-2301469 | Ericsson | Observation1: For a chosen TDD pattern, sub-band full duplex operation would increase the UL transmission in the network, increasing the level of UL interferences. Observation2: Regulators made coexistence studies assuming a certain DL/UL ratio. Any change in that ratio might have some impacts on the corresponding studies’ conclusion. Regulators might want to re-evaluate some existing coexistence studies done for TDD bands, releasing a new regulation to authorize SBFD deployment. Observation3: In Europe, operations in adjacent TDD spectrum are supposed to be synchronized. Observation4: In certain circumstances preventing any interference (e.g. factory indoor), it would be possible to deploy unsynchronized TDD network. Observation5: For some 5G bands, Regulators have considered unsynchronized (or semi-synchronized) TDD operation between adjacent operators by introducing more stringent parameters. Observation6: More stringent Regulatory requirements might impact BS feasibility, final cost, size and weight, especially if SBFD DL is considered during “legacy” UL slots. Observation7: In USA and Canada, Regulator has not mandated any TDD pattern but operators are strongly encouraged to coordinate their adjacent TDD networks to avoid any interference. Observation8: In USA and Canada, SBFD operation might be possible when deployed in environment preventing interference in the adjacent spectrum. Observation9: In China, to avoid interference, adjacent TDD networks are supposed to be synchronized using a same predefined TDD pattern. It should be expected MIIT will provide some guidance (e.g. some indoor deployment) when operating SBFD in adjacent spectrum. Observation10: In Japan, Regulator has not mandated any TDD pattern but operators are required to coordinate their adjacent TDD networks to avoid any interference. Observation11: In Japan, SBFD operation might be possible when deployed in environment preventing interference in the adjacent spectrum. And we make the following proposal: Proposal: Approve the TP to TR 38.858 proposed in Annex |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1: Text Proposal on regulatory aspects

Sub-topic description:

[Moderator] As one of the objectives in WID as below, RAN4 is tasked to summarize the regulatory aspects for deploying the duplex enhancement in TDD unpaired spectrum:

The detailed objectives are as follows: - ... - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In previous RAN4 meetings, the discussion on the survey of regulatory aspects for different regions was provided. In this meeting, the discussion continues.

Open issues and candidate options before e-meeting:

#### Issue 4-1-1: Text Proposal on regulatory aspects

- Proposals

  - Option 1 (Nokia, R4-2216204):

    - Outside Europe, national regulations do not appear to represent a major bottleneck for the deployment of SBFD in the 3400-3800 MHz range. For deployments within CEPT countries, SBFD may require changes to the current regulations. Therefore, it is envisioned that co-existence between SBFD and static TDD is thoroughly investigated in RAN4 so that performance results can be used as a basis for discussions with regulatory bodies defining harmonised standards at least for the 3400-3800MHz band.

  - Option 2 (Ericsson, R4-2301469):

    - Regulators always pay high attention to any new technology that might create interference to incumbent services operating in or adjacent to the considered spectrum, specifying new conditions to prevent any such interference.

    - When allocating spectrum to IMT TDD operation, Regulators made coexistence studies with incumbent services assuming a certain TDD pattern. Based on the conclusions of those studies, Regulators have then specified the corresponding specific parameters to enable such deployment.

    - In most of the countries, operators are expected to synchronize their adjacent TDD networks. Some Regulators have even recommended specific TDD frame structure usage to facilitate this, addressing then cross-border issues between countries (e.g. in Europe).

    - To enable unsynchronized TDD deployments without creating interference in the adjacent network(s), some Regulators have specified more stringent parameters (e.g. CEPT specified below and above the block edge a restricted baseline of -34dBm/5 MHz EIRP for non AAS BS or -43dBm/MHz TRP for AAS BS), increasing BS design’s complexity significantly.

    - Regulators might revise existing regulatory rules to allow SBFD operations and/or mandate more stringent requirements.

    - Nevertheless, when deployed in environments which guarantee and prevent any interference in the adjacent spectrum (like isolated indoor deployment), no specific condition nor recommendation have been specified by the Regulators, allowing any TDD deployment in such environments as long as no interference disturbs adjacent services. For such type of deployments, existing regulation rules should not be impacting when operating SBFD.

  - Samsung (R4-2300447):

11.1		Region 1

11.1.1	Europe

Regulators made coexistence studies assuming a certain DL/UL ratio for IMT TDD band 3.4-3.8GHz band in Europe. The evolution of NR duplex operation would bring changes to the frame structures of legacy TDD operation and consequently may affect TDD synchronisation.

In many CEPT countries, the same frame format is effectively mandated both indoor and outdoor in the 3400-3800 MHz frequency band. Several frame structures for TDD MFCN networks have been recommended by ECC to facilitate synchronisation in the frequency band 3400-3800 MHz at boarder areas. However, unsynchronised or semi-synchronised operation of TDD MFCN networks are not precluded with certain requirements and/or procedures of cross-border coordination between administrations.

However, unsychronised or semi-synchronised operation of TDD MFCN networks are not precluded with certain requirements and/or procedures of cross-border coordination between administrations. It is already possible today to use different TDD frame structure for isolated deployment.

11.2		Region 2

11.2.1	North America

No TDD pattern has been mandated in US, nor in Canada, but operators are encouraged to coordinate their network deployment and make sure they don’t interfere with each other.

Unsynchronized operation is allowed, more stringent regulation parameters have not been specified for such case but, again, operators would have to work their differences to avoid any claim to FCC/ISED.

11.3		Region 3

11.3.1	China

In China, spectrum is allocated with clearly stating it for TDD or FDD operation. There is no SBFD regulatory requirements in China until now. MIIT mainly cares interference between different operators. Necessary interference coordination mechanism and solutions may be proposed by MIIT to avoid interference before any SBFD deployment.

11.3.2	Japan

No TDD pattern has been mandated in Japan but operators are required to coordinate their network deployment to avoid interference. Operators are allowed to use unsynchronized operation as far as there is no interference with the adjacent network(s), e.g. for indoor usage.

11.4		Summary

The evolution of NR duplex operation would bring changes to the frame structures of legacy TDD operation and consequently may affect TDD synchronisation which will lead to potential interference to incumbent services.

Changes to current regulations may be required to allow the operation of SBFD. Therefore, suggestions to relevant administrative authorities are needed based on the results of co-existence studies between SBFD and legacy TDD system, as well as the consequent performance results defined for the operation of SBFD.

- Recommended WF

  - Discussion based on TP draft directly in 1st round

  - One TP could be chosen as to be further revised based on the inputs and discussion.

# Topic #5: TR 38.858 for RAN4 Part

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300444 | Samsung | Proposal 1: TR skeleton update for TR38.858 for RAN4 Part is provided as below. Proposal 2: RAN4 discuss the work splitting for TR 38.858 drafting (Section 10 and 11) by considering the following subsections: Section for TR 38.858; Responsible Company. 10.1 SBFD Feasibility of RF Requirements from BS Aspect. 10.2 SBFD Feasibility of RF Requirements from UE Aspect. 10.3 RF Requirements Impact Analysis. 10.4 Adjacent Channel Coexistence Evaluation for RF requirements. 10.5 Summary. 11 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum. |
| R4-2300792 | CMCC | TP for TR 38.858. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 5-1: TR 38.858 RAN4 Part Update

Sub-topic description:

[Moderator] Two Text Proposals on TR skeleton update are proposed in this meeting, and it is proposed to further discuss TR work splitting (R4-2300444) based on the updated TP skeleton.

Open issues and candidate options before e-meeting:

#### Issue 5-1-1: Text Proposal on TR Skeleton Update

- Text Proposals:

  - CMCC (R4-2300792).

  - Samsung (R4-2300444).

- Recommended WF

  - Discussion based on two TPs for TR skeleton update firstly.

  - One TP could be chosen as to be further revised based on the inputs and discussion.

#### Issue 5-1-2: TR Work Spliting Discussion

- Proposal from Samsung (R4-2300444): RAN4 discuss the work splitting for TR 38.858 drafting (Section 10 and 11) by considering the following subsections:

| Section for TR 38.858 | Responsible Company |
| --- | --- |
| 10.1 SBFD Feasibility of RF Requirements from BS Aspect |  |
| 10.2 SBFD Feasibility of RF Requirements from UE Aspect |  |
| 10.3 RF Requirements Impact Analysis |  |
| 10.4 Adjacent Channel Coexistence Evaluation for RF requirements |  |
| 10.5 Summary |  |
| 11 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum |  |

- Recommended WF

  - No need to discuss this Issue in 1st round: After TR skeleton update discussion completed (in Issue 5-1-1), TR works (Issue 5-1-2) can be allocated to voluntary companies based on the approved skeleton, by email discussion during this meeting or by post-meeting manner.