3GPP TSG-RAN WG4 Meeting # 106												R4-2303763

Athens, Greece, 27 February – 03 March, 2023

Agenda item:			5.4

Source:	Moderator (Apple)

Title:	Topic summary for [106][318] Demod_Maintenance_Part1

Document for:	Information

# Introduction

In this summary of Demod Maintenance the following topics will be covered:

- Up to Rel-16 demod maintenance for LTE and NR (4.5) – LTE

- Up to Rel-16 demod maintenance for LTE and NR (4.5) – NR BS Demod

- Up to Rel-16 demod maintenance for LTE and NR (4.5) – NR UE Demod

- Rel-17 FeMIMO Demod maintenance (5.2.3.3)

- Rel-17 RedCap Demod maintenance (5.2.5.4)

- Rel-17 Demod maintenance of other LTE/NR WI (5.2.8.4)

# Topic #1: Up to Rel-16 maintenance for LTE Demod

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/ Changes |
| --- | --- | --- |
| R4-2300615 | Rohde & Schwarz | Correction of RMCs for power imbalance TC Correct values for “Binary Channel Bits Per Sub-Frame” and “Max. Throughput averaged over 1 frame” for R.49-1 and R.49-2. |
| R4-2300616 | Rohde & Schwarz | Correction of RMCs for power imbalance TC R13 Cat-A CR |
| R4-2300617 | Rohde & Schwarz | Correction of RMCs for power imbalance TC R14 Cat-A CR |
| R4-2300618 | Rohde & Schwarz | Correction of RMCs for power imbalance TC R15 Cat-A CR |
| R4-2300619 | Rohde & Schwarz | Correction of RMCs for power imbalance TC R16 Cat-A CR |
| R4-2300620 | Rohde & Schwarz | Correction of RMCs for power imbalance TC R17 Cat-A CR |
| R4-2300621 | Rohde & Schwarz | Correction of RMCs for power imbalance TC R18 Cat-A CR |
| R4-2302178 | Huawei, HiSilicon | CR: Updates to applicability rules for LTE 8Rx requirements in TS 36.101 (Rel-16) Corrected the section number for 8Rx tests listed in Table 8.1.2.12.1-3 |
| R4-2302179 | Huawei, HiSilicon | CR: Updates to applicability rules for LTE 8Rx requirements in TS 36.101(Rel-17) R17 Cat-A CR R18 Cat-A CR needs to be requested if CR is agreed |
| R4-2302189 | Huawei, HiSilicon | CR: Corrections on FRC for LTE SDR test for cases with 6DL_7DL CCs in TS 36.101 (Rel-15) Changed the FRC for test 27,28,29 and 30 in Table 8.7.5.1-3 and Table 8.7.5.2-3 from R.68-2 to R.68 |
| R4-2302190 | Huawei, HiSilicon | CR: Corrections on FRC for LTE SDR test for cases with 6DL_7DL CCs in TS 36.101 (Rel-16) R16 Cat-A CR |
| R4-2302191 | Huawei, HiSilicon | CR: Corrections on FRC for LTE SDR test for cases with 6DL_7DL CCs in TS 36.101 (Rel-17) R17 Cat-A CR R18 Cat-A CR needs to be requested if CR is agreed |

## Open issues summary

N/A

# Topic #2: Up to Rel-16 maintenance for NR BS Demod

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/ Changes |
| --- | --- | --- |
| R4-2301208 | NEC, NTT DOCOMO INC. | Discussion on Fixed Reference Channel number issues Observation 1: In some cases, core and conformance requirements are defined with different FRC numbers. Proposal 1: Void annex A.7 to A.10 in TS 38.104, TS 38.141-1, and TS 38.141-2 and reintroduce FRCs in new annex A.11 to A.14 to renumber them to be consistent between specifications. |
| R4-2301209 | NEC, NTT Docomo Inc. | CR to 38.104: FRC number corrections (Rel-16) G-FR2-A7-xx and G-FR1-A8-xx are renumbered to G-FR2-A11-xx and G-FR1-A12-xx, respectively. Annex A.7 and A.8 are voided and renumbered to A.11 and A.12. Added void annexes, A.9 and A.10. Table numbers are correctred. |
| R4-2301210 | NEC, NTT Docomo Inc. | CR to 38.104: FRC number corrections (Rel-17) G-FR2-A7-xx, G-FR1-A8-xx, G-FR1-A9-xx, G-FR2-A10-xx are renumbered to G-FR2-A11-xx, G-FR1-A12-xx, G-FR1-A13-xx, G-FR2-A14-xx, respectively. Annex A.7, A.8, A.9, and A.10 are voided and renumbered to A.11, A.12, A.13 and A.14, respectively. Table numbers are correctred. |
| R4-2301295 | NEC, NTT DOCOMO INC. | CR to 38.104: FRC number corrections (Rel-18) R18 Cat-A CR |
| R4-2301212 | NEC, NTT Docomo Inc. | CR to 38.141-1: FRC number corrections (Rel-16) G-FR1-A7-xx are renumbered to G-FR1-A12-xx. Annex A.7 is voided and renumbered to A.12. Added void annexes, A.8, A.9, A.10, and A.11. |
| R4-2301213 | NEC, NTT Docomo Inc. | CR to 38.141-1: FRC number corrections (Rel-17) G-FR1-A7-xx and G-FR1-A8-xx are renumbered to G-FR1-A12-xx and G-FR1-A13-xx, respectively. Annex A.7 and A.8 are voided and renumbered to A.12 and A.13, respectively. Added void annexes, A.9, A.10, and A.11. |
| R4-2301214 | NEC, NTT DOCOMO INC. | CR to 38.141-1: FRC number corrections (Rel-18) R18 Cat-A CR |
| R4-2301215 | NEC, NTT Docomo Inc. | CR to 38.141-2: FRC number corrections (Rel-16) G-FR2-A7-xx and G-FR1-A8-xx are renumbered to G-FR2-A11-xx and G-FR1-A12-xx, respectively. Annex A.7 and A.8 are voided and renumbered to A.12 and A.13, respectively. Added void annexes, A.9, A.10, and A.11. |
| R4-2301216 | NEC, NTT Docomo Inc. | CR to 38.141-2: FRC number corrections (Rel-17) G-FR2-A7-xx, G-FR1-A8-xx, G-FR1-A9-xx, G-FR2-A10-xx are renumbered to G-FR2-A11-xx, G-FR1-A12-xx, G-FR1-A13-xx, G-FR2-A14-xx, respectively. Annex A.7, A.8, A.9, and A.10 are voided and renumbered to A.11, A.12, A.13 and A.14, respectively. |
| R4-2301217 | NEC, NTT DOCOMO INC. | CR to 38.141-2: FRC number corrections (Rel-18) R18 Cat-A CR |
| R4-2301756 | Keysight Technologies UK Ltd | CR to 38.141-1: BS Demod Step size correction on 2-Step RACH R16 (8.2.9) In 2-step RACH PUSCH msgA (clause 8.2.9) test, Time Offset specification value is shown in five places, on value for 30k SCS, one shows 0,0.1,2 (start:step:end), but other places shows 0.2 as step size which 0.2 is conflicting and incorrect for 30k SCS. Five places are; - Table 8.2.9.4.2-2: Test parameters for testing msgA PUSCH for 2-step RA type (note, this table shows correct value) - Step 6) in procedure, which describes 0.2 us step for both 15k and 30k (note, 30k SCS case should be corrected to have 0.1 us step increase) - Figure 8.2.9.4.2-1: Timing offset scheme for MsgA transmission for BS type 1-C and type 1-H with 30 kHz SCS (note, this figure shows incorrect step size as 0.2) - Table 8.2.9.5-2: Test requirements for msgA PUSCH for 2-step RA type, Type A, 30 kHz SCS (note, this table shows incorrect size 0.2) - Table 8.2.9.5-4: Test requirements for msgA PUSCH for 2-step RA type, Type B, 30 kHz SCS (note, this table shows incorrect size 0.2) For TS38.141-2, these are values are correct (0.1 us for 30k SCS) |
| R4-2301757 | Keysight Technologies UK Ltd | CR to 38.141-1: BS Demod Step size correction on 2-Step RACH (8.2.9) R17 Cat-A CR Cat-A CR shall be uploaded after Cat-F CR is approved. |
| R4-2301758 | Keysight Technologies UK Ltd | CR to 38.141-1: BS Demod Step size correction on 2-Step RACH (8.2.9) R18 Cat-A CR Cat-A CR shall be uploaded after Cat-F CR is approved. |
| R4-2301777 | Keysight Technologies UK Ltd | FRC reference numbering alignment between BS specs Observation 1: Between FR1 conducted and OTA testing, the same FRC is referred by different numbering which causes confusion and difficulty to compare result by conducted performance and radiated performance. Observation 2: For FR1 testing, core requirement document TS38.104 and conformance test document TS38.141-1 use different reference numbering on the same FRC for FR1, which causes confusion. Observation 3: This misalignment continues for adding new FRC with new annex clause added and getting worse. Better to do correction as sson as possible. Observation 4: It is understandable to have test cases only for FR2 but not for FR1. Which makes gap between FRC numnering and some numbering rule is needed to prevent the same thing repeated in future. Proposal 1, make alignment on FRC reference numnering by re-numbering on A7/A8/A9 to A11/A12 described in Table 2, Please agree with proposed CRs Proposal 2 Define future rule to maintain aligned FRC reference numbering. Rule proposed - Use aligned FRC reference number between TS38.104 and TS38.141-1, TS38.141-2 (this makes clause number and FRC reference number becomes different for the case of FR2 only FRC and tests added) - Also, not to try to use the same number between Annex number and FRC number Method proposed - Method option 1. Maintain actual FRC numbering in alignment such as G-FR1-Axx-x between TS38.104 and TS38.141-1/TS38.141-2, ignore continuous increment on FRC numbering in each one document. - Method option 2. Any other method to achieve continuous future alignment. Proposal 3. Propose to document this rule for maintaining future alignment. - For appropriate place to write such rule, we would liket consult this to MCC or expert on documentation rule. |
| R4-2301778 | Keysight Technologies UK Ltd | CR to 38.104: BS Conformance, FRC numbering alignment between BS specs For TS38.104, TS38.141-2 Rel-16 following changed - G-FR1-A8-x renamed as G-FR1-A11-x |
| R4-2301779 | Keysight Technologies UK Ltd | CR to 38.104: BS Conformance, FRC numbering alignment between BS specs For TS38.104, TS38.141-2 Rel-17 following changed - G-FR1-A8-x renamed as G-FR1-A11-x G-FR1-A9-x renamed as G-FR1-A12-x |
| R4-2301780 | Keysight Technologies UK Ltd | CR to 38.104: BS Conformance, FRC numbering alignment between BS specs R18 Cat-A CR Cat-A CR shall be uploaded after Cat-F CR is approved. |
| R4-2301781 | Keysight Technologies UK Ltd | CR to 38.141-1: BS Conformance, FRC numbering alignment between BS specs For TS38.141-1 Rel-16 following changed - G-FR1-A7-x renamed as G-FR1-A11-x |
| R4-2301782 | Keysight Technologies UK Ltd | CR to 38.141-1: BS Conformance, FRC numbering alignment between BS specs For TS38.141-1 Rel-17, following changed - G-FR1-A7-x renamed as G-FR1-A11-x G-FR1-A8-x renamed as G-FR1-A12-x |
| R4-2301783 | Keysight Technologies UK Ltd | CR to 38.141-1: BS Conformance, FRC numbering alignment between BS specs Cat-A CR Cat-A CR shall be uploaded after Cat-F CR is approved. |
| R4-2301784 | Keysight Technologies UK Ltd | CR to 38.141-2: BS Conformance, FRC numbering alignment between BS specs For TS38.104, TS38.141-2 Rel-16 following changed - G-FR1-A8-x renamed as G-FR1-A11-x Additionally, coding rate error in Annex A.8 table titles corrected. |
| R4-2301785 | Keysight Technologies UK Ltd | CR to 38.141-2: BS Conformance, FRC numbering alignment between BS specs For TS38.104, TS38.141-2 Rel-17 following changed - G-FR1-A8-x renamed as G-FR1-A11-x - G-FR1-A9-x renamed as G-FR1-A12-x Additionally, error on coding rate in Annex A.8 table title corrected |
| R4-2301786 | Keysight Technologies UK Ltd | CR to 38.141-2: BS Conformance, FRC numbering alignment between BS specs Cat-A CR Cat-A CR shall be uploaded after Cat-F CR is approved. |
| R4-2300641 | ROHDE & SCHWARZ | Update to PUSCH performance test cases UL symbols within the special slot need not be used for measurements. Specification is not clear on whether the UL symbols from the special slot is needed for measurements |
| R4-2300644 | ROHDE & SCHWARZ | Update to PUSCH performance test cases R16 Cat-A CR |
| R4-2300645 | ROHDE & SCHWARZ | Update to PUSCH performance test cases R17 Cat-A CR |
| R4-2300646 | ROHDE & SCHWARZ | Update to PUSCH performance test cases R18 Cat-A CR |

## Open issues summary

### Sub-topic 2-1 - FRC Numbering alignment in NR BS Demod specs

There is a misalignment between FRC numbering between 38.104, 38.141-1, 38.141-2

| FRC description by modulation and coding rate | TS38.104 FRC reference | TS 38.141-1 FRC reference | TS38.141-2 FRC reference | Reason of addition |
| --- | --- | --- | --- | --- |
| 16QAM R=434/1024 | Annex A7 G-FR2-A7-x | None | Annex A7 G-FR2-A7-x | FR2 only for R16 enhancement to add 16QAM w/ new MCS in FR2 (both R16 and R17) |
| QPSK R=157/1024 | Annex A8 G-FR1-A8-x | Annex A7 G-FR1-A7-x | Annex A8 G-FR1-A8-x | FR1 only for 2-step RACH (both R16 and R17) |
| 256QAM R=682.5/1024 | Annex A9 G-FR1-A9-x | Annex A8 G-FR1-A8-x | Annex A9 G-FR1-A9-x | FR1 only for UL256QAM (R17 only) |
| 64QAM R=517/1024 | Annex A10 G-FR2-A10-x | None | Annex A10 G-FR2-A10-x | FR2 only for FR2 High Speed Train (R17 only) |

Issue 2-1: FRC Numbering alignment BS Demod specs

- Proposals

  - Option 1: Void annex A.7 to A.10 in TS 38.104, TS 38.141-1, and TS 38.141-2 and reintroduce FRCs in new annex A.11 to A.14 to renumber them to be consistent between specifications. (NEC, NTT DoCoMo)

|  | Annex A.x corresponding to FRC: G-FRx-Ax-xx |  |  | From |
| --- | --- | --- | --- | --- |
|  | 38.104 | 38.141-1 | 38.141-2 | Rel-X |
| FRCs for refsens level, ACS, in-band blocking, OOB blocking, rx intermodulation and in-channel selectivity (QPSK, R=1/3) | A.1 | A.1 | A.1 | 15 |
| FRCs for dynamic range (16QAM, R=2/3) | A.2 | A.2 | A.2 | 15 |
| FRCs for performance requirements (QPSK, R=193/1024) | A.3 | A.3 | A.3 | 15 |
| FRCs for performance requirements (QPSK, R=99/1024) | A.3A | A.3A | A.3A | 16 |
| FRCs for performance requirements (QPSK, R=308/1024) | A.3B | A.3B | A.3B | 16 |
| FRCs for performance requirements (16QAM, R=658/1024) | A.4 | A.4 | A.4 | 15 |
| FRCs for performance requirements (64QAM, R=567/1024) | A.5 | A.5 | A.5 | 15 |
| PRACH Test preambles | A.6 | A.6 | A.6 | 15 |
| FRCs for performance requirements (16QAM, R=434/1024) | A.11 | - | A.11 | 16 |
| FRCs for performance requirements (QPSK, R=157/1024) | A.12 | A.12 | A.12 | 16 |
| FRCs for performance requirements (256QAM, R=682.5/1024) | A.13 | A.13 | A.13 | 17 |
| FRCs for performance requirements (64QAM, R=517/1024) | A.14 | - | A.14 | 17 |

  - Option 2: Align the FRC reference numbering by re-numbering of A7/A8/A9 to A11/A12 without changing the Annex numbering (Keysight)

| FRC description | TS38.104 FRC reference | TS 38.141-1 FRC reference | TS38.141-2 FRC reference | Reason of addition |
| --- | --- | --- | --- | --- |
| 16QAM R=434/1024 | Annex A.7 G-FR2-A7-x | None | Annex A.7 G-FR2-A7-x | FR2 only, for R16 enhancement to add 16QAM w/ new MCS in FR2 (both R16 and R17) |
| QPSK R=157/1024 | Annex A.8 G-FR1-A8-x Re-number as G-FR1-A11-x | Annex A.7 G-FR1-A7-x Re-number as G-FR1-A11-x | Annex A.8 G-FR1-A8-x Re-number as G-FR1-A11-x | FR1 only, for 2-step RACH (both R16 and R17) Re-numbering A7/A8 as aligned number A11 |
| 256QAM R=682.5/1024 | Annex A.9 G-FR1-A9-x Re-number as G-FR1-A12-x | Annex A.8 G-FR1-A8-x Re-number as G-FR1-A12-x | Annex A.9 G-FR1-A9-x Re-number as G-FR1-A12-x | FR1 only, for UL256QAM (R17 only) Re-numbering A8/A9 as aligned number A12 |
| 64QAM R=517/1024 | Annex A10 G-FR2-A10-x | None | Annex A10 G-FR2-A10-x | FR2 only, for FR2 High Speed Train (R17 only) |

- Recommended WF

  - Further discuss the options and capture the outcome in CRs

  - Recommendation for CRs with changes addressing same issue by 2 companies:

    - Company A takes CRs for 38.104

    - Company B takes CRs for 38.141-1, 38.141-2

Issue 2-2: Rule for maintaining future FRC Numbering

- Proposals

  - Option 1 (Keysight): Define future rule to maintain aligned FRC reference numbering.

Rule proposed

- Use aligned FRC reference number between TS38.104 and TS38.141-1, TS38.141-2 (this makes clause number and FRC reference number becomes different for the case of FR2 only FRC and tests added)

- Also, not to try to use the same number between Annex number and FRC number

Method proposed

- Method option 1. Maintain actual FRC numbering in alignment such as G-FR1-Axx-x between TS38.104 and TS38.141-1/TS38.141-2, ignore continuous increment on FRC numbering in each one document.

- Method option 2. Any other method to achieve continuous future alignment.

Document this rule for maintaining future alignment.

  - For appropriate place to write such rule, we would liket consult this to MCC or expert on documentation rule.

- Recommended WF

Further discuss the proposed method to make FRC numbering aligned in the future between different specs.

# Topic #3: Up to Rel-16 maintenance for NR UE Demod

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/ Changes |
| --- | --- | --- |
| R4-2300622 | Rohde & Schwarz | Update of FDD PMI test cases Change Aperiodic Report Slot Offset from 4 to 5 Change CQI/RI/PMI delay from 6 to 8 |
| R4-2300623 | Rohde & Schwarz | Update of FDD PMI test cases R16 Cat-A CR |
| R4-2300624 | Rohde & Schwarz | Update of FDD PMI test cases R17 Cat-A CR |
| R4-2302175 | Huawei,HiSilicon | CR on 38.101-4: Updates to precoder configuration for PDSCH, PDCCH and SDR tests. Clarified that for 2Tx, the precoder is randomly selected form Table 5.2.2.2.1-1 rather than variable i1, i2 |
| R4-2302176 | Huawei, HiSilicon | CR: Updates to precoder configuration for PDSCH, PDCCH and SDR tests in TS 38.101-4 (Rel-16) R16 Cat A CR |
| R4-2302177 | Huawei, HiSilicon | CR: Updates to precoder configuration for PDSCH, PDCCH and SDR tests in TS 38.101-4 (Rel-17) R17 Cat A CR |
| R4-2302181 | Huawei, HiSilicon | CR: Updates to V2X FRC in TS 38.101-4 (Rel-16) Added the note for CP-OFDM symbols for slot without PSFCH in PSSCH FRC Table |
| R4-2302182 | Huawei, HiSilicon | CR: Updates to V2X FRC in TS 38.101-4 (Rel-17) R17 Cat A CR |
| R4-2302487 | Qualcomm | CR to clarify and align random precoder configuration for PDSCH/PDCCH requirements Clarify precoder configuration with number of Tx = 1; Clarify random precoder behavior in time and frequency |
| R4-2302488 | Qualcomm | Cat.A CR to clarify PDSCH/PDCCH Random Precoding configuration R16 Cat A CR |
| R4-2302489 | Qualcomm | Cat.A CR to clarify PDSCH/PDCCH Random Precoding configuration R17 Cat A CR |
| R4-2300850 | Qualcomm | HST-SFN and HST-DPS model clarification Capture NOTE 2 in clause B.3.2 and NOTE 1 in clause B.3.3 by replacing fc*v/C by fd in equation B.3.2.5 and B.3.3.5, and removing the notes. |
| R4-2300851 | Qualcomm | FR1 HST demod test description correction R17 mirror R17 Cat-A CR |

Recommendations for CRs with same or changes in same sections

| T-doc Number | Company | Comments |
| --- | --- | --- |
| R4-2302175 | Huawei, HiSilicon | Capture changes from R4-2302487 if agreeable |
| R4-2302487 | Qualcomm | Merge with R4-2302175 if agreeable |

## Open issues summary

N/A

# Topic #4: Rel-17 FeMIMO Demod maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2300835 | Samsung | correction CR on UE demodulation and CSI requirement for FeMIMO - Fix the typo of reference table number in 5.1.1.5 - Fix channel profile typo of SFN Scheme A and Scheme B in Clause 5.2.2.1.20, 5.2.2.1.21, 5.2.2.2.21, 5.2.2.2.22 5.2.3.1.19, 5.2.3.1.20, 5.2.3.2.20, 5.2.3.2.21 - Remove squre brackets for the requirments in Clause 5.2.2.1.20, 5.2.2.1.21, 5.2.2.2.21, 5.2.2.2.22 5.2.3.1.19, 5.2.3.1.20, 5.2.3.2.20, 5.2.3.2.21 5.3.2.1.5, 5.3.2.2.5, 5.3.3.1.4, 5.3.3.2.4, 6.3.2.1.7, 6.3.2.2.8, 6.3.3.1.7, 6.3.3.2.7 |
| R4-2300976 | Qualcomm Incorporated | Views on Performance Requirements for Multi-TRP PDCCH Tests Observation 1: Averaging over all results without considering the receiver implementation leaves an inherent bias in the average SNR as there are unequal number of companies who considered with and without LLR combining in their implementation. Proposal 1: In order to derive requirements, first obtain the average SNRs over results that considered same receiver assumptions and then again average over the average SNRs from the first step to remove the inherent bias. Proposal 2: RAN4 should consider the average SNR from Table 2. Proposal 3: Existing additional margin should be added to the average SNR results from Table 2 to derive requirements. |
| R4-2300977 | Qualcomm Incorporated | Simulation Results on HST-SFN Scheme B Tests |
| R4-2302197 | Huawei, HiSilicon | CR on PDSCH requirement for HST-SFN scheme B For removing square brackets, update clause 5.2.2.1.21, 5.2.2.2.21, 5.2.3.1.21, 5.2.3.2.21, B.3.x2. |
| R4-2302715 | Qualcomm Incorporated | Draft CR on m-TRP PDCCH Performance Requirements Updated m-TRP PDCCH requirements based on R4-2300976. The results may be revised. |

## Open issues summary

### Sub-topic 4-1

Deriving requirements for mTRP PDCCH with different receiver assumptions.

Issue 4-1: mTRP PDCCH requirements derivation

- Proposals

  - Option 1 (Qualcomm): Derive requirements for PDCCH in mTRP by taking average of results with each receiver assumption first.

γ=p1+p2+p33+q1+q222

- Recommended WF

  - Discuss if the proposed method of deriving requirements is acceptable and capture the updated requirements in the CR.

Recommendations for CRs with same or changes in same sections

| T-doc Number | Company | Comments |
| --- | --- | --- |
| R4-2300835 | Samsung | Capture changes from R4-2302197, R4-2302715 if agreeable |
| R4-2302197 | Huawei, HiSilicon | Merge with R4-2300835 if agreeable |
| R4-2302715 | Qualcomm Incorporated | Merge with R4-2300835 if agreeable |

# Topic #5: Rel-17 RedCap Demod maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2301847 | Nokia, Nokia Shanghai Bell | Miscellaneous corrections to RedCap requirements - Miscellaneous editorial corrections to RedCap requirements are done. - The duplicated reference to TS 38.521-4 in ref [2] and ref [16] is removed. - The second reference ref [16] is set to void in clause 2. - The 4 occurences of ref [16] in subclause 4.5.4 are replaced by ref [2]. - The spec title of ref [2] is corrected in clause 2. |
| R4-2300210 | MediaTek inc. | CR to TS38.101-4, Corrections to RedCap CSI requirements - Square brackets for SNR are removed in Table 6.2.2.1.2.4-1 - In Table 6.2.2.1.2.4-1, modifty CSI-RS periodicity and offset from 10/1 to 10/5 and modify CQI/RI/PMI delay from 14 to 10. - In Table 6.3.2.1.1-1, modify “4” to “4 for FDD” and add “3 for HD-FDD” in the filed of Periodic Report Slot Offset. |

## Open issues summary

N/A

# Topic #6: Rel-17 Demod maintenance of other LTE/NR WI

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| LTE-BS Demod |  |  |
| R4-2302612 | Samsung | Correction CR on NPUSCH format1 demodulation requirements for TS 36.104 Correct the repetiton number in Table 8.5.1.1.1-5 |
| R4-2302615 | Samsung | Correction CR on NPUSCH format1 demodulation requirements for TS 36.104 R18 Cat-A CR |
| NR-UE Demod |  |  |
| R4-2300126 | China Telecom | Maintenance of R17 demodulation performance requirements for NR_demod_enh2-Perf Remove all square brackets and correct the interference model reference for all Rel-17 WI NR_demod_enh2-Perf test requirements. |
| R4-2302180 | Huawei,HiSilicon | CR on 38.101-4: Corrections and cleanup brackets for MU-MIMO MMSE-IRC requirements - Cleanup all brackets - Change “DMRS sequence is same for both target UE and co-scheuled UE” to “DMRS scrambling is same for both target UE and co-scheuled UE” - Clarify that “ensure orthogonality with the precoder” means any column is orthogonal - Descripe the precoder selection for 2Tx and 4Tx separately |
| NR-BS Demod |  |  |
| R4-2302523 | ROHDE & SCHWARZ | Update to HST Model FR2 requirements Minor correction withn text in Section J.3 (High Speed Train condition) |

Recommendations for CRs with same or changes in same sections

| T-doc Number | Company | Comments |
| --- | --- | --- |
| R4-2300126 | China Telecom | Capture changes from R4-2302180 if agreeable |
| R4-2302180 | Huawei, HiSilicon | Merge with R4-2300126 if agreeable |

## Open issues summary

N/A