3GPP TSG-RAN WG4 Meeting # 106	R4-2300008

Athens, Greece, 27 February –03 March, 2023

3GPP TSG RAN WG1 #111			R1-2212919

Toulouse, France, November 14th – 18th, 2022

Title:	LS on NR support for dedicated spectrum less than 5MHz for FR1

Response to:	-

Release:	Rel-18

Work Item:	NR_FR1_lessthan_5MHz_BW

Source:	RAN1

To:	RAN4

Cc:

Contact Person:

#### Name:	Timo Lunttila

E-mail Address:	<timo.lunttila@nokia.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN WG1 have started working on the WI NR support for dedicated spectrum less than 5MHz for FR1 at RAN1#111 and have reached the following agreements:

Agreement In an LS to RAN4, in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW. Agreement RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as any input from RAN1 for RAN4’s answer to this question. Agreement Before getting RAN4 responses, RAN1 assume maximum transmission bandwidth, 15RBs or 16RBs for 3 MHz channel BW for evaluation and analysis. Note: include agreement into the LS Agreement Before getting RAN4 responses, RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis. Note: it does not mean indication signaling is needed. Note: include this agreement into the LS Agreement For transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs. - FFS which PRBs are used and how to use the PRBs - Note: PRBs for PSS/SSS are not punctured. Agreement For CORESET#0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, following options are for study, - Opt.1: Existing configuration table for 15kHz SCS, 5MHz minimum channel BW (i.e., table 13-1 in TS38.213) is reused for configuration - Opt.2: A new CORESET#0 configuration table is to be introduced for the configuration. Conclusion No enhancements are required for PRACH to operate NR on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. - Note: PRACH formats and configurations not fitting into the transmission BW are not applicable Agreement Short PRACH formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS are supported for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth. Conclusion No enhancements are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, - FFS: the necessity for PUCCH FH disabling. Agreement Study whether and how to recover PDCCH detection performance of CORESET#0 for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Non-interleaved CCE-to-REG mapping - Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum - Opt.4: New aggregation level(s) for fit in the spectrum - Opt.5: PDCCH rate matching - Opt.6.: no enhancement specified Agreement Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Multiple PBCH receptions - Opt.3: PBCH remapping - Opt.4: PBCH payload reduction - Opt.5: PBCH rate matching around the punctured PRBs - Opt.6: no enhancement specified

Based on the agreements, RAN1 have the following questions to RAN4:

Question 1: RAN1’s understanding is that in addition to reusing 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 response on the maximum transmission bandwidth (the number of PRBs) for this channel BW.

Question 2: RAN1 have discussed aspects related to synch raster in the spectrum of interest. RAN1 would like to ask RAN4 if finer synch raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as if RAN4 needs any input from RAN1.

2. Actions: RAN1 kindly requests RAN4 to answer Question 1 and Question 2.

3. Dates of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #112	February 27th – March 3rd,, 2023	Athens, Greece

TSG RAN WG1 Meeting #113	May 2023	TBD