3GPP TSG-RAN WG4 Meeting # 106		R4-2300011

Athens, Greece, 27 February –03 March, 2023

3GPP TSG-RAN WG1 #111			R1-2212999

Toulouse, France, November 14 – 18, 2022

Title:	LS to RAN4 on low-power wake-up receiver architectures

Response to:

Release:	Rel-18

Work Item:	FS_NR_LPWUS

Source:	TSG-RAN WG1

To:	TSG-RAN WG4

Cc:

Contact Person(s):

#### Name:	Sigen Ye

E-mail Address:	sigen_ye@apple.com

#### Name:	Xiaodong Shen

E-mail Address:	xiaodong.shen@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

- Overall Description:

RAN1 has discussed the low-power wake-up receiver (LP WUR) architectures and made some agreements as shown in the Appendix. In addition, there are RAN1 agreements for the study item made under agenda items other than the LP WUR architectures, which are not included in the Appendix.

RAN1 kindly asks RAN4 to take RAN1 agreements into account, study at least the LP WUR architectures that RAN1 identifies and provide feedback, potentially considering the aspects including but not limited to:

- The reasonable assumption on adjacent channel selectivity (ACS) for the study and the impact on the LP WUR architectures and signal design

- The impact of adjacent subcarrier interference suppression/rejection on the LP WUR architectures if LP WUS is multiplexed with other signals/channels in frequency, including e.g.

  - The necessity of guard band (if needed, the minimum guard band) between LP WUS subcarriers and adjacent subcarriers

  - Whether it is feasible to have LP WUS location flexible within the carrier

- The feasible noise figure(s) for each type of LP WUR architectures

- Impact, if any, LP-WUS transmission on existing gNB emissions/compliance requirements

- The potential RF impairments to be considered include e.g. timing error, frequency error, image impact, LO leakage (DC offset) and flicker (1/f) noise

- Whether certain LP WUR architectures can support multi-band capability

- Note: RAN1 may or may not identify further architecture(s) for the study.

2. Actions:

To TSG-RAN WG4

ACTION: 	RAN1 respectfully asks RAN4 to take the above into consideration, study at least the LP WUR architectures that RAN1 identifies and provide feedback.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #112                              27 February – 3 March 2023	  	 	Athens, Greece

TSG-RAN WG1 Meeting #112bis-e                      17 – 26 April 2023	  	                Online

Appendix:

Conclusion

RAN1 does not intend to mandate the implementation of any specific type(s) of LP WUR architecture at the UE.

- Note: this does not prevent RAN4 from defining requirements for LP WUR in the normative phase.

Agreement

Study at least the following three types of receiver architectures for LP-WUR:

- Architecture with RF envelope detection

- Heterodyne architecture with IF envelope detection

- Homodyne/zero-IF architecture with baseband envelope detection

- Note: The details of each type of receiver architecture are discussed separately.

- Note: Above receiver architectures are considered suitable for OOK modulation. Some of the architectures

can be applicable for other modulations such as FSK.

Agreement

Study the architecture with RF envelope detection based on at least the following diagram for LP-WUR.

- The RF signal is converted into baseband signal directly via an RF envelope detector.

- There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL).

- 1-bit or multi-bit ADC is applied.

- Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied.

- High-Q matching network and/or RF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- FFS the support of band and/or carrier tuning

Agreement

Study the heterodyne architecture with IF envelope detection based on at least the following diagram for LP-WUR.

- The RF signal is down converted into IF signal via an RF mixer with a LO. The IF signal is converted into baseband signal via an IF envelope detection.

  - There may be one or multiple IF stages depending on design.

- The choice of the LO is one of the major factors that determines the power consumption.

  - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation.

  - FLL (frequency locked loop) may replace PLL for non-coherent detection.

- 1-bit or multi-bit ADC is applied.

- High-Q matching network and/or RF BPF and/or IF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Some component(s), e.g., RF LNA and/or IF AMP and/or BB AMP, can be optionally applied.

- Image rejection filter or an image rejection mixer is required.

- FFS the support of band and/or carrier tuning

- FFS the choice of IF frequency range

Agreement

Study the homodyne/zero-IF architecture with baseband envelope detection based on at least the following diagram for LP-WUR.

- The RF signal is directly down converted into baseband signal via an RF mixer with a LO.

- Baseband envelope detection can be done either in analog domain or in digital domain depending on design, which is not explicitly shown in the diagram.

- The choice of the LO is one of the major factors that determines the power consumption.

  - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation.

  - FLL (frequency locked loop) may replace PLL for non-coherent detection.

- 1-bit or multi-bit ADC is applied.

- High-Q matching network and/or RF BPF and/or BB BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- No image rejection filter is required.

- Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied.

- FFS the support of band and/or carrier tuning

Agreement

Further study the receiver architectures for FSK, with two examples shown below:

- Example 1: parallel OOK receivers and a comparator circuit, e.g.,

  - Each path can be implemented using either of [the architecture with RF envelope detection,] heterodyne architecture with IF envelope detection, or homodyne/zero-IF architecture with baseband envelope detection.

- Example 2: using an FM-to-AM detector [or an FM detector]

  - Alt 1: Use an analog FM-to-AM detector with a similar architecture as for OOK (e.g. heterodyne or zero-IF architecture), except that the envelope detector is replaced by a FM-to-AM detector.

    - Analog FM-to-AM detector can be implemented at least in BB or low-IF.

  - Alt 2: Use a FM-to-AM detector [or an FM detector] implemented in digital domain after ADC, with a heterodyne or zero-IF architecture.

    - Digital FM-to-AM detector implementation can be considered as part of digital baseband processing.

    - Here is an example of using zero-IF architecture:

  - The FM-AM detector can be implemented using a frequency discriminator, which converts frequency variations into amplitude changes. It can be implemented in either analog domain (as in Alt 1) or digital domain (as in Alt 2).

    - One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal.

- Note: Other architectures are not precluded.

Agreement

For the analysis of a receiver architecture, companies are encouraged to provide at least the following (when applicable):

- Details of the receiver

  - Receiver architecture type

  - Assumed modulation/waveform/coding

  - Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any

  - Local oscillator

    - Type of oscillator and the corresponding frequency accuracy/drifting

  - Handling of time/frequency impairments

  - Presence of PLL or FLL

  - ADC: sampling rate, bit-width

  - Assumed signal bandwidth and guard band, and frequency location within a carrier (including whether it is fixed or can be flexible)

  - RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies), if any

  - Baseband processing (e.g., sequence correlation detection / decoding, other signal processing, if any)

  - Assumed frequency band(s) and the support of band and/or carrier tuning

  - Duty cycle handling of WUS and other signals (if any)

  - Interference rejection capability (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS)

  - Handling of inter-cell interference

  - Whether there is any mobility support function, e.g. measurement capability

- Performance metrics

  - Power consumption during active monitoring/reception and during off state (and breakdown if possible)

  - Noise figure

  - Sensitivity/coverage

  - Data rate

  - FFS: other performance metrics for, e.g., cost/complexity, interference rejection capability and inter-cell interference handling

- Note: The performance and design of receiver architecture is expected to be dependent on WUS design. This list can be updated later when the discussion on WUS signal/procedure design (AI 9.13.3) starts.

Agreement

The following observation to be captured in TR38.869:

For the architecture with RF envelope detection,

- It can achieve relatively low power consumption due to the removal of LO/PLL.

- Interference suppression for adjacent channel interference requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- Interference suppression for interference from legacy NR signals and/or other LP WUS on adjacent subcarriers, if performed in RF, requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- The support of multiple bands and/or carriers may require multiple high-Q matching networks and/or RF BPFs or multiple off-chip components.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The noise figure can be relatively high.

Agreement

The following observation to be captured in TR38.869:

For homodyne/zero-IF architecture with baseband envelope detection,

- For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency.

- The matching network and RF BPF for LP WUR may or may not reuse those of the main radio.

- It is more effective and less complex to use BB BPF/LPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Using FLL instead of PLL consumes less power, but it may result in larger frequency error.

- It can suffer from LO leakage (DC offset) and flicker (1/f) noise. The impact may be alleviated by using BB BPF in some cases.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The baseband envelope detection can be done in either analog domain (before ADC) or digital domain (after ADC).

Agreement

The following observation to be captured in TR38.869:

For heterodyne architecture with IF envelope detection,

- For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency.

- The matching network and RF BPF for LP WUR may or may not reuse those of the main radio.

- It is more effective and less complex to use IF BPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Using FLL instead of PLL consumes less power, but it may result in larger frequency error.

- The IF frequency can be properly selected to avoid LO leakage (DC offset) and flicker (1/f) noise.

- Image rejection can be done via either image rejection filter or image rejection mixer.

  - Image rejection filter can be done in either RF or IF, which may require high-Q filter.

  - Image rejection mixer requires two-branch (I/Q) mixing with good matching in gain and phase, which consumes additional power.

- RF LNA and/or IF AMP can be applied to improve sensitivity, with the cost of additional power consumption.