# Operator Acceptance Values for Device Antenna Performance

# Version 4.1

# 22 July 2021

This is a Non-binding Permanent Reference Document of the GSMA

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2022 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Compliance Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

This Permanent Reference Document is classified by GSMA as an Industry Specification, as such it has been developed and is maintained by GSMA in accordance with the provisions set out in GSMA AA.35 - Procedures for Industry Specifications.

Table of Contents

1	Introduction	3

1.1	Overview	3

1.2	Scope	3

1.3	Definition of Terms	4

1.4	Document Cross-References	5

2	Antenna Performance Requirements	5

2.1	Antenna Performance Calculation Formula	6

2.2	Operator Antenna Performance Acceptance Values for 2G and 3G Bands	6

2.3	Operator Antenna Performance Acceptance Values for LTE Bands	8

2.4	Operator Antenna Performance Acceptance Values for 5G NR FR1	10

3	Moving Forward	14

Annex A	Measured Values	15

A.1	RAG Status Tables	15

A.1.1 Test Methodology	15

A.1.2 Interpretation	15

A.1.3 Measured Results Tables	16

A.1.4 Further Analysis	18

A.2	Study on Wide Grip Hand	18

A.3	Data Collection Campaign on 5G NR FR1 Bands	0

A.4	Test Campaign on 5G NR FR1 Bands	2

A.5	Survey on EN-DC Combinations	2

Annex B	Document Management	3

B.1	Document History	3

Other Information	3

# Introduction

## Overview

Mobile handset design has changed significantly over the past few years. Antennas, which were external in many cases, moved inside the terminal and evolved to be light-weight and low-volume.

The increasing focus on design conflicts with radio performance of the antenna systems can lead to service disruption and increased dropped call rates impacting customer experience. Increasing network density is not an option as it may not be viable in many cases.

Whilst the 3GPP standards define the absolute global minimum OTA performance requirements to be met by devices, numerous operators are already requesting higher values in their own procurement activities to serve customer needs and network rollout plans.

However, the test methods used, and antenna performance values requested by the various operators differ widely, leaving vendors with a fragmented requirements landscape.

To facilitate this alignment and agreement, the antenna experts of operators within the GSMA Terminal Steering Group have reviewed results of antenna performance tests (conducted within their own organisations as well as by the GSMA) and have aligned test methods and agreed on performance values to be used as guidelines for acceptable and achievable performance of antennas in Mobile devices. The values are supported by operators representing America, Europe, and Asia.

Following the publication of version four in February 2019 [TS.24], the TSG operators have now aligned performance - in addition to 2G/3G/LTE - also for 5G NR FR1 bands.

## Scope

This document defines the Operator Acceptance Values for Device Antenna Performance. The values are determined taking into consideration

devices being held close to the head with left and right hand (BHH)

devices being held with one hand only for the purpose of Browsing

devices being measured with the Free Space method

The requirements (which include measurement uncertainty) are relevant for mobile devices, including feature phones, smart phones, tablets, and dongles. More details can be found in the relevant sections.

These GSMA Operator Acceptance Values for Device Antenna Performance are non-binding, and operators are free to request alternative values as required. It will be up to each individual operator to adopt and enforce the GSMA Operator OTA requirements.

These values serve to align, enhance and promote antenna performance requirements across the industry while seeking to improve device antenna performance and network interoperability.

Note 1:  	The alignment of the GSMA Operator Acceptance Values for Device Antenna Performance does not impact the test process; this will continue as it is the case today in Operator-Manufacturer bilateral agreements.

Each phase of the work carried out in updating TS.24 has also involved independent tests on commercially available smart phones. The purpose of these tests was to benchmark the Acceptance Values defined in TS.24. For 5G NR FR1 bands, network operators voluntarily provided antenna performance data for devices used in their device portfolio pool. Test campaign has been done in across 8 different labs.

In this version of TS.24 we publish the results of these tests and data collection campaign in the appendix.

## Definition of Terms

| Term | Description |
| --- | --- |
| 3GPP | 3rd Generation Partnership Project |
| BHH | Beside Head and Hand |
| BHHL | Beside Head and Hand Left |
| BHHR | Beside Head and Hand Right |
| BW | Bandwidth |
| CBW | Channel bandwidth |
| CC | Component Carrier |
| CCSA | China Communications Standards Association |
| CS | Circuit Switched (voice mode) |
| CTIA | CTIA Certification™ |
| EN-DC | E-UTRA New Radio-Dual Connectivity |
| EPS | Equal Power Sharing. In EN-DC mode Tx power is equally shared between NR and LTE bands (Option B) |
| FDD | Frequency Division Duplex |
| E-UTRA | Evolved Universal Terrestrial Radio Access |
| FS | Free Space |
| FR1 | 5G Frequency Range 1 (410 MHz – 7125 MHz) |
| FR2 | 5G Frequency Range 2 (24250 MHz – 52600 MHz) |
| GSM / E-GSM | Global System for Mobile Communication/ Extended-GSM |
| GSMA | GSM Association |
| HL | Browsing mode with Hand Left Only |
| HR | Browsing mode with Hand Right Only |
| LTE | Long Term Evolution |
| MIMO | Multiple Input Multiple Output |
| MS | Mobile Station |
| NR | New Radio |
| NR FR1 | New Radio in frequency range 1 bands |
| NR FR2 | New Radio in frequency range 2 bands |
| NSA | Non-Standalone |
| OEM | Original Equipment Manufacturer |
| Option A/B | Power sharing. Option A: max power on NR and min power on LTE. Option B: power is shared equally between NR and LTE |
| OTA | Over The Air (without cable) |
| RAG | Red, Amber, Green |
| PC | Power class. PC3=23 dBm; PC2=26 dBm conducted Tx power |
| RB | Resource Block |
| RMC | Reference Measurement Channel |
| TDD | Time Division Duplex |
| SA | 5G Standalone |
| TRP | Total Radiated Power |
| TRS (TIS) | Total Radiated (Isotropic) Sensitivity |
| TSG | Terminal Steering Group |
| TSGFM | Terminal Steering Group Full Members |
| UE | User Equipment |
| VoLTE | Voice over LTE |
| VoNR | Voice over NR |
| VoIP | Voice over Internet Protocol |
| WCDMA | Wideband Code Division Multiple Access |

## Document Cross-References

| Ref | Document Number | Title |
| --- | --- | --- |
| CTIA | Latest Active version | The latest active version of the Test Plan, as identified at https://ctiacertification.org/test-plans/ © 2001-2022 CTIA Certification. Referenced with permission. All testing referencing this test plan must be conducted in a CTIA Certification Authorized Test Lab. |
| 3GPP | TS 25.144 | User Equipment (UE) and Mobile Station (MS) over the air performance requirements |
| 3GPP | TS 38.101 | User Equipment (UE) transmission and reception for NR |

# Antenna Performance Requirements

The following table serves as an internal guideline for mobile operators, who are encouraged to apply the required values in their bilateral agreements with their vendors.

| Req ID | Requirement |
| --- | --- |
| TS.24_2.0_REQ_001 | For the scenario “Head and Hand” (BHH) and in line with CTIA, the terminal SHALL support an average value of the low, medium and high frequency channels, on both, the left (BHHL) and right (BHHR) hand-side, a performance equal to or above the GSMA performance requirements for that frequency band listed in table 1 and 2 in section 2.2 for 2G/3G and table 3 and 4 in section 2.3 for LTE bands. |
| TS.24_2.0_REQ_002 | For the scenario “Browsing” (HL or HR) and in line with CTIA, the terminal SHALL support an average value of the low, medium and high frequency channels, on both the left and right hand-side, a performance equal to or above the GSMA performance requirements for that frequency band listed in table 1 and 2 in section 2.2 for 2G/3G and table 3 and 4 in section 2.3 for LTE bands. |
| TS.24_2.0_REQ_003 | For the scenario “Free Space” (FS) and in line with CTIA, the terminal SHALL support an average value of the low, medium and high frequency channels a performance equal to or above the GSMA performance requirements for that frequency band listed in table 1 and 2 in section 2.2 for 2G/3G and table 3 and 4 in section 2.3 for LTE bands. |
| TS.24_2.0_REQ_004 | No measured channel SHALL be 1 dB worse than the performance values defined for that frequency band. |
| TS.24_2.0_REQ_005 | The CTIA/3GPP PDA hand SHALL be used for testing a device which is between 56 mm and 72 mm (inclusive) wide. |
| TS.24_2.0_REQ_006 | The CTIA Wide Grip hand SHALL be used for testing a device which exceeds 72 mm but does not exceed 92 mm in width. |
| TS.24_2.0_REQ_007 | TRP and TRS values agreed for 5G NR FR1 bands in EN-DC and SA modes |

## Antenna Performance Calculation Formula

Note 2:

The values include measurement uncertainty.

Operators may accept alternative values for bands outside of their home market.

BHH and Browsing acceptance values are the same for devices tested using the PDA and Wide Grip hand phantom.

## Operator Antenna Performance Acceptance Values for 2G and 3G Bands

The following tables list the Operator Antenna Performance Values per test scenario and frequency band (2G and 3G).

Test scenario:

Head and Hand (BHH):

Relevant for devices that support voice and do not exceed the maximum dimensions specified for hand phantom (92 mm) [CTIA]. The values are defined considering head and hand and are relevant for left or right hand.

PDA hand is used for testing devices with widths 56 – 72 mm

Wide Grip hand is used for testing devices with widths >72 - 92 mm

Browsing (HL or HR):

Relevant for devices where the display is visible to the end user for data usage and where width ranges are between 56 mm and 92 mm [CTIA]. The values are defined considering one-hand only and are relevant for left or right hand. The 3G frequencies are measured, using a 12.2 Kbps reference measurement channel (RMC).

PDA hand is used for testing devices with widths 56 – 72 mm

Wide Grip hand is used for testing devices with widths >72 - 92 mm

Free Space:

Relevant for any device that embeds an antenna and that supports voice and/or data. The applicable measurement method is voice (CS) mode. The 3G frequencies are measured, using a 12.2 Kbps reference measurement channel (RMC).

These acceptance values include measurement uncertainty.

| Frequency Band 2G/3G | GSMA Operator Acceptance Values for TRP [dBm] |  |  |
| --- | --- | --- | --- |
|  | BHH (Note 3) | Browsing | Free Space |
| GSM 850 | 20.0 | 24.0 | 27.0 |
| EGSM 900 | 20.0 | 25.0 | 28.0 |
| GSM 1800 | 21.0 | 23.0 | 26.0 |
| GSM 1900 | 21.0 | 23.0 | 26.0 |
| WCDMA Band 1 | 15.0 | 17.0 | 20.0 |
| WCDMA Band 2 | 16.5 | 17.0 | 20.0 |
| WCDMA Band 3 | 13.5 | 14.7 | 17.7 |
| WCDMA Band 5 | 11.0 | 14.7 | 17.7 |
| WCDMA Band 8 | 11.0 | 15.0 | 18.0 |
| WCDMA Band 19 | 11.0 | 15.5 | 18.0 |

: GSMA Operator Acceptance Values for TRP for 2G and 3G bands

| Frequency Band 2G/3G | GSMA Operator Acceptance Values for TRS [dBm] |  |  |
| --- | --- | --- | --- |
|  | BHH (Note 3) | Browsing | Free Space |
| GSM 850 | -97.0 | -99.0 | -103.0 |
| EGSM 900 | -95.0 | -99.0 | -103.0 |
| GSM 1800 | -99.0 | -100.0 | -104.0 |
| GSM 1900 | -99.5 | -100.0 | -103.0 |
| WCDMA Band 1 | -101.0 | -103.0 | -106.0 |
| WCDMA Band 2 | -98.5 | -101.0 | -104.0 |
| WCDMA Band 3 | -98.5 | -99.0 | -102.0 |
| WCDMA Band 5 | -94.5 | -100.0 | -103.0 |
| WCDMA Band 8 | -96.0 | -101.0 | -104.0 |
| WCDMA Band 19 | -96.0 | -102.0 | -104.5 |

: GSMA Operator Acceptance Values for TRS for 2G and 3G bands

## Operator Antenna Performance Acceptance Values for LTE Bands

The following tables list the Operator Antenna Performance Values per test scenario and frequency band (LTE).

Test scenario:

Head and Hand (BHH):

Relevant for devices that support voice (e.g. VoLTE, VoIP). The relevant hand phantom is to be used according to the device’s width:

PDA hand is used for testing devices with widths 56 – 72 mm

Wide Grip hand is used for testing devices with widths >72 - 92 mm

The values are relevant for left or right hand.

Browsing (HL or HR):

Relevant for devices where the display is visible to the end user for data usage and relevant hand phantom to be used according to the device’s width:

PDA hand is used for testing devices with widths 56 – 72 mm

Wide Grip hand is used for testing devices with widths >72 - 92 mm

The values are defined considering one-hand only and are relevant for left or right hand.

Free Space:

Relevant for any device that embeds an antenna and supports voice (e.g. VoLTE, VoIP) and /or data.

These acceptance values include measurement uncertainty.

Settings during testing

TRP:

Single antenna transmitting

Uplink RB Allocation: 12

TRS:

All receivers/antennas active

Downlink RB Allocation: 50

Bandwidth: 10 MHz

| Frequency Band LTE | GSMA Operator Acceptance Values for TRP [dBm] |  |  |
| --- | --- | --- | --- |
|  | BHH (Note 3) | Browsing | Free Space |
| FDD Band 1 | 13.5 | 15.5 | 18.5 |
| FDD Band 2 | 13.5 | 15.5 | 18.5 |
| FDD Band 3 | 13.5 | 15.5 | 18.5 |
| FDD Band 4 | 13.5 | 15.5 | 18.5 |
| FDD Band 5 | 9.8 | 14.3 | 18.0 |
| FDD Band 7 | 13.5 | 15.5 | 18.5 |
| FDD Band 8 | 9.8 | 14.3 | 18.0 |
| FDD Band 11 | 11.5 | 14.5 | 18.0 |
| FDD Band 12 | 9.8 | 14.3 | 18.0 |
| FDD Band 13 | 9.8 | 14.3 | 18.0 |
| FDD Band 17 | 9.8 | 14.3 | 18.0 |
| FDD Band 18 | 9.8 | 14.3 | 18.0 |
| FDD Band 19 | 9.8 | 14.3 | 18.0 |
| FDD Band 20 | 9.8 | 14.3 | 18.0 |
| FDD Band 21 | 11.5 | 14.5 | 18.0 |
| FDD Band 25 | 13.5 | 15.5 | 18.5 |
| FDD Band 26 | 9.8 | 14.3 | 18.0 |
| FDD Band 28 | 9.8 | 14.3 | 18.0 |
| TDD Band 38 | 13.5 | 15.5 | 18.5 |
| TDD Band 39 | 13.5 | 15.5 | 18.5 |
| TDD Band 40 | 13.5 | 15.5 | 18.5 |
| TDD Band 41 | 13.5 | 15.5 | 18.5 |
| TDD Band 42 | 13.5 | 15.5 | 18.5 |
| TDD Band 43 | 13.5 | 15.5 | 18.5 |

- : GSMA Operator Acceptance Values for TRP for the LTE Bands

| Frequency Band LTE | GSMA Operator Acceptance Values for TRS [dBm] |  |  |
| --- | --- | --- | --- |
|  | BHH (Note 3) | Browsing | Free Space |
| FDD Band 1 | -89.0 | -91.0 | -94.0 |
| FDD Band 2 | -89.0 | -91.0 | -94.0 |
| FDD Band 3 | -89.0 | -91.0 | -94.0 |
| FDD Band 4 | -89.0 | -91.0 | -94.0 |
| FDD Band 5 | -85.0 | -89.5 | -93.5 |
| FDD Band 7 | -89.0 | -91.0 | -94.0 |
| FDD Band 8 | -85.0 | -89.5 | -93.5 |
| FDD Band 11 | -87.0 | -90.0 | -93.5 |
| FDD Band 12 | -85.0 | -89.5 | -93.5 |
| FDD Band 13 | -85.0 | -89.5 | -93.5 |
| FDD Band 17 | -85.0 | -89.5 | -93.5 |
| FDD Band 18 | -85.0 | -89.5 | -93.5 |
| FDD Band 19 | -85.0 | -89.5 | -93.5 |
| FDD Band 20 | -85.0 | -89.5 | -93.5 |
| FDD Band 21 | -87.0 | -90.0 | -93.5 |
| FDD Band 25 | -89.0 | -91.0 | -94.0 |
| FDD Band 26 | -85.0 | -89.5 | -93.5 |
| FDD Band 28 | -85.0 | -89.5 | -93.5 |
| TDD Band 38 | -89.0 | -91.0 | -94.0 |
| TDD Band 39 | -89.0 | -91.0 | -94.0 |
| TDD Band 40 | -89.0 | -91.0 | -94.0 |
| TDD Band 41 | -89.0 | -91.0 | -94.0 |
| TDD Band 42 | -89.0 | -91.0 | -94.0 |
| TDD Band 43 | -89.0 | -91.0 | -94.0 |

GSMA Operator Acceptance Values for TRS for the LTE Bands

Note 3: 	BHH and Browsing acceptance values are the same for PDA and Wide Grip hand phantom.

## Operator Antenna Performance Acceptance Values for 5G NR FR1

The following tables list the Operator Antenna Performance Values per test scenario and frequency band for 5G NR FR1.

Requirements are defined for EN-DC (NSA) and 5G SA.

If a device supports both NSA and SA it’s up to the MNO to request which configuration they would like to test the device.

However, it is recommended for test optimization perspectice to perform the full OTA test (TRP and TRS) in SA mode and in addition to test TRP at a mid channel in EN-DC mode.

TRP requirements for PC3 are defined for all NR bands listed in this document.

Although 3GPP has not yet defined PC2 conducted values for the FDD bands, TRP requirements have been defined for PC2 in all FDD bands listed in this documents.

Test scenario:

Head and Hand (BHH):

Relevant for devices that support voice (e.g. VoIP, VoNR). The relevant hand phantom is to be used according to the device’s width:

PDA hand is used for testing devices with widths 56 – 72 mm

Wide Grip hand is used for testing devices with widths >72 - 92 mm

The values are relevant for left or right hand.

Browsing (HL or HR):

Relevant for devices where the display is visible to the end user for data usage and relevant hand phantom to be used according to the device’s width:

PDA hand is used for testing devices with widths 56 – 72 mm

Wide Grip hand is used for testing devices with widths >72 - 92 mm

The values are defined considering one-hand only and are relevant for left or right hand.

Note 4: Head and hand phantoms used for 2G/3G/LTE bands can also be used for the defined NR bands in this document.

Free Space:

Relevant for any device that embeds an antenna and supports voice (e.g. VoIP, VoNR) and /or data.

These acceptance values include measurement uncertainty.

Settings during testing

TRP:

Single antenna transmitting.

Option A: Max Tx power on NR, min Tx power on LTE (10 dBm regardless of device’s PC for NR band).

Option B: Tx Power equally shared between LTE and NR (EPS).

TRS:

All receivers/antennas active.

Bandwidth: see table

Converting a measured TRS value with BW1 to a TRS value with BW2 is possible:

= 10*log(BW2/BW1)

Example:  BW1= 100 MHz; BW2 = 20 MHz

 = 10*log(20/100) = -7 dB

-86 dBm @ (100 MHz)  -93 dBm @ (20 MHz)

| NR FR1 Frequency Band | GSMA Operator Acceptance Values for TRP [dBm] in EN-DC Mode for PC3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BHH (Note 4) |  | Browsing (Note 4) |  | Free Space |  |
| Option | A | B | A | B | A | B |
| N1 | 14 | 12 | 16 | 14 | 18.5 | 16.5 |
| N3 | 14 | 12 | 16 | 14 | 18.5 | 16.5 |
| N7 | 14 | 12 | 16 | 14 | 18.5 | 16.5 |
| N8 | 10.5 | 8.5 | 14.5 | 12.5 | 18.5 | 16.5 |
| N20 | 10.5 | 8.5 | 14.5 | 12.5 | 18.5 | 16.5 |
| N28 | 10.5 | 8.5 | 14.5 | 12.5 | 18.5 | 16.5 |
| N40 | 14 | 12 | 16 | 14 | 18.5 | 16.5 |
| N77 | 14 | 12 | 16 | 14 | 18.5 | 16.5 |
| N78 | 14 | 12 | 16 | 14 | 18.5 | 16.5 |

GSMA Operator Acceptance Values for TRP for the NR FR1 Bands in EN-DC mode

| NR FR1 Frequency Band (BW) | GSMA Operator Acceptance Values for TRS [dBm] in EN-DC Mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BHH (Note 4) |  | Browsing (Note 4) |  | Free Space |  |
| Option | A | B | A | B | A | B |
| N1 (20 MHz) | -90 | -90 | -92 | -92 | -94 | -94 |
| N3 (20 MHz) | -90 | -90 | -92 | -92 | -94 | -94 |
| N7 (20 MHz) | -90 | -90 | -92 | -92 | -94 | -94 |
| N8 (10 MHz) | -86 | -86 | -90 | -90 | -94 | -94 |
| N20 (10 MHz) | -86 | -86 | -90 | -90 | -94 | -94 |
| N28 (10 MHz) | -86 | -86 | -90 | -90 | -94 | -94 |
| N40 (20 MHz) | -90 | -90 | -92 | -92 | -94 | -94 |
| N77 (20 MHz) | -90 | -90 | -92 | -92 | -94 | -94 |
| N78 (20 MHz) | -90 | -90 | -92 | -92 | -94 | -94 |

GSMA Operator Acceptance Values for TRS for the NR FR1 Bands in EN-DC mode

| NR FR1 Frequency Band | GSMA Operator Acceptance Values for TRP [dBm] in SA Mode for PC3 |  |  |
| --- | --- | --- | --- |
|  | BHH (Note 4) | Browsing (Note 4) | Free Space |
| N1 | 14 | 16 | 19 |
| N3 | 14 | 16 | 19 |
| N7 | 14 | 16 | 19 |
| N8 | 10.5 | 15 | 18.5 |
| N20 | 10.5 | 15 | 18.5 |
| N28 | 10.5 | 15 | 18.5 |
| N40 | 14 | 16 | 19 |
| N77 | 14 | 16 | 19 |
| N78 | 14 | 16 | 19 |

GSMA Operator Acceptance Values for TRP for the NR FR1 Bands in SA mode for PC3 (23 dBm)

| NR FR1 Frequency Band | GSMA Operator Acceptance Values for TRP [dBm] in SA Mode for PC2 |  |  |
| --- | --- | --- | --- |
|  | BHH (Note 4) | Browsing (Note 4) | Free Space |
| N1 | 17 | 19 | 22 |
| N3 | 17 | 19 | 22 |
| N7 | 17 | 19 | 22 |
| N8 | 13 | 17.5 | 21 |
| N20 | 13 | 17.5 | 21 |
| N28 | 13 | 17.5 | 21 |
| N40 | 17 | 19 | 22 |
| N77 | 17 | 19 | 22 |
| N78 | 17 | 19 | 22 |

GSMA Operator Acceptance Values for TRP for the NR FR1 Bands in SA mode for PC2 (26 dBm)

| NR FR1 Frequency Band (BW) | GSMA Operator Acceptance Values for TRS [dBm] in SA Mode |  |  |
| --- | --- | --- | --- |
|  | BHH (Note 4) | Browsing (Note 4) | Free Space |
| N1 (20 MHz) | -90 | -92 | -94 |
| N3 (20 MHz) | -90 | -92 | -94 |
| N7 (20 MHz) | -90 | -92 | -94 |
| N8 (10 MHz) | -86 | -90 | -94 |
| N20 (10 MHz) | -86 | -90 | -94 |
| N28 (10 MHz) | -86 | -90 | -94 |
| N40 (20 MHz) | -90 | -92 | -94 |
| N77 (20 MHz) | -90 | -92 | -94 |
| N78 (20 MHz) | -90 | -92 | -94 |

GSMA Operator Acceptance Values for TRS for the NR FR1 Bands

# Moving Forward

The GSMA operators will progress:

To run tests with EN-DC mode in low-low bands (e.g. LTE 20 + N28). If necessary, to add acceptance limits for special EN-DC combinations.

To run tests with PC2 in EN-DC mode. Add acceptance limits for PC2 in EN-DC mode.

Define acceptance limits for 5G NR FR2

In addition, operators will continue to test antenna performance of market devices and actively monitor and/or engage in work driven in relevant industry bodies.

Measured Values

As part of the process to derive the harmonised Operator Acceptance Values published in this document the GSMA has, for each phase of the work, carried out independent tests on commercially available Smartphones. These devices are taken from consumer stock, and are not prototype or engineering samples provided by the OEMs.

In this appendix we have published the results from these tests.

The purpose of the tests was to benchmark the acceptance values defined in TS.24 and to ensure that they represent a realistic achievable target that manufacturers of devices can readily achieve.

All device results have been anonymised.

RAG Status Tables

The tables published in this appendix include a RAG (Red Amber Green) colour coding. This RAG status can be interpreted as follows:

Red	= fails to meet the GSMA TS.24 acceptance value by a value greater than 2 dB

Amber	= within 2 dB of the GSMA TS.24 acceptance value

Green	= meets or surpasses the GSMA TS.24 acceptance value

The 2 dB margin used to define the Amber category reflects that the acceptance values published in this document are a harmonised average of the values contributed by operators who took part in this work.

It also recognises that some operators have a small tolerance in their acceptance values so that they do not block the launch of a device that marginally fails to meet their requirements. These tolerances are usually defined on a bilateral basis and are reviewed per device.

Because of the above the tables below try to reflect this reality through use of the 2 dB margin which defines values in Amber.

- A.1.1 Test Methodology

Device selection was based on a random selection of commercially available devices at the time of measurement, typically these devices are from Tier 1 OEMs and are their Flagship or high-volume selling products.

The tests were carried out in an independent antenna lab, accredited by both CTIA (e.g. a CTIA Certification Authorized Test Lab) and GCF (Global Certification Forum).

The specific test methodologies used, followed the recommendations issued by CTIA.

- A.1.2 Interpretation

For the purpose of this work these tables were used to assess the values defined in TS.24, and to ensure that when the 2 dB margin is taken into account, the significant majority of devices are able to meet or surpass the acceptance values published in TS.24.

- A.1.3 Measured Results Tables

: TRP and TRS RAG status of tested devices in 2G/3G bands for the use cases Browsing and Free Space (status: 2013)

: TRP and TRS RAG status of tested devices in 2G/3G bands for head & hand use case (status: 2013)

: TRP RAG status of tested devices in LTE bands for head & hand use case (status: 2015)

: TRS RAG status of tested devices in LTE bands for head & hand use case (status: 2015)

- A.1.4 Further Analysis

As part of the analysis, it was noticed that there could be a perceived trend that demonstrates a difference between left hand and right hand performance. Further analysis of this trend is required to establish if this is a genuine trend or just an anomaly in the results. The TSGFM Group would like to draw attention to this as it was noted that accreditation in some regions is still limited to testing with the right hand, and therefore there could be a correlation between this and some products being optimized for performance in the right hand.

The sample of measured product is too small to establish if this trend is real, but it has been noted for further study during future measurement and test campaigns carried out by GSMA.

Study on Wide Grip Hand

More and more devices with big screens (including many flagship models) are introduced to the market and there are still no requirements on devices wider than 72 mm. Therefore CCSA (China Communications Standards Association) made a big study with many devices with widths between 72 – 92 mm using the CTIA Wide Grip hand phantom. TRP and TIS values in BHHR (“Talk” right) and HR (“Browsing” right) modes have been measured in 2G/3G/LTE bands used in China.

The study has shown an excellent correlation between GSMA BHH requirements for PDA hand phantom and results of devices tested with Wide Grip hand phantom. The values are selected so that 80% of the devices could pass the CCSA limit. The results show that there are no obvious differences for test results between devices with widths between 72 mm and 92 mm under CTIA Wide Grip hand phantom and devices with widths below 72 mm under PDA hand phantom. So, the limit values can stay unchanged with the test of the PDA hand phantom.

| Band (Test Scenario) | TRP (80%) | TRP (TS.24) | TIS (80%) | TIS (TS.24) |
| --- | --- | --- | --- | --- |
| GSM 900 (BHHR) | 17 | 20 | -93 | -95 |
| GSM1800 (BHHR) | 19 | 21 | -97 | -99 |
| WCDMA Band 1 (BHHR) | 13.5 | 15 | -100 | -101 |
| LTE Band 3（HR） | 15.5 | 15.5 | -91 | -91 |
| LTE Band 38（HR） | 15 | 15.5 | -86 | -91 |
| LTE Band 39（HR） | 16 | 15.5 | -88 | -91 |
| LTE Band 40（HR） | 15 | 15.5 | -86 | -91 |
| LTE Band 41（HR） | 14.5 | 15.5 | -85 | -91 |

: Test results of devices with widths between 72 mm and 92 mm under CTIA Wide Grip hand phantom in comparison to TS.24 limits from the CCSA (status: 2017)

Data Collection Campaign on 5G NR FR1 Bands

We at first run a data collection campaign and asked the MNOs to provide TRP and TRS data of devices from their device pool. We received in total 313 TRP and 282 TRS values for more than 100 devices. Below is an extract with values for N78 band:

:  TRP and TRS data from MNOs for N78 band (Free Space, Option A). Green: best performing device; Red: worst performing device

We then calculated the average of all received data (see below overview):

: Average of all  data from MNOs (313 TRP and 282 TRS values). TRS values for N78 are normalized to 20 MHz bandwidth.

* TRS for N78 normalized to 20 MHz bandwidth

Test Campaign on 5G NR FR1 Bands

We performed a small test campaign with 6 devices tested in 8 different labs.

: TRP results of test campaign with 6 devices tested in 8 different labs. Left: Option A; Right: Option B.

: TRS results of test campaign with 6 devices tested in 8 different labs

Survey on EN-DC Combinations

MNOs have been asked to provide their prioritized EN-DC combinations. The table below shows the most used EN-DC-combinations.

: Most used EN-DC combinations from survey among MNOs

Document Management

Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| V1.0 | 14 May 2013 | Published on GSM world | TSG | Katrin Jordan, DT |
| V2.0 | Feb 2014 | Updated scope, references, requirements and added performance values for Free Space and Browsing. | TSG | Katrin Jordan, DT |
| V3.0 | Sept. 2015 | Performance values for LTE and Annex A added | TSG | Xiaolong Zhou, China Unicom Abbas Alpaslan, Vodafone |
| V4.0 | February 2019 | Clarifying the requirements for devices wider than 72 mm | TSG | Xiaolong Zhou, China Unicom Abbas Alpaslan, Vodafone Momar Goumballe, Orange |
| V4.1 | June 2021 | Updated in line with CTIA licence agreement | TSG | Paul Gosden GSMA |
| V5.0 | October 2022 | Antenna performance limits for 5G NR FR1 bands added | TSG#49 | Abbas Alpaslan, Vodafone Momar Goumballe, Orange |

Other Information

| Type | Description |
| --- | --- |
| Document Owner | This document is owned and approved by TSG Full Members |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com.

Your comments or suggestions & questions are always welcome.