3GPP TSG-RAN WG4 Meeting #106-e	R4-2303630

Athens, Greece, 27 Feb-03 Mar, 2023

Title:	Reply LS on clarification for ue-PowerClassPerBandPerBC-r17 (R4 16-8)

Response to:	R2-2211023

Release:	Rel-17

Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Yuanyuan Zhang

E-mail Address:  tina55.zhang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 would like to thank RAN2 for the LS to inquire the interaction between ue-PowerClassPerBandPerBC-r17 (R4 16-8) and other existing different power class parameters.

Regarding RAN2’s request to RAN4 for the responses to the following questions:

- Whether R4 16-8 is applicable to only inter-band CA?

- What is the interaction between R4 16-8 and the existing power class capabilities (i.e. ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClassNRPart-r16 (if R4 16-8 is also applicable to the cases other than inter-band CA) and powerClass/powerClass-v1610)?

After discussion during RAN4#106, RAN4 would like to respectfully provide the following responses for RAN2 consideration.

Response to 1): Yes. It is applicable to only NR inter-band UL CA, i.e. when there is uplink configured in two different operating bands. Each uplink band contains only single UL CC or intra-band contiguous UL CA.

Response to 2): ue-PowerClassPerBandPerBC-r17 indicates the power class that a UE supports for each individual band within a given band combination, while powerClass or powerClass-v1610 indicates the power class for this band combination, in other words, the maximum total output power. If indicated, ue-PowerClassPerBandPerBC-r17 shall supersede other power class capabilities such as ue-PowerClass/powerClass and its extensions in determining the power class of the individual bands within a band combination.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take above RAN4 responses into consideration.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #106-bis-e	17 - 26 April 2023             E-meeting

TSG-RAN WG4 Meeting #107	22 - 26 May 2023         Incheon, Korea