3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2304007

Online Meeting, 17 – 26 April 2023

3GPP TSG RAN WG1 #112	R1-2302194

Athens, Greece, February 27th – March 3rd, 2023

# Title:	LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM

# Response to:

# Release:	Rel-18

# Work Item:	NR_mob_enh2-Core

Source:	RAN1

To:	RAN2, RAN3, RAN4

Cc:

Contact Person:

Name:	Yosuke Akimoto

E-mail Address:	akimoto.yosuke at fujitsu.com

Name:	Xin Su

E-mail Address:	suxin at catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

- L1 measurement RS configuration

- Regarding the configuration for L1 measurement RS, RAN1 achieved the following agreement

- For L1-RSRP measurement RS configuration

  - For SSB based L1-RSRP measurement:

    - As a starting point, at least the following information needs to be provided to a UE, e.g.

      - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst)

      - For inter-frequency: frequency domain location (e.g. center frequency), SCS

      - FFS: transmission power (for pathloss calculation)

    - Note: other parameters included in the configuration can be further discussed

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112:

- Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells

  - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there.

- Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells

  - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells]

- Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells

  - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

RAN1 believes this is a RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design for LTM L1 measurement configurations. It is noted that RAN2 has a full flexibility to design the whole RRC structure design, and RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations.

Note: CellGroupConfig above is the RAN1 assumption of the LTM RRC model for each candidate cell configuration as of the RAN1 discussion.

- PDCCH ordered RACH

Regarding the configuration/indication of RAR reception for PDCCH ordered-RACH, RAN1 achieved the following agreement

For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

- If reception of RAR is not configured/indicated (without RAR)

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- If reception of RAR is configured/indicated (with RAR), FFS

  - whether RAR is received from serving cell or candidate cell

    - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE

  - content of RAR

- FFS: signaling for configuration/indication of whether RAR needs to be received

- UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- Send LS to RAN2 and RAN3 to check the feasibility about this agreement

- Note: Definition of candidate cells is up to RAN2

As the feasibility of schemes included in the agreement above is related to the designs of RAN2 and RAN3, RAN 1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN 3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in this agreement.

- RAN1 #112 agreements

The agreements achieved in RAN1#112 are captured below for information.

Agreement

  - RAN1 shares the same understanding as RAN2 on agreement:

    - The LTM mobility trigger information is conveyed in a MAC CE

  - The same MAC CE is used for the LTM triggering.

Agreement

  - The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following:

    - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

Agreement

- For L1-RSRP measurement RS configuration

  - For SSB based L1-RSRP measurement:

    - As a starting point, at least the following information needs to be provided to a UE, e.g.

      - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst)

      - For inter-frequency: frequency domain location (e.g. center frequency), SCS

      - FFS: transmission power (for pathloss calculation)

    - Note: other parameters included in the configuration can be further discussed

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

Agreement

- At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s)

- FFS: beam indication for mTRP case

Agreement

For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell.

Note: the detailed signalling is left to RAN2

Agreement

The PDCCH order from the source cell contains the indication of candidate cell.

- The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

Agreement

For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

- If reception of RAR is not configured/indicated (without RAR)

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- If reception of RAR is configured/indicated (with RAR), FFS

  - whether RAR is received from serving cell or candidate cell

    - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE

  - content of RAR

- FFS: signaling for configuration/indication of whether RAR needs to be received

- UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- Send LS to RAN2 and RAN3 to check the feasibility about this agreement

- Note: Definition of candidate cells is up to RAN2

Agreement

- For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported

Agreement

on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives.

- Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1)

- Alt 2: UE autonomous Re-transmission of PRACH is allowed,

  - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax

Agreement

If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell.

- The maximum number of TA values memorized by UE is a UE capability

- FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR

Agreement

Whether RAR needs to be received is configured by RRC.

Agreement

study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured

- Whether gap between the DCI and PRACH longer than timeline defined in spec is needed

- Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

Working Assumption

UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

2. Actions:

To RAN2 and RAN3 group.

ACTION: 	RAN 1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN 3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in the agreement described in section B. Also, RAN1 respectfully asks RAN2 and RAN3 to take the RAN1 agreements into consideration for their work.

To RAN4 group.

ACTION: 	RAN1 respectfully asks RAN4 to take the RAN1 agreements into consideration for their work.

3. Date of Next TSG RAN WG1 Meetings:

TSG RAN WG1 Meeting #112-bis-e	17th – 26th Apr. 2023		Online

TSG RAN WG1 Meeting #113	22nd May – 26th May 2023	Incheon, KR