3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2304095

Electronic meeting, 17th – 26th April 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 					      n26 A-MPR for 3 MHz channel

Agenda Item: 			5.15.3

Document for:	Approval

# 1	Introduction

This contribution presents simulation results and proposals for A-MPR for n26 3 MHz channel. The proposals are also captured in CR [2].

# 2	Simulation scenario and assumptions

We used the simulation assumptions below.

- Total number of resource blocks is NRB = 15

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

We simulate the A-MPR in the presence of network signalling values NS_12, NS_13, NS_14, and NS_15. The requirements are presented in the tables below [1].

Table 6.5.3.3.17-1: Additional requirements NS_12

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Table 6.5.3.3.18-1: Additional requirements NS_13

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 817 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Table 6.5.3.3.19-1: Additional requirements NS_14

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz, 15 MHz, 20MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 824 MHz NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Table 6.5.3.3.20-1: Additional requirements NS_15

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

# 3	Simulation results

Here we present some examples of the simulation results. A complete set of A-MPR results at the closest possible positions from the additional masks is provided in the appendix.

## 3.1	NS_12

An example of the simulated A-MPR with the channel at FC = 815.5 MHz is presented below. This is at the lower edge of the band. No A-MPR is needed when FC ≥ 817.5 MHz.

Figure 1. Examples of required total backoff for NS_12 at FC = 815.5 MHz.

## 3.2	NS_13

An example of the simulated A-MPR with the channel at FC = 818.5 MHz is presented below. This is the closest possible position from the additional mask. No A-MPR is needed when FC ≥ 820 MHz.

Figure 2. Examples of required total backoff for NS_13 at FC = 818.5 MHz.

## 3.3	NS_14

No A-MPR is needed for NS_14. This is not surprising since no A-MPR is specified for 5 MHz channel either.

## 3.4	NS_15

An example of the simulated A-MPR with the channel at FC = 847.5 MHz is presented below. This is at the upper edge of the band. No A-MPR is needed when FC ≤ 844.5 MHz.

Figure 3. Examples of required total backoff for NS_15 at FC = 847.5 MHz.

# 4	Proposed A-MPR

Based on the simulation results we propose the following A-MPR.

Proposal 1: If NS_12 is signaled apply the A-MPR from tables 1 and 2.

Table 1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A2 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 |
| DFT-s-OFDM 256 QAM | ≤ 5 |
| CP-OFDM QPSK | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 6 |
| CP-OFDM 256 QAM |  |

Proposal 2: If NS_13 is signaled apply the A-MPR from tables 3 and 4.

Table 3: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |

Table 4: A-MPR for NS_13

| Modulation/Waveform | A4 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 5.5 |
| CP-OFDM 256 QAM |  |

Proposal 3: Specify no A-MPR for 3 MHz channel in NS_14.

Proposal 4: If NS_15 is signaled apply the A-MPR from tables 5 and 6.

Table 5: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.62 | >0 | A5 |

Table 6: A-MPR for NS_15

| Modulation/Waveform | A5 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 |
| DFT-s-OFDM QPSK | ≤ 7.5 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 |
| DFT-s-OFDM 256 QAM | ≤ 7.5 |
| CP-OFDM QPSK | ≤ 8 |
| CP-OFDM 16 QAM | ≤ 8 |
| CP-OFDM 64 QAM | ≤ 8 |
| CP-OFDM 256 QAM | ≤ 8.5 |

# 5	Conclusions

We have provided simulations results on the A-MPR for n26 3 MHz channel. Based on the results we have proposed following A-MPR.

Proposal 1: If NS_12 is signaled apply the A-MPR from tables 1 and 2.

Table 1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A2 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 |
| DFT-s-OFDM 256 QAM | ≤ 5 |
| CP-OFDM QPSK | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 6 |
| CP-OFDM 256 QAM |  |

Proposal 2: If NS_13 is signaled apply the A-MPR from tables 3 and 4.

Table 3: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |

Table 4: A-MPR for NS_13

| Modulation/Waveform | A4 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 5.5 |
| CP-OFDM 256 QAM |  |

Proposal 3: Specify no A-MPR for 3 MHz channel in NS_14.

Proposal 4: If NS_15 is signaled apply the A-MPR from tables 5 and 6.

Table 5: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.62 | >0 | A5 |

Table 6: A-MPR for NS_15

| Modulation/Waveform | A5 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 |
| DFT-s-OFDM QPSK | ≤ 7.5 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 |
| DFT-s-OFDM 256 QAM | ≤ 7.5 |
| CP-OFDM QPSK | ≤ 8 |
| CP-OFDM 16 QAM | ≤ 8 |
| CP-OFDM 64 QAM | ≤ 8 |
| CP-OFDM 256 QAM | ≤ 8.5 |

The proposals are also captured in CR [2].

# 6	References

[1] TS 38.101-1 (V18.0.0), 3GPP

[2] R4-2304042	draftCR to 38.101-1: Introduction of dedicated spectrum less than 5MHz for FR1, RAN4#106-bis-e

# 7	Appendix: A-MPR triangle plots