3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304109

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Advanced Receivers - Simulation results

Agenda item:	5.19.1.2

Document for:	Discussion

Introduction

RAN4#106 was the first meeting handling the WID of advanced receiver for MU-MIMO scenario.

The following main agreements were made in RAN4#106 [1] for facilitating companies to provide simulation results for Phase I:

Issue 2-2: Rank allocation for the target and co-scheduled UEs, with 1 co-scheduled UE - Agreement on ‘rank number for target UE + rank number for co-scheduled UE’: - 2Rx UE: 1+1 - 4Rx UE: 2+2 and 1+3 […] Issue 2-4: DMRS port configurations for the target and co-scheduled UEs - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - rank 1 (DMRS port 3) +3 (port 0, 1, 2) - rank 1 (DMRS port 0 for target UE) +1 (port 1) +1 (port 2) - Use the same CDM group for rank 1+1 […] Issue 2-8: Antenna configuration - For initial simulation in Phase I - For rank 1+1: cover 2T2R - For rank 2+2, rank 1+3, rank 1+1+1: 4T4R […] Issue 2-9: Channel model - For initial simulation assumptions: - Use TDLC300-100 when the rank of the target UE is 1 - Use TDLA30-10 and TDLC300-100 when the rank of the target UE is 2 - The assumption can be updated later based on available results. […] Issue 2-10: Antenna correlation - For initial simulation in Phase I only: - Rank 1+1: ULA medium - Rank 1+1+1: ULA medium A, XPL medium - Rank 2+2, 1+3: ULA Low - The assumptions can be updated later based on available results […] Issue 2-12: Precoder selection target and co-scheduled UEs - Single panel Type 1 - Random PMI selection for the target UE - Cover both orthogonal and random PMI selection (same as Rel-17 approach) for the co-scheduled UE in phase I - The assumption can be updated later base on the available results. […] Issue 2-13: QCL assumptions - Assume all scheduled DMRS ports have same QCL assumptions […] Issue 2-15: Evaluation metric - The SNR @ %70 of maximum throughput as the phase I evaluation metric and use the MMSE-IRC receiver as the baseline […] Issue 2-16: Other parameters and assumptions - Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point.

In this document, we will provide our current simulation results to be used for alignment.

Simulation assumptions for Phase I

Table 1 provides an overview of the already agreed simulation assumptions for Phase I taken from the WF [1]:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver assumption for E-MMSE-IRC | WRX,1k,l=H1H(k,l)R-1 R= P1H1k,lH1Hk,l+Pjj=2NUEHjk,lHjHk,l+1Nrek,l∈DMRS of NUEr(k,l)r(k,l)H, rk,l=rk, l-j=1NUEHjk,ldjk,l . |
| --- | --- |
| Reference receiver for phase I simulation | Consider both R-ML and E-IRC in initial evaluation stage |
| Rank allocation for the target and co-scheduled UEs, with 1 co-scheduled UE | 2Rx UE: 1+1 4Rx UE: 2+2 and 1+3 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - rank 1 (DMRS port 3) +3 (port 0, 1, 2) - rank 1 (DMRS port 0 for target UE) +1 (port 1) +1 (port 2) - Use the same CDM group for rank 1+1 |
| Antenna configuration | For rank 1+1: cover 2T2R For rank 2+2, rank 1+3, rank 1+1+1: 4T4R |
| Channel model | Use TDLC300-100 when the rank of the target UE is 1 Use TDLA30-10 and TDLC300-100 when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium Rank 1+1+1: ULA medium A, XPL medium Rank 2+2, 1+3: ULA Low |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 Random PMI selection for the target UE Cover both orthogonal and random PMI selection (same as Rel-17 approach) for the co-scheduled UE in phase I |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | At least 1 co-scheduled UE |
| DMRS sequence for the co-scheduled UE | For initial simulation in phase I, assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| MCS for the target UE | MCS 13 for rank 1 and rank 2 for initial simulation |
| Modulation order for the co-scheduled UE | For rank 1+1: QPSK (high priority for the next meeting) For rank 2+2: 64QAM (high priority for the next meeting) For rank 2+2: QPSK (high priority for the next meeting) |
| PDSCH resource allocation for the target and co-scheduled UE | Maximum transmission bandwidth configuration for all UEs. Full OFDM symbol allocation |
| Assumptions on the required information | Assume the needed parameters of the co-scheduled UE are all known to UE (upper bound for the potential performance gain). |
| MCS | Table 1 |

Based on the information in Table 1, we provide the simulation results on the 10 highest priority cases for this meeting, listed in Table 2.

Table 2 - Simulation Overview – high priority cases

| Case | # Co-scheduled UE | Rank target UE | Rank Co-scheduled UE | Modulation order co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | orthogonal |
| 2 |  |  |  |  |  |  | random |
| 5 |  | 2 | 2 | 64QAM | 4Tx 4Rx ULA Low |  | orthogonal |
| 6 |  |  |  |  |  |  | random |
| 7 |  |  |  | QPSK |  |  | orthogonal |
| 8 |  |  |  |  |  |  | random |
| 9 |  |  |  | 64QAM |  | TDLA30-10 | orthogonal |
| 10 |  |  |  |  |  |  | random |
| 11 |  |  |  | QPSK |  |  | orthogonal |
| 12 |  |  |  |  |  |  | random |
| Note1: Case 3 and Case 4 pertain to the Rank 1+3 simulations which are planned to be presented in the next coming meetings. Note2: All tests consider the target UE modulation order to be 16 QAM (MCS 13), while the co-scheduled UE assumes a modulation order of QPSK (MCS 4), 16 QAM (MCS 13) and 64 QAM (MCS 19) using MCS index Table 1. |  |  |  |  |  |  |  |

We have also provided simulation results for additional cases studied which are included separately in this document.

Simulation Results

High priority cases

This section displays our current simulation results. Normalized throughput curves are shown for the baseline receiver (MMSE-IRC) against the E-MMSE-IRC and R-ML receivers. A black reference line is included to highlight the SNR @ 70% of maximum throughput metric for each receiver, captured in Table 3.

Cases 1-2: R1+1, TDLC300-100, medium, QPSK, orthogonal/random precoder selection for the co-scheduled UE

| Figure 1 – Rank 1+1 TDLC300-100, medium correlation, MCS13-MCS4, orthogonal precoder. | Figure 2 - Rank 1+1 TDLC300-100, medium correlation, MCS13-MCS4, random precoder. |
| --- | --- |

Cases 5-6: R2+2, TDLC300-100, low, 64 QAM, orthogonal/random precoder selection for the co-scheduled UE

| Figure 3 – Rank 2+2 TDLC300-100, low correlation, MCS13-MCS19, orthogonal precoder. | Figure 4 – Rank 2+2 TDLC300-100, low correlation, MCS13-MCS19, random precoder. |
| --- | --- |

Cases 7-8: R2+2, TDLC300-100, low, QPSK, orthogonal/random precoder selection for the co-scheduled UE

| Figure 5 – Rank 2+2 TDLC300-100, low correlation, MCS13-MCS4, orthogonal precoder. | Figure 6 – Rank 2+2 TDLC300-100, low correlation, MCS13-MCS4, random precoder. |
| --- | --- |

Cases 9-10: R2+2, TDLA30-10, low, 64 QAM, orthogonal/random precoder selection for the co-scheduled UE

| Figure 7 – Rank 2+2 TDLA30-10, low correlation, MCS13-MCS19, orthogonal precoder. | Figure 8 – Rank 2+2 TDLA30-10, low correlation, MCS13-MCS4, random precoder. |
| --- | --- |

Cases 11-12: R2+2, TDLA30-10, low, QPSK, orthogonal/random precoder selection for the co-scheduled UE

| Figure 9 – Rank 2+2 TDLA30-10, low correlation, MCS13-MCS4, orthogonal precoder. | Figure 10 – Rank 2+2 TDLA30-10, low correlation, MCS13-MCS4, random precoder. |
| --- | --- |

Table 3 offers a summary of the results above, along with each receiver’s gain over the baseline MMSE-IRC (referred to as IRC in Table 3).

Table 3 – Summary of Receiver performance

| Case | # Co-scheduled UE | Rank target UE | Rank co-scheduled UE | Modulation order co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | Nokia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNR@70%maxTP (dB) |  |  | Gain of R-ML | Gain of E-IRC | Gain of R-ML over E-IRC |
|  |  |  |  |  |  |  |  | R-ML | E-IRC | IRC (baseline) |  |  |  |
| 1 | 1 | 1 | 1 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | orthogonal | 10.70 | 16.56 | 19.43 | 8.73 | 2.86 | 5.87 |
| 2 |  |  |  |  |  |  | random | 11.27 | 19.49 | 22.96 | 11.69 | 3.46 | 8.22 |
| 3 |  |  | 3 | 16QAM | 4Tx 4Rx ULA Low |  | orthogonal |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  | random |  |  |  |  |  |  |
| 5 |  | 2 | 2 | 64QAM |  |  | orthogonal | 10.43 | 10.85 | 13.60 | 3.17 | 2.75 | 0.42 |
| 6 |  |  |  |  |  |  | random | 14.11 | 14.66 | N/A | - | - | 0.55 |
| 7 |  |  |  | QPSK |  |  | orthogonal | 7.76 | 10.86 | 13.59 | 5.82 | 2.72 | 3.10 |
| 8 |  |  |  |  |  |  | random | 9.56 | 14.99 | N/A | - | - | 5.43 |
| 9 |  |  |  | 64QAM |  | TDLA30-10 | orthogonal | 9.61 | 10.24 | 11.84 | 2.23 | 1.61 | 0.63 |
| 10 |  |  |  |  |  |  | random | 13.25 | 14.21 | 17.69 | 4.43 | 3.47 | 0.96 |
| 11 |  |  |  | QPSK |  |  | orthogonal | 6.97 | 10.24 | 11.83 | 4.86 | 1.59 | 3.27 |
| 12 |  |  |  |  |  |  | random | 8.71 | 14.72 | 18.36 | 9.65 | 3.64 | 6.00 |
| Note1: Case 3 and Case 4 (empty grey fields) pertain to the Rank 1+3 simulations which are planned to be presented in the next coming meetings. Note2: N/A expresses the cases where a receiver is not able to reach 70% of the max throughput. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Some observations follow below:

- Results show that both E-MMSE-IRC (E-IRC in Table 3) and R-ML offer improved receiver performance over the baseline for all the tested cases, with R-ML consistently outperforming E-MMSE-IRC.

- The precoder selection for the co-scheduled UE has a visible impact on receiver performance:

  - orthogonal precoder selection offers higher gains for all receivers.

  - IRC performance is particularly vulnerable to random precoder selection in Rank 2+2 for TDLC channels - it fails to reach 70% maximum throughput (while reached with orthogonal precoding).

Additional cases studied

Additional cases are also studied beyond the agreed simulation assumptions, to consider different modulation orders for the target UE. The results of the additional cases are shown in Table 4:

Table 4 Results from additional cases studied

| Additional Case | # Co-scheduled UE | Rank target UE | Rank co-scheduled UE | Modulation order target UE | Modulation order co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | Nokia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | SNR@70%maxTP (dB) |  |  | Gain of R-ML | Gain of E-IRC | Gain of R-ML over E-IRC |
|  |  |  |  |  |  |  |  |  | R-ML | E-IRC | IRC (baseline) |  |  |  |
| A1 | 1 | 1 | 1 | MCS4 | QPSK | 2Tx 2Rx ULA Medium | TDLC300-100 | random | 3.44 | 6.60 | 11.36 | 7.92 | 4.76 | 3.44 |
| A2 |  |  |  | MCS4 | 16 QAM |  |  |  | 5.86 | 6.61 | 11.37 | 5.52 | 4.76 | 5.86 |
| A3 |  |  |  | MCS13 | 16 QAM |  |  |  | 15.26 | 19.48 | 22.99 | 7.73 | 3.51 | 15.26 |
| A4 |  | 2 | 2 | MCS19 | QPSK | 4Tx 4Rx ULA Low |  | orthogonal | 12.33 | 17.04 | N/A | - | - | 12.33 |
| A5 |  |  |  | MCS19 | 64 QAM |  |  |  | 17.20 | 17.03 | N/A | - | - | 17.20 |
| A6 |  |  |  | MCS19 | QPSK |  | TDLA30-10 |  | 11.36 | 16.27 | 17.33 | 5.97 | 1.06 | 11.36 |
| A7 |  |  |  | MCS19 | 64 QAM |  |  |  | 16.80 | 16.27 | 17.34 | 0.53 | 1.07 | 16.80 |

The most relevant results are displayed below:

Case A1: R1+1, TDLC300-100, medium, QPSK-QPSK, random precoder selection for the co-scheduled UE

Figure 11 – Rank 1+1 TDLC300-100, medium correlation, MCS4-MCS4, random precoder.

Case A6-A7: R2+2, TDLC300-100/TDLA30-10, low, 64QAM-64QAM, orthogonal precoder selection for the co-scheduled UE

| Figure 12 – Rank 2+2 TDLC300-100, low correlation, MCS19-MCS19, orthogonal precoder. | Figure 13 – Rank 2+2 TDLA30-10, low correlation, MCS19-MCS19, orthogonal precoder. |
| --- | --- |

The results indicate good improvement from the reference for both investigated receiver types.

Conclusion

This paper presents Nokia's simulation results for Application Layer Throughput. It includes the configurations agreed in RAN#106 to be highest priority. Also included are the results of our additional studied cases.

- References

- R4-2302929 WF on advanced receiver for MU-MIMO scenario.docx