3GPP TSG-RAN WG4 Meeting # 106bis-e		R4-2304150

# Online, April 17 – April 26, 2023

Agenda item:	5.9.3

Source:	Apple

Title:	On RRM requirements for FR1-FR1 NR-DC

Document for:	Discussion

# Introduction

In RAN4 #106 meeting, RRM requirements for FR1-FR1 NR-DC scenarios have been discussed. The WF of eFeRRM has been agreed in [1], and the open issues were also captured. In this paper, we further discuss the RRM requirements for FR1-FR1 NR-DC scenario.

# Discussion on FR1+FR1 NR-DC RRM

## SCG activation/deactivation

The options we discussed in last meeting were:

## Sub-topic 1-4: SCG activation/deactivation Issue 1-4-1: Tsearch for RACH-less SCG activation Agreement: - For RACH-less based SCG activation for FR1+FR1 NR-DC, - [if TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell] - FFS on requirements for additional scenarios including FR1 PSCell is unknown

During the last meeting discussion, companies mentioned the definition in RAN2 spec that: only when RLM/BFD is configured before the reception of the NR RRC message containing the RRCReconfiguration message and lower layer doesn’t indicate RACH is needed, the RACH-less SCG activation can be used.

In addition, the known condition for PSCell and TCI was defined in SCG activation requirement as:

Thus, the other scenarios for FR1+FR1 SCG activation can includes:

- PSCell is unknown

  - TCI is known

  - TCI is unknown

- PSCell is known

  - TCI is known (existing one)

  - TCI is unknown

Observation: there are 3 scenarios missing in the previous discussion: (1) target PSCell is unknown but TCI is known, (2) target PSCell is unknown and TCI is unknown, (3) target PSCell is known but TCI is unknown.

(1) target PSCell is unknown but TCI is known

When PSCell is categorized to “unknown”, it could be due to different causes, e.g., UE measured but no valid report back to network, or no SSB is detectable at UE, and therefore it’s still possible that PSCell is unknown but the TCI is known, i.e., UE has not reported measurement to network in a certain period before SCG activation but UE’s TCI is still good. However, the RAN4 requirement shall be defined based on a common knowledge between UE and network, otherwise the requirement will not be able to verify. In this case, network cannot differentiate what causes the ‘unknown PSCell’ in the requirement(no reporting or SSB is not detectable), and then even TCI is known at UE, we shall still focus on the condition of unknown PSCell. The existing RRM requirement for RACH-less SCG activation is defined as following, and it is only specified for FR1+FR2 NR-DC.

### 8.17.2 SCG Activation Delay Requirement The requirements in this clause shall apply for the UE configured with one deactivated SCG in NR-DC and when PScell in one SCG is being activated. The delay within which the UE shall be able to activate the deactivated SCG depends upon the specified conditions. Upon receiving SCG activation command in slot n, the UE shall be capable to transmit PRACH preamble or PUCCH or PUSCH towards PSCell no later than in slot n+Tactivation_timeNR slot length , where: Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. When PSCell is activated from deactivated state, if any PSCell parameter is modified, Tprocessing = 20ms. Otherwise, Tprocessing = 5 ms. Tsearch is the time for AGC settling and PSS/SSS detection. Tsearch is the time for AGC settling and PSS/SSS detection. For RACH based PSCell activation, if the target cell is a known NR FR2 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. For RACH-less based PSCell activation, if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR2 PScell. There are no requirements if PSCell is unknown. T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms. TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3]. When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH]. Trs is the SMTC periodicity of the PSCell if the UE has been provided with an SMTC configuration for the target cell in SCG activation command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5.

If PSCell is unknown, no requirement shall be applied, and we think this condition shall also be used for FR1+FR1 NR-DC since the TA is also not available for an unknown PSCell during SCG activation. And in this case, we also think UE shall fall back to RACH based activation to avoid any ambiguity between UE behavior and network expectation.

So the SCG activation for FR1+FR1 NR-DC shall be:

For RACH-less based PSCell activation, if FR1 PSCell is unknown and TCI is known, UE shall fall back to RACH based SCG activation and RACH based unknown PSCell activation can be applied.

(2) target PSCell is unknown and TCI is unknown

Similar as the case (1), RACH based requirement shall be defined for this unknown PSCell activation. However, the difference here is: when TCI is unknown, it implies beam failure on PSCell. In the RAN2 TS38.331, it was specified that UE reports beam failure of the PSCell to the network while the SCG is deactivated, and it’s unclear if UE shall also report such beam failure of the PSCell during PSCell activation (after receiving the SCG activation message).

In our view, it’s still necessary to indicate such failure to network and network can understand the UE condition even during the activation, e.g., network can try to reconfigure the TCI for the target PSCell before real scheduling.

Thus, for RACH-less based PSCell activation, if FR1 PSCell is unknown and TCI is unknown, UE shall fall back to RACH based SCG activation and RACH based unknown PSCell activation can be applied. UE shall indicate network the beam failure in this case.

(3) target PSCell is known but TCI is unknown

Based on the definition of unknown TCI, that means UE may encounter the beam failure of the PSCell. Even though we don’t need to consider Rx beam for FR1 PSCell, the problematic TCI of timing and frequency may result into the transmission failure of PUCCH/PUSCH on the target PSCell. There might be different ways to perform beam recovery on the PSCell, e.g., indicate the beam failure to network via PCell link or fall back to RACH based SCG activation. In RAN4, we think it would be desirable to have a requirement for such case to avoid ambiguity when such case happens in reality. Thus, we propose to allow UE fall back to RACH based SCG activation and RACH based known PSCell activation can be applied and UE can indicate network the beam failure as same as the beam failure indication of the PSCell to the network while the SCG is deactivated.

Proposal 1:

For RACH-less based PSCell activation,

- if FR1 PSCell is unknown and TCI is known or unknown, UE shall fall back to RACH based SCG activation, and RACH based unknown PSCell activation requirement can be applied.

- if FR1 PSCell is known but TCI is unknown, UE shall fall back to RACH based SCG activation, and RACH based known PSCell activation requirement can be applied.

Proposal 2:

During FR1 PSCell activation, if TCI is unknown due to beam failure, UE shall indicate network the beam failure as same as the beam failure indication of the PSCell to the network while the SCG is deactivated.

Regarding proposal 2, if companies think it needs RAN2 expertise to clarify, we are also fine to send LS to RAN2 for clarification.

# Conclusion

In this paper, we further discuss the RRM requirements for FR1-FR1 NR-DC scenario.

Observation: there are 3 scenarios missing in the previous discussion: (1) target PSCell is unknown but TCI is known, (2) target PSCell is unknown and TCI is unknown, (3) target PSCell is known but TCI is unknown.

Proposal 1:

For RACH-less based PSCell activation,

- if FR1 PSCell is unknown and TCI is known or unknown, UE shall fall back to RACH based SCG activation, and RACH based unknown PSCell activation requirement can be applied.

- if FR1 PSCell is known but TCI is unknown, UE shall fall back to RACH based SCG activation, and RACH based known PSCell activation requirement can be applied.

Proposal 2:

During FR1 PSCell activation, if TCI is unknown due to beam failure, UE shall indicate network the beam failure as same as the beam failure indication of the PSCell to the network while the SCG is deactivated.

Regarding proposal 2, if companies think it needs RAN2 expertise to clarify, we are also fine to send LS to RAN2 for clarification.

# References

[1] R4-2303304, Way forward on R18 RRM enhancement (Part 2), OPPO, RAN4 #106