3GPP TSG-RAN WG4 Meeting # 106-bis-e							       R4-2304211

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.36.2

Source: 	CMCC

Title: 	Discussion on NR UAV requirements

Document for:	Discussion

# 1. Introduction

In the RAN#99 meeting, the last WID on NR support for UAV has added RAN4 [1] as a secondary WG to study and specify necessary UE types and additional OOBE requirements for aerial UEs in 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz based on the operational and technical conditions as defined in ECC Decision (22)07. This paper will focus on these issues and put forward some suggestions.

# 2. Discussion

1. OOBE limits

There are some spectrum operational restrictions to achieve co-existence for UAV. One of them is defining a geographical area where aerial UE is not allowed to operate in a certain frequency band at the national level. Another measure is to define additional OOBE limits to avoid interference to other services.

ECC Decision (22)07 has also listed the OOBE limits for aerial UE operating in the frequency bands 1710-1785 MHz, 2500-2570 MHz, and 2570-2620 MHz to protect some other services. The details are listed below:

- 1710-1785 MHz: Protection of MetSat operating in the 1675-1710 MHz frequency band - Out-of-band limit: -40 dBm/MHz in the frequency range 1675-1710 MHz for aerial UE operating in 1710-1785 MHz. - 2500-2570 MHz / 2570-2620 MHz: Protection of RAS operating in 2690 – 2700 MHz and radars operating in 2700-2900 MHz - Out-of-band limit: -50 dBm/MHz in the frequency range 2690-2900 MHz for aerial UE operating in 2500-2570 MHz or 2570-2620 MHz.

It's worth noting that the OOBE limit for the frequency range 1675-1710 MHz is -40 dBm/MHz, but it is hard for legacy UE to achieve this requirement without any updating. So we recommend a filter with better performance. Consider using NS signaling to support OOBE requirements, this signaling applies only to UAV but not to legacy UE.

Proposal 1: Since it is hard for legacy UE to achieve -40dBm/MHz, we suggest using the filter with better performance.

Proposal 2: Consider using NS signaling to support OOBE, this signaling applies only to UAV but not to legacy UE.

In China, State Radio Regulation Committee also does the co-existence study between UAV and other concurrent services in frequency bands 1710-1785 MHz, 2500-2570 MHz, and 2570-2620 MHz. The detailed analysis is listed below. final results show frequency bands 1710-1785MHz, and 2515-2675MHz can be used for UAV without any additional OOBE requirements.

Table 1. Coexistence analysis

| Frequency band | Interference service | Coexistence conclusion |
| --- | --- | --- |
| 1710-1735MHz | 1718.8-1722.2MHz radio astronomy 1690-1710MHz Meteorological satellite Earth BS | Be able to coexist |
| 1735-1765MHz | 1718.8-1722.2MHz radio astronomy 1690-1710MHz Meteorological satellite Earth BS | Be able to coexist |
| 1765-1785MHz | TDD private network system | Be able to coexist |
| 2515-2675MHz | 2483.5-2500MHz satellite system 2655-2700MHz radio astronomy 2700-2900MHz Radio navigation and positioning | Be able to coexist |

Observation 1: Based on the co-existence study from SRRC, there is no need to define additional OOBE limits for aerial vehicles in the frequency bands 1710-1785MHz, and 2515-2675MHz in China.

Proposal 3: Additional OOBE requirements from ECC are only regional requirements and are not applicable to all UAV UE.

2. UAV type

It’s noted although the UAV and legacy UE could be used in the same frequency bands, there are different sets of additional OOBE requirements for UVA and UE.  These differences would be differentiated by capability signaling from RAN2. so there is no need for another kind of UAV type.

Proposal 4: The UAV type could be differentiated by capability signaling from RAN2, RAN4 does not need to define a new type for UAV.

# 3. Conclusions

In this contribution, some issue for RAN4 are discussed with the following proposals.

Proposal 1: Since it is hard for legacy UE to achieve -40dBm/MHz, we suggest using the filter with better performance.

Proposal 2: Consider using NS signaling to support OOBE, this signaling applies only to UAV but not to legacy UE.

Observation 1: Based on the co-existence study from SRRC, there is no need to define additional OOBE limits for aerial vehicles in the frequency bands 1710-1785MHz, and 2515-2675MHz in China.

Proposal 3: Additional OOBE requirements from ECC are only regional requirements and are not applicable to all UAV UE.

Proposal 4: The UAV type could be differentiated by capability signaling from RAN2, RAN4 does not need to define a new type for UAV.

# 4. References

[1] RP-230782, Revised WID on NR support for UAV, Nokia, Nokia Shanghai Bell