3GPP TSG-RAN WG4 Meeting #106-bis-e									         R4-2304297

Electronic, April 17 – 26, 2023

Agenda item:	5.26.2.1

Source:	Apple

Title:	On R18 MUSIM - General aspects

Document for:	Discussion

# Introduction

R18 MUSIM - General aspects were discussed during the previous RAN4 meetings. The last agreements can be found in [1], in which there are still some open issues.

In this contribution, we continue discussing the open issues.

# Discussion

The remaining issue is mainly about the scope:

Issue 1-1-1: Clarification on the scope

- Proposals

  - P1: Add the following note for the sentence “Case 2: Collisions between MUSIM gap and SMTC” (Qualcomm vivo Huawei)

    - Note: The scope collisions between MUSIM gap and SMTC will be limited to RRM procedures for which collisions between legacy measurement gaps and SMTC are taken into account in the existing requirements

Recommended WF

No consensus

Issue 1-1-2: Considerations on one-shot RRM mobility procedures

- Proposals

  - P1: RAN4 to define the priorities for each procedure in either NW-A or NW-B in desending order as follow. The gaps or resources for higher priority procedures should be kept once the collision happens.

    - Level 1: One-shot RRM mobility procedures in NW-A, such as Handover/ Re-establishment/RRC redirection/SCell activation/SI update;

    - Level 2: Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading;

    - Level 3: Measurements procedures for both NW-A and NW-B

  - P2: No need to consider the collision between SMTC and MUSIM gaps for RRC Re-establishment, RRC Connection Release with Redirection. (vivo)

- Agreements:

  - No need to consider the collision between SMTC and MUSIM gaps for RRC Re-establishment, RRC Connection Release with Redirection

  - FFS on collision between SMTC and MUSIM gaps for handover and Scell activation

The simplest solution is to follow legacy gap design. However, we do understand the concern from proponents of P1 in issue 1-1-2. The intention is to minimize the negative impact on NW A. The difference here compared to the legacy gap scenario includes:

- As more and more gaps can be configured, it becomes more and more challenging for NW A to avoid collision between e.g. SCell activation and measurement gaps. Considering two concurrent gaps in NW A and additional three periodic MUSIM gaps for NW B, there are already five periodic gaps.

- Dropping NW A procedure due to MUSIM gaps may not bring any benefit to NW A. This is unlike legacy gap, wherein at least mobility performance in NW A can be guaranteed in case of collision. If NW A may degrade too much due to MUSIM gaps, NW A may lose interest in providing MUSIM gaps to the UE.

Too avoid complicated design, one possible compromise could be that RAN4 clarifies in high level in our spec that during some one-shot procedure, MUSIM gaps are not expected to be enabled unless the one-shot procedure is not interrupted and can be completed in time. For instance, during SCell activation, UE is not expected to enable MUSIM gaps unless the existing SCell activation delay can still be met.

Proposal 1: add a high-level clarification in RAN4 spec that during one-shot procedure such as SCell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met.

# Conclusion

In this contribution, we provide further discussion on improvement on R18 MUSIM - general aspects. After discussion, the following conclusions are provided:

Proposal 1: add a high-level clarification in RAN4 spec that during one-shot procedure such as SCell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met.

# References

- R4-2303310, WF on NR Dual TxRx Multi-SIM, vivo