3GPP TSG-RAN WG4 Meeting # 106	R4-2304378

# Electronic Meeting, April 17 – April 26, 2023

Agenda Item:	5.9.2.2

Source: 		Qualcomm Incorporated

Title: 			Discussion on L1 part enhancement for FR2 SCell activation

Document for:	Discussuion

# 1.	Introduction

In this paper, we discuss remaining issue about RRM requirements RRM requirements about FR2 unknown Scell activation enhancements from the last RAN4 meeting [1].

# 2. 	Discussion

From the last meeting, RAN4 agreed to report L3 measurement report for unknown FR2 Scell.

Issue 2-1-2: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation (for the case when no valid L3 measurement result is reported after SCell activation command)?

From our understanding, cell detection and L1 measurement can share the same delay requirements which is X TSSB if SSB periodicity is used instead of SMTC period. Some UE may strict follow each delay requirement component, some UE may perform overall process with during delay requirement. By sharing the same delay requirement, it is up to UE implementation how to perform cell detection and L1 measurement during the given measurement requirement.

Proposal: Defining union delay requirements for cell detection and L1 measurement instead of discussing whether skipping L1 measurement. e.g ) X*TSSB where X = total beam sweeping factor for both cell detection and L1 measurement, FFS : X value.

# 3. Conclusions

Proposal: Defining union delay requirements for cell detection and L1 measurement instead of discussing whether skipping L1 measurement. e.g ) X*TSSB where X = total beam sweeping factor for both cell detection and L1 measurement, FFS : X value.

# 4. References

[1] R4-2303228, Way forward on R18 RRM enhancement (part 1), Apple, RAN4 #106