3GPP TSG-RAN WG4 Meeting#106-bis-e	R4-2304393

Online, 4th Apr – 17th Apr, 2023

Source: 	Qualcomm Incorporated

Title: 	Views on PMI Reporting Requirements for FR2 Multi-Rx

Agenda item:	5.8.4.3

Document for:	Discussion

# 1. Introduction

In RAN4#106, it was agreed to introduce performance requirement for CSI reporting. To this end, RAN4 agreed to introduce PMI reporting requirement [1]. After the conclusion of RAN4#106, there were a few open issues that need to be further discussed. In this paper, we share our views on these open issues for PMI reporting requirement.

2. PMI Reporting Requirements

- Proposals

  - Option 1: 4-port per TRP.

  - Option 2: 4 or 8 port per TRP.

Number of CSI-RS Ports

We are of the opinion that 4-port per TRP may not offer much throughput gain for PMI reporting requirement, especially for FR2. Therefore, we prefer to consider 8-port per TRP.

Proposal 1: For PMI reporting requirement, consider 8-port per TRP.

- Proposals

  - Option 1: MCS13.

MCS

In our opinion, the assumptions about channel modelling may influence the choice of MCS selection. Therefore, we think that MCS selection should be discussed once we have finalized the channel modelling.

Proposal 2: RAN4 to discuss MCS selection after finalizing the channel modelling.

- Proposals

  - Option 1: Assume no time and frequency offsets between TRPs.

Time and frequency offsets between TRPs

We would like to note that in Rel-17 performance requirement for mTRP (FR1), it was agreed not to consider time and frequency offsets between TRPs for CSI reporting requirement. Similarly, we support the view of assuming no time and frequency offsets between TRPs for multi-Rx FR2 (option 1).

Proposal 3: Support option 1.

  - Layer indicator (LI) reporting

The LI can be exploited to decide which port PTRS should map to. Therefore, report quantity should include LI in its reporting for FR2.

Proposal 4: CSI report quantity should include LI reporting for FR2.

3. Conclusions

In this paper, we share our views on PMI reporting requirements for FR2 multi-RX tests. The following has been proposed.

Proposal 1: For PMI reporting requirement, consider 8-port per TRP.

Proposal 2: RAN4 to discuss MCS selection after finalizing the channel modelling.

Proposal 3: Support option 1.

Proposal 4: CSI report quantity should include LI reporting for FR2.

4. References

[1] R4-2302941, “WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception,” 3GPP TSG RAN WG4 Meeting #106, Athens, GR, Feb 27 – Mar 3, 2023.