3GPP TSG-RAN WG4 Meeting # 106bis-e                                                       R4-2304431

# Online, April 17 – April 26, 2023

Title: 	TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects”

Source: 	CATT

Agenda item:	5.20.2.2.1

Document for:	Approval

Background

In the draft TR skeleton [1], feasibility is analysed for WA, MR and LA BS. There’s no formal agreement on the feasibility, but for LA BS, the challenge is much less than WA BS as it is analysed in [2]. For the TP on the feasibility, it’ll be better that the spec structures for WA, MR and LA analysis are aligned. It may be a good approach that the general technical analysis method is provided in “10.1 Background for analysis”. For the clauses of WA, MR and LA analysis, only the specific analysis is provided. This contribution provides a draft TP for LA BS feasibility for discussion assuming that general analysis is provided in “10.1 Background for analysis”.

Reference

[1] R4-2302887, “draft TR skeleton of RAN4 part for 38858 and work splitting for TR drafting”, RAN4#106, CMCC

[2] R4-2304433, “Analysis on Local area BS feasibility for duplex evolution”, RAN4#106b-e, CATT

TP for TR 38.858

## 10.4 Feasibility of FR1 Local Area BS aspects

### 10.4.1	Self-interference analysis

For FR1 LA BS SI analysis, the following assumptions are used,

Table 10.4.1-1: SI analysis assumptions for Local area BS

| Parameters | Local Area BS |
| --- | --- |
| Reference sensitivity level degradation due to SI | 1dB |
| Channel bandwidth | 100MHz |
| Subband configuration | {DUD} |
| DL subband width BWDL | 40MHz*2 |
| UL subband width BWUL | 20MHz |
| Tx output power over whole channel PTx | 24dBm |
| Adjacent subband Leakage Power Ratio | 45dB |
| Noise Figure | 13dB |

Using the analysis framework in 10.1, the following analysis can be made.

Table 10.4.1-2: SI analysis for LA BS

| Item |  | Parameters and calculations | example |
| --- | --- | --- | --- |
| General assumption |  | BS total Tx power @ DL SB (dBm) ① | 24 |
|  |  | SBFD configuration | DUD |
|  |  | UL subband width (MHz) ② | 20 |
|  |  | DL subband width (MHz) ③ (Note1) | 40 |
|  |  | Guard band assumption |  |
|  |  | BS NF (dB) ④ | 13 |
| Tx leakage contribution |  | Tx Frequency isolation capability (dB) ⑤ | 45 |
|  |  | DL subband RF filter suppression capability (dB) ⑥ | 0 |
|  |  | Antenna isolation capability (dB) ⑦ | 70 |
|  |  | Tx Beam nulling/isolation capability (dB) ⑧ | NA |
|  |  | DL EIRP impact due to beam nulling in TX sub-band | —— |
|  |  | Digital IC for Tx leakage capability (dB) ⑨ | 3 |
|  |  | Overall RSIC for Tx leakage (dB) ⑩ |  |
|  |  | ⑩=⑤+⑥+⑦+⑨ | 118 |
|  |  | Tx leakage contribution to SI (dBm/UL subband) ⑪ |  |
|  |  | ⑪=①-10*log10(③②)-⑩ | -97.01 |
| Rx path noise contribution | Blocking analysis | Rx beam nulling/isolation capability (dB) ⑫ | NA |
|  |  | RX sensitivity degradation caused by RX beam nulling | —— |
|  |  | UL subband RF filter suppression capability (dB) ⑬ | 0 |
|  |  | Overall RSIC for blocking capability (dB) ⑭ |  |
|  |  | ⑭=⑦+⑬ | 70 |
|  |  | Tx SB interference signal level at Rx path (dBm) ⑮ |  |
|  |  | ⑮=①-⑭ | -46 |
|  | Rx IMD | Rx IIP3 capability (dBm) ⑯ | -16 |
|  |  | Rx IM3 contribution (dBm) ⑰ |  |
|  |  | ⑰=⑮-2*(⑯-⑮) | -106 |
|  | Rx ACS | ACS capability (dB) ⑱ | 55 |
|  |  | Rx ACS contribution (dBm) ⑲ |  |
|  |  | ⑲=⑮-⑱ | -101 |
|  |  | Any other RX impacts if significant (dBm) ⑳(Note 2) | -127 |
|  | Rx path overall noise contribution (dBm) ㉑ |  |  |
|  | ㉑=10*log10(10⑰10+10⑲10+10⑳10) |  | -99.80 |
| Total SI contribution (dBm) ㉒ |  |  |  |
| ㉒=10*log10(10⑪10+10㉑10) |  |  | -95.17 |
| Noise floor (dBm/UL subband) ㉓ |  |  |  |
| ㉓=-174+10*log10②*106+④ |  |  | -87.99 |
| Overall REFSENS degradation (dB) ㉔ |  |  |  |
| ㉔=10*log1010㉒10+10㉓10-㉓ |  |  | 0.76 |
| Note 1：③ is single DL subband width |  |  |  |
| Note 2: If ⑳ is very lower than ⑰ and ⑲, the value of it inserted in the table must be at least 10dB lower than ⑰ and ⑲ |  |  |  |

From the analysis in Table 10.4.1-2, the following capabilities are needed,

Overall RSIC for Tx leakage capability is 118dB

Overall RSIC for blocking capability is 70dB

ACS is 55dBc

IIP3 is -16dBm

The above capabilities are improved compared with legacy LA BS, but it’s feasible from implementation point of view.

### 10.4.2	Co-channel inter-sub-band inter-site interference analysis

This section captures the typical assumption of RF requirements and analysis results

### 10.4.3	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.