3GPP TSG-RAN WG4 Meeting # 106bis-e                                                       R4-2304438

Online, April 17 – April 26, 2023

Title: 	Discussion on NCR-MT feature list

Source: 	CATT

Agenda item:	5.29.1.2

Document for:	Approval

- Background

This contribution provides our views on NCR-MT feature list.

- Discussion

2.1 R-15 feature list

According to the current RAN4 discussion, many NCR-MT requirements may reuse IAB-MT requirements. Although NCR-MT behavior is like UE, BS can identify NCR-MT from normal UE, which is the same as IAB-MT. In the IAB-MT feature list discussion, a minimum set of the feature list were discussed and agreed [2]-[7]. IAB-MT R15 feature list agreement can be considered as the starting point of NCR-MT.

Proposal 1: For R15 feature list, IAB-MT R15 feature list agreement can be considered as the starting point.

2.2 R-16 feature list

The followings are the R16 feature list for UE. All of the features are optional for UE. Considering the features

| Features | NCR-MT |
| --- | --- |
| 4. NR-based access to unlicensed spectrum | Not supported |
| 5. Mobility Enhancement | Not supported |
| 6. LTE_NR_DC_CA_enh | Not supported |
| 7. RF requirements for NR frequency range 1 (FR1) | Not supported |
| 8. NR RF Requirement Enhancements for FR2 | Not supported. It was agreed that beam correspondence requirements are not defined for NCR-MT. |
| 9. Rel-16 NR RRM Enhancement | Not supported |
| 10. NR HST | Not supported |
| 11. NR Positioning Support | Not supported |
| B. Physical layer enhancements for NR URLLC | Not supported |
| C. Enhancements on MIMO for NR | Not supported |
| 12. NR RRM requirements for CSI-RS based L3 measurement | Not supported |
| 2-18 Maximum uplink duty cycle for TDD+TDD EN-DC power class 2 (maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16) | Not supported |
| 2-19 FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum | Not supported |
| 2-20 Maximum uplink duty cycle for FDD+TDD EN-DC power class 2 | Not supported |
| 2-21 Transparent Tx Diversity | Not supported |
| 2-22 (1) powerClass-v1610 (2) ue-powerClass-v1610 | Not supported |
| 2-23 powerClassNRPart-r16 | Not supported |
| 13. 5G_V2X_NRSL | Not supported |

Proposal 2: All of the R16 features can be considered not supported for NCR-MT,.

2.3 R-17 feature list

The followings are the R17 features. All of the features are optional for UE.

NR_pos_enh

NR_ext_to_71GHz

NR_RF_FR1_enh

NR_RF_FR2_req_enh2

NR_HST_FR1_enh

NR_MG_enh

NR_SAR_PC2_interB_SUL_2BUL

NR_PC2_UE_FDD

NR_HST_FR2

NR_UE_pow_sav_enh

NR_demod_enh2-Perf

NR_NTN_Solutions

Higher Power Limit CA_DC

For NCR-MT, none of the R17 features is needed considering the minimum set of features to support the NCR-MT behavior. So all of the R17 features can be considered not supported for NCR-MT.

Proposal 3: All of the R17 features can be considered not supported for NCR-MT.

- Summary

This contribution provides our view for NCR-MT feature list. We have the following proposals.

Proposal 1: For R15 feature list, IAB-MT R15 feature list agreement can be considered as the starting point.

Proposal 2: All of the R16 features can be considered not supported for NCR-MT,.

Proposal 3: All of the R17 features can be considered not supported for NCR-MT.

Reference

[1] R4-2302903, “WF for system parameters and NCR requirements”, RAN4#106, ZTE, CMCC

[2] R4-2005606, “WF on IAB-MT Features”, RAN4#94 Bis-e, Qualcomm

[3] R4-2005608, “Draft LS on RAN4 IAB-MT feature list agreement”, RAN4#94 Bis-e

[4] R4-2009067, “WF on IAB-MT feature list”, RAN4#95e, CATT

[5] R4-2009051, “LS on RAN4 IAB-MT feature list agreement”, RAN4#95e, Qualcomm

[6] R4-2012563, “LS to RAN2 on IAB-MT feature list”, RAN4#96e

[7] R4-2100019, “Reply LS from RAN2 (R4-2100019)”, RAN2

[8] TS 38.822