3GPP TSG-RAN WG4 Meeting #106-bis-e 	R4-2304595

E-meeting, April 17th – 26th, 2023

Agenda Item: 5.11.2.2

Source: MediaTek inc.

Title: Discussion on BWP operation without BW restrictions – Option B-1-1

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#99 and defined a new WI with objectives captured below [1]:

## Objective ### Objective Core part WI ### The work item includes following objectives: - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1)

Therefore, in this contribution paper, we provide analysis and discussion on how to specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions.

# Discussion on RRM requirements impact/workload in RAN4

The existing RAN4 requirements specify that the UE is not required to perform BM/RLM/BFD outside the active BWP. However, in the previous RAN plenary #99, a new WI was introduced to allow the UE to perform L1 (BM/RLM/BFD) measurements based SSB outside the active BWP. To our understanding, the existing requirements for BM/RLM/BFD measurements can be reused and hence RAN4 needs to update the applicability. However, RAN4 should further study the impact on L3 measurements.

- the requirements in sections 8.1, 8.5, and 7.1 for RLM, BFD and UE transmit timing and the requirements for L1 measurements are applicable with CD-SSB outside the active BWP with no changes required.

- RAN4 should further study the impact on L3 measurements.

# Summary

In this contribution, we have the following proposal:

Proposal 1: the requirements in sections 8.1, 8.5, and 7.1 for RLM, BFD and UE transmit timing and the requirements for L1 measurements are applicable with CD-SSB outside the active BWP with no changes required.

Proposal 2: RAN4 should further study the impact on L3 measurements.

# References

- RP-230805, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Rotterdam, Netherlands, March 2023.