# 3GPP TSG-RAN WG4 Meeting # 106bis-e                               R4-2304668

# Electronic Meeting, April 17 – April 26, 2023

# Title: 	Discussion on BS demodulation performance requirements

# Source: 	ZTE Corporation

# Agenda item:	5.14.5.3

# Document for:	Approval

# 1 Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package.

During the last RAN4 meeting, some conclusions has been reached for ATG UL demodulation. which are listed below[2].We want to share some views on ATG BS demodulation performance requirements.

- 2 Discussion

2.1 Test scope

Issue 2-4-1: Test scope Agreement: - PUSCH/PUCCH/PRACH - Option 1: New dedicated requirements required - Option 2: Existing requirements can be applied for ATG BS - For PRACH format: FFS only long preamble format can be considered or both long and short format need to be considered

In last meeting, we made a conclusion that UE can perform pre-compensate for uplink transmission. Based on the conclusion, there is no necessary to consider lager Doppler shift for ATG demodulation. And we made the following conclusions for ATG channel model.

Channel model Agreement: - Candidate channel models for ATG deployment: - Option 1: Single path AWGN channel - Option 2: NTN-TDL-C (FFS for the delay spread and K factor) - Doppler shift assumption: 0.1 ppm

From this agreement, we can find that AWGN or NTN-TDLC channel model are optional for ATG scenario. Based on our observation, whether to consider new requirements for PUSCH/PUCCH/PRACH related to ATG channel model. Next, we will analyze whether to define new requirements for the two channel models one by one. Firstly, If the single tap AWGN channel is used for demodulation evaluation, we notice that the current TS 38.104 only specifies PRACH requirements for AWGN channel model. From that we understand, we need to consider new dedicated requirements for PUSCH/PUCCH /PRACH for AWGN channel model.

In general, UE timing shift and frequency shift pre-compensation are supported for ATG scenario. The cell coverage and UL timing for PRACH would be greatly relaxed. So long formats and short formats could be feasible for ATG scenario. And for long formats, the coverage should be greater than that of the short formats. Also format 0 has a better frequency offset resistance capability. Based on our analysis, we prefer to use format 0 for ATG scenario.

Observation 1. There is no PUSCH and PUCCH requirement defined for AWGN channel model in TS 38.104.

Proposal 1: To define new dedicated requirements for PUSCH/PUCCH for AGWN channel model.

Proposal 2: Only consider long format 0 for PRACH.

- 2.2 Bandwidth & SCS

Issue 2-4-2: Bandwidth & SCS Agreement: - Apply 15kHz SCS for FDD, 30kHz SCS for TDD. FFS channel bandwidth: - For FDD 15kHz, consider all or part of following bandwidths: - 5MHz, 20MHz and 40MHz - For TDD 30kHz, consider all or part of following bandwidths: - 10MHz, 40MHz, 60MHz and 100MHz

In general, we can keep alignment with UE demodulation which support 10MHz for FDD and 40MHz or 100MHz for TDD band.

Proposal 3. To consider 10MHz for FDD band and 40MHz or 100MHz for TDD band.

- 2.3  TDD pattern

Issue 2-4-3: TDD pattern - Option 1: Define new TDD pattern (e.g. 30D4S6U, S=14G) which only applied for ATG scenario. - Option 2: Do not consider TDD pattern impact in ATG demodulation requirements because it is not relevant to receiver demodulation algorithm.

With regard to TDD pattern, it was pointed out that the demodulation requirement is not so relevant to the TDD pattern since the test metric is 70% of max TP. On the other hand, actual deployment and traffic characteristics should also be considered. Therefore a new TDD pattern that reflects actual deployment is more preferred.

Proposal 4. To consider a new TDD pattern( e.g. 30D4S6U, S=14G) for ATG scenario.

- 2.4  Antenna Configuration

Issue 2-4-4: Antenna Configuration - Option 1: 1x2, 1x4, 1x8, 2x2, 2x4, and 2x8. - Option 2: Use same antenna configurations and manufacture declarations as TN BS for ATG demodulation requirements, e.g., 1/2/4Tx and 2/4/8 Rx for 1-C/1-H; and 1/2Tx and 2Rx for 1-O. Agreement: - For BS 1-C/1-H - 1x2, 1x4, 1x8, - FFS 2x2, 2x4, and 2x8 - For BS 1-O - 1x2 - FFS 2x2

- In general, antenna configuration could support 2Tx for BS 1-C/1-H or BS 1-O.

- Proposal 5. To consider 2Tx for BS 1-C/1-H and BS 1-O.

- 2.5  Rank and MCS

Issue 2-4-5: Rank and MCS - Option 1: for rank, both rank1 and rank2; for MCS, 16QAM, 64QAM and 256QAM. - Option 2: For MCS and rank should be selected based on link budget evaluation after the impact of the TN network to the ATG network is clear - Option 3: More robust MCS scheme than HST UE can be considered for PDSCH/PUSCH performance requirements.

With regard to MCS and rank, the MCS corresponding to 16QAM, 64QAM and 256QAM can be considered with rank 1 to be supported.

- Proposal 6. To consider rank1 and 16QAM, 64QAM and 256QAM for ATG performance requirements.

2.6 Transform precoding

Issue 2-4-6: Transform precoding - Option 1: Consider CP-OFDM and DFT-S-OFDM waveform for requirement definition.

- Proposal 7. To consider CP-OFDM and DFT-S-OFDM waveform for requirement definition.

2.7  Other parameters for PUSCH

Table 2-1 . Simulation assumptions for PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric |  | SNR @ %70 of maximum Throughput |

Proposal 8. To consider Table 2-1 for PUSCH simulation assumptions.

- 3 Conclusion

In this contribution, we give some discussions on BS demodulation performance requirements for ATG , The conclusions are:

Observation 1. There is no PUSCH and PUCCH requirement defined for AWGN channel model in TS 38.104.

Proposal 1. To define new dedicated requirements for PUSCH/PUCCH for AGWN channel model.

Proposal 2. Only consider long format 0 for PRACH.

Proposal 3. To consider 10MHz for FDD and 40MHz or 100MHz for TDD band.

Proposal 4. To consider a new TDD pattern( e.g. 30D4S6U, S=14G) for ATG scenario.

Proposal 5. To consider 2Tx for BS 1-C/1-H and BS 1-O.

Proposal 6. To consider rank1 and 16QAM, 64QAM and 256QAM for ATG performance requirements.

Proposal 7. To consider CP-OFDM and DFT-S-OFDM wave form for requirement definition.

Proposal 8. To consider Table 2-1 for PUSCH simulation assumptions.

# 4	Reference

[1]  RP-221369, WID on Air-to-ground network for NR, CMCC.

[2]  R4-2302996, WF for ATG demodulation requirements, CMCC