3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2304686

Online, April 17 – April 26, 2023

Source:	China Telecom

Title:	Phase I study for the advanced receiver for MU-MIMO: simulation results

Agenda Item:	5.19.1.2

Document for:	Information

- Introduction

RAN4 #106 meeting is the first meeting for us to discuss the advanced receiver to cancel inter-user interference for MU-MIMO within the NR_demod_enh3-Perf WI. The WF is approved in [1] as the discussion outcome.

In this paper, our initial simulation results are provided based on the agreed assumptions in [1].

- Simulation Results

| Co-scheduled UE number | Rank for the target UE | Rank for the C-scheduled UE | Modulation order for the co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | SNR (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | R-ML | E-IRC | IRC (baseline) | Gain of R-ML | Gain of E-IRC |  |
| 1 | 1 | 1 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | orthogonal |  |  |  |  |  |  |
|  |  |  |  |  |  | random | 19.6 | 20.7 | 25.5 | 5.9 | 4.8 |  |
|  |  | 3 | 16QAM | 4Tx 4Rx ULA Low |  | orthogonal |  |  |  |  |  |  |
|  |  |  |  |  |  | random |  |  |  |  |  |  |
|  | 2 | 2 | 64QAM |  |  | orthogonal |  | 14.3 | 15.3 |  | 1 |  |
|  |  |  |  |  |  | random |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | orthogonal | 11.8 | 14.3 | 15.3 | 3.5 | 1 |  |
|  |  |  |  |  |  | random |  |  |  |  |  |  |
|  |  |  | 64QAM |  | TDLA30-10 | orthogonal |  | 12.9 | 13.5 |  | 0.6 |  |
|  |  |  |  |  |  | random |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | orthogonal | 11.3 | 12.8 | 13.5 | 2.2 | 0.7 |  |
|  |  |  |  |  |  | random |  |  |  |  |  |  |
| 2 | 1 | 1 for each Co-scheduled UE | 64QAM and QPSK | 4Tx 4Rx ULA medium A | TDLC300-100 | orthogonal |  |  |  |  |  |  |
|  |  |  |  |  |  | random | 29.9 | 35.4 | inf | inf | inf |  |
|  |  |  | 64QAM and 16QAM |  |  | orthogonal |  |  |  |  |  |  |
|  |  |  |  |  |  | random | 27.7 | 35.5 | inf | inf | inf |  |
|  |  |  | 64QAM and QPSK | 4Tx 4Rx XPL medium |  | orthogonal |  |  |  |  |  |  |
|  |  |  |  |  |  | random | 12.6 | 14.4 | 16.1 | 3.5 | 1.7 |  |
|  |  |  | 64QAM and 16QAM |  |  | orthogonal |  |  |  |  |  |  |
|  |  |  |  |  |  | random | 13.6 | 14.4 | 16 | 2.4 | 1.6 |  |

- References

- [1] R4-2302929, WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106, March, 2023