3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304703

# Online, April 17 – 26, 2023

Source:	LG Electronics

Title:	Discussion on co-channel coexistence for LTE SL and NR SL

Agenda item:	5.31.3.3

Document for:	Discussion

Introduction

In [1], RAN4 agreed to identify and specify RRM requirements for co-channel coexistence for LTE SL and NR SL. In this contribution, we provide our views on this case based on RAN1 progress.

Discussion

Based on the RAN1 discussion, some agreements have been made in RAN1

RAN1#109-e Agreement For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed. RAN1#110bis-e Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE Agreement For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission. - FFS: which layer carries out the resource determination: PHY layer or MAC layer. Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: Value of Tmax - FFS: any discussion on the earliest information, if needed RAN1#111 Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms. Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set. RAN1#112 Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharingAlt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied

Based on these RAN1 agreements, the new procedure of resource selection/reselection for NR SL would be defined in RAN1 using some information shared by the LTE SL module, and RAN1 is still under discussion on the detailed procedure. Also, so far, no new UE measurement behavior related to RRM requirements has been identified for this scenario. Therefore, RAN4 needs to wait for RAN1 conclusion.

- Proposal: Wait for further RAN1 progress and conclusion for co-channel coexistence for LTE SL and NR SL.

Conclusion

In this contribution, we provide views on co-channel coexistence for LTE SL and NR SL, and, we propose

- Proposal: Wait for further RAN1 progress and conclusion for co-channel coexistence for LTE SL and NR SL.

Reference

[1] R4-2303265, “WF on R18 NR SL RRM requirements,” LG Electronics.