3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2304752

Electronic Meeting, 17 – 26 April 2023

Source:	Ericsson, Rogers

Title:	TP for TR 38.718-02-01: to include band combination CA_n7A-n12A

Agenda item:			4.10.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.718-02-01to add CA_n7A-n12A configuration as specified in WID [1].

# 2. Text Proposal

---Start of changes---

## 5.X	CA_n7-n12

### 5.X.1	Common for 1 band UL and 2 bands UL CA

#### 5.X.1.1		Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n7 and n12

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500MHz | – | 2570 MHz | 2620MHz | – | 2690 MHz | FDD |
| n12 | 699MHz | – | 716MHz | 729MHz | – | 746MHz | FDD |

#### 5.X.1.2		Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported bandwidths per CA band combination of band n7+n12

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n12A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |

#### 5.X.1.3		UE co-existence studies

Table 5.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n7-n12. It is shown:

There are no harmonic and harmonic mixing issues for CA_n7A-n12A combination.

Table 5.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |

Table 5.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2918 | 2984 | 3645 | 3730 |

#### 5.X.1.4		∆TIB and ∆RIB values

For CA_n7-n12, the TIB,c and RIB,c values are reused from CA_7-12 as specified in TS 36.101 and are given in the tables below.

Table 5.X.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n12 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.X.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n12 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.X.1.5 		REFSENS requirements

As the co-existence studies in 5.X.1.3 show, there are no harmonics and harmonic mixing issues. So, there is no need to define any MSD values.

#### 5.X.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

---End of changes---

# Reference

[1]		RP-230133, Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE