3GPP TSG-RAN WG4 Meeting #106-bis-e                              	R4-2304770

E-meeting, 17th Apr. – 26th Apr., 2023

Source:	Xiaomi

Title:	Discussion on RRM requirements for FR2 HST CA

Agenda Item:	5.13.4.2

Document for:	Discussion

# 1 Introduction

In RAN4#106 meeting, RAN4 had some discussion on RRM impact due to FR2 HST CA operation, and the related WF was approved in [1]. In this contribution, we would like to discuss the RRM requirements for FR2 HST CA and provide our proposals.

- Discussion

- Issue 2-1-2: Discussion on the necessity of the requirements Way forward: - For Rel-18 FR2 PC6, the necessity of the following requirements shall be FFS: - PSS/SSS detection, Time index detection for Intra-frequency and inter-frequency measurements - FFS IDLE mode in inter-frequency measurement requirements - Beam failure recovery on SCell - L1-RSRP measurement on SCell - TCI state switch in SCell - Active SCell measurement requirement (measurement period) - Other requirements are not precluded

In Rel-18 FR2 HST, only intra-band CA scenario is supported. According to the principle defined in Rel15, if there is at least one active serving cell on one FR2 band, the timing of the active serving cell can be applied to other neighbour cell of the same band. Thus, UE is not required to detect PSS/SSS and SSB index for the neighbour cell in intra-band CA scenario.

Proposal 1: RAN4 not to define PSS/SSS detection and SSB index detection requirement for intra-band CA scenario in FR2 HST.

In Rel-17, the requirements of PSS/SSS detection and intra-/inter-frequency measurement are enhanced by a configurable scaling factor under the case of non-DRX and DRX cycle ≤ 80ms, and the FR2 HST enhancement is applied to SMTC periodicity ≤ 40ms. From our perspective, similar enhancement on PCell measurement in R17 FR2 HST could be reused for active SCell measurement in Rel-18 FR2 HST enhancement.

Table 9.2.5.1-11: Time period for PSS/SSS detection when [highSpeedMeasFlagFR2-r17] is configured, (Frequency range FR2) when SMTC period <= 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80ms | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | ceil(1.5 x Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if [highSpeedMeasFlagFR2-r17 = set1] or M1 = 18 if [highSpeedMeasFlagFR2-r17 = set2] NOTE 3: Mpss/sss_sync_w/o_gaps =24. |  |

Proposal 2: For active SCell measurement requirement, the enhanced requirement defined in R17 FR2 HST can be reused in Rel-18 FR2 HST.

In Rel-17 FR2 HST, the BFD requirement was enhanced by a configurable scaling factor under the case of non-DRX and DRX cycle ≤ 80ms, while CBD requirement has not been enhanced. According to TS38.306, the beam failure recovery on SCell is an optional feature as shown in below, RAN4 need to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell. If yes, RAN4 need to define the enhanced requirement for BFD and the enhanced BFD requirement defined in Rel-17 FR2 HST can be reused.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxNumberSCellBFR-r16 Defines the maximum number of SCells configured for SCell beam failure recovery simultaneously. The UE indicating support of this also indicates the capabilities of maxNumberCSI-RS-BFD, maxNumberSSB-BFD and maxNumberCSI-RS-SSB-CBD. | Band | No | N/A | N/A |

Table 8.5.2.2-3: Evaluation period TEvaluate_BFD_SSB for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N Note2) TSSB) |
| DRX cycle ≤ 80ms | Max(50, Ceil(7.5 P N Note2) Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. Note 2: scaling factor N=2 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=6 when highSpeedMeasFlagFR2-r17 is configured to set2.ctor N=6 when [highSpeedMeasFlagFR2-r17] is configured to [set2]. |  |

Observation 1: RAN4 needs to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell.

Proposal 3: The enhanced BFD requirement defined in Rel-17 FR2 HST can be reused in Rel-18 FR2 HST if the FR2 HST UE supports the beam failure recovery on SCell.

- Sub-topic 2-2 Network signaling for Rel-18 FR2 HST CA Scenario Way forward: - HST signaling for CA enhancement: - Option 1: RAN4 postpones the agreement on network signaling until the CA requirements are defined - Option 2: Reuse PCell signaling for SCell in HST - Option 3: Other options are not precluded

Regarding the signaling for FR2 HST CA, we prefer to introduce a new network signaling instead of reusing the existing signaling, as the existing signaling is only applied to non-CA case. And we also fine to postpone the discussion on network signaling until the related CA requirements are defined.

Proposal 4: RAN4 to postpone the agreement on network signaling until the CA requirements are defined.

- Conclusion

In this contribution, we discuss the RRM requirements for intra-band CA in FR2 HST and provide our proposals as follows:

Proposal 1: RAN4 not to define PSS/SSS detection and SSB index detection requirement for intra-band CA scenario in FR2 HST.

Proposal 2: For active SCell measurement requirement, the enhanced requirement defined in R17 FR2 HST can be reused in Rel-18 FR2 HST.

Observation 1: RAN4 needs to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell.

Proposal 3: The enhanced BFD requirement defined in Rel-17 FR2 HST can be reused in Rel-18 FR2 HST if the FR2 HST UE supports the beam failure recovery on SCell.

Proposal 4: RAN4 to postpone the agreement on network signaling until the CA requirements are defined.

- Reference

[1]	R4-2303173,	WF on NR FR2 HST Enhancements RRM requirements, Samsung