3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2304828

# Online, April 17 – April 26, 2023

Title:	Discussion on TRP concept and test method for multi-antenna UE

Source:	Samsung

Agenda item:	5.16.2.1

Document for:	Discussion

1.	Introduction

2TX TRP test method has been discussed for a long time since Rel-17 including TAS, TxD and UL MIMO. In Rel-18 the discussion is focused on TxD and UL MIMO. Till now there is still no convergence on 2Tx test method yet. It becomes more complicated when dynamic TPMI was involved into discussion, as the dynamic TPMI configuration changes the traditional TRP concept from static pattern TRP to an envelope-like combined TRP.

It is a fundamental issue for the group to clarify TRP concept before moving forward. In this contribution, we further discuss the TRP concept for multi-antenna UE and propose how to move forward.

2. 	Discussion

Previous discussion shows that companies have the consensus that TRP test for UL MIMO with dynamic TPMI is testing with dynamic antenna pattern and results in an envelope based TRP. However, existing TRP definition is only based on static antenna pattern even for multiple antenna UE. For example, UE supporting TAS actually show dynamic radiation pattern in field but the TRP test in lab was tested with TAS off and thus its final TRP is based on static radiation pattern. Even for FR2, TRP is already defined with fixed beam transmitting with beam locked mode.

Observation 1:	TRP of TAS UE and TRP of FR2 UE already are measured with static radiation pattern by TAS off and beam locked mode respectively.

When discussing TRP of TxD and UL MIMO, with TxD on and dynamic TPMI approach respectively, actually it also shows dynamic radiation pattern. Without special configuration, we can see that all these scenarios show dynamic radiation pattern including TAS, TxD, UL MIMO, and FR2.

Observation 2:	Without special configuration, all these scenarios show dynamic radiation pattern including TAS, TxD, UL MIMO, and FR2.

For TxD and UL MIMO, if TRP is tested with TxD on and dynamic TPMI respectively, it causes confusion. People could ask why TAS and FR2 are tested with static radiation pattern but TxD and UL MIMO will be tested with dynamic pattern.

Observation 3:	To specify TRP for TxD and UL MIMO with dynamic radiation pattern while specifying TRP for TAS and FR2 with static radiation pattern, cause confusion to the TRP concept.

If TxD on and dynamic TPMI are to be considered for TRP measurement, a new TRP concept has to be defined so that TRP based on static radiation pattern and TRP with dynamic pattern are distinguishable. After looking into this issue with legacy TRP concept and potential new TRP concept separately, current status of TRP status is summarized in Table 1 for comparison.

Table. 1 TRP concept comparison between legacy and new

| Multi-TX scenarios | Legacy TRP |  | Envelop combined TRP |  |
| --- | --- | --- | --- | --- |
| TAS | TRP = max (TRP1, TRP2) | TAS off | Not defined | TAS on |
| TxD | Not defined | ? | Not defined | TxD on |
| UL MIMO | Not defined | ? | Not defined | Dynamic TPMI |
| FR2 | TRP with fixed beam | Beam locked | Not defined | Beam unlocked |

The two kinds of TRP concept are totally different. It is natural way to complete the legacy test method firstly and then consider the new TRP. Especially for TxD and UL MIMO, there is no legacy TRP test method for them yet but now test method for new envelope combined TRP are already under discussion. In our view, it is better not to jump to new envelope combined TRP test method discussion when legacy TRP test method is not available yet for TxD and UL MIMO.

Proposal 1:	For each multi-TX scenario, test method for legacy TRP should be prioritized. RAN4 should not jump to new envelope combined TRP test method discussion when legacy TRP test method is not available yet for TxD and UL MIMO.

From Table 1, it can be observed that legacy TRP are measured with some special configuration (TAS off and beam lock mode) to make the radiation pattern static, while the new envelope combined TRP is based on dynamic condition which is the same as UE behaviour in field.

According to proposal 1, for TxD and UL MIMO, first of all RAN4 need to define legacy TRP test method.

For TxD, there is always controversial view on testing directly with TxD on, because phase variation results into dynamic radiation pattern. Though the conduction test of TxD is measured with TxD on directly, it does not mean also applicable for radiative test. As we can see from RAN5 reply LS on TxD [1], conductive MOP of TxD is measured by summing the mean power of the UE at each antenna connector:

Question 3: Is conductive RF MOP testing for TxD based on testing 1 antenna port at a time and summing two ports or testing 2 antenna ports transmitting simultaneously. Response: Based on the test procedure (snippet below) in sub-clause 6.2G.1.4.2 of TS 38.521-1 V17.7.0, it is seen that the MOP is measured by summing the mean power of the UE at each antenna connector in the channel bandwidth.

In such test method for conductive TxD MOP, phase variation between two antenna connector has no impact to the final summed MOP, but the impact will emerge in radiative test.

So in our view, it is still controversial and there is no valid the test method for legacy TRP of TxD until now. According to the approved work plan [2], test mode can be considered when no other valid test method identified till the end of this meeting.

- RAN4 #106-bis-e (Apr 2023) - Core part - Anechoic chamber test methodology - Conclude AC test methodology to support 2Tx configuration - Test mode can be considered when no other valid test method identified till the end of this meeting

To move forward, we propose to adopt test mode for legacy TRP of TxD, so that phase variation between antennas could be eliminated by either fixing the phase in simultaneous transmission or testing each antenna one by one sequentially.

Proposal 2:	It is proposed to adopt test mode for legacy TRP of TxD, so that phase variation between antennas could be eliminated by either fixing the phase in simultaneous transmission or testing each antenna one by one sequentially.

For UL MIMO, it is simpler as the 2TX radiation pattern could be achieved by TPMI index configuration. For FR2 case RAN4 has following agreement in TR38.884: “2Tx TPMI index 2-5 can force UE single-layer transmission using two antenna ports. Among them, only one proper precoding matrix is selected for EIRP measurement”. In our view it is enough to measure legacy TRP for single layer UL MIMO with TPMI index 2.

Proposal 3:	It is proposed to adopt TPMI index 2 for legacy TRP of single layer UL MIMO.

After progress on test method for legacy TRP of TxD and UL MIMO based on proposal 2 and 3 is made, then RAN4 can further discuss the new envelope combined TRP.

When discussing the new TRP concept for UL MIMO, it is important that the envelop combined TRP concept should be aligned for all multiple antenna UE supporting dynamic radiation pattern including TAS, TxD, UL MIMO and even FR2. With that, the new TRP concept is distinguishable from legacy TRP without confusion.

Proposal 4:	Before discussing the new TRP concept for UL MIMO, envelop combined TRP concept should be aligned for all multiple antenna UE supporting dynamic radiation pattern including TAS, TxD, UL MIMO and even FR2.

Above observation and proposals can be summarized in following table:

| Multi-TX scenarios | Legacy TRP |  | Envelop combined TRP |  |
| --- | --- | --- | --- | --- |
| TAS | TRP = max (TRP1, TRP2) | TAS off | FFS | TAS on |
| TxD | To be defined | Test mode | FFS | TxD on |
| UL MIMO | To be defined | TPMI index 2 | FFS | Dynamic TPMI |
| FR2 | TRP with fixed beam | Beam locked | FFS | Beam unlocked |

In case there would be two kinds of TRP concepts (legacy and new), then there would be issue how to handle the TRP requirements for them. In our view, we don’t think requirement would be different depending on different test method.

Proposal 5:	If new envelop combined TRP concept would be introduced, no new TRP requirements are needed.

3. 	Conclusion

Observation 1:	TRP of TAS UE and TRP of FR2 UE already are measured with static radiation pattern by TAS off and beam locked mode respectively.

Observation 2:	Without special configuration, all these scenarios show dynamic radiation pattern including TAS, TxD, UL MIMO, and FR2.

Observation 3:	To specify TRP for TxD and UL MIMO with dynamic radiation pattern while specifying TRP for TAS and FR2 with static radiation pattern, cause confusion to the TRP concept.

Proposal 1:	For each multi-TX scenario, test method for legacy TRP should be prioritized. RAN4 should not jump to new envelope combined TRP test method discussion when legacy TRP test method is not available yet for TxD and UL MIMO.

Proposal 2:	It is proposed to adopt test mode for legacy TRP of TxD, so that phase variation between antennas could be eliminated by either fixing the phase in simultaneous transmission or testing each antenna one by one sequentially.

Proposal 3:	It is proposed to adopt TPMI index 2 for legacy TRP of single layer UL MIMO.

Proposal 4:	Before discussing the new TRP concept for UL MIMO, envelop combined TRP concept should be aligned for all multiple antenna UE supporting dynamic radiation pattern including TAS, TxD, UL MIMO and even FR2.

Above observation and proposals can be summarized in following table:

| Multi-TX scenarios | Legacy TRP |  | Envelop combined TRP |  |
| --- | --- | --- | --- | --- |
| TAS | TRP = max (TRP1, TRP2) | TAS off | FFS | TAS on |
| TxD | To be defined | Test mode | FFS | TxD on |
| UL MIMO | To be defined | TPMI index 2 | FFS | Dynamic TPMI |
| FR2 | TRP with fixed beam | Beam locked | FFS | Beam unlocked |

Proposal 5:	If new envelop combined TRP concept would be introduced, no new TRP requirements are needed.

4. 	References

- R4-2304018    “LS response on UE TxD for OTA testing”, RAN5

- R4-2217455    “Workplan of Rel-18 TRP TRS WI”, vivo