3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304834

Electronic Meeting, April 17-26, 2023

Source:	Ericsson

Title:	On timing requirements in ATG

Agenda item:	5.14.4.3

Document for:	Approval

# 1	Introduction

In WF from RAN4#106 in Athens it is stated [1]:

… 3. Timing and frequency adjustment 3.1 Timing requirements Issue 3-1-1: How to introduce UE based Timing pre-compensation Agreement: - UE-based timing pre-compensation for ATG network should be supported in case of 15KHz, 30kHz and 60kHz SCS. - Network is expected to provide location information to assist UE-based timing pre-compensation for ATG network - Option 1: Reuse SIB19 for ATG. - Other options are not precluded Issue 3-1-2: Initial transmit timing requirements Te Agreement: - For Initial transmit timing requirements Te, UE pre-compensation timing error should be considered. - The GNSS accuracy is FFS. - Option 1: 50m - Option 2: 30m - Others are not precluded. Issue 3-1-3: Gradual timing adjustment Agreement: - Option 1: the Tq_ATG and Tp_ATG should be 5.5*64*Tc for all SCSs - Option 2: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS. - Other options are not precluded. … 3.2 Guard period as Large TDD cell impaction Issue 3-2-1: Guard period issue due to large TDD cell and coexistence demand - Option 1: RAN4 to investigate remedies to large TDD cells and ATG and TN TDD coexistence like: TDD stand alone, surrendering slots or Full Duplex at ATG UE terminal. - The cell radius can be more than 200 km for ISD 200km. - The Guard period, TGUARD > 1.33 ms for a cell radius of 200 km. - Large TDD cells are inefficient due to large GP, if we switch often. - The largest cell radius will define the GP for all TDD cells and UL/DL configuration has to be the same for all coexisting (overlapping) TDD networks. - If the gNB receives information on the propagation delay to the UE, then at least half of the GP could be removed. - Other Options are not precluded. …

# 2	Discussion

## 2.1	How to introduce UE based Timing pre-compensation

The NTN standard from 3GPP release-17 gives the most flexible and general standard fulfilling the maximum range for any deployment, pointing towards horizon, pointing upwards or general linear deployment for the agreed STS of 200 km.

Observation 1: 	The NTN standard from 3GPP release-17 gives the most flexible and general standard fulfilling the maximum range for any deployment.

Proposal 1: 	Introduce UE based Timing pre-compensation using the NTN standard.

Given that we propose NTN as a solution for UE based Timing pre-compensation, then the Network is expected to provide location information to assist UE-based timing pre-compensation for ATG network. It is possible to reuse a subset of SIB19 for ATG, as a new SIB given that SIB19 is dedicated to NTN.

Proposal 2: 	The Network is expected to provide location information to assist UE-based timing pre-compensation for ATG network using reuse a subset of SIB19 for ATG, as a new SIB given that SIB19 is dedicated to NTN.

## 2.2	Initial transmit timing requirements Te

In NTN the allowed UE position error is 50 m. This is implicit and used to derive the NTN Te requirements. We propose the same values for ATG.

Proposal 3: 	Allowed UE position error is 50 m.

## 2.3	Gradual timing adjustment

We think that option 1: the Tq_ATG and Tp_ATG should be 5.5*64*Tc for all SCSs. This is feasible given that the time drift due to UE movement will be compensated by NTA,UE-specific, therefore, the legacy Tp and Tq value can be reused, which is 5.5Ts, as discussed in [2].

Observation 2: 	Time drift due to UE movement will be compensated by NTA,UE-specific.

Proposal 4: 	The Tq_ATG and Tp_ATG should be 5.5*64*Tc for all SCSs.

## 2.4	Guard period issue due to large TDD cell and coexistence demand

An ISD of 200 km will imply a cell radius of at least 100 km and up to more than 200 km depending on BS site deployment. Figure 1 show upwards deployment where cell radius is at least 100 km, assuming ISD up to 200 km and equal EIRP and radio propagation. Figure 1 also shows a unidirectional arrangement where all beams point to horizon and the cell radius can be larger than the ISD of 200 km depending on aircraft altitude and cell overlap. In fact, cell radius may be independent of the ISD.

Figure 1: ATG BS antenna pointing upwards or using beams towards the horizon

Observation 3: 	The cell radius can be more than 200 km for ISD 200km.

### 2.1.1 Dimensioning of TDD GP

In [3] the TDD GP dimensioning rules are outlined in Figure 2:

Figure 2: TDD GP dimensioning

Where:

- TGUARD :  	Total allocated GP (for both DL to UL and UL to DL guards)

- TAoffset :  Guard period allocated for the UL to DL switch

- TDL_UL :    	Guard period allocated for the DL to UL switch

With some assumptions and simplification listed in [3] we get:

TGUARD  ≥ 2* TSync + 2*Tprop_cell edge +max ((TBS onè off), (TUE offè on)) + max ((TBS offè on), (TUE onè off))

Where:

- TSync: Cell Phase Sync = 3 µs

- Tprop_cell edge: The time to traverse the cell, for the longest distance between the BS site and any UE at the far away cell border.

Figure 3: BS-to-BS interference distance

In our case ISD of up to 200 km results in a Tprop_cell edge time corresponding to a distance of more than 200 km, for BS antenna using beams towards the horizon (Figure 1) or Tprop_cell edge > 100 km in a linear deployment with ATG BS antenna pointing upwards (Figure 1). Finally, Tprop_cell edge > 200*(2/3) km > 133 km for hexagonal deployment (Figure 3). The GP is dimensioned using RTT = 2* Tprop_cell edge for all cases.

Observation 4: 	The Guard Period, TGUARD > 1.33 ms for a cell radius of 200 km.

## 2.1.2 Large TDD cells

The Guard Period in TDD normally has to consider the RTT = 2* Tprop_cell edge to make room for the UE moving UL transmission time, to an earlier point in time, based on DL timing reference and TA command, when it moves further away from the BS. This means that we have to set aside a Guard Period (GP) each time we switch from DL to UL. This is shown in Figure 4.

Figure 4: Guard Period in TDD cells

Observation 5: 	Large TDD cells are inefficient due to large GP, if we switch often.

TDD networks are nominally all synchronized to the same source and carry the same UL/DL configuration and periodicity. This is an issue for a TDD network with large cells, coexisting with a network with normal to small cells.

Observation 6: 	The largest cell radius will define the GP for all TDD cells and UL/DL configuration has to be the same for all coexisting (overlapping) TDD networks.

## 2.1.3 Large ATG cells and potential remedies

Potential remedies to solve the fact that we have large GP for large TDD cells and that this GP has to be the same across coexisting networks include:

- FDD: An FDD-network will always avoid the need for GP altogether. FDD is full duplex at the expense of separate UL and DL bands. This will not solve issue for TDD band like ATG WID model band in 4.8 GHz.

- TDD ATG stand alone: A stand-alone ATG network which does not coexist can switch UL and DL direction less often. If we switch infrequently, then the large GP overhead is not so large, on average. However, we will lose latency due to infrequent TDD switching.

- TDD ATG coexisting with regular terrestrial (TN) TDD: ATG TDD and TN TDD synchronize to common source and coordinate UL and DL configurations and the ATG TDD network surrender DL slots (x) to the TN TDD network:

ATG: DDDDDDDDDDxxxDDDDDDDDDDxxxDDDDDDDDDDxxxDDDDDDDDDDssssssssssUDDDDDDDDDDxxx
TN:    DDDDDDDDDDuuuDDDDDDDDDDuuuDDDDDDDDDDuuuDDDDDDDDDDssssssssssUDDDDDDDDDDuuu

- UE TDD Full Duplex: If TGUARD is deliberately under dimensioned then UE DL (RX) and UE UL (TX) will meet in time, occasionally. This is a complex arrangement, even if an aircraft fuselage might offer some possible TDD TX and RX isolation. This is shown in Figure 5.

Figure 5: Under dimensioned Guard Period in TDD cells

### 2.1.4	Reduction of the overhead of the guard period

Consider an example of a 300 km ATG cell as a baseline scenario. The round-trip time is 2msec. The TDD frame is organized as 10 slots of 0.5msec each, numbered from 1 to10, for SCS = 30 kHz. Slots 1-7 should be downlink slots, slot 8 a “special” slot (with most symbols configured for downlink) and slots 9 and 10 are allocated for uplink.

Due to the propagation time to the cell edge, the gNB needs to allocate slots 5, 6, 7 and 8 as a guard period, for this SCS = 30 kHz example. However, all SCS is possible, i.e., 15, 20, 60, 120, 240, 480 and 960 kHz and differs only by scaling the number of slots based on example considered here.

The TDD structure from the UE perspective is as depicted in the figure 6. The UL begins to receive the downlink part of each frame with 2 slots, or 1msec delay. In order that the UE UL transmission arrives at the gNB at the correct time, the UE needs to switch to UL after receiving DL slot 4. After transmitting the two UL slots, at the UE the timing is such that slots 7 and 8, which are guard slots occur. Following the guard slots, the following two slots, marked as “X” in the figure are slots in which the UE does not receive any data because they correspond to the slots during which the gNB received the UL transmission, delayed by 1msec.

Figure 6: Large TDD cell with large GP.

If the gNB receives information on the propagation delay to the UE, the gNB allocates a guard period starting at twice the propagation delay (i.e., the RTT) earlier than the UL period for a duration equal to the UL period. However, immediately after this guard period, the gNB resumes transmitting DL data to the UE until the UL period commences.

At the UE, the UE is configured such that it applies timing advance to the UL signal (as usual) and then assumes that DL data can commence immediately after it has finished transmitting UL. This is shown in figure 7.

In this way, half of the guard period slots can be removed. From the UE perspective, there is no guard period, however there are two slots within which the UE does not receive downlink (due to no DL transmission during those slots because of UL reception at the gNB).

Figure 7: Large TDD cell with large GP reduced by a factor of 2, ATG UE far away.

If the cell size is 300 km, but the ATG UE is located at a distance of 150 km from the gNB. The propagation time is 0.5msec or 1 slot. This is shown in figure 8. The gNB is made aware that the propagation time is 0.5 msec. The gNB provides a guard period equal to the propagation time of 0.5msec at 2 slots, or one RTT prior to the UL slots at the gNB, i.e., in slot 7. This is as before, in first example, but scaled to smaller RTT. Due to the fact that the UL is 2 slots in duration, the gNB cannot transmit to the UE in slot 8 because, after propagation delay, the UE will still be transmitting UL during that time.

From the UE perspective, the UE receives the first 6 DL slots and then switches to UL. After transmitting UL, the UE is does not receive any DL in the following 2 slots, as these slots correspond to the time at which the gNB was receiving the UL signal.

Figure 8: Large TDD cell with large GP reduced by a factor of 2. ATG UE closer.

If the UE is located at 75 km from the gNB. The propagation time is now 0.25 msec, or half of one slot. This is shown in figure 9.

The gNB transmits DL for slots 1-7. A half slot guard period, again equal to the propagation time in slots, is inserted at 1 RTT = 0.5 msec (1 slot) prior to the UL period at the start of slot 8. The gNB cannot transmit during the remainder of slot 8 because the DL transmission would overlap with the UE UL transmission.

From the UE perspective, the UE receives the first 7 DL slots and then switches to UL. After the UL slots, the UE does not receive anything for 1 slot, since this slot corresponds to the second UL slot reception at the gNB.

Figure 9: Large TDD cell with large GP reduced by a factor of 2. ATG UE even closer.

Observation 7: 	If the gNB receives information on the propagation delay to the UE, then at least half of the GP could be removed.

# 3 	Summary

Observation 1: 	The NTN standard from 3GPP release-17 gives the most flexible and general standard fulfilling the maximum range for any deployment.

Proposal 1: 	Introduce UE based Timing pre-compensation UE based timing pre compensation using the NTN standard.

Proposal 2: 	The Network is expected to provide location information to assist UE-based timing pre-compensation for ATG network using SIB19, or a new SIB which may be a subset of SIB19.

Proposal 3: 	Allowed UE position error is 50 m and allowed aircraft position error is 30 meters.

Observation 2: 	Time drift due to UE movement will be compensated by NTA,UE-specific.

Proposal 4: 	The Tq_ATG and Tp_ATG should be 5.5*64*Tc for all SCSs.

Observation 3: 	The cell radius can be more than 200 km for ISD 200km.

Observation 4: 	The Guard Period, TGUARD > 1.33 ms for a cell radius of 200 km.

Observation 5: 	Large TDD cells are inefficient due to large GP, if we switch often.

Observation 6: 	The largest cell radius will define the GP for all TDD cells and UL/DL configuration has to be the same for all coexisting (overlapping) TDD networks.

Observation 7: 	If the gNB receives information on the propagation delay to the UE, then at least half of the GP could be removed.

# References

[1]	 	R4-2303226, 	WF on NR ATG RRM requirements, CMCC

[2]		R4-2300781, Discussion on RRM timing requirements for ATG, CMCC

[3]		R4-1703013, CR on study TR proposal, TDD timing budget, Ericsson