3GPP TSG-WG RAN4 Meeting #106-bis-e	R4-2304878

# Online, April 17 – 26, 2023

Source: 				Nokia, Nokia Shanghai Bell

Title: 				LTE based 5G UE RF open issues

Agenda Item: 	6.3.3

Document for:	Approval

# Introduction

In this contribution, open issues on band plan are discussed according to the agreed WF [1] in RAN4#106 as shown below.

# Band plan From [1] two band plan options not mutually exclusive were identified - Define a single band covering the entire UHF frequency range 470 – [698/702] MHz - The requirements are relaxed for this band (TBD which requirements and how much) and/or - This band is assumed to be implemented with more than one filter in the UE (TBD how many filters and the characteristics of those filters) - Define smaller bands - At least one of the bands overlaps with Band n105 DL to reuse n105 Rx filter - Specify the following bands 470-542 MHz, 540-606 MHz and 602-702 MHz. - Other Agreement from online session: - Define the band plan based on available data for BS and UE. - Full UHF band, i.e., 470 – 698MHz, - with reduced UE blocking - Narrow band, i.e., 612 – 652 MHz, which is the same as the downlink operating band of n105. - If data for other frequency ranges becomes available before the conclusion of the work item, it can be considered either as a separate band or as a modification to initially defined bands. Way Forward - Further clarification and study is needed to elaborate on and quantify requirements with respect to reduced UE blocking for the full UHF band. - All technical requirements for these bands are still to be defined.

# Discussion

Full UHF band with reduced UE blocking is being discussed although there has been no consensus what relaxation is allowed for the UE blocking. 700 MHz band 28 is widely deployed nowadays; thus coexistence with this adjacent band is crucial for the deployment of 5G broadcast service. However, only 5 MHz separation is there between 698 MHz and 703 MHz (band 28 lower band edge).

Observation 1: Coexistence with band 28 is crucial for 5G broadcast band.

As we have already discussed in [2], 5G broadcast receiver requirement in EN standard [3] considers the coexistence with band 28 (Test case 3 in Table 5), where 9 MHz frequency separation between 694 to 703 MHz is used. A single full band filter would not provide sufficient attenuation of such band 28 signals. At least upper part of the full UHF band needs to be covered by a sub-band filter that can ensure the coexistence with band 28.

Observation 2: At least upper part of the full UHF band needs to be covered by a sub-band filter that can ensure the coexistence with band 28.

For lower part of the full UHF band, stringent out-of-band blocking would not be required; so relatively large bandwidth in sub-band filter would be allowed. The number of filters can be left for implementation.

Proposal 1: For full UHF band, it is proposed to assume multiple UE Rx filters.

# Conclusion

Observation 1: Coexistence with band 28 is crucial for 5G broadcast band.

Observation 2: At least upper part of the full UHF band needs to be covered by a sub-band filter that can ensure the coexistence with band 28.

Proposal 1: For full UHF band, it is proposed to assume multiple UE Rx filters.

# Reference

[1] R4-2303489 WF on RF requirements for 5G Broadcast, Qualcomm, RAN4#106

[2] R4-2219708 LTE based 5G broadcast band definition, Nokia, Nokia Shanghai Bell, RAN4#105

[3] ETSI EN 303 340 Digital Terrestrial TV Broadcast Receivers; Harmonised Standard for access to radio spectrum