3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2304890

Electronic Meeting, Apr 17- Apr 26, 2023

Agenda Item:	5.10.3.1

Source:	Ericsson

Title:	Discussion on measurement for NeedForGaps

Document for:	Discussion

# Introduction

In last meeting, RAN4 discussed NeedForGaps requirement [1]. The following terminologies are used during the discussion.

- Case 1: without gap and no interruption (e.g. ’[TBD1]’ indicated in [TBD new signaling]) - Case 2: without gap but interruption allowed (e.g. ’[TBD1]’ indicated in [TBD new signaling])

In this contribution, we will continue to discuss the requirement on NeedForGaps measurement.

# Interruption

Interruption length

In last meeting, RAN4 agreed to define the interruption length.

Issue 1-1-2: Framework of the interruption requirements < Agreement >: - The following aspects will be defined in the requirements of interruption: - Interruption length

In our understanding, the measurement behaviour for NeedForGaps is similar as deactivated SCell measurement which defined both interruption length and the interruption ratio in the specification. UE will only choose one of SMTCs to perform deactivated SCell measurement during the configured measCycleSCell period. The interruption for NeedForGaps measurement is also due to RF switching which is the same as deactivated SCell measurement. Thus, we suggest to define the interruption length as 0.5ms.

Proposal 1: The interruption length equals 0.5ms for both FR1 and FR2 when UE reports ‘interruption’ in NeedForGaps.

Interruption ratio

In last meeting, RAN4 agrees an WF to use interruption ratio to define the NeedForGaps UE behaviours.

Issue 1-1-2: Framework of the interruption requirements < Way forward >: - Interruption ratio can be defined depending on the measurement cycle length and interruption length as: - Option 1: - with up to [1.25%] probability of interruption per a UE measurement sample cycle when it is NOT less than [160ms] ms - FFS on whether and how to define the interruption ratio requirements when the UE measurement sample cycle is less than [160ms] - Other options not precluded - FFS on possible measurement delay requirements extension - FFS whether there is a need to define the interruption location

In our understanding, UE will use the similar implementation for both NCSG and NeedForGaps feature. If RAN4 defines NeedForGaps requirement with the dedicated interruption location, the NeedForGaps feature will be the same as NCSG feature. When NeedForGaps capability is defined based on interruption ratio other than interruption location, it means both UE and NW can have an alternative for NCSG.

Proposal 2: When UE supports NeedForGaps capability, RAN4 to define UE behaviours based on the interruption ratio.

When UE reports ‘no-gap’ measurements with interruption, the interruption may be expected before and after any SMTC outside gap. The network doesn’t know the dedicated SMTC occasions in which UE performs the measurements. If the interruption ratio follows the SMTC periodicity, it will result in too much performance degradation once several frequency layers’ measurements are configured with ‘no gap’. Thus, RAN4 needs to discuss how to control the total interruption ratio for NeedForGaps.

Proposal 3: RAN4 to agree that the total interruption ratio for NeedForGaps should be controlled instead of performing measurement on each possible SMTC occasion.

To control the total interruption ratio, a possible solution is to introduce an additional scaling factor to extend the measurement delay for NeedForGaps’ measurement. When UE performs NeedForGaps intra-/inter-frequency measurement with a longer delay, the relative interruption ratio will be reduced.

Proposal 4: RAN4 to introduce a NW indicator KNeedForGaps to reduce the total interruption ratio.

On the other hand, the lower bound in delay requirement can also be a good method to restrict the interruption ratio. When the measurement delay requirement is defined based on a lower bound, it’s equivalent to introduce a higher bound for the total interruption.

Proposal 5: RAN4 to introduce the lower bound for NeedForGaps measurement periodicity, such as 80ms.

General principle for interruption ratio

As we discussed above, the general interruption ratio is determined by the interruption control factor, measurement lower bound. Furthermore, the interruption ratio for each frequency layer is also based on the CSSF which represents UE to perform one shot measurement within multiple SMTC samples.

Proposal 6: RAN4 to define the interruption ratio for NeedForGaps at least considering the following factors

- Interruption higher bound

- CSSF

- Interruption controller factor

When no DRX is configured, the detail equation of interruption ratio for frequency layer #i is shown as follow.

Proposal 7: When no DRX is configured, the interruption ratio for frequency layer #i in which UE reports ‘interruption needed’ is min(K, 2*L/(KNeedForGaps,i*SMTCi*CSSFi)) in NeedForGaps.

Where,

L is the single interruption length for measurements with NeedForGaps capability;

KNeedForGaps,i is the interruption ratio controller indicator for frequency layer #i;

SMTCi is the SMTC periodicity for frequency layer #i;

CSSFi is the CSSF for frequency layer #i;

K=2*L*measurement samples/(lower bound of the measurement delay) is the higher bound of the interruption ratio.

DRX based interruption

Another important issue for NeedForGaps measurement is DRX-based requirement. NeedForGaps capability is defined to not limit the interruption location which means UE can have a more flexible design than other features to perform measurement in any SMTC occasion regardless of NW configuration. Thus, it’s important to trade-off between the NeedForGaps flexible design and the interruption ratio. We see the possibility to introduce a zero interruption in DRX mode.

On the one hand, considering SMTC is broadcast signals, but DRX is UE specific configuration, it is hardly for NW to align the SMTC duration and the DRX ON duration for UEs. Consequently, it means the NeedForGaps measurement will always misalign with the DRX ON duration and no interruption is expected. On the other hand, when NW configures a long DRX(DRX>320ms), multiple SMTCs will be contained within one DRX cycle. Thus, it’s easily to find a suitable SMTC outside DRX ON duration. The only issue is for short DRX(DRX<=320ms).

Proposal 8: RAN4 to discuss the DRX-based NeedForGaps requirement based on the following scenarios

- Short DRX(DRX<=320ms)

- Long DRX(DRX>320ms)

When UE is configured a short DRX, frequent wake-up will result in additional power consumption. One possible solution for UE is to utilize the DRX ON duration to perform both data reception and SMTC measurement. However, it means UE had to endure the performance loss due to interruption and inaccurate AGC. Another solution is to wake up before the DRX ON duration for SMTC measurement. After that, UE needs to wake up again for data reception. In Rel-15, this issue has widely discussed and a further scaling factor 1.5 was introduced to trade-off the additional SMTC wake-up and power consumption. Thus, UE shall wake up to perform the measurement before the DRX ON duration. The total measurement delay requirement is extended, and less interruption is expected.

We calculate the worst interruption ratio for DRX cycle=320ms, with interruption length equals 1ms, no interruption controller indicated and CSSF=2. The upper bound of the interruption ratio will be 0.3%. Considering the DRX ON misalignment and power consumption scaling factor, the real interruption ratio for short DRX will be in a very low level.

Observation 1: In Rel-15, RAN4 had already solved the power consumption issue for short DRX measurement by further scaling factor 1.5.

Another important scenario for DRX case is that the SMTC occasions are misaligned with DRX ON duration. SMTC configuration is a cell specific configuration, but DRX configuration is UE specific which implies the SMTC configuration may highly misalign with UE DRX configuration. That’s the main reason to introduce further scaling factor in short DRX scenario. Thus, RAN4 shall further consider at least the case when SMTC occasions are fully misaligned with DRX ON duration. We think zero interruption is also expected in this case when DRX is equal or smaller than 320ms.

Figure 1. DRX ON duration misaligned with SMTC occasions

Proposal 9: RAN4 to discuss the possible interruption ratio when DRX is configured,

- When DRX cycle is equal or smaller than 320ms,

  - no interruption is expected when configured SMTC occasions are misalignment with DRX ON duration;

  - otherwise, the interruption ratio is min(K, 2*L/(KNeedForGaps,i *1.5* max(DRX cycle, SMTCi) *CSSFi)).

- When DRX cycle is larger than 320ms, no interruption is expected.

# Measurement delay requirement

Case 1: Measurement delay requirement

The open issue related to measurement delay requirement when UE reports ‘no gap no interruption’ in NeedForGaps is shown as follow. In our understanding, all the candidate proposals are not mutual exclusive to each other.

Issue 1-2-2: Requirement for inter-freq measurement without gap (Inter-f case 1) < Way forward >: - Proposal 1: CATT, CMCC, Huawei - to update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates ‘no-gap’ via interFreq-needForGap - Proposal 2: Intel, CATT, Huawei - Updates/Clarification on CSSFoutside_gap. - Proposal 3: Nokia, ZTE, Huawei - Define measurement reporting delay requirements for UEs indicating no-gap with interruption considering both deriveSSB-IndexFromCellInter-r17 enabled and disabled

The delay requirement when UE reports ‘no gap no interruption’ will follow inter-frequency without gap requirement. The CSSFoutside_gap needs to update to include the additional number of frequency layers which UE reports ‘no gap no interruption’ in NeedForGaps. deriveSSB-IndexFromCellInter-r17 should also be considered in scheduling restriction.

Proposal 10: RAN4 to agree the measurement requirement for NeedForGaps case 1 in FR1 as follow.

Table 1. Measurement period when UE reports ‘no gap no interruption’ in NeedForGaps for inter-frequency measurements (FR1)

| DRX cycle | T SSB_measurement_period_NeedForGaps_wo_interuption |
| --- | --- |
| No DRX | max(200ms, ceil( 8 x Kp) x SMTC period) x CSSFNeedForGaps_no_interrupt |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 8 x Kp) x max(SMTC period, DRX cycle)) x CSSFNeedForGaps_no_interrupt |
| DRX cycle>320ms | ceil( 8 x Kp ) x DRX cycle x CSSFNeedForGaps_no_interrupt |

Proposal 11: RAN4 to agree the following bullets CSSFNeedForGaps_no_interrupt as follow.

- CSSFNeedForGaps_no_interrupt is determined according to CSSFoutside_gap,i for measurement conducted outside MG or according to CSSFwithin_gap,i for measurement conducted within measurement gaps.

Proposal 12: RAN4 to update the CSSFoutside_gap,i to add additional factor NNeedForGaps_no_interrupt , where NNeedForGaps_no_interrupt is the number of configured MOs in the bands which UE reports ‘no gap no interruption’ in NeedForGaps and not fully overlapping with the MG; otherwise, it is 0.

Case 2: Measurement delay requirement

The open issue related to case 2 measurement delay requirement when UE reports ‘no gap with interruption’ in NeedForGaps is shown as follow.

Issue 1-2-1 Requirement for intra/inter-freq measurement without gap when interruption allowed (case 2) < Way forward >: - Option 1: - Take requirements NCSG requirements in TS38.133 clause 9.3.10 as a starting point - The other aspects can be FFS. e.g. - The time slot alignment among the measurement objects and interruption location - Option 2: - The deactivated SCell measurement requirement can be the start point in case of interruption location is unknown. - Option 2a: - The deactivated SCell measurement except the measCycleSCell can be a start point - To reduce the total interruption ratio, some trade-off solutions for extending the measurement can be - introducing a lower bound, such as [80]ms, or - introducing a scaling factor KNeedForGaps, such as KNeedForGaps =[2] - Option 3: - Take requirements in 38.133, clause 9.3.9 as a starting point

We can see that the measurement period is related to a clear VIRP pattern in NCSG, but NeedForGaps doesn’t have any pattern. Thus, RAN4 cannot follow NCSG to define NeedForGaps’ measurement requirement directly.

Observation 2: RAN4 cannot follow NCSG to define NeedForGaps’ measurement requirement since no pattern design for NeedForGaps.

In Rel-15, deactivated SCell is measured without gap but with interruption. The deactivated SCell is counted in CSSF outside gap and the interruption requirement is also defined. In our understanding, the frequency layer with ‘no gap with interruption’ can follow the same behaviour as deactivated SCell measurement.

Proposal 13: RAN4 to agree the lower bound as 640ms for interruption control in NeedForGaps case 2.

Proposal 14: RAN4 to update the CSSFoutside_gap,i to add additional factor NNeedForGaps_with_interrupt , where NNeedForGaps_with_interrupt is the number of configured MOs in the bands which UE reports ‘no gap with interruption’ in NeedForGaps and not fully overlapping with the MG; otherwise, it is 0.

Proposal 15: RAN4 to agree the measurement requirement for NeedForGaps case 2 outside gap in FR1 as follow.

Table 2: Measurement period when UE reports ‘interruption’ in NeedForGaps for inter-frequency measurements (FR1)

| DRX cycle | T SSB_measurement_period_NeedForGaps_with_interuption |
| --- | --- |
| No DRX | max([640ms], 8 × KNeedForGaps × SMTC period) × CSSFoutside_gap |
| DRX cycle≤ 320ms | max([640ms], ceil(1.5x 8) x KNeedForGaps × max(SMTC period, DRX cycle)) x CSSFoutside_gap |
| DRX cycle>320ms | 8 x DRX cycle x CSSFoutside_gap |

When UE reports ‘NeedForGaps’ for a specific band, however, the MOs within the band have fully overlapping with MG. In this case, the MOs had to be measured within MG. In our opinion, the KNeedForGaps is invalid in this case and the requirements shall be as follow.

Proposal 16: RAN4 to agree the measurement requirement for NeedForGaps case 2 within gap as the same with legacy measurement within gap as follow.

Table 3: Measurement period when UE reports ‘interruption’ in NeedForGaps for inter-frequency measurements (FR1)

| Condition | T SSB_measurement_period_NeedForGaps_with_interuption |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP , SMTC period)) CSSFwithin_gap |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFwithin_gap |
| DRX cycle > 320ms | 8 DRX cycle CSSFwithin_gap |

# Scheduling restriction

When RAN4 defines the scheduling restriction for NeedForGaps, the general principles can be the same as NCSG. The new capability ‘deriveSSB-IndexFromCellInter-r17’ should also be considered in NeedForGaps.

Issue 1-4-1: General principles to define scheduling restriction requirements < Way forward/ >: - Option 1: Nokia, vivo, OPPO - whether the UE supports simultaneousRxTxInterBandCA in FR1. - whether deriveSSB-IndexFromCellInter-r17 is enabled and supported by the UE in FR1 and FR2. - Option 1a: Nokia, OPPO - whether the UE supports simultaneousRxTxInterBandCA in FR1. - whether deriveSSB-IndexFromCellInter-r17 is enabled and supported by the UE in FR1 and FR2. - whether IBM is supported in FR2. - Option 2: Qualcomm - No need to introduce scheduling restriction due to interruption for performing inter-frequency measurements. Issue 1-4-2: On top of which existing requirements to define scheduling restriction requirements < Way forward >: - Option 1: Apple, CMCC, Intel, OPPO, Huawei, MTK, E/// - take the similar requirements for NCSG (TS38.133 v17.6.0 9.3.10.3) as baseline to define scheduling availability - Option 1a: Huawei - The scheduling restriction applies regardless of whether interruption is allowed (for both case 1 and case 2) - Option 2: CATT - Reuse the scheduling availability requirements from intra/inter-frequency without gaps 9.2.5.3 or 9.3.9.3 for UEs reporting no-gap but with interruption. - Option 3: vivo - If RAN4 agrees to define total interruption ratio without specifying location and length, no need to define scheduling restriction

Proposal 17: Define scheduling restriction requirements for NeedForGaps similar as NCSG.

# NeedForGaps and NCSG mapping and mismatch

There is a possible mismatch issue between NW and UE with different gapless capability. Especially, when both UE and NW support NCSG and NeedForGaps. A typical use case is when UE transfers from DRX to non-DRX, to avoid the interruption to data, NW may configure a NCSG pattern. On the convey, when UE transfer from non-DRX to DRX, to reduce the interruption, NW may further configure NeedForGaps and release NCSG pattern. Frequent large signalling interaction is needed when NW change the measurement configuration. To simplify the signalling interaction, when UE reports NCSG, it’s better to allow NW to understand UE’s behaviours for NeedForGaps.

Issue 1-3-1: Mapping between NeedForGap and NCSG capabilities when UE supports both of them < Way forward >: - FFS on: - [NeedForGapsInfoNR] and NeedForGapNCSG-InfoNR are not expected to be enabled for the same UE at the same time - No need to establish the mapping between UE’s indication for NeedForGaps and NCSG Issue 1-3-2: UE behaviors mismatch between UE and NW < Way forward >: - FFS on: - No impact on Rel-18 NFG requirements because of mismatch scenarios where either UE or NW support Rel-17 or earlier release. - The requirements of Rel18 NFG will not be applicable to these mismatch scenarios - Rel-17 UE which supports NCSG in a Rel-16 NW which only supports NeedForGaps - Rel-16 UE which supports NeedForGaps in a Rel-17 NW which supports NCSG - Both UE and NW support NCSG and NeedForGaps - Others are not precluded

For example, if UE supports both NeedForGaps and NCSG, UE reports the following gap status in NCSG.

Table 4. The example of gap status indication for UE supporting NCSG

| CC | B1 | B2 | B3 | B4 | B5 | B6 |
| --- | --- | --- | --- | --- | --- | --- |
| B1+B2 (Pcell+Scell) | 2 | 1 | 0 | 0 | 0 | 1 |

Note: (‘0’: gap, ‘1’: NCSG, ‘2’: no gap no interruption)

The gap status indication in NeedForGaps should have 1-to-1 mapping with the gap status in NCSG with the following rules.

- UE should report ‘no gap’ in the same band for NeedForGaps if reporting ‘no gap no interruption’ or ‘no gap no interruption’ in a band for NCSG

- UE should report ‘gap’ in the same band for NeedForGaps if reporting ‘gap’ in a band for NCSG

The mapping is mainly used to save the signalling interaction

In last meeting, after some further clarification with other companies, we see the concern due to the uncertain requirement and unclear UE behaviours for NeedForGaps feature. Thus, we suggest to postpone the discussion until RAN4 has a clear understanding on NeedForGaps requirement.

Observation 3: The benefits of 1-to-1 mapping between NeedForGaps and NCSG is to avoid the frequent large signalling interaction.

Proposal 18: NeedForGaps and NCSG are not expected to be enabled for the same UE at the same time, but NW can alternatively switch between NeedForGaps and NCSG once both UE and NW support NeedForGaps and NCSG.

Proposal 19: RAN4 to postpone the 1-to-1 mapping between NeedForGaps and NCSG capabilities until RAN4 has a clear understanding on NeedForGaps requirement.

# Conclusion

In the contribution, we discuss the NeedForGaps in Rel-18. We have the following proposals:

Observation 1: In Rel-15, RAN4 had already solved the power consumption issue for short DRX measurement by further scaling factor 1.5.

Observation 2: RAN4 cannot follow NCSG to define NeedForGaps’ measurement requirement since no pattern design for NeedForGaps.

Observation 3: The benefits of 1-to-1 mapping between NeedForGaps and NCSG is to avoid the frequent large signalling interaction.

Proposal 1: The interruption length equals 0.5ms for both FR1 and FR2 when UE reports ‘interruption’ in NeedForGaps.

Proposal 2: When UE supports NeedForGaps capability, RAN4 to define UE behaviours based on the interruption ratio.

Proposal 3: RAN4 to agree that the total interruption ratio for NeedForGaps should be controlled instead of performing measurement on each possible SMTC occasion.

Proposal 4: RAN4 to introduce a NW indicator KNeedForGaps to reduce the total interruption ratio.

Proposal 5: RAN4 to introduce the lower bound for NeedForGaps measurement periodicity, such as 80ms.

Proposal 6: RAN4 to define the interruption ratio for NeedForGaps at least considering the following factors

- Interruption higher bound

- CSSF

- Interruption controller factor

Proposal 7: When no DRX is configured, the interruption ratio for frequency layer #i in which UE reports ‘interruption needed’ is min(K, 2*L/(KNeedForGaps,i*SMTCi*CSSFi)) in NeedForGaps.

Where,

L is the single interruption length for measurements with NeedForGaps capability;

KNeedForGaps,i is the interruption ratio controller indicator for frequency layer #i;

SMTCi is the SMTC periodicity for frequency layer #i;

CSSFi is the CSSF for frequency layer #i;

K=2*L*measurement samples/(lower bound of the measurement delay) is the higher bound of the interruption ratio.

Proposal 8: RAN4 to discuss the DRX-based NeedForGaps requirement based on the following scenarios

- Short DRX(DRX<=320ms)

- Long DRX(DRX>320ms)

Proposal 9: RAN4 to discuss the possible interruption ratio when DRX is configured

- When DRX cycle is equal or smaller than 320ms,

  - no interruption is expected when configured SMTC occasions are misalignment with DRX ON duration;

  - otherwise, the interruption ratio is min(K, 2*L/(KNeedForGaps *1.5* max(DRX cycle, SMTCi) *CSSFi)).

- When DRX cycle is larger than 320ms, no interruption is expected.

Proposal 10: RAN4 to agree the measurement requirement for NeedForGaps case 1 in FR1 as follow.

Table. Measurement period when UE reports ‘no gap no interruption’ in NeedForGaps for inter-frequency measurements (FR1)

| DRX cycle | T SSB_measurement_period_NeedForGaps_wo_interuption |
| --- | --- |
| No DRX | max(200ms, ceil( 8 x Kp) x SMTC period) x CSSFNeedForGaps_no_interrupt |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 8 x Kp) x max(SMTC period, DRX cycle)) x CSSFNeedForGaps_no_interrupt |
| DRX cycle>320ms | ceil( 8 x Kp ) x DRX cycle x CSSFNeedForGaps_no_interrupt |

Proposal 11: RAN4 to agree the following bullets CSSFNeedForGaps_no_interrupt as follow.

- CSSFNeedForGaps_no_interrupt is determined according to CSSFoutside_gap,i for measurement conducted outside MG/NCSG or according to CSSFwithin_gap,i for measurement conducted within measurement gaps.

Proposal 12: RAN4 to update the CSSFoutside_gap,i to add additional factor NNeedForGaps_no_interrupt , where NNeedForGaps_no_interrupt is the number of configured MOs in the bands which UE reports ‘no gap no interruption’ in NeedForGaps and not fully overlapping with the MG; otherwise, it is 0.

Proposal 13: RAN4 to agree the lower bound as 640ms for interruption control in NeedForGaps case 2.

Proposal 14: RAN4 to update the CSSFoutside_gap,i to add additional factor NNeedForGaps_with_interrupt , where NNeedForGaps_with_interrupt is the number of configured MOs in the bands which UE reports ‘no gap with interruption’ in NeedForGaps and not fully overlapping with the MG; otherwise, it is 0.

Proposal 15: RAN4 to agree the measurement requirement for NeedForGaps case 2 outside gap in FR1 as follow.

Table: Measurement period when UE reports ‘no gap with interruption’ in NeedForGaps for inter-frequency measurements (FR1)

| DRX cycle | T SSB_measurement_period_NeedForGaps_with_interuption |
| --- | --- |
| No DRX | max([640ms], ceil(8 × Kp) × KNeedForGaps × SMTC period) × CSSFoutside_gap |
| DRX cycle≤ 320ms | max([640ms], ceil(1.5x 8 x Kp) x KNeedForGaps × max(SMTC period, DRX cycle)) x CSSFoutside_gap |
| DRX cycle>320ms | ceil(8 x Kp ) x DRX cycle x CSSFoutside_gap |

When UE reports ‘NeedForGaps’ for a specific band, however, the MOs within the band have fully overlapping with MG. In this case, the MOs had to be measured within MG. In our opinion, the KNeedForGaps is invalid in this case and the requirements shall be as follow.

Proposal 16: RAN4 to agree the measurement requirement for NeedForGaps case 2 within gap as the same with legacy measurement within gap as follow.

Table: Measurement period when UE reports ‘no gap with interruption’ in NeedForGaps for inter-frequency measurements (FR1)

| Condition | T SSB_measurement_period_NeedForGaps_with_interuption |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP , SMTC period)) CSSFwithin_gap |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFwithin_gap |
| DRX cycle > 320ms | 8 DRX cycle CSSFwithin_gap |

Proposal 17: Define scheduling restriction requirements for NeedForGaps similar as NCSG.

Proposal 18: NeedForGaps and NCSG are not expected to be enabled for the same UE at the same time, but NW can alternatively switch between NeedForGaps and NCSG once both UE and NW support NeedForGaps and NCSG.

Proposal 19: RAN4 to postpone the 1-to-1 mapping between NeedForGaps and NCSG capabilities until RAN4 has a clear understanding on NeedForGaps requirement.

# Reference

- R4-2303305, “WF on Measurement without gaps for UEs reporting NeedForGapsInfoNR”, Intel