3GPP TSG-RAN WG4 Meeting #106bis-e                                           	R4-2304900

Online, April 17 – April 26, 2023

Source:	Samsung

Title:	simulation results on 8Rx UE demodulation requirements

Agenda item:	5.5.4.1.1

Document for:	Discussion

- Introduction

In last RAN4 #106 meeting, further more discussion about test scope and simulation assumptions for 8Rx UE demodulation requirements based on RAN4 #105 WF[1], and have agreements in WF [2]. In this contribution, we share our simulation results.

- Simulation Assumptions

As agreed in last RAN4 #106 meeting, companies are encouraged to bring simulation results based on assumptions listed in Table 2-1.

Table 2-1: Common simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | 30/40 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |

Additionally, other basic simulation assumptions used for this contribution are as below.

- MIMO layers: Rank 2, 4, 8

- Propagation conditions: TDLA30-10 for rank>=2 and TDLC300-100 for rank=2

- Antenna configuration: rank2 2T8R, rank4 4T8R, rank8 8T8R

- Correlation matrix: ULA Low for rank4 and rank8, ULA Medium B for rank2

- DMRS symbol indexes for rank 8: (2, 3, 10, 11)

- Rank8 N1=4, N2=1

- MCS index table for PDSCH: Table 5.1.3.1-1 in 3GPP38.214

Other parameters specified in 3GPP 38.101-4 Table 5.2-1. Test cases for rank2, rank4 and rank8 are specified as Table 2-1 - Table 2-3.

Table 2-1 Test cases for rank2

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | TBD |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | TBD |
| 1-3 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| 1-4 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 1-5 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| 1-6 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |

Table 2-2 Test cases for rank4

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |

Table 2-3 Test cases for rank8

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |

- Simulation results

Simulation results for cases defined in Table 2-1 - Table 2-3 are as below.

Figure 3-1 Simulation results of test cases 1-1 - 1-6

Thus the SNR point of 70% maximum throughput for cases 1-1 and 1-2 could be acquired as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 5.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 6.60 |
| 1-3 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 6.58 |
| 1-4 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 7.25 |
| 1-5 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 8.37 |
| 1-6 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.42 |

Figure 3-2 Simulation results of test cases 2-1 and 2-2

Thus the SNR point of 70% maximum throughput for cases 2-1 and 2-2 could be acquired as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 5.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 8.93 |

Figure 3-3 Simulation results of test cases 3-1, 3-2 and 3-3

Thus the SNR point of 70% maximum throughput for cases 3-1, 3-2 and 3-3 could be acquired as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 0.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 12.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 18.74 |

In summary, the SNR point of 70% maximum throughput for rank2, 4, 8 cases could be summarized as table 3-1.

Table 3-1 SNR point of 70% maximum throughput for rank2, 4, 8 cases

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 5.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 6.60 |
| 1-3 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 6.58 |
| 1-4 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 7.25 |
| 1-5 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 8.37 |
| 1-6 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.42 |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 5.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 8.93 |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 0.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 12.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 18.74 |

- Conclusion

According simulation results in section 3, we could get the simulation results of SNR point of 70% of max throughput as below.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 5.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 6.60 |
| 1-3 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 6.58 |
| 1-4 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 7.25 |
| 1-5 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 8.37 |
| 1-6 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.42 |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 5.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 8.93 |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 0.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 12.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 18.74 |

- Reference

[1] R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei, HiSilicon, 3GPP RAN4 #105 meeting

[1] R4-2302942, “WF for 8RX UE performance requirements”, Huawei, HiSilicon, 3GPP RAN4 #106 meeting