3GPP TSG-RAN WG4 Meeting #106-bis-e 	 R4-2304936

E-meeting, 17th – 26th April, 2023

Title:	RF simulation results for non-transparent schemes

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	5.25.2.4

Document for:	Approval

# 1	Introduction

This contribution relates to a work item agreed in RAN#94-e, namely “Further NR coverage enhancements” [1]. We consider power domain enhancements and the following objectives captured in the WID:

- Study and if necessary specify following power domain enhancements

  - […]

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this contribution we provide results of the UE transmitter performance for power domain enhancements (objective related to MPR/PAR reduction). We evaluate the performance of different filter-extension combinations for FDSS in FR1 and FR2 from the transmitter point of view. Furthermore, we evaluate tone reservation for DFT-s-OFDM waveform. We provide the net gain results in APPENDIX.

In this paper we focus on non- transparent schemes. The corresponding results for transparent schemes are shown in [5]. Based on the following agreement made in RAN1 #110bise, FDSS w/ SE and Tone reservation are the non-transparent scheme according to WID (FDSS w/o SE is a transparent scheme). Hence, we’ll focus on those two.

Agreement

At least the following candidate solutions for MPR/PAR reduction will be studied in RAN1.

- Frequency domain spectrum shaping w/ spectrum extension

- Frequency domain spectrum shaping w/o spectrum extension

- Tone reservation (which can only be w/ spectrum extension)

The following agreement was made in RAN4 #104bis-e. Based on that, we use DFT-s-OFDM for evaluations.

Agreement:

- DFT-s-OFDM is considered for future study for DFT-s-OFDM for FDSS w SE or w/or SE and Tone Reservation.

- Whether CP-OFDM for tone reservation can be discussed is FFS.

# 2	Simulation parameters

Simulation parameters used in these simulations are shown in Table 1 for FR1, and in Table 2 for FR2, respectively.

Table 1 Simulation parameters for FR1 for FDSS

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | Case 1: 20 MHz Case 2: 100 MHZ |
| SCS | Case 1: 15/30/60 kHz Case 2: 30 kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of RBs | 16, 32, 64, … < NRB |
| Allocation type | Sweep over the channel |
| Extension factors | 0, 0.125,0.25,0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap, FD implementation - (0.335 1 0.335) - (0.28 1 0.28) - Truncated RRC - No filter (reference case) |
| Power class | PC 3 |

Table 2 Simulation parameters for FR2 for FDSS

| Carrier frequency | 28GHz |
| --- | --- |
| Channel BW | 400MHz |
| SCS | 120kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of RBs | 16, 32, 64, … < NRB |
| Allocation type | Sweep over the channel |
| Extension factors | 0, 0.125, 0.25, 0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap, FD implementation - (0.335 1 0.335) - (0.28 1 0.28) - Truncated RRC - No filter (reference case) |
| Power class | PC 3 |

# 3	Simulation results, QPSK

## 3.1 FR1 MPR results for FDSS

Simulation results with required MPR for FR1 are shown in this section. For completeness, the required MPR for legacy DFT-s-OFDM (i.e., without FDSS) is included in the figures.

### 3.1.1 FR1 MPR results for 20 MHz channel, 15 kHz SCS

Figure 1 shows the required MPR for the different combinations of filter and extensions in the case of 16, 32, 64 and 96 PRB. It can be seen that different filter-extension combinations provide the lowest MPR depending on the allocation size. A filter-extension combination that performs well in this channel is the [0.335 1 0.335] filter with 25% extension. But the same filter with 37.5% extension provides the lowest MPR in the smaller allocations. Overall, up to 1.5 dB lower MPR can be obtained with respect to legacy DFT-s-OFDM.

Figure 1 Required MPR for 20 MHz channel, 15 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (bottom left) and 96 PRB (bottom right)

Figure 2 shows the required MPR for the different combinations of filter and extensions in the case of 16, 32, 64 and 96 PRB. The emphasize in Figure 2 (compared to Figure 1) is that it includes results for [0.28 1 0.28] filter (in addition to three other filtering options (No filter, TRCC, [0.335 1 0.335]). It can be noted that less aggressive [0.28 1 0.28] filter provides good performance especially for inner RB regions and small RB allocation. On the other hand, more aggressive [0.335 1 0.335] filter provides the smallest MPR for outer RB allocations and larger RB allocations.

Figure 2 Required MPR for 20 MHz channel, 15 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (bottom left) and 96 PRB (bottom right)

Observation 1: Transmitter performance varies with the allocation size in 20 MHz channel: small allocations benefit for larger extension than larger allocations.

Observation 2: Less aggressive filters provide good performance especially for inner RB regions and small RB allocations. On the other hand, more aggressive filter provides the smallest MPR for outer RB allocations and larger RB allocations.

Observation 3: Non-transparent schemes outperform transparent schemes at least in terms of the amount of MPR reduction

### 3.1.2 FR1 MPR results for 20 MHz channel, 30 kHz SCS

Figure 3 shows the required MPR for the different combinations of filter and extensions in the case of 8, 16, 32 and 48 PRB. When compared to results with 15 kHz SCS (Figure 2), it can be noted that for similar FDRA, the OBO behaviour is very similar between 15 kHz and 30 kHz SCSs.

Figure 3 Required MPR for 20 MHz channel, 30 kHz SCS with the evaluated filters, for 8 PRB (top left), 16 PRB (top right), 32 PRB (bottom left) and 48 PRB (bottom right).

Observation 4: For similar FDRA, OBO behaviour is very similar between different SCSs.

### 3.1.3 FR1 MPR results for 20 MHz channel, 60 kHz SCS

Figure 4 shows the required MPR for the different combinations of filter and extensions in the case of 4, 8, 16 and 24 PRB. When compared to results with 15 kHz SCS (Figure 2) and with 30 kHz SCS (Figure 3), it can be noted again, that for similar FDRA, the OBO behaviour is very similar between different SCSs.

Figure 4 Required MPR for 20 MHz channel, 30 kHz SCS with the evaluated filters, for 8 PRB (top left), 16 PRB (top right), 32 PRB (bottom left) and 48 PRB (bottom right).

### 3.1.2 FR1 MPR results for 100 MHz channel, 30 kHz SCS

Figure 5 shows the required MPR for different combinations of filter and extensions in the case of 16, 32, 64, 128 and 256 PRB. Similarly, as for the 100 MHz channel, different filter-extension combinations offer the lowest MPR depending on the allocation size. Overall, TRRC and [0.335 1 0.335] with 25% extension offer a good performance. In the small allocations, [0.335 1 0.335] with 37.5% has the lowest MPR, and for fully allocated channel (256 PRB) [0.335 1 0.335] with 25% extension performs the best.

Figure 5 Required MPR for 100 MHz channel, 30 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (middle left) and 128 PRB (middle right) and 256 PRB (bottom)

Figure 6 shows the required MPR for the different combinations of filter and extensions in the case of 16, 32, 64, 128, 256  PRBs. The emphasize in Figure 6 (compared to Figure 5) is that it includes results for [0.28 1 0.28] filter (in addition to three other filtering options (No filter, TRCC, [0.335 1 0.335]). It can be noted that less aggressive [0.28 1 0.28] filter provides good performance especially for inner RB regions and small RB allocation. On the other hand, more aggressive [0.335 1 0.335] filter provides the smallest MPR for outer RB allocations and larger RB allocations.

Figure 6 Required MPR for 100 MHz channel, 30 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (middle left) and 128 PRB (middle right) and 256 PRB (bottom)

Observation 5: Transmitter performance varies with the allocation size in 100 MHz channel: small allocations benefit for larger extension than larger allocations.

Observation 6: Up to 1.5 dB lower MPR can be obtained with respect to legacy DFT-s-OFDM in both 20 MHz and 100 MHz channels.

## 3.2 FR2 MPR results for FDSS

## 3.2.1 FR2 MPR results for FDSS

Figure 7 shows the required MPR for different combinations of filter and extensions in the case of 16, 32, 64, 128 and 256 PRBs. Similarly, as for FR1, different filter-extension combinations offer the lowest MPR depending on the allocation size. Overall, [0.335 1 0.335] with 37.5% extension offers a good performance over the evaluated allocations. TRRC and [0.335 1 0.335] with 25% extension are overall a good filter-extension choice.

Figure 7 Required MPR for 400 MHz channel, 120 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (middle left) and 128 PRB (middle right) and 256 PRB (bottom)

## 3.3 FR1 MPR results for tone reservation for DFT-s-OFDM

Figure 8 shows the required MPR for QPSK and 16QAM for the 100 MHz channel with 30 kHz SCS for an allocation in the channel centre. For tone reservation, the portion of the reserved resources to carry the peak cancellation signal (PCS) is the same as for FDSS. It can be seen from the figures that for QPSK, tone reservation has larger OBO than FDSS. On the other hand, for 16QAM, tone reservation offers the lowest OBO. This is due to the fact that 16QAM is more prone to be EVM limited, and while FDSS introduces some degradation to the EVM, tone reservation does not modify the inband, thus higher order modulations may benefit from tone reservation techniques, at least from the transmitter point of view.

Figure 8 Required MPR for 100 MHz channel, 30 kHz SCS with QPSK (left) and 16QAM (right)

Observation 7: From the transmitter point of view, tone reservation does not offer gains with respect to FDSS with spectral extension for QPSK modulation and DFT-s-OFDM.

Observation 8: Higher order modulations (than QPSK) may benefit from tone reservation over FDSS from the transmitter point of view.

## 3.4 Summary of the OBO results (non-transparent schemes)

In the following, we provide summarize the OBO results shown in Section 3.1 (FR1), and Section 3.2 (FR2):

- Power gain from non-transparent schemes (compared to the case without filter) can be up-to 2.5 dB (see e.g., Figure 6).

  - The largest power gain from non-transparent schemes (compared to the case without filter) is achieved for the largest RB allocations. However, considerable power gain is available also for smaller allocations (see e.g., Figure 1, Figure 5).

  - Depending on the filter, power gain from non-transparent schemes can be bigger for outer allocations (compared to inner allocations).

  - Power gain from non-transparent scheme is available for both FR1 and FR2

  - For similar allocations OBO behaviour is very similar between different SCSs.

  - There are no major differences in OBO performance between 20 MHz CBW and 100 MHz CBW cases.

- Power boosting is mandatory to take the full power gain benefit from non-transparent schemes.

## 3.5 Summary of the net gain results (see Appendix C)

In the following, we summarize the net gain results for provided in APPENDIX C. The net gain result combines transmitter results (APPENDIX A), Receiver results (APPENDIX B) and they’re given with respect to a reference waveform, which corresponds to legacy DFT-s-OFDM without FDSS and without spectrum extension.

- Net gain from non-transparent scheme varies according to Tx filter, extension size, code rate, and RB allocation.

  - Considerable net gains are observed for all RB allocations. For inner allocations, the gains can be more than 1.5 dB (see e.g., Figure C8). For outer allocations, the gains can be more than 2 dB (see e.g., Figure C4).

  - Net gain is biggest for the largest RB allocations. However, net gain around 1 dB is available also for the smallest RB allocation (16RBs) considered in simulation scenarios (see Figures C2, C6).

  - Net gain reduces with when the code rate increases. However, for large RB allocations notable net gain is available even for the highest code rates considered in the simulation scenarios (see e.g., Figure C9).

  - Trend: Less aggressive filters (such as TRRC) maximize the net gain for small/inner allocations, while more aggressive filters (such as [0.335 1 0.335]) maximize the net gain for large/outer allocations.

  - 25% extension maximizes the net gain in most of the simulation scenarios.

  - 20 MHz CBW and 100 MHz CBW provide similar net gains.

  - Similar net gains are available for both FR1 and FR2.

- Net gain from transparent scheme is typically 1-1.5 dB smaller compared to that of non-transparent schemes. For the largest allocations, the difference is up-to 2 dB (see e.g., Figure C10). In many cases (esp. with FR2) the net gain from transparent schemes is close to zero.

# 4.	Conclusion

In this contribution we provided results of the UE transmitter performance for power domain enhancements. The focus of this paper was on non-transparent schemes. Based on the results we make the following observations:

Observation 1: Transmitter performance varies with the allocation size in 20 MHz channel: small allocations benefit for larger extension than larger allocations.

Observation 2: Less aggressive filters provide good performance especially for inner RB regions and small RB allocations. On the other hand, more aggressive filter provides the smallest MPR for outer RB allocations and larger RB allocations.

Observation 3: Non-transparent schemes outperform transparent schemes at least in terms of the amount of MPR reduction

Observation 4: For similar FDRA, OBO behaviour is very similar between different SCSs.

Observation 5: Transmitter performance varies with the allocation size in 100 MHz channel: small allocations benefit for larger extension than larger allocations.

Observation 6: Up to 1.5 dB lower MPR can be obtained with respect to legacy DFT-s-OFDM in both 20 MHz and 100 MHz channels.

Observation 7: From the transmitter point of view, tone reservation does not offer gains with respect to FDSS with spectral extension for QPSK modulation and DFT-s-OFDM.

Observation 8: Higher order modulations (than QPSK) may benefit from tone reservation over FDSS from the transmitter point of view.

# References

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom

- R4-2215515, “Enhancements to reduce MPR/PAR”, Nokia, Nokia, Nokia Shanghai Bell

- R4-2217745, “WF on enhancements to reduce MPR/PAR”, Nokia, Nokia, Nokia Shanghai Bell

- R1-2210326, “Final FL summary of power domain enhancements (AI 9.14.2)”, Moderator (Nokia, Nokia, Nokia Shanghai Bell)

- R4-2304935, “RF simulation results for transparent schemes”, Nokia, Nokia, Nokia Shanghai Bell

# APPENDIX

The following was agreed in RAN4 #104bis-e

Agreement:

Actual conclusion of the MPR/PAR reduction methods should be based on net coverage gain results combining transmitter and receiver performance

In this appendix, we provide net gain analysis for both FDSS w/o SE, and Tone reservation. We provide receiver performance results in Appendix A, transmitter performance results in Appendix B and net gain results in Appendix C.

Appendix D provides simulation results for pi/2 BPSK with and without SE.

# APPENDIX A:

## Receiver performance results

In this appendix we provide results of the BS receiver performance for power domain enhancements.

Studies earlier done in 3GPP in coverage enhancement context in Rel-16/17 timeframe are coverage enhancements SI/WI and pi/2 BPSK optimizations WI. It would be beneficial to keep the simulation assumptions as similar to those used in these studies as possible.

The simulation assumptions used for coverage enhancement SI are captured in Annex A.1 for FR1 and in Annex A.2 for FR2 [3]. Table A1 and Table A2 show the proposed parameters selected based on those.

The main difference between Tables A1/A2 and Annexes of [3] is that NR coverage study considers only one data rate for the selected scenario (such as 1Mbps for UL in Urban scenario), and only one PRB allocation (such as 30 PRBs). In here, we propose to study the coding rate and bandwidth aspects in more details to see e.g. how channel estimation works in different scenarios. For that reason, we propose to consider 6 different coding rates and several different PRB allocations, respectively.

Some of the proposed methods for power domain enhancements (spectrum extension, tone reservation) use frequency band allocation extension to improve PAPR performance. It is beneficial to be able to define excess bands that are full PRBs and hence the extension factor values in Table A1 and Table A2 are proposed.

In this appendix we provide results for spectrum shaping with and without spectrum extension. Spectrum extension is a method which uses excess band allocation in addition to inband to reduce PAPR of the signal. Basic principle is that part of the inband signal is copied to excess band, hence extending the used bandwidth. In these simulations the receiver used with extension is combining signals from inband and excess bands to improve receiver performance and utilize the excess band power.

Frequency domain spectrum shaping is a PAPR reduction method where signal is filtered in frequency domain. It has been used in pi/2 BPSK optimization work. It can be applied similarly to signal with spectrum extension, just the total bandwidth is filtered. Two filters are used in these simulations, FD implementation of three tap filter [0.335,1,0.335] and truncated RRC. First one is used here as an example of very aggressive filter whereas latter is less aggressive as can be seen in Figure A1.

As can be seen several bandwidths and extension factors are used in simulations to get comprehensive picture of the performance of each of the methods. Extension factor (a) used here is defined as excess bandwidth divided by sum of inband bandwidth and excess band bandwidth.

Both spectrum extension and tone reservation use excess band to reduce PAPR. When comparing such methods with each other or to some methods not using the excess band it is important to do the comparison in a fair manner. This can be ensured by using the same bandwidth for all compared cases. Also spectral efficiency should be kept the same i.e. the TBS used should be the same. This means that the code rates for different cases should be changed according to inband size. To do this a set of code rate schemes have been developed as shown in Table A3. Since the total bandwidth and the TBS remain the same as extension size increases the code rate increases within the same scheme (see Table 1). Schemes used are named cr0-cr5 and the code rate increases from low to high as the scheme index increases.

Observation A1: Ensure fair comparison between different methods by keeping the total bandwidth and the spectral efficiency the same for all compared cases.

Table A1 Simulation parameters for FR1

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | 20, 100MHz |
| SCS | 15, 30kHz |
| Channel model | TDL-C 300ns |
| UE speed | 3km/h |
| Channel estimation | Frequency domain |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| DMRS config | ZC, 2 symbols |
| Waveform | DFT-S-OFDM |
| HARQ config | No retransmissions |
| Num PRBs | 20MHz: 16,32,64,96 100MHz: 16,32,64,128,256 |
| Extension factors | 0.125,0.25,0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Frequency hopping | No |
| BLER | 10 % |
| Spectral shaping filter | 3-tap, FD implementation Truncated RRC |

Table A2 Simulation parameters for FR2

| Carrier frequency | 28GHz |
| --- | --- |
| Channel BW | 400MHz |
| SCS | 120kHz |
| Channel model | TDL-A 30ns |
| UE speed | 3km/h |
| Channel estimation | Frequency domain |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2 |
| DMRS config | ZC, 2 symbols |
| Waveform | DFT-S-OFDM |
| HARQ config | No retransmissions |
| Num PRBs | 400MHz: 16,32,64,128,256 |
| Extension factors | 0.125,0.25,0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Frequency hopping | No |
| BLER | 10 % |
| Spectral shaping filter | 3-tap, FD implementation Truncated RRC |

Figure A1 Positive side of the frequency-domain allocation with different spectrum shaping filters.

Table A3 Code rate schemes used for simulations

| Scheme | QPSK CR | QPSK extension CR |  |  |
| --- | --- | --- | --- | --- |
|  |  | extension factor |  |  |
|  |  | 0.125 | 0.25 | 0.375 |
| cr0 | 1/16 | 1/14 | 1/12 | 1/10 |
| cr1 | 1/8 | 1/7 | 1/6 | 1/5 |
| cr2 | 1/6 | 4/21 | 2/9 | 4/15 |
| cr3 | 1/4 | 2/7 | 1/3 | 2/5 |
| cr4 | 1/3 | 8/21 | 4/9 | 8/15 |
| cr5 | 1/2 | 4/7 | 2/3 | 4/5 |

Simulation results for FR1 20MHz CBW case are shown in Figure A2. Results are shown for baseline QPSK case, truncated RRC (TRRC) without extension and with three different extension factors and similar set for three tap FD filter.

As can be seen the introduction of shaping and extension decreases receiver performance compared to the baseline QPSK. This is an expected result and the coverage gain of these methods should be coming from increased Tx power, which should exceed the reduced receiver performance.

Performance reduction is higher for the more aggressive filter and for the higher code rates. In general performance reduction is lower for case with extension compared to the case without extension.

Best performance for TRRC filter is obtained with lower extension factors 0.125 and 0.25, 0.375 seems to be too much for most cases. For more aggressive three tap filter the small extension is the worst case. The middle value 0.25 looks like a good compromise in all cases. In general differences between methods increase with higher bandwidth.

Simulation results for FR1 100MHz CBW and FR2 400MHz CBW are shown in Figures A3 and A4, respectively. Despite some difference the results are in big picture similar to the FR1 20MHz ones. Anyway, the actual conclusion of the methods cannot be made based on receiver performance but net gain results combining transmitter and receiver performance should be investigated.

Observation A2: Receiver performance reduction is higher for the more aggressive filter.

Observation A3: With spectrum shaping the receiver performance reduction is lower for case with extension compared to the case without extension.

Observation A4: Extension factor 0.25 looks like a good compromise.

Observation A5: Actual conclusion of the methods should be based on net gain results combining transmitter and receiver performance.

Figure A2 SNR corresponding to 10% BLER for bandwidths 16,32,64 and 96 for FR1 20MHz CBW.

Figure A3 SNR corresponding to 10% BLER for bandwidths 16,32,64,128 and 256 for FR1 100MHz CBW.

Figure A4 SNR corresponding to 10% BLER for bandwidths 16,32,64,128 and 256 for FR2 400MHz CBW.

# APPENDIX B:

## Transmitter performance results

In this appendix we provide results of the UE transmitter performance for power domain enhancements. We evaluate the performance of different filter-extension combinations for FDSS in FR1 and FR2 from the transmitter point of view. Furthermore, we evaluate tone reservation for DFT-s-OFDM waveform.

Simulation parameters used in these simulations are shown in Table B1 for FR1, and in Table B2 for FR2, respectively. As discussed in Appendix A the intention with the parameters is to keep them as similar as possible to the ones used in coverage enhancement study and pi/2 BPSK optimization.

In this appendix we provide results for spectrum shaping with and without spectrum extension. Spectrum extension is a method which uses excess band allocation in addition to inband to reduce PAPR of the signal. Basic principle is that part of the inband signal is copied to excess band, hence extending the used bandwidth. The filters used for evaluations are shown in Figure B1.

Table B1 Simulation parameters for FR1 for FDSS

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | 20, 100MHz |
| SCS | 15, 30kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of RBs | 20MHz: 16, 32, 64, 96 100MHz: 16, 32, 64, 128, 256 |
| Allocation type | Sweep over the channel |
| Extension factors | 0.125,0.25,0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | 3-tap [0.335 1 0.335], FD implementation Truncated RRC |
| Power class | PC 3 |

Table B2 Simulation parameters for FR2 for FDSS

| Carrier frequency | 28GHz |
| --- | --- |
| Channel BW | 400MHz |
| SCS | 120kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of RBs | 400MHz: 16, 32, 64, 128, 256 |
| Allocation type | Sweep over the channel |
| Extension factors | 0.125, 0.25, 0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | 3-tap [0.335 1 0.335], FD implementation Truncated RRC |
| Power class | PC 3 |

Frequency domain spectrum shaping is a PAPR reduction method where signal is filtered in frequency domain. It has been used in pi/2 BPSK optimization work. It can be applied similarly to signal with spectrum extension, just the total bandwidth is filtered. Two filters are used in these simulations, FD implementation of three tap filter [0.335,1,0.335] and truncated RRC. First one is used here as an example of very aggressive filter whereas latter is less aggressive as can be seen in Figure B1.

As can be seen several bandwidths and extension factors are used in simulations to get comprehensive picture of the performance of each of the methods. Extension factor (a) used here is defined as excess bandwidth divided by sum of inband bandwidth and excess band bandwidth.

Figure B1 Positive side of the frequency-domain allocation with different spectrum shaping filters.

## B.1 FR1 MPR results for FDSS

Simulation results with required MPR for FR1 are shown in this section. For completeness, the required MPR for legacy DFT-s-OFDM (i.e., without FDSS) is included in the figures.

### B.1.1 FR1 MPR results for 20 MHz channel, 15 kHz SCS

Figure B2 shows the required MPR for the different combinations of filter and extensions in the case of 16, 32, 64 and 96 PRB. It can be seen that different filter-extension combinations provide the lowest MPR depending on the allocation size. A filter-extension combination that performs well in this channel is the [0.335 1 0.335] filter with 25% extension. But the same filter with 37.5% extension provides the lowest MPR in the smaller allocations. Overall, up to 1.5 dB lower MPR can be obtained with respect to legacy DFT-s-OFDM.

Figure B2 Required MPR for 20 MHz channel, 15 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (bottom left) and 96 PRB (bottom right)

Observation B1: Transmitter performance varies with the allocation size in 20 MHz channel: small allocations benefit for larger extension than larger allocations.

### B.1.2 FR1 MPR results for 100 MHz channel, 30 kHz SCS

Figure B3 shows the required MPR for different combinations of filter and extensions in the case of 16, 32, 64, 128 and 256 PRB. Similarly, as for the 100 MHz channel, different filter-extension combinations offer the lowest MPR depending on the allocation size. Overall, TRRC and [0.335 1 0.335] with 25% extension offer a good performance. In the small allocations, [0.335 1 0.335] with 37.5% has the lowest MPR, and for fully allocated channel (256 PRB) [0.335 1 0.335] with 25% extension performs the best.

Figure B3 Required MPR for 100 MHz channel, 30 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (middle left) and 128 PRB (middle right) and 256 PRB (bottom)

Observation B2: Transmitter performance varies with the allocation size in 100 MHz channel: small allocations benefit for larger extension than larger allocations.

Observation B3: Up to 1.5 dB lower MPR can be obtained with respect to legacy DFT-s-OFDM in both 20 MHz and 100 MHz channels.

## B.2 FR2 MPR results for FDSS

## B.2.1 FR2 MPR

Figure B4 shows the required MPR for different combinations of filter and extensions in the case of 16, 32, 64, 128 and 256 PRBs. Similarly, as for FR1, different filter-extension combinations offer the lowest MPR depending on the allocation size. Overall, [0.335 1 0.335] with 37.5% extension offers a good performance over the evaluated allocations. TRRC and [0.335 1 0.335] with 25% extension are overall a good filter-extension choice.

Figure B4 Required MPR for 400 MHz channel, 120 kHz SCS with the evaluated filters, for 16 PRB (top left), 32 PRB (top right), 64 PRB (middle left) and 128 PRB (middle right) and 256 PRB (bottom)

## B.3 FR1 MPR results for tone reservation for DFT-s-OFDM

Figure B5 shows the required MPR for QPSK and 16QAM for the 100 MHz channel with 30 kHz SCS for an allocation in the channel centre. For tone reservation, the portion of the reserved resources to carry the peak cancellation signal (PCS) is the same as for FDSS. It can be seen from the figures that for QPSK, tone reservation has larger OBO than FDSS. On the other hand, for 16QAM, tone reservation offers the lowest OBO. This is due to the fact that 16QAM is more prone to be EVM limited, and while FDSS introduces some degradation to the EVM, tone reservation does not modify the inband, thus higher order modulations may benefit from tone reservation techniques, at least from the transmitter point of view.

Figure B5 Required MPR for 100 MHz channel, 30 kHz SCS with QPSK (left) and 16QAM (right)

Observation B4: From the transmitter point of view, tone reservation does not offer gains with respect to FDSS with spectral extension for QPSK modulation and DFT-s-OFDM.

Observation B5: Higher order modulations (than QPSK) may benefit from tone reservation over FDSS from the transmitter point of view.

# APPENDIX C:

## Net gain evaluations

In this appendix we provide evaluations of net gain for power domain enhancements.

In this appendix we characterize the net gain of different shaping filters considering transmitter performance from Appendix B and link level performance from Appendix A.

Simulation parameters used in these simulations are shown in Tables C1 and C2 for FR1 and FR2, respectively. As discussed in Appendix A the intention with the parameters is to keep them as similar as possible to the ones used in coverage enhancement study and pi/2 BPSK optimization.

In this appendix we provide results for spectrum shaping with and without spectrum extension. Spectrum extension is a method which uses excess band allocation in addition to inband to reduce PAPR of the signal. Basic principle is that part of the inband signal is copied to excess band, hence extending the used bandwidth. The filters used for evaluations are shown in Figure C1.

Table C1 Simulation parameters for FR1 for FDSS

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | 20, 100MHz |
| SCS | 15, 30kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of PRBs | 20MHz: 16,32,64,96 100MHz: 16,32,64,128,256 |
| Allocation type | Sweep over the channel |
| Extension factors | 0.125,0.25,0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | 3-tap [0.335 1 0.335], FD implementation Truncated RRC |
| Power class | PC 3 |

Table C2 Simulation parameters for FR2 for FDSS

| Carrier frequency | 28GHz |
| --- | --- |
| Channel BW | 400MHz |
| SCS | 120kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of PRBs | 400MHz: 16,32,64,128,256 |
| Allocation type | Sweep over the channel |
| Extension factors | 0.125,0.25,0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | 3-tap [0.335 1 0.335], FD implementation Truncated RRC |
| Power class | PC 3 |

Frequency domain spectrum shaping is a PAPR reduction method where signal is filtered in frequency domain. It has been used in pi/2 BPSK optimization work. It can be applied similarly to signal with spectrum extension, just the total bandwidth is filtered. Two filters are used in these simulations, FD implementation of three tap filter [0.335,1,0.335] and truncated RRC. First one is used here as an example of very aggressive filter whereas latter is less aggressive as can be seen in Figure C1.

As can be seen several bandwidths and extension factors are used in simulations to get comprehensive picture of the performance of each of the methods. Extension factor used here is defined as excess bandwidth divided by sum of inband bandwidth and excess band bandwidth.

Figure C1 Positive side of the frequency-domain allocation with different spectrum shaping filters.

### Net gain analysis:

In RAN4 #105, it was “encouraged to describe a metric used for net gain evaluation”. The metric used in our net gain evaluation is depicted below.

In order to characterize the filters, it is needed to consider both the transmitter performance (i.e., the achievable output power) from Appendix B and the link level performance, obtained in Appendix A. One important issue to note is that when different schemes are compared against each other, the amount of resources should be the same. The link simulations assume that the receiver is not aware of the used shaping filter. With transmitter and receiver results, it is possible to compute the net gain with respect to a reference waveform, which in this study corresponds to legacy DFT-s-OFDM (i.e., without FDSS or tone reservation). The net gain expression is:

Net Gain dB=PoutFilter i-PoutRef+SNR10%Ref-SNR10%Filter i

Where PoutFilter i is the achieved (TX) output power of the filtered  waveform being compared against the reference, PoutRef is the output power of the reference, SNR10%Ref is the required SNR to achieve 10% BLER with the reference, and SNR10%Filter i is the required SNR to achieve 10% BLER using the filtered  waveform being compared against the reference.

Observation C1: Actual conclusion of the methods should be based on net gain results combining transmitter and receiver performance.

## C.1 FR1 Net gain results for FDSS

Net gain results for FR1 are shown in this section.

### C.1.1 FR1 Net gain results for 20 MHz channel, 15 kHz SCS

#### C.1.1.1 20MHz/15kHz 16 PRB

Figure C2 shows the net gain for the different combinations of filter and extensions in the case of 16 PRB and all the tested code rates (see Appendix A for more details) where the spectrum efficiency lost by the extension is compensated with higher coding rate. From the results, the TRRC filter with 25% extension provides the largest net gain of up to 1 dB for small coding rate and small allocation (which are the target for coverage extension).

Figure C2 Net gain for 20 MHz channel, 15 kHz SCS with the evaluated filters, for 16 PRB for all tested coding rates.

#### C.1.1.2 20MHz/15kHz 32 PRB

For 32 PRB, the picture is similar as for 16 PRB, where TRRC with 25% extension performs the best for all coding rates. For small coding rates, more aggressive filters with larger extension (e.g., 37.5%) also offer similar net gain of up to 1.5 dB.

Figure C3 Net gain for 20 MHz channel, 15 kHz SCS with the evaluated filters, for 64 PRB for all tested coding rates.

#### C.1.1.3 20MHz/15kHz 64 PRB

For 64 PRB, more variety of filter-extension combinations offer the largest net gain for the different coding rates. For small coding rates, more aggressive filter (i.e., [0.335 1 0.335]) with 25% and 37.5% extension offer the best performance of up to 2.5 dB. However, for larger allocations, less aggressive filter with 12.5% extension provide the largest gains.

Figure C4 Net gain for 20 MHz channel, 15 kHz SCS with the evaluated filters, for 64 PRB for all tested coding rates.

#### C.1.1.4 20MHz/15kHz 96 PRB

For 96 PRB, more variety of filter-extension combinations offer the largest net gain for the different coding rates. For small coding rates, more aggressive filter (i.e., [0.335 1 0.335]) with 25% and 37.5% extension offer the best performance of up to 2.5 dB. However, for larger allocations, less aggressive filter with 25% extension provide the largest gains.

Figure C5 Net gain for 20 MHz channel, 15 kHz SCS with the evaluated filters, for 96 PRB for all tested coding rates.

Observation C2: The optimum filter-extension combination depends on both the allocation size and the coding rate

### C.1.2 FR1 Net gain results for 100 MHz channel, 30 kHz SCS

#### C.1.2.1 100MHz/30kHz 16 PRB

Figure C6 shows the net gain for the different combinations of filter and extensions in the case of 16 PRB and all the tested code rates (see Appendix A for more details) where the spectrum efficiency lost by the extension is compensated with higher coding rate. From the results, the TRRC filter with 25% extension provides the largest net gain of up to 1 dB for small coding rate and small allocation (which are the target for coverage extension).

Figure C6 Net gain for 100 MHz channel, 30 kHz SCS with the evaluated filters, for 16 PRB for all tested coding rates.

#### C.1.2.2 100MHz/30kHz 32 PRB

Figure C7 shows the net gain for the different combinations of filter and extensions in the case of 32 PRB and all the tested code rates (see Appendix A for more details). From the results, the TRRC filter with 25% extension provides the largest net gain for all tested coding rates. For small coding rates, more aggressive filter with 37.5% extension provides similar gains

Figure C7 Net gain for 100 MHz channel, 30 kHz SCS with the evaluated filters, for 32 PRB for all tested coding rates.

#### C.1.2.3 100MHz/30kHz 64 PRB

For 64 PRB and small coding rates, both filters with 25% extension provide up to 1.5 dB net gain for inner allocations, and the [0.335 1 0.335] filter with 37.5% extension has similar gain. For larger coding rates, TRRC with 25% extension is the best.

Figure C8 Net gain for 100 MHz channel, 30 kHz SCS with the evaluated filters, for 64 PRB for all tested coding rates.

#### C.1.2.4 100MHz/30kHz 128 PRB

For 128 PRB and small coding rates, [0.335 1 0.335] with 25% extension provide up to 2 dB net gain for allocations in the centre of the channel, and the [0.335 1 0.335] filter with 37.5% extension has similar gain. For larger coding rates, TRRC with 25% extension is the best for centered allocations.

Figure C9 Net gain for 100 MHz channel, 30 kHz SCS with the evaluated filters, for 128 PRB for all tested coding rates.

#### C.1.2.5 100MHz/30kHz 256 PRB

For 128 PRB and small coding rates, [0.335 1 0.335] with 25% extension provide up to 3 dB net gain and the [0.335 1 0.335] filter with 37.5% extension has similar gain. For larger coding rates, TRRC with 25% extension is the best.

Figure C10 Net gain for 100 MHz channel, 30 kHz SCS with the evaluated filters, for 256 PRB for all tested coding rates.

## C.2 FR2 Net gain results for FDSS

### C.2.1 FR2 Net gain results for 400 MHz channel, 120 kHz SCS

#### C.2.1.1 400MHz/120kHz 16 PRB

For 16 PRB and small coding rates, [0.335 1 0.335] with 25% and37.5% extension provide up to 1 dB net gain. For larger coding rates, TRRC with 25% extension is the best.

Figure C11 Net gain for 400 MHz channel, 120 kHz SCS with the evaluated filters, for 16 PRB for all tested coding rates.

#### C.2.1.2 400MHz/120kHz 32 PRB

For 32 PRB and small coding rates, [0.335 1 0.335] with 37.5% extension provide up to 1.2 dB net gain. For larger coding rates, TRRC with 25% extension is the best.

Figure C12 Net gain for 400 MHz channel, 120 kHz SCS with the evaluated filters, for 32 PRB for all tested coding rates.

#### C.2.1.3 400MHz/120kHz 64 PRB

For 64 PRB and small coding rates, [0.335 1 0.335] with 37.5% extension provides up to 1.4 dB net gain. For larger coding rates, TRRC with 25% extension is the best.

Figure C13 Net gain for 400 MHz channel, 120 kHz SCS with the evaluated filters, for 64 PRB for all tested coding rates.

#### C.2.1.4 400MHz/120kHz 128 PRB

For 128 PRB and small coding rates, [0.335 1 0.335] with 37.5% extension provides up to 1.5 dB net gain. For larger coding rates, TRRC with 25% extension is the best.

Figure C14 Net gain for 400 MHz channel, 120 kHz SCS with the evaluated filters, for 128 PRB for all tested coding rates.

#### C.2.1.5 400MHz/120kHz 256 PRB

For 256 PRB and small coding rates, [0.335 1 0.335] with 37.5% extension provides up to 1.8 dB net gain. For larger coding rates, TRRC with 25% extension is the best.

Figure C15 Net gain for 400 MHz channel, 120 kHz SCS with the evaluated filters, for 256 PRB for all tested coding rates.

## C.2 FR1 Net gain results for tone reservation

### C.2.1 FR1 Net gain results for QPSK

Figure C15 shows a coverage (net) gain results for QPSK over baseline (legacy) DFT-s-OFDM transmission comparing tone reservation and FDSS with spectrum extension. It is clear that although tone reservation provides gains for QPSK, they are smaller than the achievable gains with FDSS with spectrum extension. This holds especially for the FDSS SE scenario where receiver utilizes the signal transmitted via excess band (“QPSK FDSS, RX w/ excess band”).

Figure C15 Net results for QPSK in FR1 comparison between tone reservation and FDSS with spectrum extension

Observation C3: Tone reservation provides smaller coverage gains than FDSS with spectrum extension for QPSK.

### C.2.2 FR1 Net gain results for 16QAM

Figure C16 shows the required SNR to reach 10% BLER for 16QAM for baseline (legacy) DFT-s-OFDM transmission comparing tone reservation and FDSS with spectrum extension. The lower MPR achievable by tone reservation for 16QAM (see Appendix B.3) does not compensate for the losses in the receiver.

Figure C16 Comparison of SNR required for 10% BLER for 16QAM in FR2 between tone reservation and FDSS with spectrum extension

Observation C4: Only FDSS with spectrum extension should be considered for DFT-s-OFDM.

# APPENDIX D:

## Simulation results, pi/2 BPSK with and without SE

Based on our results, pi/2 BPSK with FDSS + SE has no room for improving MPR/PAR (compared to FDSS w/o SE). In the following we consider both link level performance (Figure D1) and RF performance (Figure D2) separately.

Figure D1 shows the link level performance for pi/2 BPSK and QPSK with and without spectrum extension. We consider four FDSS scenarios: No filter, Truncated RRC, [0.28 1 0.28] and [0.335 1 0.335]. The BW allocation is 16 RB (including the possible excess band). We consider six coding rates. The data rate is the same for each cases representing the same coding rate.

Results show that pi/2 BPSK with FDSS + SE will create considerable SNR degradation compared to FDSS w/o SE. The same holds also when comparing pi/2 BPSK FDSS SE against QPSK FDSS SE. The gap is smallest with CR0 and increases considerably with the decreasing coding rate. It is also noted that pi/2 BPSK does not support the highest coding rate option (CR5).

Figure D1. Required SNR for 10% BLER as function of spectral efficiency.

Figure D2 shows the OBO comparison for pi/2 BPSK. with and without spectrum extension. We consider four FDSS scenarios: No filter, Truncated RRC, [0.28 1 0.28] and [0.335 1 0.335].  This simulation has been done with 16 RBs (including the excess band) and for two SCS values (15 kHz and 30 kHz). Channel bandwidth is 20 MHz in this simulation.

Based on the results, pi/2 BPSK with FDSS + SE has no room for improving MPR/PAR (compared to FDSS w/o SE). This is visible in Figure D2, which indicates that for FDSS w/o SE with [0.335 1 0.335] filter, which is close to the 14 dB p-p limit can be used with full saturation even for edge allocations. The same holds for [0.28 1 0.28] filter.  Figure 10 also shows that for pi/2 BPSK the spectrum extension has slightly negative impact to OBO performance (the degradation is visible for spectrum edge allocations).

Observation D1: Pi/2 BPSK FDSS with SE does not provide gain compared to pi/2 BPSK FDSS without SE

Figure D2 OBO for pi/2 BPSK modulation, 16 RBs, [15 30] kHz SCS.

See code rates in Table A3