3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2304977

E-Meeting, April 17th – April 26th, 2023

Title:	TP to TR 38.718-02-01 Addition of CA_n40-n105

Source:	Nokia, Spark

Agenda item:	4.10.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.718-02-01 to add CA_n40A-n105A.

************************************* Start of TP*****************************************

## 5.x		CA_n40-n105

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1 Operating bands for CA

Table 5.x.1.1-1: CA band combination of band n40+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n40+n105

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n40A-n105A | CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

#### 5.x.1.3	Co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n7-n105.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11500 | 12000 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Based on above table, there is no uplink harmonic issue of CA_n40-n105

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11500 | 12000 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on above table, there is no harmonic mixing issue of CA_n40-n105

#### 5.x.1.4	∆TIB and ∆RIB values

For CA_n40-n105, the TIB,c and the RIB are based on CA_n41-n71.

Table 5.x.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n40-n105 | 0.3 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n40-n105 | - | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENs requirements

There’s no MSD exception needed.

#### 5.x.1.6	OOB blocking exception requirements

There’s no OOB exception needed.

### 5.x.2		Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n40A-n105A | 23 | +2/-3 |

#### 5.x.2.2		UE co-existence

Table 5.x.2.2-1 gives IMD interference analysis for CA_ n40-n105.

Table 5.x.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2300 | 2400 | 663 | 703 |
| DL Frequency [MHz] | 2300 | 2400 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1737 | 1597 | 2963 | 3103 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3897 | 4137 | 1074 | 894 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5263 | 5503 | 3626 | 3806 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6197 | 6537 | 411 | 191 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3194 | 3474 | 5926 | 6206 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7563 | 7903 | 4289 | 4509 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 512 | 252 | 8937 | 8497 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2491 | 2811 | 5874 | 5494 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4952 | 5212 | 9863 | 10303 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6589 | 6909 | 8226 | 8606 |

Based on the table 5.x.2.2-1, there is no IMD issues of this combination.

Table 5.x.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n40-n105 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 5.x.2.3		REFSENS requirements

There are no additional MSD requirements for this band combination.

************************************* End of TP*****************************************