3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2305087

# Electronic Meeting, April 17th – April 26th, 2023

Source: 	vivo

Title: 	Discussion of UE RF requirements for STxMP in FR2

Agenda Item:	5.30.2.1

Document for:	Discussion

Introduction

In RAN4#106, the RF requirements of NR MIMO evolution was discussed. An WF has been agreed in [1]. The LS on UE power limitation for STxMP have been approved in [2].

Though not concluded yet, up till now the tentative RF requirements are all related to STxMP. One such analysis paper is [5], and the topic summary and work plan are in [3][4].

In this paper, further analysis was provided for the requirements for STxMP in FR2.

Discussion

Background for “per-TCI”

In the approved LS, there is the following description:

“RAN4 can confirm that it can define the configured transmitted power requirement per-TCI for STxMP, but other solutions are not precluded.”

A detailed TP was also provided in [6] as following.

In the WF [1], there is also proposal to provide further views on the “per-TCI state” configured power.

‘Per-TCI state’ configured power for ‘per-panel’ power limitation

‘Per-TCI state’ configured power is currently the only comprehensive proposal for configured transmitted power for STxMP. There still some problems.

One of the most fundamental problem is that it seems that there is no good way to differentiate the power from multiple beams in case of simultaneous transmission. Thus it is difficult to differentiate the newly defined MPRf,c,k for TCI state k with other TCI state.

Observation: Currently it seems that there is no good way to differentiate the power from multiple beams.

In another word, when two beams are transmitting simultaneously, there would be some unavoidable overlapping and the extend would depending on the angle difference etc. Since those beams would occupy the same physical resources, there would be no way to further differentiate a measurement in certain direction. So, the measurement of this per-TCI state MPR seems also difficult.

Based on this situation, the following proposals were provided:

Proposal 1: Discuss whether and how to differentiate the per-beam power for different TCI-state in case of simultaneous transmission.

Proposal 2: Discuss whether and how to verify the MPR requirements in the per-TCI state power limitation.

One way to overcame this might be per panel transmission, but this clearly would be out of the scope of STxMP.

Another problem is the measured peak EIRP PUMAX,f,c,k for TCI state k. Currently, there is only one peak direction for one UE, and there is currently no testing procedure for differentiate multiple peaks. Thus a per-beam requirement is somewhat in conflict with the concept/verification of peak, since a peak mostly would corresponding to a beam, while not all beams can satisfy a peak.

Take the following figure as an example, under current test procedure, there is only one peak direction that corresponds to beam a would be accounted.

Figure 1. Illustration of multiple beams

Proposal 3: Further discuss the validity of per-TCI state peak EIRP, since currently the general understanding is still that only one peak direction is valid/testable for a UE.

Testing combination complexity & Other Requirements

If multiple per-TCI state requirements are specified separately, one of the difficult question is that whether they could be really independent? If not, then there could be a vast number of possible TCI state combinations for multiple panels. The volume could be too high for verification.

Proposal 4: Further discuss whether and how to consider joint requirements of multiple Tci-state requirement simultaneously.

In addition, there are other requirements might be also impacted by STxMP, such as beam correspondence as mentioned in [5]. The spherical coverage might be another requirement need to be considered.

Proposal 5: Further study the impact on other requirements in case of STxMP, such as beam correspondence and the spherical coverage.

‘Per-UE’ EIRP

In the WF, views on ‘per-UE’ power limitation is also welcome to be provided in case of STxMP.

Here we would like to raise some previous discussion during the study of inter-band UL CA during which the simultaneous UL transmission is done in multiple bands. However, for inter-band UL CA the EIRP was defined on a “per-band’ basis, based on the explanation of FCC.

One way of defining “per-UE” EIRP can be re-evaluated in [7]. In that paper, “per UE” EIRP in multi-CC/Beam scenario was proposed to be clarified as the sum of the EIRP of all respective CCs/Beams in a certain direction, which can be express as:

"per UE" EIRP(θ,φ)=EIRP(beam1,θ,φ)+EIRP(beam2,θ,φ)+…+EIRP(beamn,θ,φ)

As Figure 2 shows, the “per UE” EIRP of UE at (θ1, φ1) can be calculated as the EIRP of beam1 plus the EIRP of beam2 at (θ1, φ1).

Figure2 “Per UE” concept of EIRP

Actually, this “per UE” EIRP might be the only EIRP that could be actually measured.

Proposal 6: Further discuss tentative "per UE" EIRP concept, e.g. as the sum of the EIRP of all respective beams in a certain direction, which might be the only testable EIRP for STxMP compared to “per TCI state”.

Conclusion

In this paper, further analysis was provided for the requirements for STxMP in FR2. The following observation and proposals are provided:

Observation: Currently it seems that there is no good way to differentiate the power from multiple beams.

Proposal 1: Discuss whether and how to differentiate the per-beam power for different TCI-state in case of simultaneous transmission.

Proposal 2: Discuss whether and how to verify the MPR requirements in the per-TCI state power limitation.

Proposal 3: Further discuss the validity of per-TCI state peak EIRP, since currently the general understanding is still that only one peak direction is valid/testable for a UE.

Proposal 4: Further discuss whether and how to consider joint requirements of multiple Tci-state requirement simultaneously.

Proposal 5: Further study the impact on other requirements in case of STxMP, such as beam correspondence and the spherical coverage.

Proposal 6: Further discuss tentative "per UE" EIRP concept, e.g. as the sum of the EIRP of all respective beams in a certain direction, which might be the only testable EIRP for STxMP compared to “per TCI state”.

- References

[1] R4-2303495, WF on NR MIMO evolution for downlink and uplink, Samsung, RAN4#106

[2] R4-2303494, Draft LS on UE power limitation for STxMP in FR2, vivo, RAN4#106

[3] R4-2302838, Topic summary for [106][145] NR_MIMO_evo_DL_UL_UERF, Samsung, RAN4#106

[4] R4-2301929, Work plan for Rel-18 WI on MIMO evolution, Samsung, RAN4#106

[5] R4-2301540, Initial analysis on UE RF requirements for NR MIMO evolution, vivo, RAN4#106

[6] R4-2300706, Reply LS to RAN1 on UE power limitation for STxMP in FR2, Qualcomm Incorporated, RAN4#106

[7] R4-2104525, Discussion on per UE concept of FR2 UL CA, vivo, RAN4#98bis-e