3GPP TSG-RAN WG4 Meeting # 106bis-e 	R4-2305095

Online, April 17 – April 26, 2023

Source: 	vivo

Title: 	Discussion on beam type and requirement of beam correspondence

Agenda Item:	5.7.3.2

Document for:	Approval

- Introduction

In [1], several options for requirement design were captured:

- Option 1: Do not specify the min peak EIRP requirements + Same spherical coverage as RRC_Connected mode

- Option 2: Lower than the min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode

- Option 3: Same as min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode

In this contribution, we provide our views on the requirement design.

- Discussion

  - Beam type and RF requirement

The beam type issue has been discussed for a long time but hard to be concluded because it is really an implementation specific issue. To further validate whether the fine beam is necessary for msg1, we perform a simulation to show the misdetection of msg1 between rough beam and fine beam. The simulation results are shown in below:

Figure 1 Misdetection of msg1 between rough beam and fine beam

Considering that beam sweeping is not considered in msg1, which means UE cannot ensure transmit the msg 1 on the best correspondence beam, and a random beam that can receive SSB may be used which may also lead to fail reception at BS side. The results show that the rough beam even have better performance on misdetection under the assumption.

Observation 1: The Rough beam is enough to finish initial access in some scenario, whether rough beam is used or not is a UE implantation issue.

Based on this, we still insist that the requirement should accommodate both rough beam and fine beam.

Proposal 1: The RF requirement should accommodate both rough beam and fine beam.

For different beam type, the main difference in requirement is the min peak EIRP, which is already considered as 7 dB in RRM spec, so if the min peak EIRP need to be defined, 7dB relaxation is need. As for the spherical coverage, reuse the legacy requirement is meaningful to guarantee the minimum UE performance.

Proposal 2: RF requirement can be defined as:

- Option 1: Do not specify the min peak EIRP requirements + Same spherical coverage as RRC_Connected mode

- Option 2: 7 dB lower than the min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode

  - Side condition

Another issue related beam type is side condition. The legacy side condition for beam correspondence in RRC_CONNECTED state is:

Table 1: Conditions for SSB based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -93.2 | ≥6 |
|  | n258 | -93.2 |  |
|  | n261 | -93.2 |  |
| NOTE 1: Void NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

It is noted that the 6dB SNR is defined based on that only fine beam is considered, but if the rough beam also exists in initial access, this low SNR will cause UE cannot find any rough beam, so to accommodate different beam type, the SNR should be relaxed 7 dB also.

Proposal 3: The SSB minimum SNR should be relaxed 7 dB to align with the gain difference in TS 38.133.

- Conclusion

In this contribution, we provide our view on beam type and related RF requirement:

Observation 1: The Rough beam is enough to finish initial access in some scenario, whether rough beam is used or not is a UE implantation issue.

Proposal 1: The RF requirement should accommodate both rough beam and fine beam.

Proposal 2: RF requirement can be defined as:

- Option 1: Do not specify the min peak EIRP requirements + Same spherical coverage as RRC_Connected mode

- Option 2: 7 dB lower than the min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode

Proposal 3: The SSB minimum SNR should be relaxed 7 dB to align with the gain difference in TS 38.133.

- References

- R4-2303563   WF on beam correspondence requirements for initial access and RRC_inactive mode, Nokia, RAN4#106

- Annex

Table 1A simulation assumptions

| Parameter | Values |
| --- | --- |
| Carrier Frequency | 28GHz |
| System bandwidth | 100MHz |
| Subcarrier spacing | 120kHz |
| UE TX power | 23dBm |
| Channel model | CDL-A (DS 100ns) |
| UE speed | 30km/h for O2O |
| ISD | 200m |
| BS antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 4, 2, 1, 1) for fine beam (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) for rough beam |
| UE array orientation | Spherical angles θ and ϕ in global coordinate system is uniformly distributed in [0,π] and [0,2π]. |
| PRACH format | B4 |