3GPP TSG-RAN4 Meeting #106bis-e	R4-2305113

# Online, April 17th – April 26th, 2023

Agenda item:			5.4.2.1

Source:	vivo

Title:	Discussion on UE architectures and RF requirements impact for CA_n5A-n105A

Document for:	Discussion

- Introduction

After the introduction of APT 600MHz band (band n105), combining 600MHz with other sub-1GHz frequency bands to obtain better performance such as higher transmission speed is the direction of further research. Therefore, after the RAN#99 meeting, a new study item [1] is proposed to evaluate the related feasibility and other RF requirements to support the sub-1GHz band combinations.

In the study item [1], the specific following aspects need to be studied:

- Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands with 2Tx and simultaneous reception on two or three bands for the following sub-1GHz band combinations for smart phone form factor - UE RF architecture including n-plexing, PA - Study feasibility of low band wideband antenna and multiple low band narrowband antennas. - Evaluate the MSD requirements if needed. - Evaluate if UE complexity can be reduced based on the frequency range and BCS proposed by operators. - Identify potential impacts to relevant RAN4 requirements.

In this paper, we’d like to preliminary discuss the multiple potential antenna architecture and MSD requirements for CA_n5A-n105A combination.

- Discussion

The summaries of CA_n5A-n105A combination from SI [1] are as shown in the following tables:

Table 1: Summary of band combinations considered in the SI

| Configuration | Uplink Configuration | fallback configurations (Status) | Supported operators |
| --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | DL_n5A-n105A_UL_n5A_BCS0 (New) DL_n5A-n105A_UL_n105A_BCS0 (New) | Spark NZ |

Table 2: Bandwidth combination set for each band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

The spectrum range of CA_n5A-n105A is shown in Figure 1, which can be observed that there is no frequency overlap between n5 and n105. After calculating the UL harmonic frequency and IMD products of band n5 and n105, no corresponding interference falling into DL bands can be observed.

Figure 1. CA_n5A-n105A spectrum range

Observation 1：For CA_n5A-n105A combination, there is no IMD component or harmonics frequency falling into DL frequency range.

As to the UE architecture for CA_n5A-n105A, two kinds of potential structure are proposed in the contribution, separately using 2 antennas and 3 antennas.

- The 2-antenna architecture is listed as below:

1) Main TRx antenna 1 with quadplexer supporting n105UL+n105DL+n5UL+n5DL

2) Diversity Rx antenna 2 with duplexer supporting n5DL+n105DL

Figure2. Potential UE architecture with 2 antennas, supporting CA_n5A-n105A

For the 2-antenna architecture, the two antennas should cover the entire frequency band from n105DL to n5DL at the same time, with the length of 282MHz and a bandwidth ratio of up to 37%, obviously higher than that of the other typical CA band combinations in 700/800/900MHz, as shown in Table 3. As mentioned in TR [2], it will exceed the bandwidth ratio of typical planar antenna in smartphones, therefore the radiative performance for CA_n5A-n105A would be affected even more obviously than that for CA_n5-n28. At the same time, as the frequency gap between n105DL and n105UL is 10 MHz, the isolation requirement is high, and the effect of insertion loss on in-band signal is also difficult to solve.

Observation 2: For CA_n5A-n105A using the UE architecture with two antennas, the implementation of a well-performing quadplexer is difficult, and the radiation performance will be affected by the high bandwidth ratio.

Table 3: Bandwidth ratio of CA band combinations

| CA band combinations type | CA_n5A-n105A | CA_n5-n8 | CA_n5-n28 |
| --- | --- | --- | --- |
| Bandwidth ratio | 37.5% | 5.16% | 23.9% |

- The 3-antenna architecture is listed as below:

1) Main TRx antenna 1 with duplexer supporting n105UL+n105DL

2) Main TRx antenna 2 with duplexer supporting n5UL+n5DL

3) Diversity TRx antenna 3 with duplexer supporting n105DL+n5DL

Figure3. Potential UE architecture with 3 antennas, supporting CA_n5A-n105A

For the 3-antenna architecture, there are two antennas on the main signal path, the use of duplexer eases the difficulty of multiplexer design and implement. The frequency range for each main antenna is narrower compared with 2-antenna architecture, optimizing the radiation performance. At the same time, the isolation between antennas can weaken the UL interference of another band to DL.

However, due to the lower frequency band of n105, the size of the antenna will be relatively larger, and the size of the 3-antenna structure will increase more obviously than 2-antenna structure. To consider the dimension of the mobile phone, some radiation performance may be sacrificed. On the other hand, it can be seen that the diversity antenna also needs to cover the entire band with the length of 282MHz, so the challenges to radiation performance still remain.

Observation 3: 3-antenna architecture will avoid more complex multiplexer implementation than 2-antenna architecture. However, due to the lower frequency band of n105, the size of the 3-antenna structure will be relatively larger, the challenge to the dimension of the mobile phone is also greater than 2-amtenna architecture.

- Conclusion

Observation 1：For CA_n5A-n105A combination, there is no IMD component or harmonics frequency falling into DL frequency range.

Observation 2: For CA_n5A-n105A using the UE architecture with two antennas, the implementation of a well-performing quadplexer is difficult, and the radiation performance will be affected by the high bandwidth ratio.

Observation 3: 3-antenna architecture will avoid more complex multiplexer implementation than 2-antenna architecture. However, due to the lower frequency band of n105, the size of the 3-antenna structure will be relatively larger, the challenge to the dimension of the mobile phone is also greater than 2-amtenna architecture.

- References

- RP-230789 Study on enhancement for sub-1GHz NR band combinations, Spark NZ, Huawei, HiSilicon, #RAN 99.

- RP-230425 TR, Editorial update for 1.0.0 version, Study on enhancement for 700/800/900MHz band combinations (Release 18), #RAN 99.