3GPP TSG-RAN4 Meeting #106bis-e	R4-2305115

# Online, April 17th – April 26th, 2023

Agenda item:			5.4.2.3

Source:	vivo

Title:	Discussion on UE architectures and RF requirements impact for CA_n26A-n28A

Document for:	Discussion

- Introduction

In the previous SI, the UE structure and RF requirements of CA_n5-n28 and CA_n5-n8 have been investigated in detail, but the related UE performances of CA_n26-n28 have not been evaluated yet. So in the new study item [1], the research on CA_n26-n28 will be carried out, specifically including the following aspects:

- Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands with 2Tx and simultaneous reception on two or three bands for the following sub-1GHz band combinations for smart phone form factor - UE RF architecture including n-plexing, PA - Study feasibility of low band wideband antenna and multiple low band narrowband antennas. - Evaluate the MSD requirements if needed. - Evaluate if UE complexity can be reduced based on the frequency range and BCS proposed by operators. - Identify potential impacts to relevant RAN4 requirements.

In this paper, we’d like to preliminary discuss the multiple potential antenna architecture and MSD requirements for CA_n26A-n28A combination.

- Discussion

The summaries of CA_n26A-n28A combination from SI [1] are as shown in the following tables:

Table 1: Summary of band combinations considered in the SI

| Configuration | Uplink Configuration | fallback configurations (Status) | Supported operators |
| --- | --- | --- | --- |
| CA_n26A-n28A | CA_n26A-n28A | DL_n26A-n28A_UL_n28A_BCS0 (Completed in RAN4#106) DL_n26A-n28A_UL_n26A_BCS0 (New) | CA_n26A-n28A |

Table 2: Bandwidth combination set for each band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n28A | CA_n26A-n28A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |

The spectrum range of CA_n26A-n28A is shown in Figure 1, which can be observed that there is no frequency overlap between n26 and n28. However, there are 3rd order IMD products falling into n26DL band, part of the frequency data is presented in Table 3. Other MSD interferences need to be further evaluated.

Figure 1. CA_n26A-n28A spectrum range

Table 3: Band n26 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 814 | 849 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 682 | 880 | 995 |

Observation 1：For CA_n26A-n28A combination, 3rd order IMD products overlap with the DL band of n26, which will cause interference to a certain extent. So the MSD requirements for this CA band combination should be evaluated.

As to the UE architecture for CA_n26A-n28A, two kinds of potential structure are proposed in the contribution, separately using 2 antennas and 3 antennas.

- The 2-antenna architecture is listed as below:

1) Main TRx antenna 1 with quadplexer supporting n26UL+n26DL+n28UL+n28DL

2) Diversity Rx antenna 2 with duplexer supporting n26DL+n28DL

Figure2. Potential UE architecture with 2 antennas, supporting CA_n26A-n28A

For the 2-antenna architecture, the main antenna should cover the entire frequency band from n28UL to n26DL at the same time, with a length of 191MHz and a bandwidth ratio of up to 23.9%, which is comparable to that of CA_n5-n28, as shown in Table 4. As mentioned in TR [2], it will exceed the bandwidth ratio of typical planar antenna in smartphones, so the radiation performance of CA_n26A-n28A will be affected.

On account of n28 band is as long as 45MHz, for consideration of performance, it needs to be divided to band n28A and band n28B. As TR [2] mentioned, ‘n28 alone requires a dual duplexer to reduce the band range to duplex gap ratio in order to achieve the acceptable filter isolation and insertion loss performance’. So for the main path, the design for multiplexer may become more complicated. At the same time, as all the frequency gaps between the four adjacent UL/DL bands are close to 10 MHz, the isolation requirement is high, and the effect of insertion loss of filter on in-band signal is also difficult to solve.

Observation 2: For consideration of performance, band n28 is likely to be divided to band n28A and band n28B in CA_n26A-n28A band combination.

Observation 3: For CA_n26A-n28A using the UE architecture with two antennas, the implementation of a well-performing quadplexer is difficult, and the radiation performance will be affected by the high bandwidth ratio.

Table 4: Bandwidth ratio of CA band combinations

| CA band combinations type | CA_n26A-n28A | CA_n5-n8 | CA_n5-n28 |
| --- | --- | --- | --- |
| Bandwidth ratio | 23.9% | 5.16% | 23.9% |

- The 3-antenna architecture is listed as below:

1) Main TRx antenna 1 with duplexer supporting n26UL+n26DL

2) Main TRx antenna 2 with duplexer supporting n28UL+n28DL

3) Diversity TRx antenna 3 with duplexer supporting n26DL+n28DL

Figure3. Potential UE architecture with 3 antennas, supporting CA_n5A-n105A

For the 3-antenna architecture, there are two antennas on the main signal path, the use of duplexer eases the difficulty of multiplexer design. The frequency range for each main antenna is narrower compared with 2-antenna architecture, optimizing the radiation performance. At the same time, the isolation between antennas can weaken the UL interference of another band to DL.

However, the size of the 3-antenna structure is much bigger than 2-antenna structure, especially for low band. To consider the dimension of the mobile phone, some radiation performance may be sacrificed.

Observation 4: 3-antenna architecture will avoid more complex multiplexer implementation than 2-antenna architecture. However, the size of the 3-antenna structure is much bigger than 2-antenna structure, especially for low band. To consider the dimension of the mobile phone, some radiation performance may be sacrificed.

- Conclusion

Observation 1：For CA_n26A-n28A combination, 3rd order IMD products overlap with the DL band of n26, which will cause interference to a certain extent. So the MSD requirements for this CA band combination should be evaluated.

Observation 2: For consideration of performance, band n28 is likely to be divided to band n28A and band n28B in CA_n26A-n28A band combination.

Observation 3: For CA_n26A-n28A using the UE architecture with two antennas, the implementation of a well-performing quadplexer is difficult, and the radiation performance will be affected by the high bandwidth ratio.

Observation 4: 3-antenna architecture will avoid more complex multiplexer implementation than 2-antenna architecture. However, the size of the 3-antenna structure is much bigger than 2-antenna structure, especially for low band. To consider the dimension of the mobile phone, some radiation performance may be sacrificed.

- References

- RP-230789 Study on enhancement for sub-1GHz NR band combinations, Spark NZ, Huawei, HiSilicon, #RAN 99.

- RP-230425 TR, Editorial update for 1.0.0 version, Study on enhancement for 700/800/900MHz band combinations (Release 18), #RAN 99.