3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2305169

e-Meeting, April 17 – April 26,2023

Agenda item:	5.12.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on RRM requirements for non-collocated EN-DC and NR-CA

Document for:	Discussion

- Introduction

At the last RAN4 #106 meeting, RRM requirements for intra-band non-contiguous non-collocated EN-DC/NR-CA was discussion and WF was agreed [1]. There are two remaining issues as following.

- FR1 non-collocated EN-DC/NR-CA for Type 2 UE for 2 layer MIMO

- FR1 non-collocated EN-DC/NR-CA for New Type UE for 4 layer MIMO

In this contribution, we propose our view on each remaining issue.

- Discussion

  - RRM impact for Type 2 UE

During last meeting, the discussion that impact on SCell activation requirements for Type 2 UE. The part of SCell activation requirements is duplicated as follows [1]

Issue 1-3-1: Impacts on SCell activation requirements - Proposals - Option 1: (MTK) - For a known FR1 intra-band non-collocated SCell, the existing activation delay requirement can be re-used - RAN4 shall revise the unknown SCell activation delay for activating an FR1 intra-band non-collocated SCell. For an unknown FR1 intra-band non-collocated SCell, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,. - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - For a non-collocated SCell without SSB, the exiting delay requirement is not applicable - Option 2: (Nokia) - The existing SCell activation delay for intra-band contiguous scenario needs to be clarified with co-location assumption - RAN4 shall define the SCell activation delay for activating an FR1 intra-band non-collocated SCell. The SCell activation delay requirement for inter-band CA can be applied to intra-band non-colocated SCell for Type 2 UE - Option 3: (NTT DOCOMO) - RAN4 may consider that whether for intra-band non-contiguous non-collocated EN-DC/NR-CA scenario case can be included as inter-band. - Option 4: (Huawei) - The existing SCell activation delay requirements in FR1 can also be applied for FR1 intra-band non-contiguous CA supporting non-collocated deployment for Type 2 UE.

At the last meeting, many companies suggested how the SCell activation requirements need to be defined. RAN4 RRM session discussed and agreed on MRTD/MTTD non-collocated for intra-band non-contiguous EN-DC/NR-CA in FR1. However, since there are only two remaining meetings until Rel-18, the April and May meetings, we need to consider the minimum other requirements (e.g., including SCell start-up delay requirements).

Proposal 1: RAN4 RRM session can consider that what specific requirement need to be determined to achieve non-collocation for Type 2 UE. (e.g., including SCell activation delay)

  - RRM impact for new Type UE for 4 layer MIMO

During last meeting, the discussion that impact on RRM requirement for new Type UE for 4 layer MIMO. The part of new Type UE for 4 layer MIMO is duplicated as follows [2].

Issue 2-1-1: MRTD/MTTD requirements for New Type UE for 4 layer MIMO - Proposals - Option 1: (MTK) - For New Type UE for 4-layer MIMO, the RRM requirement and MRTD/MTTD requirement need to wait for RF’s progress. - Option 2: (Samsung) - Joint session to evaluate the Rx requirement relaxation under 25dB power imbalance could be assessed for Type-3 UE, to compensate the signal distortion under the assumption MRTD＞CP - Option 3: (Nokia) - RAN4 will also develop requirements for UE type 4a/4b. - For Type 4 UE, reuse the same MRTD/MTTD requirement as Type 2 UE. - Option 4: (NTT DOCOMO) - RAN4 RRM’s session may avoid that problem exceeding the CP length for the New Type UE 4 layer 3a/3b by using ECP. - Option 5: (Ericsson) - If type 3 UE with shared LNA are added then power imbalance should be = 25 dB and MRTD > CP to allow deployment flexibility and reuse of existing site infrastructure. - Option 6: (Huawei) - For 4-layer MIMO with Type 3a/3b UE, the MRTD requirements for non-collocated FR1 intra-band non-contiguous CA and inter-band EN-DC with overlapping DL bands can be defined as (3us +ΔT), where ΔT is the maximum propagation delay difference determined by the agreed power balance for Type 3a/3b UE.

At the last RAN4 RF session meeting, there are still many remaining issues as follows for Type 3a/3b UE [2].

< Issue 3-2-1: Whether to keep TAE/network synchronization requirement for Type 3a/3b > - Option.1 Keep TAE 3us - Option.2 MRTD [3]us - Option.3 MRTD < CP Way Forward: Continue further discussions in the next meeting. < Issue 3-2-2: Whether to discuss to cope with both 25dB power imbalance (including the relaxation<25dB) and MRTD>CP Length for Type 3a/3b > Way Forward: - Continue to discuss the following both options in the next meeting. - Option.1: Cope with 25dB (including the relaxation<25dB) and MRTD>CP. - Option.2: RTD should be within CP to enable type 3a/3b UE. And power imbalance should be reduced accordingly. < Issue 3-2-3: With the assumption MRTD＞CP, discuss the RF requirement/performance for 25dB power imbalance (including the relaxation＜25dB) for type3a/3b based on companies’ input. > Way Forward: - Continue further discussion and introduce sub-AI for simulation in the next meeting. < Issue 3-2-4: Whether it is feasible to restrict the n77/n78 SCS=15kHz for Type 3a/3b > Way Forward: - Agree not to preclude SCS 30kHz for n77/n78. < Issue 3-2-5: Whether to discuss the power imbalance along with the assumption MRTD＜CP. > Way Forward: - Continue further discussions in the next meeting.

RAN4 RF session remain many issues for Type 3 UE. According to the RF discussion staus, it may be difficult to realize Type 3 requirement within Rel-18. Also, the RAN4 RRM session needs to discuss issues that exceed the CP length (MRTD > CP). However, the RAN4 RRM session needs to wait about RF discussions on TAE and MRTD issues.

Observation 1: According to the RF discussion status, there are many issues to consider for Type 3 UE specification and remaining discussion time is limited.
Proposal 2: Whether RAN4 RRM discussion about Type 3a/3b/4 UE can be continued within Rel-18 timeline or not should follow the RF session conclusion.

- 3. Conclusion

Proposal 1: RAN4 RRM session can consider that what specific requirement need to be determined to achieve non-collocation for Type 2 UE. (ex. including SCell activation delay)

Observation 1: According to the RF discussion status, there are many issues to consider for Type 3 UE specification and remaining discussion time is limited.
Proposal 2: Whether RAN4 RRM discussion about Type 3a/3b/4 UE can be continued within Rel-18 timeline or not should follow the RF session conclusion.

- 4. Reference

[1] 3GPP, R4-2303225, Huawei, HiSilicon “WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA”

[2] 3GPP, R4-2303696, KDDI “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”