3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305174

Electronic meeting, 17 April – 26 April 2023

Agenda Item:	5.19.2

Source:	Ericsson

Title:	Open issues on absolute physical layer throughput requirements

Document for:	Discussion

# 1	Introduction

RAN4#106 agreed with the way forward on physical layer throughput requirements [1]. This contribution discusses the open issues on the application layer throughput requirements based on the simulation summary [2].

# 2	Discussion

## 2.1	Test points and requirements

The remaining open issue is the SNR test point(s) and throughputs UE need to achieve. According to the WF, RAN4 has already agreed with the high-level criteria to set the requirements:

Average SNR of impairments results to achieve T% of maximum throughput + X dB margin - Use Gspan = [2.5] dB to check if the results are aligned - Use X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM - The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank Y for 2Rx/4Rx UE, e.g., Y=2 for both 2Rx/4Rx UEs.

We first discuss the criteria to select test points and requirements. Table 1 lists the average of relative throughput and reported rank values from the simulation summary [2].

Table 1	Average of Relative throughput and rank from the simulation result summary [2].

|  | FR1 FDD 2Rx |  | FR1 TDD 2Rx |  | FR2 TDD 2Rx |  | FR1 FDD 4Rx |  | FR1 TDD 4Rx |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Relative Tput | Average rank | Relative Tput | Average rank | Relative Tput | Average rank | Relative Tput | Average rank | Relative Tput | Average rank |
| 0 | 0.05 | 1.00 | 0.05 | 1.00 | 0.08 | 1.00 | 0.08 | 1.00 | 0.08 | 1.00 |
| 2 | 0.08 | 1.00 | 0.07 | 1.00 | 0.11 | 1.00 | 0.12 | 1.00 | 0.12 | 1.11 |
| 4 | 0.11 | 1.00 | 0.10 | 1.00 | 0.14 | 1.00 | 0.15 | 1.78 | 0.15 | 1.89 |
| 6 | 0.13 | 1.00 | 0.13 | 1.00 | 0.18 | 1.00 | 0.20 | 1.89 | 0.20 | 1.89 |
| 8 | 0.16 | 1.00 | 0.16 | 1.11 | 0.22 | 1.00 | 0.26 | 2.00 | 0.26 | 2.00 |
| 10 | 0.20 | 1.11 | 0.19 | 1.11 | 0.26 | 1.13 | 0.32 | 2.00 | 0.32 | 2.00 |
| 12 | 0.24 | 1.11 | 0.22 | 1.22 | 0.31 | 1.25 | 0.38 | 2.00 | 0.39 | 2.00 |
| 14 | 0.28 | 1.33 | 0.26 | 1.78 | 0.36 | 1.75 | 0.45 | 2.00 | 0.45 | 2.00 |
| 16 | 0.32 | 1.89 | 0.30 | 2.00 | 0.42 | 2.00 | 0.51 | 2.00 | 0.51 | 2.00 |
| 18 | 0.37 | 2.00 | 0.35 | 2.00 |  |  | 0.58 | 2.00 | 0.57 | 2.00 |
| 20 | 0.41 | 2.00 | 0.39 | 2.00 |  |  | 0.65 | 2.00 | 0.63 | 2.00 |

From the average we have the following observations.

Observation 1:

- For 2Rx scenario, companies’ results show the UE reports mainly rank 1 for SNR ≤ 6 dB, and mainly rank 2 for SNR ≥ 16 dB

- For 4Rx scenario, companies’ results show the UE reports mainly rank 2 for SNR ≤ 2 dB, and mainly rank 2 for SNR ≥ 8 dB

The reason we focus on the average rank is to get the stabilized MCS/PMI. In RAN4#106, we agreed to enable the HARQ retransmission during the ATP test. This means UE may report different RI/PMI/CQI during the HARQ retransmission period. Our preference is to select SNR test points where the UE can report the same rank as much as possible to avoid reporting different rank every CSI reporting.

Proposal 1: For SNR test criteria,

- For 2Rx, set two test points: one for rank 1 in the range SNR = [0-6] dB and another for rank 2 in the range SNR = [16-20] dB

- For 4Rx case, set one or two test points; both corresponds to rank 2, in the range SNR = [8-20] dB

Regarding the T%, we have the following observations from the average of results in Table 1.

Observation 2:

- For FR1 2Rx,

  - 10-15% of peak throughput is achieved in the range SNR = [0-6] dB

  - 35% of peak throughput is achieved in the range SNR = [16-20] dB

- For FR2 2Rx,

  - 10-20% of peak throughput is achieved in the range SNR = [0-8] dB

  - 40% of peak throughput is achieved in the range SNR = [14-16] dB

- For FR1 4Rx,

  - 30% of peak throughput is achieved in range SNR = [8-12] dB

  - 60% of peak throughput is achieved in range SNR = [16-20] dB

Proposal 2: Choose the following SNR test points to satisfy T% of the maximum throughput:

- For FR1 2Rx, T=10% and 35%

- For FR2 2Rx, T=15% and 40%

- For FR1 4Rx, T=30% and T=60%

  - If we set one test point only, choose higher test point.

## 2.2	Applicability and release independency

Issue 1-4-1: Applicability and release independent - Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15 - Option 2: The requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15 considering that companies are providing the latest results.

As we discussed in RAN4#106, the motivation of ‘release independent way’ was to specify the UE performance requirements in a release N for the features which has already introduced in a release M, where M<N. Examples of the NR features are Type-II PMI reporting test or URLLC-related UE performance requirements. These performance requirements are specified in Rel-16 in RAN4, but these requirements should be satisfied for Rel-15 UEs also because the physical layer specification has been set from Rel-15 core part.

The test parameters of the absolute physical layer throughput requirements discussed so far are almost same as Rel-15 RI reporting requirements. In this sense, the performance requirements can be applicable from Rel-15 UEs. However, Rel-15 specification has been frozen long time ago, and many Rel-15 UE chipsets have already been in the market.

We think it is sufficient to define the new absolute physical layer throughput requirements from Rel-18 UEs because the legacy UE should be able to pass the absolute physical layer throughput requirements as far as the UE passes the Rel-15 RI reporting test. If there are companies want to define ATP requirements from the earlier release, one compromise is to define from Rel-17, but not from Rel-15 or 16.

Proposal 3: Absolute physical layer (ATP) throughput requirements should be applicable from Rel-18.

# 4	Summary

Observation 1:

- For 2Rx scenario, companies’ results show the UE reports mainly rank 1 for SNR ≤ 6 dB, and mainly rank 2 for SNR ≥ 16 dB

- For 4Rx scenario, companies’ results show the UE reports mainly rank 2 for SNR ≤ 2 dB, and mainly rank 2 for SNR ≥ 8 dB

Proposal 1: For SNR test criteria,

- For 2Rx, set two test points: one for rank 1 in the range SNR = [0-6] dB and another for rank 2 in the range SNR = [16-20] dB

- For 4Rx case, set one or two test points; both corresponds to rank 2, in the range SNR = [8-20] dB

Observation 2:

- For FR1 2Rx,

  - 10-15% of peak throughput is achieved in the range SNR = [0-6] dB

  - 35% of peak throughput is achieved in the range SNR = [16-20] dB

- For FR2 2Rx,

  - 10-20% of peak throughput is achieved in the range SNR = [0-8] dB

  - 40% of peak throughput is achieved in the range SNR = [14-16] dB

- For FR1 4Rx,

  - 30% of peak throughput is achieved in range SNR = [8-12] dB

  - 60% of peak throughput is achieved in range SNR = [16-20] dB

Proposal 2: Choose the following SNR test points to satisfy T% of the maximum throughput:

- For FR1 2Rx, T=10% and 35%

- For FR2 2Rx, T=15% and 40%

- For FR1 4Rx, T=30% and T=60%

  - If we set one test point only, choose higher test point.

Proposal 3: Absolute physical layer (ATP) throughput requirements should be applicable from Rel-18.

# References

- R4-2302930, “WF for Absolute physical layer throughput requirements”, Intel Corporation.

- R4-2300484, “Summary of simulation results for NR UE absolute physical layer throughput requirements with link adaptation”, Intel Corporation.