3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305331

Electronic Meeting, 17 – 26 April, 2023

Title: 	Discussion on NW-A requirements with MUSIM gaps

Source: 	Huawei, HiSilicon

Agenda item:	5.26.2.3

Document for:	Discussion

Introduction

RRM requirements for NW A with MUSIM gaps configured are discussed in RAN4#106 and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- General principle

- Parameters for L1/L3 measurement requirements

- Time window W for aperiodic gap

In this paper we will provide our views on RRM requirements for NW A with MUSIM gaps.

Discussion

General principle

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling - Proposals - P1: Frameworks of LBT failure in NR-U design can be used as starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps (Apple) - P2: Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured. (CMCC xiaomi vivo ZTE oppo Huawei Ericsson Qualcomm MTK Nokia) - P3: Since principle used in Rel-17 concurrent gaps WI on layer 1 and layer 3 measurement requirements is derived after gap collision handling, it can be reused for even the gap handling solution within MUSIM gap is not fully determined, no need to postpone the discussion. (vivo) - P4: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority. (Ericsson) Recommended WF Continue discussion

In Rel-17 con-MG discussion, RAN4 developed a more generic approach to account for the impact of multiple MGs when defining measurement requirements. The scaling factor accounting for multiple MG impact is defined by counting the total number of measurement occasion in a window and the number of us-able occasions in the same window. It is straightforward to re-use the same approach for MUSIM gaps.

Proposal 1: Re-use the Rel-17 con-MG approach as baseline to define NW-A measurement requirements with MUSIM gaps, i.e. counting Navailable, Noutside_MG and Ntotal.

Parameters for L1/L3 measurement requirements

Issue 3-1-2: On parameters for L1/L3 measurement requirements - Proposals - P1: (CMCC): - For L3 measurement, Navailable need to be updated to cover MUSIM gaps - For L1 measurement, Noutside_MG and Navailable need to be updated to cover MUSIM gaps - P2: The following parameters need to be updated to account for collisions with MUSIM gaps (Qualcomm MTK): - Kp for intra-frequency and inter-frequency measurements without gaps (Xiaomi) - Kgap for intra-frequency and inter-frequency measurements with gaps (Xiaomi) - Kgap_EUTRA for inter-RAT measurements - Kp_CSI-RS for CSI-RS L3 measurements - Kp,PRS,I for NR positioning measurements - CSSFintra for intra-frequency measurements - CSSFinter for intra-frequency measurements - CSSFinterRAT for intra-RAT measurements - P scaling factor for L1-RSRP and L1-SINR measurements (xiaomi) - P3: Suggest the following update on parameters for L1/L3 measurement requirements (vivo ZTE) - For SSB based or CSI-RS based RLM, BFD and CBD, scaling factor P can be reused without any update, the definition of Ntotal, Noutside_MG and Navailable need updated. - For intra-frequency and inter-frequency measurements without gaps, scaling factor Kp can be reused and the definition of Ntotal and Navailable will be updated. - For intra-frequency and inter-frequency measurements with gaps, scaling factor Kgap can be reused and the definition of Ntotal and Navailable will be updated. - For inter-RAT E-UTRAN TDD/FDD measurement, scaling factor Kgap_EUTRA can be reused and the definition of Ntotal and Navailable will be updated. - For L1-RSRP/L1-SINR measurement, scaling factor P can be reused and the definition of Ntotal, Noutside_MG and Navailable will be updated. - For NR measurement for positioning, scaling factor Kp,PRSIi can be reused and the definition of Ntotal and Navailable will be updated. - For CSI-RS based L3 measurements, scaling factor Kp_CSI-RS can be reused and the definition of Ntotal and Navailable will be updated. - P4: (Huawei) - For L3 measurement outside MG, Kp in the requirements is updated - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W. - For L3 and positioning measurement with MG, existing requirements can be re-used. - For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated - Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W - P5: Clarification for L3 measurement without gap, SMTC should not be fully overlapping with MUSIM gap (oppo) Recommended WF Depends on outcome of issue 3-1-1

For L3 measurement outside MG, Kp is used in the requirements and with con-MG, Kp is defined as Kp = Ntotal / Navailable. In our view, the only change needed with MUSIM gaps is that the Navailable should exclude SMTC occasions that are overlapped with MUSIM gaps.

Proposal 2: For L3 measurement outside MG, Kp in the requirements is updated

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

For L3 measurement with MG, Kgap is used in the requirements and with con-MG, Kp is defined as Kp = Ntotal / Navailable. In our view, the existing definition of Ntotal and Navailable can be re-used with MUSIM gaps because existing Navailable is already based on non-dropped MG occasions. The same also applies to positioning measurement with MG.

Proposal 3: For L3 and positioning measurement with MG, existing requirements can be re-used.

For L1 measurement, P is used in the requirements and with con-MG, P is defined as Ntotal / Navailable

- Ntotal / Noutside_MG in FR1

- Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

- Ntotal / Navailable in FR2 with Navailable > 0.

In our view, the only change needed with MUSIM gaps is that the Noutside_MG and Navailable should exclude SMTC occasions that are overlapped with MUSIM gaps.

Proposal 4: For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W

In last meeting, some companies proposed that CSSF for L3 measurements may also need to be updated. So far we have not seen impact of MUSIM gaps on CSSF calculation, but we are open to further discussions.

Time window W for aperiodic gap

Issue 3-1-3: On the time window W for aperiodic gap - Proposals - P1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer. (vivo ZTE oppo Huawei) - P2: max(SMTC period, MGRP_max)+[M], where MGRP_max is the largest periodicity among all the periodic gaps and [M] is a time margin for the one-shot aperiodic MUSIM gap. (MTK) - P2-1: If P3 is used, the new window length proposed by P3 will only apply during the window where aperiodic gap locates. (vivo) Recommended WF Continue discussion

We support option 1. In our view, aperiodic MUSIM gap is an irregular event, and it may puncture one occasion of L1 or L3 measurement resources or MG for NW-A. It would be sufficient to clarify that the measurement period can be longer. This is also aligned with the approach to handle aperiodic CSI-RS impact, e.g. as the following requirements from cl. 9.2.5.1.

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

Proposal 5: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer.

Conclusions

In this paper we provided our views on RRM requirements for NW A with MUSIM gaps.

Proposal 1: Re-use the Rel-17 con-MG approach as baseline to define NW-A measurement requirements with MUSIM gaps, i.e. counting Navailable, Noutside_MG and Ntotal.

Proposal 2: For L3 measurement outside MG, Kp in the requirements is updated

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

Proposal 3: For L3 and positioning measurement with MG, existing requirements can be re-used.

Proposal 4: For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W

Proposal 5: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer.

Reference

- R4-2303248, WF on NR Dual TxRx Multi-SIM, vivo