3GPP TSG-RAN WG4 Meeting # 106bis-e                           R4-2305377

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF impacts and requirements for CA_n5-n28

Agenda Item:	4.30.2.1

Document for:	Approval

Introduction

A new follow-up WI [1] for Enhancement for 700/800/900MHz band combinations was approved in RAN#99 meeting after the completion of SI. Based on the outcomes of TR 38.872 [2], I would like to discuss the RF impacts and requirements for CA_n5-n28.

Discussion on ΔTIB,c and ΔRIB,c values

Referring to TR 38.872 [2], ∆TIB,c and ∆RIB,c values and REFSENS degradation due to cross band isolation interference need to be further down-selected. In study phase, since UE RF architectures with both two and three antennas for CA_n5-n28 are assumed for evaluation, the RF requirements based on UE RF architectures with both two and three antennas for CA_n5-n28 should be considered.

Observation 1: since UE RF architectures with both two and three antennas for CA_n5-n28 are assumed for evaluation, the RF requirements based on UE RF architectures with both two and three antennas for CA_n5-n28 should be considered.

During the study phase, company proposed to reuse the DC_28_n5 ΔTIB,c and ΔRIB,c values with an additional 0.2dB to all to enable the two-antenna n-plexers in the future and including the support of full band n28, which are shown below:

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 |
|  | n28 | 0.7 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.2 |
|  | n28 | 0.2 |

I think the additional 0.2dB for TIB,c and ∆RIB,c values can be acceptable to enable the two-antenna n-plexers in the future.

Proposal 1: additional 0.2dB for TIB,c and ∆RIB,c values can be considered to enable the two-antenna n-plexers in the future for CA_n5-n28.

Discussion on MSD due to cross band isolation from one UL band.

For CA_n5-n28, the MSD due to cross band isolation from n5 UL impacts have been specified in the spec TS 38.101-1 as below.

Table 3 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

During the study phase, the MSD value (18.6dB) is also evaluated by assuming three antenna solution. As the difference is very small and TIB,c and ∆RIB,c values have been agreed considering two-antenna n-plexers in the future. There is no need to further change current spec.

Proposal 2: Keep the MSD due to cross band isolation from n5 UL impacts for CA_n5-n28 unchanged in the spec.

Table 3 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

Discussion on MSD due to cross band isolation from two UL bands.

As shown in figure 1, when both band n5 UL and n28 UL are configured for CA_n5-n28, band n28 DL receiver will be impacted by the leakage interference from band n5 UL and n28 UL.

Figure 1 The relationship for band n28 DL impacted by both band n5 UL and n28 UL

Generally, when 30MHz channel bandwidth is configured for band n28, the REFSENS degradation for band n28 DL has been analysed referring to TR 38.888 [3]. 25 RB (15kHz SCS) was defined for n28 UL configuration. In addition, it’s assumed that 50 dB Transmit port to receiver port duplexer rejection in n28 RX band as a worst case according to contribution [4], and -24 dBm TX noise level in RX band was observed. Finally, -78.5dBm REFSENS was specified for band n28 30MHz considering the interference from band n28 UL.

Observation 2: Currently, -78.5dBm REFSENS was specified for band n28 30MHz considering the interference from band n28 UL with 25 RB (15kHz SCS) UL configuration.

Observation 3: to adopt the following configuration for MSD due to cross band interference from two UL bands for CA_n5-n28 assuming 30MHz channel bandwidth in band n28.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | TBD | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

In contribution [5], some measurement results were proposed for band n28 30MHz case. -32.4dBm Tx noise in n28 Rx band can be observed from band n5 PAout. Since 45dB Tx-Rx isolation between n5 UL band and n28 DL band was assumed, 2.9dB MSD can be observed based on n28 30MHz -78.5dBm REFSENS.

Proposal 3: To consider the following MSD configuration due to cross band interference from two UL bands for CA_n5-n28 as reference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 2.9 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

- Summary

Observation 1: since UE RF architectures with both two and three antennas for CA_n5-n28 are assumed for evaluation, the RF requirements based on UE RF architectures with both two and three antennas for CA_n5-n28 should be considered.

Proposal 1: additional 0.2dB for TIB,c and ∆RIB,c values can be considered to enable the two-antenna n-plexers in the future for CA_n5-n28.

Proposal 2: Keep the MSD due to cross band isolation from n5 UL impacts for CA_n5-n28 unchanged in the spec.

Table 3 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

Observation 2: Currently, -78.5dBm REFSENS was specified for band n28 30MHz considering the interference from band n28 UL with 25 RB (15kHz SCS) UL configuration.

Observation 3: to adopt the following configuration for MSD due to cross band interference from two UL bands for CA_n5-n28 assuming 30MHz channel bandwidth in band n28.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | TBD | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

Proposal 3: To consider the following MSD configuration due to cross band interference from two UL bands for CA_n5-n28 as reference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 2.9 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

- References

[1] RP-230764, New WID on enhancement for 700/800/900MHz band combinations, CATT, China Telecom

[2] TR 38.872, Study on enhancement for 700/800/900MHz band combinations

[3] TR 38.888, Adding wider channel bandwidths in NR band n28

[4] R4-1916062, Experimental Measurements for n28 30MHz CBW REFSENS, Skyworks Solutions, Inc.

[5] R4-2202036, Measurements for CA_n5-n28 MSD, Skyworks Solutions, Inc.