3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305482

# Online, 17th - 26th Apr, 2023

Title: 	Simulation results on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	5.8.4.2

Document for:	Discussion

Background

During RAN4#106 meeting, WF [1] for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception was approved. In this contribution, we provide our initial simulation results on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception

Simulation

In our evaluation, low and high inter-TRP interference refer to two cases corresponding to different UE panel deployment respectively, as per our discussion paper [2].

Single-DCI based multi-TRP

| Case number | MCS | frequency offset[Hz] | timing offset[us] | Rank | Inter-TRP interference | Rx Phase noise model | SNR@70%maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 17 | 3000 | 0.25 | 1+1 | low | N/A | 13.18 |
| 2 | 17 | 3000 | 0.25 | 1+1 | low | Example 2, UE @ 30GHz | 13.33 (+0.15) |
| 3 | 17 | 3000 | 0.25 | 1+1 | low | Example 2, UE @ 47GHz | 13.61 (+0.43) |
| 4 | 17 | 3000 | 0.25 | 1+1 | high | N/A | 16.59 |
| 5 | 17 | 3000 | 0.25 | 1+1 | high | Example 2, UE @ 30GHz | 17.00 (+0.41) |
| 6 | 17 | 3000 | 0.25 | 1+1 | high | Example 2, UE @ 47GHz | 17.61 (+1.02) |
| 7 | 17 | 3000 | 0.25 | 2+2 | low | N/A | 19.96 |
| 8 | 17 | 3000 | 0.25 | 2+2 | low | Example 2, UE @ 30GHz | 20.55 (+0.59) |
| 9 | 17 | 3000 | 0.25 | 2+2 | low | Example 2, UE @ 47GHz | 21.62 (+1.66) |
| 10 | 17 | 3000 | 0.25 | 2+2 | high | N/A | None |
| 11 | 17 | 3000 | 0.25 | 2+2 | high | Example 2, UE @ 30GHz | None |
| 12 | 17 | 3000 | 0.25 | 2+2 | high | Example 2, UE @ 47GHz | None |

Multi-DCI based multi-TRP with non-overlapping scheduling

| Case number | MCS | frequency offset[Hz] | timing offset[us] | Rank | Inter-TRP interference | Rx Phase noise model | SNR@70%maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 17 | 3000 | -0.0625 | 2+2 | None | N/A | 16.76 |
| 2 | 17 | 3000 | -0.0625 | 2+2 | None | Example 2, UE @ 30GHz | 17.24 (+0.48dB) |
| 3 | 17 | 3000 | -0.0625 | 2+2 | None | Example 2, UE @ 47GHz | 18.16 (+1.4dB) |

Multi-DCI based multi-TRP with fully-overlapping scheduling

| Case number | MCS | frequency offset[Hz] | timing offset[us] | Rank | Inter-TRP interference | Rx Phase noise model | SNR@70%maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 17 | 3000 | -0.0625 | 1+1 | low | N/A | 12.89 |
| 2 | 17 | 3000 | -0.0625 | 1+1 | low | Example 2, UE @ 30GHz | 13.05 (+0.16) |
| 3 | 17 | 3000 | -0.0625 | 1+1 | low | Example 2, UE @ 47GHz | 13.33 (+0.44) |
| 4 | 17 | 3000 | -0.0625 | 1+1 | high | N/A | 16.27 |
| 5 | 17 | 3000 | -0.0625 | 1+1 | high | Example 2, UE @ 30GHz | 16.65 (+0.38) |
| 6 | 17 | 3000 | -0.0625 | 1+1 | high | Example 2, UE @ 47GHz | 17.33 (+1.06) |
| 7 | 17 | 3000 | -0.0625 | 2+2 | low | N/A | 20.14 |
| 8 | 17 | 3000 | -0.0625 | 2+2 | low | Example 2, UE @ 30GHz | 20.66 (+0.52) |
| 9 | 17 | 3000 | -0.0625 | 2+2 | low | Example 2, UE @ 47GHz | 21.69 (+1.55) |
| 10 | 17 | 3000 | -0.0625 | 2+2 | high | N/A | None |
| 11 | 17 | 3000 | -0.0625 | 2+2 | high | Example 2, UE @ 30GHz | None |
| 12 | 17 | 3000 | -0.0625 | 2+2 | high | Example 2, UE @ 47GHz | None |

Conclusion

In this contribution, we discuss on UE demodulation requirements for NR FR2 multi-Rx chain DL reception.

Reference

R4-2302941, WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception, RAN4#106, Qualcomm

R4-2305479, Discussion on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception, RAN4#106-bis-e, Huawei, HiSilicon