3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305786

Online, , April 17 – April 26, 2023

Agenda item:	5.3.2

Source:	Keysight Technologies

Title:		Testing Considerations for Multi AoA Rx Testing

Document for:	Approval

# Introduction

This contribution outlines our views on system parameter assumptions for multi-AoA reception testing. Most aspects are very similar to what is presented in the WI contribution [1].

# Discussion

In [2], a new Study Item (SI) was endorsed to define test methodologies for UE RF/demodulation/RRM testing to support reception of DL signals simultaneously from multiple Angles of Arrival (AoAs). This SI directly supports the Work Item (WI) in [3] with testability aspects to eventually enable testing of 4-layer MIMO reception which states [3]:

“The existing Rel-15 NR FR2 minimum UE requirements are defined with an assumption that UE is equipped with a single antenna panel and capable to perform DL reception using a single RX beam/chain reception. Furthermore, the UE performance requirements are limited for DL MIMO rank 1 and 2. In FR2, 4-layer MIMO reception requires beam reception from at least two directions. Although this is supported by the MIMO features since Rel-15, no performance requirements have yet been specified.” and “This work item aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier to achieve improved RF, RRM and UE demodulation performance.”

In the last two meetings, various important decisions were made to progress the multi-RX chain DL reception testing and system aspects [5][6][7][8].

This contribution addresses various open topics that were discussed last meeting without any conclusion [5][6].

# UE Orientation

For FR2 UE RF testing, different UE orientations are permissible for testing, i.e., Alignment Options 1, 2, and 3 as outlined in [10] in Clause N.2.

Table N.2-1: Test conditions and angle definitions for smartphones and tablets for Alignment Option 1 Test condition; DUT orientation; Link angle; Measurement angle; Diagram. Free space DUT Orientation 1 (default); α = 0º; β = 0º; γ = 0º; θLink; ϕLink with polarization reference PolLink = θ or ϕ; θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ. Table N.2-2: Test conditions and angle definitions for smartphones and tablets for Alignment Option 2 Test condition; DUT orientation; Link angle; Measurement angle; Diagram. Free space DUT Orientation 1 (default); α = 0º; β = -90º; γ = 0º; θLink; ϕLink with polarization reference PolLink = θ or ϕ; θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ. Table N.2-3: Test conditions and angle definitions for smartphones and tablets for Alignment Option 3 Test condition; DUT orientation; Link angle; Measurement angle; Diagram. Free space DUT Orientation 1 (default); α = 90º; β = 0º; γ = 0º; θLink; ϕLink with polarization reference PolLink = θ or ϕ; θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ.

For UE RF testing with a single feed probe (generally in the z direction), the selection of any of these orientations has no impact on the test results (peak EIRP/EIS, spherical coverage CDF, TRP, etc) ignoring any fixturing impact on the performance.

Observation 1: FR2 UE RF test cases allow multiple alignment options; the selection of any of the orientations has no impact on the test results

For FR2 multi-AoA UE RRM testing, absolute probe locations are not defined and necessary (“The absolute position of the probes is left up to implementation” per [11]) and given that most RRM testing are based on pass/fail verdicts using multiple iterations (arbitrary selection of angular offsets), any of the above UE orientation is permitted.

For FR2 NR MIMO testing, absolute probe locations are defined [12] and the test verdict is based on parametric testing, i.e., MASC defined in [13], which is closely aligned to FR2 multi-Rx testing. For these test cases, only a single device orientation is permitted [12].

Table A.3-1: Summary of possible DUT positioning options Testing condition; DUT orientation angles; Diagram. XY plane or P0 Orientation 1 (default); α=0; β=0; γ=0.

Observation 2: FR2 NR MIMO testing where absolute probe locations are defined and where the test verdict is based on parametric testing, only a single device orientation is permitted

It is expected that different device orientations/alignment options yield different multi-AoA spherical coverage test results and whether an augmentation of the test system/AoA directions allows those differences to be de-embedded would need detailed investigations from OEMs.

Proposal 1: OEMs to determine whether different device orientations/alignment options yield different multi-AoA spherical coverage test results and what augmentations of the test system/AoA directions are necessary to de-embed those differences

Regardless of the outcome of Proposal 1, to avoid any additional system complexities and potentially test-time increases required for orientation de-embedding, it is proposed to define just a single permitted DUT orientation/alignment option for FR2 multi-Rx testing.

Proposal 2: Define just a single permitted DUT orientation/alignment option for FR2 multi-Rx testing.

# Weighting of AoA Pairs

In the last meeting, it was decided to use the constant-step size grids, e.g., [7]

Issue 1-1-6: Grids for 3D scan - Proposals - Option 1 (QC, Keysight): Use constant step size grid - Option 2: Specify other option if any - Agreements: - Option 1 is agreed

and [8]

- For optimized AoA1 and AoA2 test point/perceived DL direction coverage, utilize constant-step size grids only.

For legacy spherical coverage, the PDF needs to be scaled by either sin(q) or Clenshaw-Curtis weights W(q), e.g., Clause K.1.5.0 [10]

When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(q) or the normalized Clenshaw-Curtis weights W()/W(90°), introduced in Section M.4.2.1, to account for the denser grid point distribution near the poles.

For FR2 multi-Rx testing that is not based on counting pass/fail verdicts, it is therefore proposed to apply similar corrections, i.e., scale the PDF probability contribution for each combination of AoA1&AoA2 by sin(qAoA1)*sin(qAoA2) or the normalized Clenshaw-Curtis weights W(qAoA1)/W(90°)* W(qAoA2)/W(90°)

Proposal 3: For FR2 multi-Rx testing not based on counting pass/fail verdict but based on probability contributions instead, apply a theta-dependent correction, i.e., scale the PDF probability contribution for each combination of AoA1&AoA2 by sin(qAoA1)*sin(qAoA2) or the normalized Clenshaw-Curtis weights W(qAoA1)/W(90°)*W(qAoA2)/W(90°)

# Approximate Test Time Estimates

In RAN4#106, the “go or no-go” test approach was endorsed in [8]

### Requirement Concept for UE RF - Proposal for UE RF requirement concept - Option 3 from WF R4-2220533: Only verify the UE functionality (e.g., go or no-go) under two AoAs with a fixed DL power level. In other words, the UE can achieve EIS performance not worse than YdBm on the test point pair (corresponding to 2 AoAs) and the ratio of qualified test points over the whole sphere is M%. (R4-2300196, R4-2300268, R4-2300709, R4-2301234, R4-2302250). Agreement: - Use Option 3 as baseline.

This selected approach greatly improves test time given the lack of sensitivity searches. The test effort (primarily in terms of overall test time without taking positioning and switching times into account) is summarized in Table 1 for M AoA2 probes. The test approach is closely aligned with the “Proposed Pass/Fail Test Procedure for Multi-DCI & Single-DCI Schemes” without the early/pass fail approach and while taking into account that both ±AoAoffset are tested, endorsed in [8].

Agreement: - Testing both +AoA offset and –AoA offset for each test point shall be the starting point for simulation. The intention is to accommodate the impact from directionality of the AoA1-AoA2 DL orientation vectors. In other words we want to ensure no testing bias is introduced.

The test times considered here are merely approximate (actual test time depends on many factors and can be better than stated below) and estimated as follows:

- tdwell: dwell time (per 38.521-2: “BEAM_SELECT_WAIT_TIME for the UE TX beam selection to complete”) (~3s)

- tTP: average time for a TP or JTP measurement (~1s)

Table 1: Multi-AoA DL spherical coverage effort for multi-DCI & single-DCI schemes based on non-parametric verdicts

| Power Class | Grid Type | Minimum Number of Spherical Coverage Test Points N (single DL) | Effort/Test Time for 1-DL spherical coverage test | Effort/Test Time for multi-AoA DL spherical coverage test |
| --- | --- | --- | --- | --- |
| PC1/PC3 | constant-step size | 312 (for multi AoA) 266 (for single AoA) | 266*2*(tEIS+tdwell) | 312*2*P*M*(tdwell+tTP) |

The test times estimates for a few AoA2 probes M is tabulated in Table 2.

Table 2: Multi-AoA non-parametric spherical coverage test time estimates for various number of polarization combinations, P, and AoA2 probes, M

| Power Class | Test Approach | Minimum Number of Spherical Coverage Test Points N for constant-step size grid | Number of Polarization Combinations P | Number of AoA2 Probes M | Effort/Test Time for multi-AoA DL spherical coverage test [min] |
| --- | --- | --- | --- | --- | --- |
| PC1/PC3 | Based on counting pass/fail verdicts | 312 | 4 | 4 | Min: 42 Max: 666 |
|  |  |  |  | 3 | Min: 42 Max: 499 |
|  |  |  |  | 2 | Min: 42 Max: 333 |
|  |  |  |  | 1 | Min: 42 Max: 166 |
|  |  |  | 2 | 4 | Min: 42 Max: 333 |
|  |  |  |  | 3 | Min: 42 Max: 250 |
|  |  |  |  | 2 | Min: 42 Max: 166 |
|  |  |  |  | 1 | Min: 42 Max: 83 |
| PC1/PC3 | Based on probability contributions | 312 | 4 | 4 | 666 |
|  |  |  |  | 3 | 499 |
|  |  |  |  | 2 | 333 |
|  |  |  |  | 1 | 166 |
|  |  |  | 2 | 4 | 333 |
|  |  |  |  | 3 | 250 |
|  |  |  |  | 2 | 166 |
|  |  |  |  | 1 | 83 |

Given the vast differences in test time in Table 1Table 2 based on the various test parameters (test approach, number of polarization combinations, number of AoA2 probes/directions), it is proposed to take the test time into account when making further decisions in test parameters.

Proposal 4: Take test time into account when making further decisions in test parameters (test approach, number of polarization combinations, number of AoA2 probes/directions)

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: FR2 UE RF test cases allow multiple alignment options; the selection of any of the orientations has no impact on the test results

Observation 2: FR2 NR MIMO testing where absolute probe locations are defined and where the test verdict is based on parametric testing, only a single device orientation is permitted

Proposal 1: OEMs to determine whether different device orientations/alignment options yield different multi-AoA spherical coverage test results and what augmentations of the test system/AoA directions are necessary to de-embed those differences

Proposal 2: Define just a single permitted DUT orientation/alignment option for FR2 multi-Rx testing.

Proposal 3: For FR2 multi-Rx testing not based on counting pass/fail verdict but based on probability contributions instead, apply a theta-dependent correction, i.e., scale the PDF probability contribution for each combination of AoA1&AoA2 by sin(qAoA1)*sin(qAoA2) or the normalized Clenshaw-Curtis weights W(qAoA1)/W(90°)*W(qAoA2)/W(90°)

Proposal 4: Take test time into account when making further decisions in test parameters (test approach, number of polarization combinations, number of AoA2 probes/directions)

# References

- R4-2305788, On UE RF Requirements for Multi-AoA Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #106bis-e, April 2023

- RP-220988, New SI: Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements, Qualcomm Incorporated, 3GPP TSG RAN Meeting #95e, March 2022

- RP-221753, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated, 3GPP TSG RAN Meeting #96, June 2022

- R4-2302522, System Parameter Assumptions for Multi-AoA Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #106, February 2023

- R4-2220264, WF for FR2 OTA test SI, Qualcomm, 3GPP TSG-RAN WG4 Meeting #105, November 2022

- R4-2220533, WF on FR2 multi-Rx, Apple, 3GPP TSG-RAN WG4 Meeting #105, November 2022

- R4-2302923, WF for FR2 OTA test enhancement SI, Qualcomm, 3GPP TSG-RAN WG4 Meeting # 106, March 2023

- R4-2303708, WF on NR FR2 multi-Rx chain DL reception, Samsung, Apple, 3GPP TSG-RAN WG4 Meeting # 106, March 2023

- TR 38.884, Study on enhanced test methods for FR2 NR UEs

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- TR38.810, Study on test methods

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- R4-2218755, Further views on multi-Rx chain DL reception in FR2, Sony, Ericsson, 3GPP TSG-RAN WG4 Meeting # 105, November 2022